The extraction algorithms of the final eight features selected to build the radiomic signatures are shown as follows.

First Order Features:

First-order statistics describe the distribution of voxel intensities within the image region defined by the mask through commonly used and basic metrics.

Let:

X be a set of Np voxels included in the ROI

P(i) be the first order histogram with Ng discrete intensity levels, where Ng is the number of non-zero bins, equally spaced from 0 with a width defined in the binWidth parameter.

p(i) be the normalized first order histogram and equal to P(i)Np

- Total Energy:

total energy=Vvoxel+n=1NpXn+c2

Note: Here, c is optional value, defined by voxelArrayShift, which shifts the intensities to prevent negative values in X. This ensures that voxels with the lowest gray values contribute the least to Energy, instead of voxels with gray level intensity closest to 0.

Total Energy is the value of Energy feature scaled by the volume of the voxel in cubic mm.

- 90th percentile

The 90th percentile of X

Shape Features (3D):

In this group of features we included descriptors of the three-dimensional size and shape of the ROI. These features are independent from the gray level intensity distribution in the ROI and are therefore only calculated on the non-derived image and mask. Unless otherwise specified, features are derived from the approximated shape defined by the triangle mesh. To build this mesh, vertices (points) are first defined as points halfway on an edge between a voxel included in the ROI and one outside the ROI. By connecting these vertices a mesh of connected triangles is obtained, with each triangle defined by 3 adjacent vertices, which shares each side with exactly one other triangle. This mesh is generated using a marching cubes algorithm. In this algorithm, a 2x2 cube is moved through the mask space. For each position, the corners of the cube are then marked ‘segmented’ (1) or ‘not segmented’ (0). Treating the corners as specific bits in a binary number, a unique cube-index is obtained (0-255). This index is then used to determine which triangles are present in the cube, which are defined in a lookup table. These triangles are defined in such a way, that the normal (obtained from the cross product of vectors describing 2 out of 3 edges) are always oriented in the same direction. For PyRadiomics, the calculated normals are always pointing outward. This is necessary to obtain the correct signed volume used in calculation of MeshVolume.

Let:

Nv represent the number of voxels included in the ROI

Nf represent the number of faces (triangles) defining the Mesh.

V the volume of the mesh in mm3,

A the surface area of the mesh in mm2,

- Mesh Volume:

Vi=Oai(Obi × Oci)6 (1)

V=i=1NfVi (2)

The volume of the ROI V is calculated from the triangle mesh of the ROI. For each face i in the mesh, defined by points ai,bi and ci, the (signed) volume Vf of the tetrahedron defined by that face and the origin of the image (O) is calculated. (1)

The sign of the volume is determined by the sign of the normal, which must be consistently defined as either facing outward or inward of the ROI. Then taking the sum of all Vi, the total volume of the ROI is obtained (2)

- Surface Volume Ratio:

surface to volume ratio=AV

Here, a lower value indicates a more compact (sphere-like) shape. This feature is not dimensionless, and is therefore (partly) dependent on the volume of the ROI.

- Voxel Volume

Vvoxel=k=1NvVk

The volume of the ROI Vvoxel is approximated by multiplying the number of voxels in the ROI by the volume of a single voxel Vk. This is a less precise approximation of the volume and is not used in subsequent features. This feature does not make use of the mesh and is not used in calculation of other shape features.

Gray Level Co-occurrence Matrix (GLCM) Features:

A Gray Level Size Zone (GLSZM) quantifies gray level zones in an image. A gray level zone is defined as a the number of connected voxels that share the same gray level intensity. A voxel is considered connected if the distance is 1 according to the infinity norm (26-connected region in a 3D, 8-connected region in 2D). In a gray level size zone matrix P (i, j) the (i, j)th element equals the number of zones with gray level i and size j appear in image. Contrary to GLCM and GLRLM, the GLSZM is rotation independent, with only one matrix calculated for all directions in the ROI.

Let:

Ng be the number of discrete intensity values in the image

Ns be the number of discrete zone sizes in the image

Np be the number of voxels in the image

Nz be the number of zones in the ROI, which is equal to i=1Ngj=1Ns and 1≤Nz≤Np

P(i,j) be the size zone matrix

p(i,j) be the normalized size zone matrix, defined as p(i,j)=P(i,j)Nz

- Inverse Difference Moment (IDM)

IDM=k=0Ng-1Px-y(k)1+k2

IDM (a.k.a Homogeneity 2) is a measure of the local homogeneity of an image. IDM weights are the inverse of the Contrast weights (decreasing exponentially from the diagonal i=j in the GLCM).

- Informational Measure of Correlation (IMC) 1

IMC1=HXY-HXY1maxHX, HY

IMC1 assesses the correlation between the probability distributions of i and j (quantifying the complexity of the texture), using mutual information I(x, y):

However, in this formula, the numerator is defined as HXY - HXY1 (i.e. −I(x,y)), and is therefore ≤0. This reflects how this feature is defined in the original Haralick paper.

In the case where the distributions are independent, there is no mutual information and the result will therefore be 0. In the case of uniform distribution with complete dependence, mutual information will be equal to log2(Ng).

Finally, HXY−HXY1 is divided by the maximum of the 2 marginal entropies, where in the latter case of complete dependence (not necessarily uniform; low complexity) it will result in IMC1=−1, as HX=HY=I (i, j).

- Inverse Variance:

Inverse Variance=k=1Ng-1Px-y (k)k2

Note that k=0 is skipped, as this would result in a division by 0.