Appendix: Running the Scenarios

- Scenario Overview

Six scenarios were modelled for Kenya’s power system to explore the impacts of increased renewable production, EE measures, and increased demand on the BAU baseline resolutions.

A) Business as Usual: The BAU model is used as a baseline energy system framework from Kenya to different the different scenarios. Constraints were included to produce the energy generation shares seen in Kenya from 2015 to 2021. No new investments into EE were considered and demand levels were kept as outlined in the TEMBA model. Policies such as the LCPDP and Vision 2030 are not achieved within this scenario. No new investments are included from 2015 to 2021.

B) Vision 2030: The Vision 2030 scenario follows a future where 100% renewable energy production by 2030 is achieved. The model assumes no new investments in fossil fuels, nuclear power, or energy efficiency (EE). The share of total demand to be met by each source is constrained with upper limits to guarantee the system is realistic and operational under a high proportion of renewable energy sources.

C) Vision 2030 & EE: The Vision 2030 & EE scenario follows the 100% renewable energy mix by 2030 as seen in the previous scenario. Additionally, the model sees a gradual increase in EE investments from 2022 to reach 25% of the total demand from 2030.

D) Increased Demand (LCPDP): This scenario involves a future where a revised 100% electrification target, as highlighted in the LCPDP targets, is achieved by 2025 (given the halt in progress following the COVID-19 pandemic). This is combined with a gradual increase in energy demand from 2022 to align with a 10% annual GDP growth from 2030. The model assumes no investments in EE or nuclear energy after 2021. The share of total demand to be met by each source is constrained with upper limits to ensure viability and energy diversity.

E) LCPDP & Vision 2030: This scenario involves a combination of the Vision 2030 and LCPDP scenarios, combining the targets set by both Vision 2030 and the LCPDP. Energy demand is gradually increased to achieve 100% electrification by 2025 and 10% annual GDP growth by 2030. Upper constraints are applied to all fossil fuels to achieve 100% renewable generation by 2030. Additionally, the model assumes no new investments in fossil fuels, nuclear power, or EE.

F) Clean Energy Transitions (CET): This scenario produces the prospect of achieving both Vision 2030 and LCPDP alongside increased EE measures. In addition to the constraints observed in the previous scenario, a minimum investment into EE technologies is gradually added annually from 2022 to meet 25% of Kenya’s energy demand from 2030.

2. Modelled Scenarios

Business As Usual (BAU):

The BAU scenario will reflect the optimal configuration of the energy system under which the cost is minimized, following the actual power production from the previous years (i.e., 2015 to 2021). The aim of this scenario is to show a baseline cost optimal resolution for the model in which additional scenarios can be compared. All additional scenarios are built upon from the BAU spreadsheet and therefore contain the same constraints.

Table 1 outlines the inputted data for Kenya’s 2015 to 2021 energy production by energy source. Data was acquired from nationally produced data sets by the Energy and Petroleum Regulatory Authority (2022). In order to ‘force’ the model to produce a specific output a small buffer was created using both the upper and lower limits.

| Commodity | Annual Production (PJ) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2016 | 2017 | 2018 | 2019 | 2020 | 2021 |
| Geothermal | 12.7 | 13.3 | 14.3 | 15.4 | 16.5 | 17.1 | 16.9 |
| Hydropower (Large, Medium & Off-grid) | 8.1 | 8.5 | 9.1 | 9.4 | 10.4 | 14.3 | 12.5 |
| Oil-Fired Gas Turbine (SCGT) | 2.7 | 2.8 | 3.1 | 3.3 | 4.2 | 2.5 | 4.1 |
| On-shore wind power | 3.9 | 4.2 | 4.5 | 4.8 | 5.5 | 4.5 | 6.6 |
| Solar PV (Utility) |  |  |  |  |  |  | 0.42 |
| Biomass power plant | 0.95 | 0.99 | 1.1 | 1.2 |  |  | 0.82 |

Table 1: Total Power Production (PJ) by Resource in Kenya from 2015-2021. Adapted from the Energy and Petroleum Regulatory Authority (2022).

As the national data does not breakdown the data for hydropower production into specific power plant sizes, figures could not be split between the large, medium, small and off-grid technologies to accurately reflect reality. As a result, a dummy ‘hydropower’ technology was created which allowed one figure to be inputted for all hydropower technologies, letting the model to allocate production to the four technologies based on the least cost solution in response to the existing installed hydropower plants (allocated in the parameter ‘residual capacity’). To add the dummy hydropower technology the parameters ‘InputActivityRatio’ and ‘OutputActivityRatio’ will be used:

- Go to the sets tab and change ‘TEC088’ in the technologies column to be ‘PWRHYD’ with the label ‘Hydropower Technologies’ and change ‘COM027’ to be ‘RENEW’ in the commodities column with the label ‘Hydro Renewables’.

- Then go to parameters and in column A filter out ‘InputActivityRatio’.

- In column C filter out for the dummy hydropower technology ‘PWRHYD’, and in column F for the dummy hydropower fuel ‘RENEW’.

- Change the default value (0) to 1 from 2015 to 2050.

- Additionally, in column A filter out ‘OutputActivityRatio’.

- In column C filter out for all existing hydropower technologies of: PWRHYD001, PWRHYD002, PWRHYD003 and PWRHYD004, and in column F for the dummy hydropower fuel ‘RENEW’.

- Change the default value (0) to 1 from 2015 to 2050.

- In this way one value can be added for all hydropower technologies, the model will then find a combination of hydropower production to supply the demand. Therefore, hydropower production results from 2015 to 2021 will not accurately reflect the breakdown between types of hydropower plants however this presents an option to overcome lack of available data.

The parameters ‘Total Technology Annual Activity Upper Limit’ and ‘Total Technology Annual Activity Lower Limit’ will be used to constrain the model into producing an energy mix from the years 2015 to 2021 to reflect reality:

- The figures seen in Table 1 were added to the ‘Total Technology Annual Activity Lower Limit’.

- Go to parameters and in column A filter out ‘Total Technology Annual Activity Lower Limit’.

- In column C filter out for all the technologies presented in Table 1: ‘PWRGEO’, ‘PWRHYD’, ‘PWROHC002’, ‘PWRSOL001’, ‘PWRBIO001’ and ‘PWRWND001’.

- Then change the constraints from the default value (0) to correspond with the values in Table 1. For example, for PWRGEO change the default value (0) for 2015 to ’12.7’.

- Repeat this for all six technologies from the years 2015 to 2021.

- Then go to the parameters and in column A filter out ‘Total Technology Annual Activity Upper Limit’, keeping column C filtered with the same technologies as before.

- A buffer is needed to force the model to output the specific values. Therefore, the values in Table 1 will be inputted with an additional 0.1. For example, for PWRGEO change the default value (0) for 2015 to ’12.8’ and ’13.4’ for 2016.

- Repeat this for all six technologies from the years 2015 to 2021.

Vision 2030

The aim of this scenario is to understand what would happen if no fossil fuel technologies were used in the energy system from 2030, with 100% renewable technologies producing the final energy demand.

The parameter ‘TotalTechnologyAnnualActivityUpperLimit’ is used within this scenario to inhibit all fossil fuel production from 2030:

- Go to parameters and in column A filter out ‘TotalTechnologyAnnualActivityUpperLimit’.

- In column C filter out for all fossil fuel technologies: ‘PWRCOA001’, ‘PWROHC001’, ‘PWROHC002’, ‘PWROHC003’, ‘PWRNGS001’ and ‘PWRNGS002’.

- Then change the constraints from the default value (99999) from 2030 onwards for all the fossil fuel technologies.

- Now the model will only be able to produce renewable energies from 2030 onwards.

As Kenya’s energy system operates under a high renewable resource configuration, constraints have been added in order to produce a realistic and operational final energy mix, through the parameter ‘TotalTechnologyAnnualActivityUpperLimit’. Percentage shares constraints used follow those outlined by Allington et al (2021) and are shown in Table 2.

- Go to parameters and in column A filter out ‘SpecifiedAnnualDemand’.

- In column F filter out all demand fuels: ‘COMELC’, ‘RESELC’ and ‘INDELC’.

- In a separate document copy and paste the figures from 2015 to 2050 for all three demand fuels. Create a new row called ‘ELC003’ which contains the sum of the three demand fuels, representing the accumulated demand of all three ‘COMELC’, ‘RESELC’ and ‘INDELC’ annually. For example, in 2022 the accumulated demand of COMELC (6.56), RESELC (14.46) and INDELC (22.65) is 43.67PJ.

- Now that the total accumulated annual demand has been obtained, we can work out what the maximum amount of produced energy per technology to meet the demand is for each modelled year. For example, in any given year, the total amount of geothermal energy produced cannot exceed 40% of the total demand.

- Go to parameters in column A filter out ‘TotalTechnologyAnnualActivityUpperLimit’.

- In column C filter out all renewable energy technologies which appear in the BAU energy mix: ‘PWRBIO001’, ‘PWRGEO’, ‘PWRSOL001’, ‘PWRWND001’, ‘PWRWND002’, ‘PWRSOL001S’ and ‘PWRWND001S’.

- In a separate document figure out what the maximum amount of energy produced is per technology type using the percentage shares in Table 1. For example, for PWRGEO the maximum generation cannot exceed 40% of 43.67PJ, which totals 17.47PJ.

- Repeat this for all seven technologies from the years 2022 to 2050.

| Technologies | Percentage Share (%) |
| --- | --- |
| Biomass Power Plant | 30 |
| Geothermal Power Plant | 40 |
| Solar PV (Utility) | 15 |
| Onshore Wind | 15 |
| Offshore Wind | 10 |
| Utility Scale PV with 2-hour storage | 15 |
| Onshore Wind Power Plant with Storage | 25 |

Table 2: Maximum generation share constraints added to the total technology annual activity upper limit for each technology to produce a realistic and diverse energy mix.

Vision 2030 & Energy Efficiency (EE)

The aim of this scenario is to understand what would happen if investment is made into energy efficiency measures alongside 100% renewable technology production from 2030.

Cost-optimal EE penetration percentages were not explored within this scenario as EE was not the main focus within these scenarios. Instead, a 25% base penetration into deep efficiency technologies was chosen as a starting point to begin to see the long-term impacts on both cost and energy generation. Further studies can assess different EE penetration levels within the scenarios produced.

The ‘TotalTechnologyAnnualActivityLowerLimit’ parameter is used within this scenario to ensure that demand is generated from a specified amount of EE technologies per year:

- Go to parameters and in column A filter out ‘SpecifiedAnnualDemand’.

- In column F filter out all demand fuels: ‘COMELC’, ‘RESELC’ and ‘INDELC’.

- In a separate document copy and paste the figures from 2015 to 2050 for all three demand fuels.

- The scenario introduces a gradual increase in EE technology penetration from a base 2.5% from 2022 to 2026, followed by a yearly increase of 2.5% until 2035 when 25% of demand is met by EE technologies for the remaining model period.

- Now the percentage of demand to be met by EE technologies per year can be figured out. For example, in 2035 the commercial energy demand is 14.33PJ, therefore (14.33 x 0.25) 3.58PJ are needed to be generated by EE technologies to reach 25% of the total.

- Repeat these calculations for all three demand fuels across the modelling period. These figures will then become the ‘Lower Limit’ and should be inputted into the ‘TotalTechnologyAnnualActivityLowerLimit’ parameter.

- Go to parameters and in column A filter out ‘TotalTechnologyAnnualActivityLowerLimit’ and in column C filter out for the three deep efficiency technologies of: ‘COMENEFFDP’, RESENEFFDP’ and ‘INDENEFFDP’.

- Change the constraints default values (0) to the figures obtained in the separate document for all years from 2022 to 2050, across all three EE technologies.

As Kenya’s energy system operates under a high renewable resource configuration, constraints have been added in order to produce a realistic and operational final energy mix, through the parameter ‘TotalTechnologyAnnualActivityUpperLimit’ (Instructions on how to achieve this are found under the Vision 2030 scenario).

Least Cost Power Development Plan (LCPDP)

The aim of this scenario is to understand what would happen if energy demand was to gradually increase to 10% annually by 2030 in line with GDP growth targets, and 100% electrification rates by 2025, as outlined in the LCPDP.

The parameter ‘SpecifiedAnnualDemand’ is used in this scenario to increase the demand from 2022:

- This scenario considers two separate factors which impact energy demand: residential electrification efforts and GDP growth (Table 3). Kenya’s residential electrification rates were 75% in 2021 (Kehbila et al, 2021). An increase in demand from 2022 to 2025 is needed to meet universal residential electrification targets (Schnitzer et al, 2014). Alongside this Kenya has targets to gradually increase GDP to a 10% annual growth rate by 2030 (Republic of Kenya, 2018).

- Go to parameters and in column A filter out ‘SpecifiedAnnualDemand’ and in column F filter the two demand fuels of ‘COMELC’ and ‘INDELC’.

- In a separate document copy and paste the figures for commercial (‘COMELC’) and industrial (‘INDELC) demand from 2015 to 2021.

- The new demand from 2022 will need to be calculated using the GDP percentage increases in Table 3. For example, for commercial demand the 2021 figure will need to be increased by 7.68% to meet the new increased demand (i.e. 6.227582 x 1.0768 = 6.70586). Then using the new 2022 demand figure an 8% increase can be figured out to get the new 2023 demand and so on until 2050.

- Go to parameters and in column A filter out ‘SpecifiedAnnualDemand’ and in column F filter the three demand fuels of ‘‘RESELC’.

- In a separate document copy and paste the figures for residential (‘RESELC’) demand from 2015 to 2021. Both the electrification and GDP percentage increases are needed each year from 2022 to 2025 to calculate residential demand changes.

- Residential demand was increased 6.25% annually from 75% in 2021 to reach 100% universal electrification in 2025 (Table 3). Therefore, the electrification rate increase will need to be worked out in addition to demand associated with GDP increase (as worked out for commercial and industrial demand).

- Take the 2021 figure and increase it by 6.25% (for electrification) and 7.68% (for GDP) (i.e., 14.46467 x 1.0625 x 1.0768 = 15.68674) to give the new demand for 2022. Repeat this with the new 2022 demand until you reach 100% electrification in 2025 and 10% GDP in 2030.

- Replace the ‘SpecifiedAnnualDemand’ figures for ‘COMELC’, ‘RESELC’ and ‘INDELC’ with the figures obtained in the document.

| Residential Electrification | 2022 | 2023 | 2024 | 2025 | 2026 | 2027 | 2028 | 2029 | 2030 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 81.25% (6.25%^) | 87.5% (12.5%^) | 93.75% (18.75%^) | 100% (25%^) | 100% | 100% | 100% | 100% | 100% |
| GDP | 7.68% | 8% | 8.3% | 8.6% | 8.9% | 9.2% | 9.5% | 9.8% | 10% |

Table 3: The two factors contributing to changes in demand from 2022 to 2030.  Residential electrification and GDP percentage increases in line with LCPDP targets. *GDP growth rates are taken as an indicator of demand increase within this study.

As Kenya’s energy system operates under a high renewable resource configuration, constraints have been added in order to produce a realistic and operational final energy mix, through the parameter ‘TotalTechnologyAnnualActivityUpperLimit’ (Instructions on how to achieve this are found under the Vision 2030 scenario). The figures will differ within this scenario to the Vision 2030 as the demand has been changed. Therefore, the steps will need to be repeated to account for new annual demand figures.

Least Cost Power Development Plan (LCPDP) & Vision 2030

The aim of this scenario is to understand what would happen if energy demand was to gradually increase to 10% annually by 2030 in line with GDP growth targets, 100% renewable energy production achieved from 2030 in line with Vision 2030 goals, and a 100% electrification rate was achieved by 2025 as outlined in the LCPDP.

The parameter ‘SpecifiedAnnualDemand’ is used in this scenario to increase the demand from 2022:

- This scenario considers two separate factors which impact energy demand: residential electrification efforts and GDP growth (Table 3). Kenya’s residential electrification rates were 75% in 2021 (Kehbila et al, 2021). An increase in demand from 2022 to 2025 is needed to meet universal residential electrification targets (Schnitzer et al, 2014). Alongside this Kenya has targets to gradually increase GDP to a 10% annual growth rate by 2030 (Republic of Kenya, 2018).

- Go to parameters and in column A filter out ‘SpecifiedAnnualDemand’ and in column F filter the two demand fuels of ‘COMELC’ and ‘INDELC’.

- In a separate document copy and paste the figures for commercial (‘COMELC’) and industrial (‘INDELC) demand from 2015 to 2021.

- The new demand from 2022 will need to be calculated using the GDP percentage increases in Table 3. For example, for commercial demand the 2021 figure will need to be increased by 7.68% to meet the new increased demand (i.e. 6.227582 x 1.0768 = 6.70586). Then using the new 2022 demand figure an 8% increase can be figured out to get the new 2023 demand and so on until 2050.

- Go to parameters and in column A filter out ‘SpecifiedAnnualDemand’ and in column F filter the three demand fuels of ‘‘RESELC’.

- In a separate document copy and paste the figures for residential (‘RESELC’) demand from 2015 to 2021. Both the electrification and GDP percentage increases are needed each year from 2022 to 2025 to calculate residential demand changes.

- Residential demand was increased 6.25% annually from 75% in 2021 to reach 100% universal electrification in 2025 (Table 3). Therefore, the electrification rate increase will need to be worked out in addition to demand associated with GDP increase (as worked out for commercial and industrial demand).

- Take the 2021 figure and increase it by 6.25% (for electrification) and 7.68% (for GDP) (i.e., 14.46467 x 1.0625 x 1.0768 = 15.68674) to give the new demand for 2022. Repeat this with the new 2022 demand until you reach 100% electrification in 2025 and 10% GDP in 2030.

- Replace the ‘SpecifiedAnnualDemand’ figures for ‘COMELC’, ‘RESELC’ and ‘INDELC’ with the figures obtained in the document.

| Residential Electrification | 2022 | 2023 | 2024 | 2025 | 2026 | 2027 | 2028 | 2029 | 2030 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 81.25% (6.25%^) | 87.5% (12.5%^) | 93.75% (18.75%^) | 100% (25%^) | 100% | 100% | 100% | 100% | 100% |
| GDP | 7.68% | 8% | 8.3% | 8.6% | 8.9% | 9.2% | 9.5% | 9.8% | 10% |

Table 3: The two factors contributing to changes in demand from 2022 to 2030.  Residential electrification and GDP percentage increases in line with LCPDP targets. *GDP growth rates are taken as an indicator of demand increase within this study.

The parameter ‘TotalTechnologyAnnualActivityUpperLimit’ is used within this scenario to inhibit all fossil fuel production from 2030:

- Go to parameters and in column A filter out ‘TotalTechnologyAnnualActivityUpperLimit’.

- In column C filter out for all fossil fuel technologies: ‘PWRCOA001’, ‘PWROHC001’, ‘PWROHC002’, ‘PWROHC003’, ‘PWRNGS001’ and ‘PWRNGS002’.

- Then change the constraints from the default value (99999) from 2030 onwards for all the fossil fuel technologies.

- Now the model will only be able to produce renewable energies from 2030 onwards.

As Kenya’s energy system operates under a high renewable resource configuration, constraints have been added in order to produce a realistic and operational final energy mix, through the parameter ‘TotalTechnologyAnnualActivityUpperLimit’ (Instructions on how to achieve this are found under the Vision 2030 scenario). The figures will differ within this scenario to the Vision 2030 as the demand has been changed. Therefore, the steps will need to be repeated to account for new annual demand figures.

Clean Energy Transition (CET)

The aim of this scenario is to understand what would happen if demand were to increase (through 100% electrification and GDP increase), energy as produced from 100% renewable energy sources by 2030, and a 25% EE penetration achieved by 2030.

The parameter ‘SpecifiedAnnualDemand’ is used in this scenario to increase the demand from 2022:

- This scenario considers two separate factors which will impact on energy demand: residential electrification efforts and GDP growth (Table 3). Kenya’s residential electrification rates were 75% in 2021 (Kehbila et al, 2021). An increase in demand from 2022 to 2025 is needed to meet universal residential electrification targets (Schnitzer et al, 2014). Alongside this Kenya has targets to gradually increase GDP to a 10% annual growth rate by 2030 (Republic of Kenya, 2018).

- Go to parameters and in column A filter out ‘SpecifiedAnnualDemand’ and in column F filter the three demand fuels of ‘COMELC’ and ‘INDELC’.

- In a separate document copy and paste the figures for commercial (‘COMELC’) and industrial (‘INDELC) demand from 2015 to 2021.

- The new demand from 2022 will need to be calculated using the percentage increases in Table 3. For example, for commercial demand the 2021 figure will need to be increased by 7.68% to meet the new increased demand (i.e. 6.227582 x 1.0768 = 6.70586). Then using the new 2022 demand figure an 8% increase can be figured out to get the new 2023 demand and so on until 2050, using the percentages in Table 3.

- Go to parameters and in column A filter out ‘SpecifiedAnnualDemand’ and in column F filter the three demand fuels of ‘‘RESELC’.

- In a separate document copy and paste the figures for residential (‘RESELC’) demand from 2015 to 2021. There are two steps each year from 2022 to 2025 to calculate residential demand changes.

- Residential demand was increased 6.25% annually from 75% in 2021 to reach 100% universal electrification in 2025 (Table 3). Therefore, the electrification rate increase will need to be worked out in addition to demand associated with GDP increase (as worked out for commercial and industrial demand).

- Take the 2021 figure and increase it by 6.25% (for electrification) and 7.68% (for GDP) (i.e., 14.46467 x 1.0625 x 1.0768 = 15.68674) to give the new demand for 2022. Repeat this with the new 2022 demand until you reach 100% electrification in 2025 and 10% GDP in 2030.

- Replace the ‘SpecifiedAnnualDemand’ figures for ‘COMELC’, ‘RESELC’ and ‘INDELC’ with the figures obtained in the document.

| Residential Electrification | 2022 | 2023 | 2024 | 2025 | 2026 | 2027 | 2028 | 2029 | 2030 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 81.25% (6.25%^) | 87.5% (12.5%^) | 93.75% (18.75%^) | 100% (25%^) | 100% | 100% | 100% | 100% | 100% |
| GDP | 7.68% | 8% | 8.3% | 8.6% | 8.9% | 9.2% | 9.5% | 9.8% | 10% |

Table 3: The two factors contributing to changes in demand from 2022 to 2030.  Residential electrification and GDP percentage increases in line with LCPDP targets. *GDP growth rates are taken as an indicator of demand increase within this study.

The parameter ‘TotalTechnologyAnnualActivityUpperLimit’ is used within this scenario to inhibit all fossil fuel production from 2030:

- Go to parameters and in column A filter out ‘TotalTechnologyAnnualActivityUpperLimit’.

- In column C filter out for all fossil fuel technologies: ‘PWRCOA001’, ‘PWROHC001’, ‘PWROHC002’, ‘PWROHC003’, ‘PWRNGS001’ and ‘PWRNGS002’.

- Then change the constraints from the default value (99999) from 2030 onwards for all the fossil fuel technologies.

- Now the model will only be able to produce renewable energies from 2030 onwards.

The ‘TotalTechnologyAnnualActivityLowerLimit’ parameter is used within this scenario to ensure that demand is generated from a specified amount of EE technologies per year:

- Go to parameters and in column A filter out ‘SpecifiedAnnualDemand’.

- In column F filter out all demand fuels: ‘COMELC’, ‘RESELC’ and ‘INDELC’.

- In a separate document copy and paste the figures from 2015 to 2050 for all three demand fuels.

- The scenario introduces a gradual increase in EE technology penetration from a base 2.5% from 2022 to 2026, followed by a yearly increase of 2.5% until 2035 when 25% of demand is met by EE technologies for the remaining model period.

- Now the percentage of demand to be met by EE technologies per year can be figured out. For example, in 2035 the commercial energy demand is 21.57PJ, therefore (21.57 x 0.25) 5.39PJ are needed to be generated by EE technologies to reach 25% of the total.

- Repeat these calculations for all three demand fuels across the modelling period. These figures will then become the ‘Lower Limit’ and should be inputted into the ‘TotalTechnologyAnnualActivityLowerLimit’ parameter.

- Go to parameters and in column A filter out ‘TotalTechnologyAnnualActivityLowerLimit’ and in column C filter out for the three deep efficiency technologies of: ‘COMENEFFDP’, RESENEFFDP’ and ‘INDENEFFDP’.

- Change the constraints default values (0) to the figures obtained in the separate document for all years from 2022 to 2050, across all three EE technologies.

As Kenya’s energy system operates under a high renewable resource configuration, constraints have been added in order to produce a realistic and operational final energy mix, through the parameter ‘TotalTechnologyAnnualActivityUpperLimit’ (Instructions on how to achieve this are found under the Vision 2030 scenario). The figures will differ within this scenario to the Vision 2030 as the demand has been changed. Therefore, the steps will need to be repeated to account for new annual demand figures.

- Run and check the results.

- Instructions on how to run and visualise the scenarios on OSeMOSYS can be found in the ‘Hands on 3’ document from the ‘Energy and Flexibility Modelling: OSeMOSYS & FlexTool (Windows)’ Open University course by Climate Compatible Growth.

- Additionally, CCG have produced video instructions which are accessible via YouTube.

- An example of the generated BAU total annual electricity production graph is shown below.

References

Allington L, Cannone C, Pappis I, Zachau Walker M, Ahsan A, Hirmer S, Cervantes Barron K, Usher W, Pye S, Howells M, Taliotis C, Sundin C, Sridharan V, Ramos E, Brinkerink M, Deane P, Gritsevskyi A, Moura G, Rouget A, Rogner H (2021). CCG Starter Data Kit: Kenya (v1.1.0) [Data set]. Zenodo. https://doi.org/10.5281/zenodo.5139528

Energy and Petroleum Regulatory Authority (2022) The Energy and Petroleum Statistics Report 2021: Key statistics and highlights. P1-37. Energy and petroleum regulatory authority: Upperhill, Kenya.

Kehbila AG, Masumbuko RK, Ogeya M, Osano P (2021) Assessing transition pathways to low-carbon electricity generation in Kenya: A hybrid approach using backcasting, socio-technical scenarios and energy system modelling. Renewable and Sustainable Energy Transition, 1, 1-16.

Republic of Kenya (2018b) Updated Least Cost Power Development Plan: 2017-2037. June 2018. 1-338. Government of the Republic of Kenya: Nairobi.

Schnitzer D, Lounsbury DS, Carvalo JP, Ranjit D, Apt J, Kammen DM (2014) Microgrids for Rural Electrification: A Critical Review of Best Practices Based on Seven Case Studies Microgrids for Rural Electrification. United Nations Foundation: Washington DC. 1-122. Available at: https://www.ourenergypolicy.org/wp-content/uploads/2014/02/MicrogridsReportFINAL_low.pdf (Accessed: 5th June 2022)