Supplementary data

A Box-Behnken design-based chemometric approach to optimize the photo-sono-Fenton degradation of hydroxychloroquine in water using the Fe(0)/S2O82-/UV system

H. Bendjeffal a, H. Mamine a, M. Boukachabia b, A. Aloui a, T. Metidji C, A. Djeblid, Y. Bouhedja C.

Table.S1. Main physical and chemical properties of hydroxychloroquine.

| Chemical Structure |  |
| --- | --- |
| Chemical formula | C18H26Cl N3 O * H2 SO4 |
| Molecular Weight | 433.95 g mol-1 |
| Elimination half-life | Between 30 and 60 days |
| Solubility in Water at 25°C | 26.10 mg L-1 |
| pKa1 /pKa2 | 7.28 / 9.76 |
| Main absorption wavelengths | 220, 257, 325, and (λmax) 334 nm |

Table S2. Coded Coefficients for the degradation of hydroxychloroquine using the Fe(0)/ S2O82-/UV system.

| Term | Coef | SE Coef | T-Value | P-Value | VIF |
| --- | --- | --- | --- | --- | --- |
| Constant | 74.74 | 3.48 | 21.49 | 0.000 |  |
| [Fe(0)] | 8.37 | 1.74 | 4.81 | 0.000 | 1.00 |
| pH | -6.65 | 1.74 | -3.83 | 0.002 | 1.00 |
| [S2O82-] | 8.26 | 1.74 | 4.75 | 0.000 | 1.00 |
| [HCQ] | -22.03 | 1.74 | -12.67 | 0.000 | 1.00 |
| [Fe(0)]*[Fe(0)] | -2.75 | 2.61 | -1.05 | 0.313 | 1.25 |
| pH*pH | -8.55 | 2.61 | -3.28 | 0.007 | 1.25 |
| [S2O82-]*[S2O82-] | -1.88 | 2.61 | -0.72 | 0.484 | 1.25 |
| [HCQ]*[HCQ] | -10.40 | 2.61 | -3.99 | 0.002 | 1.25 |
| [Fe(0)]*pH | -4.16 | 3.01 | -1.38 | 0.192 | 1.00 |
| [Fe(0)]* [S2O82-] | 10.18 | 3.01 | 3.38 | 0.005 | 1.00 |
| [Fe(0)]*[HCQ] | 3.70 | 3.01 | 1.23 | 0.243 | 1.00 |
| pH*[S2O82-] | -4.96 | 3.01 | -1.65 | 0.125 | 1.00 |
| pH*[HCQ] | -11.26 | 3.01 | -3.74 | 0.003 | 1.00 |
| [S2O82-]*[HCQ] | 15.62 | 3.01 | 5.18 | 0.000 | 1.00 |