Environmental Science and Pollution Research

ETDA as a legacy soil chelatant; a comparative study to a more environmentally-sensitive alternative for metal removal by Pistia stratiotes L.

Manhattan Lebrun1, Jiřina Száková2, Ondřej Drábek3, Václav Tejnecký3, Rupert Lloyd Hough4, Luke Beesley1,4, Hailong Wang5,6 and Lukáš Trakal*

1Department of Environmental Geosciences, Faculty of Environmental Sciences, Czech University of Life Sciences Prague, 165 00 Prague 6, Czech Republic

2Department of Agroenvironmental Chemistry and Plant Nutrition, Faculty of Agrobiology, Food, and Natural Resources, Czech University of Life Sciences Prague, 165 00 Prague 6, Czech Republic

3Department of Soil Science and Soil Protection, Faculty of Agrobiology, Food and Natural Resources, Czech University of Life Sciences Prague, 165 00 Prague 6, Czech Republic

4The James Hutton Institute, Craigiebuckler, Aberdeen, AB15 8QH, UK

5Biochar Engineering Technology Research Center of Guangdong Province, School of Environmental and Chemical Engineering, Foshan University, Foshan, Guangdong 528000, China

6Key Laboratory of Soil Contamination Bioremediation of Zhejiang Province, Zhejiang A&F University, Hangzhou, Zhejiang 311300, China

Table S1. Composition of the different growing solutions (mg.L-1) with time: Letters indicate significant differences between treatments at each sampling time, for the nutrients (p < 0.05) (n = 3). DW = deionized water, TA = tartaric acid, EDTA = Na-EDTA.

| Element | Treatment | 0 | 1 | 2 | 4 | 8 | 24 | 72 | 168 | 360 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DOC | DW | 5.1 ± 0.4 b | 4.8 ± 0.2 b | 4.2 ± 0.2 b | 4.5 ± 0.1 b | 4.5 ± 0.1 b | 4.3 ± 0.1 b | 5.1 ± 0.2 b | 5.6 ± 0.2 b | 35.7 ± 0.6 b |
|  | TA | 5.1 ± 0.1 b | 5.1 ± 0.1 b | 4.7 ± 0.2 b | 4.9 ± 0.1 b | 4.8 ± 0.1 b | 4.5 ± 0.1 b | 5.1 ± 0.1 b | 5.8 ± 0.0 b | 24.9 ± 0.4 c |
|  | EDTA | 24.7 ± 0.6 a | 24.4 ± 0.4 a | 23.6 ± 0.5 a | 24.1 ± 0.6 a | 24.3 ± 0.7 a | 23.9 ± 0.3 a | 22.7 ± 0.3 a | 26.7 ± 0.1 a | 56.1 ± 1.5 a |
| Calcium | DW | 12.6 ± 0.1 c | 11.9 ± 0.1 c | 11.9 ± 0.1 c | 12.3 ± 0.2 c | 12.7 ± 0.1 c | 12.0 ± 0.1 c | 16.4 ± 0.2 b | 13.2 ± 0.2 c | 4.53 ± 0.07 c |
|  | TA | 22.4 ± 0.3 a | 21.4 ± 0.3 a | 21.0 ± 0.1 a | 21.8 ± 0.2 a | 24.3 ± 0.2 a | 22.2 ± 0.3 a | 23.5 ± 0.2 a | 22.5 ± 0.2 a | 10.1 ± 0.1 b |
|  | EDTA | 15.8 ± 0.1 b | 14.9 ± 0.1 b | 15.0 ± 0.1 b | 15.7 ± 0.2 b | 16.0 ± 0.2 b | 17.0 ± 0.1 b | 13.2 ± 0.02 c | 13.8 ± 0.2 b | 12.5 ± 0.1 a |
| Magnesium | DW | 0.84 ± 0.01 b | 0.82 ± 0.00 c | 0.82 ± 0.01 b | 0.85 ± 0.01 b | 0.90 ± 0.01 c | 0.87 ± 0.01 c | 1.26 ± 0.01 b | 1.02 ± 0.01 c | 0.57 ± 0.01 c |
|  | TA | 1.33 ± 0.02 a | 1.32 ± 0.01 a | 1.32 ± 0.01 a | 1.37 ± 0.02 a | 1.43 ± 0.02 a | 1.48 ± 0.02 a | 1.60 ± 0.16 a | 1.85 ± 0.02 a | 0.95 ± 0.01 b |
|  | EDTA | 1.32 ± 0.02 a | 1.29 ± 0.01 b | 1.30 ± 0.01 a | 1.34 ± 0.01 a | 1.38 ± 0.02 b | 1.43 ± 0.01 b | 1.39 ± 0.00 ab | 1.35 ± 0.03 b | 1.56 ± 0.03 a |
| Potassium | DW | 2.14 ± 0.04 c | 2.22 ± 0.09 b | 2.20 ± 0.11 b | 1.94 ± 0.12 b | 1.86 ± 0.18 b | 1.46 ± 0.03 b | 0.71 ± 0.03 a | 0.39 ± 0.00 c | 0.16 ± 0.00 c |
|  | TA | 2.41 ± 0.04 b | 2.26 ± 0.06 b | 2.39 ± 0.12 ab | 2.21 ± 0.12 ab | 2.16 ± 0.05a | 1.66 ± 0.03 a | 0.68 ± 0.02 a | 0.50 ± 0.01 a | 0.36 ± 0.00 a |
|  | EDTA | 2.56 ± 0.07 a | 2.64 ± 0.05 a | 2.62 ± 0.13 a | 2.36 ± 0.19 a | 2.22 ± 0.03 a | 1.72 ± 0.03 a | 0.32 ± 0.02 b | 0.42 ± 0.00 b | 0.23 ± 0.00 b |
| Sodium | DW | 1.56 ± 0.02 b | 1.67 ± 0.02 b | 1.65 ± 0.01 b | 1.56 ± 0.03 b | 1.55 ± 0.05 b | 1.47 ± 0.01 b | 1.67 ± 0.01 b | 0.50 ± 0.01 c | 0.18 ± 0.00 c |
|  | TA | 1.42 ± 0.02 b | 1.48 ± 0.01 c | 1.48 ± 0.01 b | 1.41 ± 0.04 b | 1.45 ± 0.03 b | 1.40 ± 0.02 b | 1.01 ± 0.00 c | 0.70 ± 0.01 b | 0.65 ± 0.01 b |
|  | EDTA | 8.76 ± 0.11 a | 8.71 ± 0.09 a | 8.71 ± 0.22 a | 8.61 ± 0.23 a | 8.72 ± 0.08 a | 8.66 ± 0.25 a | 6.52 ± 0.04 a | 5.12 ± 0.06 a | 4.58 ± 0.02 a |
| Lactate | DW | 0.95 | 0.66 | 1.20 | 0.79 | 0.52 | 1.15 | 1.52 | 0.75 | 0.65 |
|  | TA | 0.60 | 0.80 | 1.29 | 0.79 | 1.25 | 0.93 | 0.82 | 2.43 | 1.69 |
|  | EDTA | 0.32 | 1.30 | 0.66 | 0.25 | 1.00 | 1.07 | 1.11 | 1.45 | 1.43 |
| Acetate | DW | 0.16 | 0.29 | 0.30 | 0.14 | 0.25 | 0.22 | 0.50 | 0.13 | 0.39 |
|  | TA | 0.17 | 0.14 | 0.45 | 0.30 | 0.22 | 0.25 | 0.22 | 0.21 | 0.31 |
|  | EDTA | 0.11 | 0.24 | 0.34 | 0.24 | 0.12 | 0.22 | 0.25 | 0.55 | 0.40 |
| Formate | DW | 0.07 | 0.11 | 0.19 | 0.04 | 0.31 | 0.08 | 0.30 | 0.06 | 0.07 |
|  | TA | 0.08 | 0.07 | 0.40 | 0.04 | 0.08 | 0.11 | 0.12 | 0.37 | 0.15 |
|  | EDTA | 0.05 | 0.11 | 0.12 | 0.08 | 0.07 | 0.08 | 0.12 | 0.15 | 0.11 |
| Malate | DW | 0.73 | 0.54 | 0.18 | 0.21 | 0.45 | 0.34 | 0.26 | 0.25 | 0.20 |
|  | TA | 0.67 | 0.20 | 0.19 | 0.20 | 0.29 | 0.38 | 0.24 | 0.35 | 0.22 |
|  | EDTA | 0.62 | 0.22 | 0.19 | 0.19 | 0.24 | 0.53 | 0.09 | 0.23 | 0.00 |
| Oxalate | DW | 0.14 | 0.23 | 0.17 | 0.11 | 0.12 | 0.16 | 0.38 | 2.65 | 0.44 |
|  | TA | 0.22 | 0.18 | 0.19 | 0.24 | 0.13 | 0.14 | 0.33 | 1.54 | 0.46 |
|  | EDTA | 0.16 | 0.18 | 0.17 | 0.11 | 0.13 | 0.17 | 0.31 | 0.98 | 0.48 |
| Fluoride | DW | 0.21 | 0.16 | 0.23 | 0.35 | 0.18 | 0.22 | 0.27 | 0.19 | 0.30 |
|  | TA | 0.12 | 0.16 | 0.25 | 0.15 | 0.24 | 0.18 | 0.18 | 0.47 | 0.32 |
|  | EDTA | 0.15 | 0.25 | 0.16 | 0.16 | 0.20 | 0.21 | 0.19 | 0.28 | 0.31 |
| Chloride | DW | 16.88 | 16.87 | 14.40 | 17.13 | 17.39 | 14.23 | 3.83 | 0.20 | 0.47 |
|  | TA | 31.17 | 31.34 | 31.70 | 31.36 | 30.85 | 24.85 | 20.63 | 18.12 | 0.93 |
|  | EDTA | 22.50 | 21.59 | 21.51 | 20.29 | 19.32 | 17.10 | 7.64 | 0.87 | 0.61 |
| Sulfate | DW | 2.35 | 2.38 | 1.86 | 2.52 | 5.01 | 1.80 | 3.64 | 2.76 | 4.11 |
|  | TA | 1.81 | 1.94 | 1.94 | 1.80 | 1.77 | 1.43 | 2.22 | 6.06 | 2.19 |
|  | EDTA | 5.40 | 6.67 | 5.61 | 5.23 | 5.40 | 5.05 | 6.08 | 5.30 | 5.91 |
| Phosphate | DW | 0.20 | / | / | / | / | / | 0.14 | 0.13 | / |
|  | TA | 0.15 | 0.09 | / | / | 0.23 | / | / | 0.16 | / |
|  | EDTA | 0.06 | / | 0.31 | / | / | / | / | 0.34 | / |
| Nitrate | DW | 0.785 | 0.713 | 0.676 | 0.658 | 0.672 | 0.476 | / | / | / |
|  | TA | 0.004 | 0.020 | 0.006 | / | 0.020 | 0.003 | / | / | / |
|  | EDTA | 1.249 | 1.162 | 1.162 | 1.157 | 1.104 | 0.761 | / | / | / |
| Ammonium | DW | 0.207 | 0.053 | 0.104 | 0.047 | 0.079 | 0.083 | 0.055 | 0.050 | 0.004 |
|  | TA | 0.018 | 0.008 | 0.019 | 0.031 | 0.010 | 0.068 | 0.062 | 0.055 | 0.011 |
|  | EDTA | 0.304 | 0.136 | 0.145 | 0.142 | 0.101 | 0.076 | 0.046 | 0.052 | 0.003 |

*Corresponding author: Dr. Lukáš Trakal; Department of Environmental Geosciences, Faculty of Environmental Sciences, Czech University of Life Sciences Prague, Kamýcká 129, Praha 6 Suchdol, 16500, Czech Republic. Email: trakal@fzp.czu.cz; ORCiD: 0000-0002-1216-6206