Hydro-Geochemical Conditions under Projected Climate Change Scenarios of Marshyangdi River, Nepal

Reeta Singh1*; Sadhana Pradhananga Kayastha 2; Suman Man Shrestha2; Ramesh Prasad Sapkota2

- Patan Multiple Campus, Patan Dhoka, Lalitpur, Nepal

- Central Department of Environmental Science, Kirtipur, Nepal

*Corresponding author: reetaisngh80@gmail.com; reeta.singh.pmc.tu.edu.np

APPENDIX

Table A1 Standard values of water quality parameters for aquatic life

| Parameters | Permissible limit | Reference |
| --- | --- | --- |
| DO | 5 | DoI/GoN (2008); CCME, (2001) |
| pH | 6.5-9 | CCME, (2001) |
| EC | 1000 | BBWMSIP (1994) in Regmi et al., (2017); BIS, (2012) |
| NH3 -N | 1.2 | DoI/GoN (2008) |
| NO3- | 45 | CCME, (2001) |
| PO43- | 0.1 | BBWMSIP (1994) in Regmi et al. (2017); USEPA (1986) |
| Cl- | 500 | BBWMSIP (1994) in Regmi et al. (2017) |
| COD | <20 | Chapman, (1996), DoI/GoN (2008) |
| SO42- | 200 | BIS, (2012) |
| HCO3- | 200 | BIS, (2012) |
| BOD | <15 | DoI/GoN (2008) |
| TH | >180 | DoI/GoN (2008) |
| TP | <0.05 | CCME, (2001) |
| TDS | 1000 | BBWMSIP (1994) in Regmi et al. (2017) |
| Ca 2+ | 100 | BIS, (2012) |
| Na + | 200 | BIS, (2012) |
| K + | <10 | BIS, (2012) |

Table A2a Characterization of seasonal variations in physicochemical conditions in mid-future (RCP4.5)

| Pre-monsoon |  |  |  |  | Post-monsoon |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters | Minimum | Maximum | Mean | SD | Minimum | Maximum | Mean | SD |
| Physical |  |  |  |  |  |  |  |  |
| pH | 6.1 | 9.9 | 7.6 | 7.3 | 6.1 | 9.9 | 7.6 | 1.1 |
| TDS | 43.3 | 250.1 | 119.2 | 115.2 | 43.3 | 250.1 | 119.2 | 45.5 |
| EC | 82.7 | 475.9 | 225.5 | 218.7 | 82.7 | 475.9 | 225.5 | 89.9 |
| Chemical |  |  |  |  |  |  |  |  |
| DO | 3.7 | 7.0 | 5.4 | 5.7 | 3.7 | 7.0 | 5.4 | 1.0 |
| Cl- | 7.9 | 28.3 | 17.5 | 16.9 | 7.9 | 28.3 | 17.5 | 5.6 |
| COD | 3.2 | 27.3 | 11.3 | 9.7 | 3.2 | 27.3 | 11.3 | 6.4 |
| BOD | 1.9 | 16.4 | 6.8 | 5.8 | 1.9 | 16.4 | 6.8 | 3.8 |
| Ca2+ | 29.5 | 74.3 | 56.2 | 57.0 | 29.5 | 74.3 | 56.2 | 11.3 |
| Na+ | 12.0 | 32.9 | 17.6 | 16.8 | 12.0 | 32.9 | 17.6 | 4.3 |
| K+ | 0.6 | 10.3 | 4.0 | 2.7 | 0.6 | 10.3 | 4.0 | 3.1 |
| Mg2+ | 2.2 | 15.2 | 8.8 | 4.1 | 7.6 | 24.5 | 14.6 | 4.5 |
| HCO3- | 103 | 214 | 159 | 156 | 103 | 214 | 159 | 35.2 |
| TH | 38 | 297 | 112 | 99 | 38 | 297 | 112 | 61.0 |
| SO42- | 2.83 | 4.71 | 3.48 | 3.29 | 2.83 | 4.71 | 3.48 | 0.57 |
| NH3-N | 0.02 | 0.32 | 0.07 | 0.05 | 0.02 | 0.32 | 0.07 | 0.07 |
| Nutrients |  |  |  |  |  |  |  |  |
| NO3- | 0.41 | 0.93 | 0.71 | 0.72 | 0.41 | 0.93 | 0.71 | 0.12 |
| TP | 0.03 | 1.23 | 0.26 | 0.12 | 0.03 | 1.23 | 0.26 | 0.31 |
| PO43- | 0.01 | 0.14 | 0.05 | 0.03 | 0.01 | 0.14 | 0.05 | 0.04 |

Note: All values are expressed in mg/L except Temp (°C), EC (µS/cm), and Turbidity (NTU); SD is the Standard deviation

Table A2b Characterization of seasonal variations in physicochemical conditions in mid-future (RCP8.5)

| Pre-monsoon |  |  |  |  | Post-monsoon |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters | Minimum | Maximum | Mean | SD | Minimum | Maximum | Mean | SD |
| Physical |  |  |  |  |  |  |  |  |
| pH | 6.3 | 10.7 | 7.9 | 1.1 | 6.2 | 9.9 | 7.7 | 1.1 |
| TDS | 40 | 171 | 107 | 32.7 | 43.3 | 250.2 | 120.8 | 45.7 |
| EC | 91 | 363 | 228 | 62.5 | 82.7 | 475.9 | 228.4 | 90.2 |
| Chemical |  |  |  |  |  |  |  |  |
| DO | 3.6 | 7.0 | 5.8 | 0.9 | 3.8 | 7.0 | 5.5 | 1.0 |
| Cl- | 7.2 | 29.3 | 15.1 | 5.8 | 7.9 | 28.7 | 17.7 | 5.7 |
| COD | 1.6 | 20.3 | 5.6 | 4.0 | 3.2 | 27.3 | 11.5 | 6.4 |
| BOD | 1.0 | 12.2 | 3.4 | 2.4 | 1.9 | 16.4 | 6.9 | 3.8 |
| Ca2+ | 16.0 | 56.5 | 23.9 | 9.4 | 29.5 | 74.3 | 56.9 | 11.4 |
| Na+ | 1.0 | 28.2 | 9.3 | 7.7 | 12.1 | 32.9 | 17.8 | 4.3 |
| K+ | 0.6 | 25.8 | 6.8 | 7.8 | 0.6 | 10.3 | 4.0 | 3.1 |
| Mg2+ | 2.2 | 15.5 | 8.9 | 4.1 | 7.6 | 24.9 | 14.8 | 4.6 |
| HCO3- | 45 | 208 | 96 | 34 | 113.1 | 369.1 | 238.7 | 65.4 |
| TH | 61 | 372 | 164 | 90 | 38.4 | 296.8 | 113.4 | 60.9 |
| NH3-N | 0.02 | 0.16 | 0.05 | 0.03 | 0.02 | 0.32 | 0.07 | 0.07 |
| SO42- | 2.70 | 4.54 | 3.61 | 0.47 | 2.85 | 4.71 | 3.52 | 0.55 |
| Nutrients |  |  |  |  |  |  |  |  |
| NO3- | 0.03 | 1.35 | 0.83 | 0.31 | 0.41 | 0.95 | 0.72 | 0.12 |
| TP | 0.04 | 0.48 | 0.19 | 0.13 | 0.03 | 1.25 | 0.26 | 0.32 |
| PO43- | 0.01 | 0.09 | 0.03 | 0.02 | 0.01 | 0.14 | 0.05 | 0.04 |

Note: All values are expressed in mg/L except Temp (°C), EC (µS/cm), and Turbidity (NTU); SD is the Standard deviation