Table 1 Experimental conditions for sequential methods for fractionation analysis of mercury.

| - BCR-sequential method |  |  |  |
| --- | --- | --- | --- |
| Step | Fractions | Extractants | Extraction conditions |
| 01 | Acid Soluble Fraction | 25 mL acetic acid (0.11 mol L-1) | Mixing by mechanical shaker for 16 hr then centrifugation, Save the supernatant solution and residue. |
| 02 | Reducible Fraction | 25 mL hydroxylamine hydrochloride (0.5 mol L-1), pH = 1.5 | Mixing by mechanical shaker for 16 hr then centrifugations, save extracted solution and residue for next step |
| 03 | Oxidizable Fraction | 8 mL hydrogen peroxide (30%), 30 mL ammonium acetate (1.0 mol L-1), pH = 2.0 | Residue of step 2 was digested by hot plate and extractants twice. Shaking for 16 hr then centrifugation and save the supernatant solution. |
| 04 | Residual Fraction | 10.0 mL aqua regia, 25 mL 0.2 mol L-1 nitric acid | Residue of step 3 was digested by aqua regia at 85 ºC. After filtration save supernatant solution. |
| - Single-step sequential method |  |  |  |
| Step | Fractions | Extractants | Extraction conditions |
| 01 | Acid soluble fraction | 25 mL acetic acid (0.11 mol L-1) | Mixing by mechanical shaker for 10 hr then centrifugation, Save the supernatant solution, discard the residue. |
| 02 | Reducible Fraction | 25 mL hydroxylamine hydrochloride (0.5 mol L-1), pH = 1.5 | Mixing by mechanical shaker for 10 h, then centrifugation, save the supernatant solution and residue. |
| 03 | Oxidizable fraction | 8.0 mL hydrogen peroxide (30%), 30 mL ammonium acetate (1.0 mol L-1), pH = 2.0 | Residue of step 2 was digested by hot plate twice and extractant. Shaking for 10 hr then centrifugation, save the supernatant solution, discard the residue |
| - Single-step sequential based ultrasonic-assisted method |  |  |  |
| Step | Fractions | Extractants | Extraction conditions |
| 01 | Acid soluble fraction | 25 mL acetic acid (0.11 mol L-1) | Sonication time for 15-30 min then centrifugation, save the supernatant solution for analysis. |
| 02 | Reducible fraction | 25 mL hydroxylamine hydrochloride (0.5 mol L-1), pH = 1.5 | Sonication time for 15-30 min then centrifugation, save the supernatant solution and residue for next step. |
| 03 | Oxidizable fraction | 8.0 mL hydrogen peroxide (30%), 30 mL ammonium acetate (1.0 mol L-1), pH = 2.0 | Residue of step 2 was digested on hot plate twice and extractants. Sonication time for 15-30 min then centrifugation, save the supernatant solution for fractionation analysis. |

Table 2 Validation of total mercury and fractional mercury concentration in coal.

| - Standard reference material |  |  |  |  |
| --- | --- | --- | --- | --- |
| Total mercury | Certified value |  | Experimental value | recovery (%)a |
| Mercury (µg g-1) | 0.0836 |  | 0.0812 ± 0.004b | 97.1 |
| - Standard spiking addition method |  |  |  |  |
| Added mercury | 0.0 (µg g-1) | 2.0 (µg g-1) | 4.0 (µg g-1) | 6.0 (µg g-1) |
| Acid Soluble (µg g-1) | 0.14c | 2.11 | 4.15 | 6.13 |
| Recovery (%)d | -- | 98.5 | 100.2 | 99.8 |
| Reducible (µg g-1) | 0.12c | 2.07 | 4.10 | 6.15 |
| Recovery (%)d | - | 97.5 | 99.5 | 100.5 |
| Oxidizable (µg g-1) | 0.18c | 2.15 | 4.12 | 6.12 |
| Recovery (%)d | - | 98.5 | 98.5 | 99.0 |

Key:  a Recovery %=experimental valuecertified value×100  b sample mean ± standard deviation,

c Mercury concentration in fractions at 152-154m,

d Recovery (%)=spiked mercury - unspiked mercury)added mercury standard×100

Table 3 Results of fractional concentration of mercury in coal by sequential methods

| Fractions/depth |  | 152-154 (m) | 158-160 (m) | 200-203 (m) | 209-210 (m) |
| --- | --- | --- | --- | --- | --- |
| Water soluble (µg g-1) |  | 0.022 ± 0.001 | 0.036 ± 0.002 | 0.032 ± 0.001 | 0.034 ± 0.002 |
| Pseudo Hg (µg g-1) |  | 1.65 ± 0.08 | 1.86 ± 0.10 | 2.15 ± 0.11 | - ± 0.08 |
| - BCR-sequential method |  |  |  |  |  |
| Acid Soluble (µg g-1) |  | 0.14 ± 0.006 | 0.15 ± 0.008 | 0.24 ± 0.011 | 0.19 ± 0.01 |
| Reducible (µg g-1) |  | 0.12 ± 0.005 | 0.16 ± 0.009 | 0.14 ± 0.005 | 0.13 ± 0.003 |
| Oxidizable (µg g-1) |  | 1.18 ± 0.06 | 1.24 ± 0.07 | 1.21 ± 0.05 | 1.17 ± 0.058 |
| Residual (µg g-1) |  | 0.29 ± 0.02 | 0.27 ± 0.02 | 0.24 ± 0.01 | 0.21 ± 0.01 |
| ∑ (Acid + Red + Oxid) |  | 1.34 ± 0.08 | 1.55 ± 0.06 | 1.59 ± 0.09 | 1.49 ± 0.08 |
| Cf = ⅀(Acid+Red+Oxid)Residual | 4.62 |  | 5.74 | 6.62 | 7.10 |

- Single-step sequential method

| Fractions/Depth (m) | 152-154 | 158-160 | 200-203 | 209-210 |
| --- | --- | --- | --- | --- |
| Acid Soluble | 0.15 ± 0.008 | 0.20 ± 0.01 | 0.18 ± 0.01 | 0.11 ± 0.006 |
| Reducible | 0.13 ± 0.007 | 0.17 ± 0.009 | 0.13 ± 0.007 | 0.14 ± 0.008 |
| Oxidizable | 1.23 ± 0.07 | 1.25 ± 0.07 | 1.39 ± 0.08 | 1.21 ± 0.09 |
| ⅀ (Acid + Red + Oxid) | 1.51 ± 0.085 | 1.62 ± 0.85 | 1.70 ± 0.095 | 1.46 ± 0.08 |
| Recovery (%) | 112 | 104 | 107 | 98 |

- Single-step sequential based ultrasonic-assisted method

| Fractions/Depth (m) | 152-154 | 158-160 | 200-203 | 209-210 |
| --- | --- | --- | --- | --- |
| Acid Soluble | 0.16 ± 0.008 | 0.23 ± 0.002 | 0.19 ± 0.007 | 0.15 ± 0.004 |
| Reducible | 0.14 ± 0.006 | 0.19 ± 0.01 | 0.15 ± 0.008 | 0.15 ± 0.01 |
| Oxidizable | 1.25 ± 0.07 | 1.26 ± 0.04 | 1.40 ± 0.06 | 1.24 ± 0.05 |
| ⅀ (Acid + Red + Oxid) | 1.55 ± 0.09 | 1.68 ± 0.10 | 1.74 ± 0.11 | 1.54 ± 0.08 |
| Recovery (%) | 116 | 108 | 109 | 103 |

Key: a BCR-Sequential method, b Single-step sequential method, c Single-step sequential based on ultrasonic-assisted method, d First step of BCR-SM and SSM are same, e sum of the concentration of mercury in all fractions, f contamination factor and g not determined.