Supplementary for

Development patterns, material metabolism, and life-cycle greenhouse gas emissions of high-speed railway in China

Ruichang Mao1, Yankun Wu1,2, Jian Chen2, Peng Chen1, Xiaodong Li1, *

1 Department of Construction Management, School of Civil Engineering, Tsinghua University, Beijing, China

2 China Railway 14th Bureau Group Corporation Limited, Jinan, China

*email: eastdawn@tsinghua.edu.cn

Supporting Information contains:

Number of pages: 42

Number of Tables: 43

Number of Figures: 25

Table of content

1. Data collection	1

1.1 High-speed rail line dataset	1

1.2 High-speed rail station dataset	2

1.3 Socio-economic characteristics at provincial levels in China	3

1.4 The material composition indicators of Chinese high-speed rail	5

1.5 The length and material composition indicators of Chinese road infrastructure	10

1.6 The airport numbers, terminal area, runway area, and material composition indicators of Chinese aviation infrastructure	11

1.7 The carbon replacement calculation and GHG emission factors of construction materials	12

1.8 The operational GHG emission calculation and emissions factors of HSR, road, and aviation of Chinese passenger transport	14

2.	Spatial distribution of high-speed rail and stations in China	18

3.	Characteristics of Chinese high-speed rail stocks by end-use components and material typologies	21

4.	Evolution of high-speed rail stocks with soaring speed and technology advances	24

5.	Material stock characteristics across Chinese provinces and regions	27

6.	Material flows across Chinese high-speed rail networks	31

7.	Material stocks of Chinese road and aviation networks	32

8.	Carbon replacement value among Chinese road, aviation, and high-speed rail transport	35

9.	Operational GHG emissions among Chinese road, aviation, and high-speed rail transport	39

10.	Uncertainty analysis	40

11.	Reference	42

1. Data collection

1.1 High-speed rail line dataset

Totally 152 high-speed rail lines constructed from 2008 to 2020 are contrived with various sources. The HSR line length and name are collected from official statistics. HSR line attributes, including bridge-tunnel ratio, track length, tunnel length, bridge length, culvert length, speed, geo-location, and opening date, are mainly obtained from individual construction companies and on-site surveys (see a snapshot of HSR line in Fig. S1 information and the details in the Supplementary Excel).

Figure S1 High-speed rail line dataset.

1.2 High-speed rail station dataset

The information on 1064 HSR stations is retrieved from official websites, including the name, number of platforms, number of lines, station building area, and operation year (see a snapshot of the HSR line in Fig. S2 information and the details in the Supplementary Excel).

Figure S2 High-speed rail station dataset.

1.3 Socio-economic characteristics at provincial levels in China

The provincial socioeconomics (i.e., population and GDP) and land area are mainly explored from statistical yearbooks to analyze the correlations between local HSR stocks and socioeconomic factors (see Table S1).

Table S1 The socioeconomics among Chinese provinces

| Province | Pop. 2010 (Million) | Pop. 2020 (Million) | Pop. 2035 (Million) | GDP 2010 (Billion Yuan) | GDP 2020 (Billion Yuan) | Land Area (1000 km2) |
| --- | --- | --- | --- | --- | --- | --- |
| Anhui | 60 | 61 | 63 | 1236 | 3868 | 140 |
| Beijing | 20 | 22 | 26 | 1411 | 3610 | 16 |
| Chongqing | 29 | 33 | 37 | 793 | 2500 | 82 |
| Fujian | 37 | 42 | 49 | 1474 | 4390 | 121 |
| Gansu | 26 | 25 | 24 | 412 | 902 | 426 |
| Guangdong | 104 | 126 | 160 | 4601 | 11076 | 180 |
| Guangxi | 46 | 50 | 56 | 957 | 2216 | 238 |
| Guizhou | 35 | 39 | 45 | 460 | 1783 | 176 |
| Hainan | 9 | 10 | 12 | 206 | 553 | 35 |
| Hebei | 72 | 75 | 79 | 2039 | 3621 | 189 |
| Heilongjiang | 38 | 32 | 22 | 1037 | 1370 | 473 |
| Henan | 94 | 99 | 108 | 2309 | 5500 | 167 |
| Hubei | 57 | 58 | 61 | 1597 | 4344 | 186 |
| Hunan | 66 | 66 | 68 | 1604 | 4178 | 212 |
| Inner Mongolia | 25 | 24 | 23 | 1167 | 1736 | 1183 |
| Jiangsu | 79 | 85 | 94 | 4143 | 10272 | 107 |
| Jiangxi | 45 | 45 | 46 | 945 | 2569 | 167 |
| Jilin | 27 | 24 | 19 | 867 | 1231 | 187 |
| Liaoning | 44 | 43 | 41 | 1846 | 2511 | 148 |
| Ningxia | 6 | 72 | 9 | 169 | 392 | 66 |
| Qinghai | 6 | 6 | 6 | 135 | 301 | 722 |
| Shaanxi | 37 | 40 | 43 | 1012 | 2618 | 206 |
| Continued with Table S1 |  |  |  |  |  |  |
| Shandong | 96 | 102 | 111 | 3917 | 7313 | 158 |
| Shanghai | 23 | 25 | 27 | 1717 | 3870 | 6 |
| Shanxi | 36 | 35 | 34 | 920 | 1765 | 157 |
| Sichuan | 80 | 84 | 89 | 1719 | 4860 | 486 |
| Tianjin | 13 | 14 | 15 | 922 | 1408 | 12 |
| Tibet | 3 | 4 | 5 | 51 | 190 | 1228 |
| Xinjiang | 22 | 26 | 32 | 544 | 1380 | 1665 |
| Yunnan | 46 | 47 | 49 | 722 | 2452 | 394 |
| Zhejiang | 54 | 65 | 80 | 2772 | 6461 | 102 |

Note：the provincial population data for the year 2035 is projected from the individual population trend line from 2008 to 2020.

1.4 The material composition indicators of Chinese high-speed rail

Material composition indicators (MCIs) are labor-intensively compiled the HSR from bill of quantities covering more than 50 HSR construction projects (i.e., Zhengzhou-Xi’an HSR in West China, Shijiazhang-Jinan HSR in East China, Hainan Roundabout HSR in South China, Beijing-Zhangjiakou in North China, and Wuhan-Daye in central China). Over 400 material-use processes are found in each HSR construction project. Fig. S3 illustrates a snapshot of the HSR bill of quantities.

Figure S3 A snapshot of high-speed rail bill of quantities.

These construction materials are further aggregated into seven end-use HSR components (i.e., HSR bridge, subgrade, tunnel, track, station, culvert, and EPCS system). In the end, the materials utilized in HSR construction are classified into 19 typologies.

These construction materials are further aggregated into seven end-use HSR components (i.e., HSR bridge, subgrade, tunnel, track, station, culvert, and EPCS system). In the end, the materials utilized in HSR construction are classified into 19 typologies. The details MCIs are listed across components below.

| Bridge Box girder width (m) | Cement (t/km) | Gravel (t/km) | Fly ash (t/km) | Sand (t/km) | Steel (t/km) | Water (t/km) | Water-reducing admixture (t/km) | Total (t/km) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W=24 | 111.51 | 240.4 | 12.12 | 150.51 | 46.4 | 36.6 | 2.47 | 600 |
| W=32 | 166.98 | 359.99 | 18.15 | 225.38 | 71 | 54.8 | 3.7 | 900 |
| W=40 | 183.9 | 396.46 | 19.99 | 248.21 | 87 | 60.36 | 4.07 | 1000 |

Table S2 The material composition indicators of HSR bridge box girder in China

Table S3 The material composition indicators of HSR bridge pier in China

| Bridge pier height (m) | Cement (t/unit) | Fly ash (t/unit) | Gravel (t/unit) | Sand (t/unit) | Steel (t/unit) | Water (t/unit) | Water-reducing admixture (t/unit) | Total (t/unit) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H＜7 | 28.35 | 7.06 | 94.94 | 84.20 | 6.00 | 14.13 | 0.35 | 220.55 |
| 7≤H≤20 | 63.16 | 15.74 | 211.53 | 187.60 | 8.40 | 31.48 | 0.79 | 486.43 |
| 21≤H≤30 | 93.84 | 23.38 | 314.27 | 278.73 | 11.10 | 46.77 | 1.17 | 721.32 |
| 31≤H≤40 | 127.83 | 31.85 | 428.09 | 379.68 | 14.10 | 63.70 | 1.60 | 981.55 |

Table S4 The material composition indicators of HSR bridge pile foundation in China

| Bridge pile foundation diameter (m) | Cement (t/unit) | Fly ash (t/unit) | Gravel (t/unit) | Sand (t/unit) | Steel (t/unit) | Water (t/unit) | Water-reducing admixture (t/unit) | Total (t/unit) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d=1 m | 4.15 | 2.52 | 18.07 | 13.09 | 1.08 | 2.94 | 0.07 | 41.91 |
| d=1.25 m | 6.10 | 3.70 | 26.58 | 19.25 | 1.25 | 4.33 | 0.10 | 61.30 |
| d=1.5 m | 12.44 | 7.55 | 54.21 | 39.27 | 1.81 | 8.82 | 0.20 | 124.31 |
| d=1.8 m | 18.30 | 11.10 | 79.73 | 57.75 | 2.49 | 12.98 | 0.29 | 182.63 |

Table S5 The material composition indicators of HSR bridge bearing in China

| Material | Rubber | Steel | Total |
| --- | --- | --- | --- |
| Bearing (t/unit) | 0.3 | 1.2 | 1.5 |

Table S6 The material composition indicators of HSR bridge base platform according to bridge pier height in China

| Base platform (t/unit) | Cement | Fly ash | Gravel | Sand | Steel | Water | Water-reducing admixture | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pier H＜7 | 25.38 | 15.39 | 110.55 | 80.08 | 6.62 | 17.99 | 0.41 | 256.42 |
| 7≤Pier H≤20 | 54.17 | 32.86 | 235.99 | 170.94 | 11.10 | 38.41 | 0.87 | 544.32 |
| 21≤Pier H≤30 | 78.08 | 47.36 | 340.16 | 246.40 | 11.38 | 55.36 | 1.25 | 779.99 |
| 31≤Pier H≤40 | 110.29 | 66.90 | 480.48 | 348.04 | 15.00 | 78.20 | 1.76 | 1100.65 |

Table S7 The material composition indicators of HSR subgrade in China

| HSR subgrade (t/km) | Cement | Fly ash | Geotextile | Gravel | PVC | Sand | Steel | Water | Water-reducing admixture | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subgrade ancillary | 1551 | 1687 | 0.24 | 31334 | 131 | 1922 | 177 | 4821 | 343 | 73231 |
| Rail subgrade |  |  |  | 36559 |  |  |  |  |  | 36559 |

Table S8 The material composition indicators of HSR track in China

| HSR track (t/km) | Cement | Fly ash | Geotextile | Gravel | Sand | Steel | Water | Water-reducing admixture | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Steel track |  |  |  |  |  | 120 |  |  | 120 |
| Track base | 691.71 | 75.2 | 0.52 | 1491.3 | 933.6 | 520 | 227 | 15.32 | 3954.7 |

Table S9 The material composition indicators of HSR culvert in China

| HSR culvert (t/km) | Cement | Fly ash | Gravel | Sand | Steel | Water | Water-reducing admixture | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mass | 13421.41 | 1459.2 | 28935.2 | 18115.4 | 1199.4 | 4404 | 297.34 | 67832.9 |

Table S10 The material composition indicators of HSR electrification, power, communication, and signaling systems in China

| Material | Aluminum | Cement | Copper |  | Fly ash |  | Gravel | Sand | Steel | Water | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HSR electrification, power, communication, and signaling systems (t/km) | 32 | 18 | 27 | 2 |  | 39 |  | 25 | 102 | 6 | 251 |

Table S11 The material composition indicators of HSR tunnel in China

| Material | Cement | Fly ash | Gravel | Non-woven fabric | Polyethylene board | Sand | Steel | Water | Water-reducing admixture | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HSR tunnel (t/km) | 1483 | 1613 | 31983 | 6.8 | 23 | 20024 | 1238 | 4869 | 329 | 74921 |

Table S12 The material composition indicators of HSR station in China

| HSR station | Asphalt | Brick | Cement | Ceramics | Gravel | Polished Granite | Sand | Steel | Timber | Water | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station building (kg/m2) | 1 | 12 | 482 | 75 | 742 |  | 679 | 187 | 10 | 60 | 2248 |
| Platform (t/unit) |  | 27 | 2175 |  | 4689 | 135 | 2936 |  |  | 714 | 10677 |

1.5 The length and material composition indicators of Chinese road infrastructure

The national road length across five classes from 2008 to 2020 is collected from official statistics, including expressway and first to fourth classes roads. Table S13 shows the length of Chinese road network during 2008 to 2020. The road MCIs databases were also retrieved from the Chinese local construction bills of quantities, as listed in Table S14.

Table S13 The length of Chinese road network across five classes during 2008 to 2020

| Length (km) | Expressway | First-class | Second-class | Third-class | Fourth-class |
| --- | --- | --- | --- | --- | --- |
| 2008 | 60302 | 54216 | 285226 | 374215 | 2004563 |
| 2009 | 65055 | 59462 | 300686 | 379023 | 2252038 |
| 2010 | 74113 | 64430 | 308743 | 387967 | 2469456 |
| 2011 | 84946 | 68119 | 320536 | 393613 | 2586377 |
| 2012 | 96200 | 74271 | 331455 | 401865 | 2705809 |
| 2013 | 104438 | 79491 | 340466 | 407033 | 2824138 |
| 2014 | 111936 | 85362 | 348351 | 414199 | 2940986 |
| 2015 | 123523 | 90964 | 360410 | 418237 | 3053157 |
| 2016 | 130973 | 99152 | 371102 | 424443 | 3200874 |
| 2017 | 136449 | 105224 | 380481 | 429035 | 3287372 |
| 2018 | 142593 | 111703 | 393471 | 437060 | 3381036 |
| 2019 | 149571 | 117061 | 405345 | 446107 | 3580640 |
| 2020 | 160980 | 123101 | 418300 | 457375 | 3784732 |

Table S14 The material composition indicators of different road typologies in China

| Road MCIs (t/km) | Expressway | First-class | Second-class | Third-class | Fourth-class |
| --- | --- | --- | --- | --- | --- |
| Asphalt | 1250 | 938 | 625 | 375 | 150 |
| Sand | 3759 | 2820 | 1880 | 1128 | 451 |
| Silica fume | 1185 | 888 | 592 | 355 | 142 |
| Gravel | 72824 | 54618 | 36412 | 21847 | 8739 |
| Cement | 1102 | 827 | 551 | 331 | 132 |
| Water | 2675 | 2006 | 1337 | 802 | 321 |
| Total | 82795 | 62096 | 41398 | 24839 | 9935 |

1.6 The airport numbers, terminal area, runway area, and material composition indicators of Chinese aviation infrastructure

The Chinese airport infrastructure, including the number of airports, airport runways, and airport parking bays, are collected from official statistics from 2008 to 2020. Based on the collected data and airport construction design code, we thus estimated the airport terminal areas and runway areas in China (see Table S15).

Table S15 The length of Chinese road network across five classes during 2008 to 2020

| Airport | No. of airports | No. of runways | No. of parking bays | Terminal area (km2) | Runways area (km2) |
| --- | --- | --- | --- | --- | --- |
| 2008 | 160 | 148 | 2880 | 8.83 | 41.36 |
| 2009 | 166 | 151 | 2988 | 9.20 | 42.53 |
| 2010 | 175 | 160 | 3500 | 9.63 | 47.28 |
| 2011 | 180 | 165 | 3600 | 10.13 | 48.70 |
| 2012 | 183 | 165 | 3660 | 10.07 | 49.10 |
| 2013 | 193 | 175 | 3860 | 10.62 | 51.93 |
| 2014 | 202 | 184 | 4444 | 11.21 | 57.16 |
| 2015 | 210 | 191 | 4620 | 11.61 | 59.38 |
| 2016 | 218 | 199 | 4796 | 12.08 | 61.75 |
| 2017 | 229 | 210 | 5038 | 12.60 | 65.01 |
| 2018 | 235 | 255 | 5800 | 14.55 | 76.83 |
| 2019 | 238 | 261 | 6244 | 16.29 | 80.68 |
| 2020 | 241 | 265 | 6621 | 18.00 | 83.79 |

The MCIs databases of the airport runway and terminal building were also retrieved from the Chinese local construction bills of quantities, as listed in Table S16 and S17.

Table S16 The material composition indicators of airport runway in China.

| Material | Asphalt | Sand | Silica fume | Gravel | Cement | Water |
| --- | --- | --- | --- | --- | --- | --- |
| Airport runway (t/m2) | 92.68 | 278.66 | 87.81 | 5398.10 | 81.71 | 198.26 |

Table S17 The material composition indicators of terminal building in China.

| Material | Cement | Ceramics | Gravel | Sand | Steel |
| --- | --- | --- | --- | --- | --- |
| Terminal building (kg/m2) | 137.53 | 1.38 | 523.48 | 341.99 | 83.50 |

1.7 The carbon replacement calculation and GHG emission factors of construction materials

The carbon replacement value (CRV) of Chinese HSR, road, and aviation systems, implying the GHG emissions generated if current technologies and materials replace existing infrastructure, are calculated based on GHG emission factors of materials and the estimated stocks. The carbon replacement value of Chinese HSR, road, and airport stocks was estimated with GHG emission factors and the bottom-up model. Equation 1 illustrates the bottom-up model to quantify CRV across transportation systems.

CRVm,i =m,i(EFm × MSm,i)      (1)

Where CRVm,i is the total carbon replacement value of material m in transport network i; EFm  represents the emission factor of material m; and MSm,i refers to the material stocks of material type m in transport network i.

The China-specific GHG emission factors (measured in CO2 equivalent, CO2-eq/t), which represent cradle-to-gate requirements for raw material extraction, manufacture, and processing, are modeled from a local Chinese Life Cycle Database in eBalance software1 or from the latest literature to estimate the CRV in HSR, road, and aviation networks. See the GHG emission factors of construction materials in Table S18.

Table S18. The GHG emission factors of construction materials.

| Materials | Aluminum2 | Asphalt3 | Brick4 | Cement4 | Ceramics1 |
| --- | --- | --- | --- | --- | --- |
| Impact factor (kg CO2-eq/t) | 7950 | 240 | 330 | 880 | 740 |
| Materials | Copper1 | Fly ash1 | Geotextile5 | Gravel1 | Polished Granite1 |
| Impact factor (kg CO2-eq/t) | 7800 | 180 | 1980 | 2.425 | 15 |
| Materials | Non-woven fabric 1 | Polyethylene board6 | PVC1 | Rubber1 | Sand1 |
| Impact factor (kg CO2-eq/t) | 1470 | 570 | 1720 | 2560 | 2.796 |
| Materials | Steel product7 | Steel bar7 | Timber1 | Water1 | Water-reducing admixture4 |
| Impact factor (kg CO2-eq/t) | 2670 | 4080 | 178 | 0.168 | 28.49 |

Note: the source of material GHG emission factors were cited accordingly.

1.8 The operational GHG emission calculation and emissions factors of HSR, road, and aviation of Chinese passenger transport

The operational GHG emission in China’s HSR, road, and aviation passenger transport is estimated based on the annual passenger turnover volume and the operational emissions factors. Equation 2 illustrates the bottom-up model to quantify operational GHG emissions across passenger transport.

OEt =t(OEFt × PTt)      (2)

Where   OEt  is the operational GHG emission of passenger transport typology t; OEFt  represents the operational emission factor of passenger transport t; and PTt refers to the passenger turnover volume of passenger transport t.

Table S19 and S20 depicts the passenger turnover volume of Chinese passenger transport across HSR, road, and aviation from 2008 to 2020.

| Road turnover (billion passenger * km) | Large vehicle | Medium vehicle | Small vehicle | Minicar vehicle | Total |
| --- | --- | --- | --- | --- | --- |
| 2008 | 33 | 47 | 1063 | 105 | 1248 |
| 2009 | 30 | 41 | 1184 | 96 | 1351 |
| 2010 | 29 | 36 | 1349 | 89 | 1502 |
| 2011 | 28 | 33 | 1530 | 84 | 1676 |
| 2012 | 26 | 27 | 1715 | 79 | 1847 |
| 2013 | 14 | 12 | 1060 | 39 | 1125 |
| 2014 | 12 | 10 | 1048 | 29 | 1100 |
| 2015 | 11 | 7 | 1035 | 22 | 1074 |
| 2016 | 9 | 5 | 994 | 15 | 1023 |
| 2017 | 8 | 4 | 954 | 11 | 977 |
| 2018 | 7 | 3 | 909 | 8 | 928 |
| 2019 | 6 | 3 | 870 | 7 | 886 |
| 2020 | 3 | 1 | 457 | 3 | 464 |

Table S19 The annual passenger turnover volume of Chinese road passenger transport.

Table S20 The annual passenger turnover volume of Chinese HSR and aviation passenger transport.

| Turnover volume (billion passenger * km) | HSR | Aviation |
| --- | --- | --- |
| 2008 | 1.56 | 288.28 |
| 2009 | 16.22 | 337.52 |
| 2010 | 46.32 | 403.90 |
| 2011 | 105.84 | 453.70 |
| 2012 | 144.61 | 502.57 |
| 2013 | 214.11 | 565.68 |
| 2014 | 282.50 | 633.42 |
| 2015 | 386.34 | 728.26 |
| 2016 | 464.10 | 837.81 |
| 2017 | 587.56 | 951.30 |
| 2018 | 687.19 | 1071.23 |
| 2019 | 774.67 | 1170.53 |
| 2020 | 484.49 | 631.13 |

The operational GHG emissions factors of HSR, road, and aviation in China’s passenger transport are retrieved and managed from various literatures8–11 (see Table S21 to S24 of the collected factors).

| Operational GHG emissions of road | Gasoline of small and minicar vehicle (L/100 km) | Gasoline of medium and large vehicle (L/100 km) | GHG emissions of small and minicar vehicle (g/passenger*km) | GHG emissions of medium and large vehicle (g/passenger*km) |
| --- | --- | --- | --- | --- |
| 2008 | 9.6 | 28.1 | 198.1 | 581.4 |
| 2009 | 9.3 | 28.0 | 192.6 | 579.4 |
| 2010 | 8.9 | 28.0 | 183.2 | 576.4 |
| 2011 | 8.8 | 27.8 | 181.6 | 575.3 |
| 2012 | 8.5 | 27.7 | 176.1 | 573.2 |
| 2013 | 8.3 | 27.6 | 170.6 | 571.2 |
| 2014 | 8.3 | 27.5 | 171.6 | 568.6 |
| 2015 | 7.7 | 27.4 | 159.6 | 567.1 |
| 2016 | 7.5 | 27.3 | 154.1 | 565.1 |
| 2017 | 7.2 | 27.2 | 148.6 | 563.0 |
| 2018 | 6.9 | 27.1 | 143.1 | 561.0 |
| 2019 | 6.7 | 27.0 | 137.6 | 559.0 |
| 2020 | 6.3 | 27.0 | 130.7 | 560.1 |

Table S21 The gasoline requirement and GHG emissions factors of road passenger transport across minicar, small, medium, and large vehicles.

Table S22 The operational GHG emissions factors of HSR passenger transport across 200 km/h, 250 km/h, and 300 km/h train speed.

| Train speed (km/h) | Seat | GHG emissions (g/PKT) | Energy type |
| --- | --- | --- | --- |
| 200 | 600 | 273 | Electricity |
| 250 | 1200 | 385 |  |
| 300 | 600 | 403 |  |

| 350km/h high-speed train occupancy rate (%) | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GHG emissions (g/ passenger*km) | 130 | 99 | 80 | 65 | 57 | 49 | 44 | 40 |

Table S23 The operational GHG emissions factors of HSR passenger transport in 350 km/h train speed.

Table S24 The operational GHG emissions factors of aviation passenger transport

| Year | GHG emissions (kg/ton flight weight*km) | Energy type |
| --- | --- | --- |
| 2008 | 1.08 | Aviation gasoline |
| 2009 | 1.02 |  |
| 2010 | 0.995 |  |
| 2011 | 0.92 |  |
| 2012 | 0.94 |  |
| 2013 | 0.95 |  |
| 2014 | 0.93 |  |
| 2015 | 0.93 |  |
| 2016 | 0.8798 |  |
| 2017 | 0.8606 |  |
| 2018 | 0.8414 |  |
| 2019 | 0.8222 |  |
| 2020 | 0.803 |  |

- Spatial distribution of high-speed rail and stations in China

Figure S4 The spatial distribution of HSR network in 2008.

The spatial distributions of Chinese high-speed rail network in 2008, 2010, 2015, 2020, and 2035 are displayed as below.

Figure S5 The spatial distribution of HSR network in 2010.

Figure S6 The spatial distribution of HSR network in 2015.

Figure S7 The spatial distribution of HSR network in 2020.

Figure S8 The spatial distribution of HSR network in 2035.

- Characteristics of Chinese high-speed rail stocks by end-use components and material typologies

(a)

(b)

Figure S9 The HSR stock distribution across construction materials. (a) the material ratio across seven main construction materials. (b) the other construction materials that account for 0.5% of total HSR stocks.

Table S25 The weight of HSR stock across construction materials

| Material | Stock (Mt) | Ratio (5%) |
| --- | --- | --- |
| Aluminum | 1.25 | 0.03% |
| Asphalt | 0.02 | 0.00% |
| Brick | 0.35 | 0.01% |
| Cement | 559.99 | 15.32% |
| Ceramics | 1.61 | 0.04% |
| Copper | 1.05 | 0.03% |
| Fly ash | 91.01 | 2.49% |
| Geotextile | 0.05 | 0.00% |
| Gravel | 1756.11 | 48.06% |
| Polished Granite | 0.45 | 0.01% |
| Non-woven fabric | 0.05 | 0.00% |
| Polyethylene board | 0.16 | 0.00% |
| PVC | 1.50 | 0.04% |
| Rubber | 0.18 | 0.00% |
| Sand | 891.26 | 24.39% |
| Steel | 132.36 | 3.62% |
| Timber | 0.20 | 0.01% |
| Water | 205.25 | 5.62% |
| Water-reducing admixture | 11.33 | 0.31% |
| Total | 3654.16 | 100% |

Figure S10 The HSR stock distribution across end-use components.

| End-use component | Detailed component | Detailed stock (Mt) | End-use stock (Mt) | Ratio |
| --- | --- | --- | --- | --- |
| Track | Steel rail | 10.12 | 343.78 | 9.4% |
|  | Track base | 333.66 |  |  |
| Subgrade | Subgrade ancillary | 837.44 | 1255.52 | 34.4% |
|  | Rail subgrade | 418.07 |  |  |
| Tunnel |  | 523.56 | 523.56 | 14.3% |
| Bridge | Box girder | 525.99 | 1358.59 | 37.2% |
|  | Bearing | 0.91 |  |  |
|  | Pier | 283.63 |  |  |
|  | Pile foundation | 250.48 |  |  |
|  | Base platform | 297.59 |  |  |
| Culvert |  | 79.71 | 79.71 | 2.2% |
| EPCS systems |  | 9.77 | 9.77 | 0.3% |
| HSR station | Station building | 47.85 | 83.24 | 2.3% |
|  | Platform | 35.38 |  |  |
| Total |  | 3654.16 | 3654.16 | 100.0% |

Table S26 The weight of HSR stock across end-use components

- Evolution of high-speed rail stocks with soaring speed and technology advances

| Year | 200 (km/h) | 250 (km/h) | 300 (km/h) | 350 (km/h) | Total |
| --- | --- | --- | --- | --- | --- |
| 2008 | 362 | 156 | 0 | 117 | 635 |
| 2009 | 146 | 1118 | 0 | 1068 | 2332 |
| 2010 | 65 | 831 | 0 | 982 | 1878 |
| 2011 | 120 | 0 | 0 | 1420 | 1540 |
| 2012 | 759 | 46 | 1229 | 1036 | 3070 |
| 2013 | 1463 | 1146 | 0 | 923 | 3532 |
| 2014 | 107 | 3546 | 952 | 1344 | 5949 |
| 2015 | 1174 | 1241 | 474 | 1429 | 4318 |
| 2016 | 1054 | 795 | 0 | 825 | 2674 |
| 2017 | 378 | 1447 | 0 | 0 | 1825 |
| 2018 | 2343 | 893 | 0 | 812 | 4048 |
| 2019 | 336 | 1742 | 235 | 2015 | 4328 |
| 2020 | 291 | 1281 | 0 | 810 | 2382 |
| Total | 8798 | 14492 | 3190 | 13130 | 38510 |

Table S27 The annual length of HSR network across different speeds

Table S28 The average HSR speed and material stock density from 2008 to 2020

| Year | Average speed (km/h) | Stock density (t/m) |
| --- | --- | --- |
| 2008 | 239.92 | 111.00 |
| 2009 | 281.38 | 103.16 |
| 2010 | 288.81 | 101.26 |
| 2011 | 300.75 | 96.96 |
| 2012 | 297.72 | 94.88 |
| 2013 | 286.22 | 94.47 |
| 2014 | 284.17 | 93.50 |
| 2015 | 282.46 | 93.10 |
| 2016 | 280.26 | 93.16 |
| 2017 | 277.59 | 92.73 |
| 2018 | 272.95 | 93.06 |
| 2019 | 275.64 | 92.23 |
| 2020 | 275.78 | 91.76 |

The Chinese national HSR design code requires the minimum small radius curves, the at least subgrade surface width, the track load requirement, and the lowest tunnel clearance sectional area with various operational speeds12. The material consumption across HSR components is thus evaluated according to the design code. Table S29 to S32 show the requirements of the HSR track, according to the Chinese HSR design code.

Table S29 The width and load requirement of HSR track according to design code

| Track typology | Width (m) | Track load (kN/m2) | Train load (kN/m2) | Total load (kN/m2) |
| --- | --- | --- | --- | --- |
| CRTS Ⅰ Slab | 3 | 12.6 | 41.7 | 54.3 |
| CRTS Ⅱ Slab | 3.25 | 11.6 | 38.5 | 50.1 |
| CRTS Ⅲ Slab | 3.1 | 13.7 | 40.4 | 54.1 |
| CRTS Ⅰ double-block | 3.4 | 13.7 | 36.8 | 50.5 |
| Ballasted | 3.4 | 17.3 | 36.8 | 54.1 |

Table S30 The HSR subgrade surface width across tracks and HSR speed according to design code

| Track typology | Speed (km/h) | Distance between lines (m) | Subgrade width (m) |
| --- | --- | --- | --- |
| Ballasted track | 250 | 4.6 | 13.2 |
|  | 300 | 4.8 | 13.4 |
|  | 350 | 5 | 13.6 |
| Unballasted track | 250 | 4.6 | 1.34 |
|  | 300 | 4.8 | 13.6 |
|  | 350 | 5 | 13.8 |

Table S31 The HSR minimum small radius curves across tracks and HSR speed according to design code

| HSR speed (km/h) |  | 350 | 300 | 250 |
| --- | --- | --- | --- | --- |
| Ballasted track | Normal (m) | 7000 | 500 | 3500 |
|  | Difficult(m) | 6000 | 4500 | 3000 |
| Unballasted track | Normal(m) | 7000 | 5000 | 3200 |
|  | Difficult(m) | 5500 | 4000 | 2800 |

Table S32 The lowest tunnel clearance sectional area across HSR speed according to design code

| HSR speed (km/h) | 200 | 250 | 300 | 350 |
| --- | --- | --- | --- | --- |
| Lowest tunnel clearance sectional area (m2) | 90 | 90 | 100 | 100 |

- Material stock characteristics across Chinese provinces and regions

Table S33 The material stocks at Chinese provincial level in 2010, 2020, and 2035

|  | Stocks in 2010 (Mt) | MS in 2020 (Mt) | MS in 2035 (Mt) |
| --- | --- | --- | --- |
| Anhui | 35 | 200 | 218 |
| Beijing | 7 | 28 | 39 |
| Chongqing | 1 | 84 | 165 |
| Fujian | 36 | 154 | 154 |
| Gansu | 0 | 146 | 180 |
| Guangdong | 32 | 188 | 234 |
| Guangxi | 1 | 152 | 184 |
| Guizhou | 0 | 127 | 153 |
| Hainan | 33 | 65 | 68 |
| Hebei | 11 | 170 | 238 |
| Heilongjiang | 0 | 142 | 155 |
| Henan | 34 | 178 | 215 |
| Hong Kong | 0 | 2 | 2 |
| Hubei | 32 | 154 | 257 |
| Hunan | 54 | 178 | 230 |
| Inner Mongolia | 0 | 65 | 222 |
| Jiangsu | 36 | 198 | 232 |
| Jiangxi | 13 | 162 | 201 |
| Jilin | 17 | 83 | 83 |
| Liaoning | 54 | 241 | 241 |
| Ningxia | 0 | 29 | 59 |
| Qinghai | 0 | 25 | 40 |
| Shaanxi | 15 | 91 | 212 |
| Shandong | 43 | 195 | 266 |
| Shanghai | 10 | 13 | 13 |
| Shanxi | 14 | 104 | 153 |
| Sichuan | 23 | 149 | 277 |
| Tianjin | 7 | 23 | 28 |
| Tibet | 0 | 0 | 0 |
| Xinjiang | 0 | 79 | 79 |
| Yunnan | 0 | 90 | 119 |
| Zhejiang | 46 | 138 | 153 |

Table S34 The material stock density at Chinese provincial level in 2010, 2020, and 2035

|  | Stock density in 2010 (1000 t/m2) | Stock density in 2020 (1000 t/m2) | Stock density in 2035 (1000 t/m2) |
| --- | --- | --- | --- |
| Anhui | 248 | 1430 | 1553 |
| Beijing | 455 | 1684 | 2366 |
| Chongqing | 8 | 1018 | 1997 |
| Fujian | 295 | 1269 | 1269 |
| Gansu | 1 | 343 | 423 |
| Guangdong | 179 | 1044 | 1303 |
| Guangxi | 4 | 640 | 776 |
| Guizhou | 0 | 722 | 871 |
| Hainan | 933 | 1838 | 1928 |
| Hebei | 57 | 901 | 1260 |
| Heilongjiang | 0 | 301 | 327 |
| Henan | 201 | 1068 | 1286 |
| Hong Kong | 0 | 2057 | 2057 |
| Hubei | 170 | 828 | 1383 |
| Hunan | 257 | 841 | 1085 |
| Inner Mongolia | 0 | 55 | 188 |
| Jiangsu | 339 | 1847 | 2167 |
| Jiangxi | 77 | 971 | 1202 |
| Jilin | 88 | 444 | 444 |
| Liaoning | 365 | 1626 | 1626 |
| Ningxia | 0 | 444 | 881 |
| Qinghai | 0 | 35 | 55 |
| Shaanxi | 71 | 443 | 1032 |
| Shandong | 274 | 1233 | 1682 |
| Shanghai | 1616 | 2082 | 2082 |
| Shanxi | 92 | 664 | 978 |
| Sichuan | 47 | 306 | 570 |
| Tianjin | 592 | 1880 | 2299 |
| Tibet | 0 | 0 | 0 |
| Xinjiang | 0 | 47 | 47 |
| Yunnan | 1 | 228 | 301 |
| Zhejiang | 454 | 1360 | 1505 |

Table S35 The per capita material stocks density at Chinese provincial level in 2010, 2020, and 2035

|  | Stock per capita in 2010 (kg/cap) | Stock per capita in 2020 (kg/cap) | Stock per capita in 2035 (kg/cap) |
| --- | --- | --- | --- |
| Anhui | 585 | 3282 | 3434 |
| Beijing | 381 | 1262 | 1519 |
| Chongqing | 23 | 2575 | 4388 |
| Fujian | 972 | 3710 | 3168 |
| Gansu | 11 | 5844 | 7440 |
| Guangdong | 308 | 1488 | 1464 |
| Guangxi | 19 | 3033 | 3269 |
| Guizhou | 1 | 3301 | 3424 |
| Hainan | 3808 | 6453 | 5640 |
| Hebei | 150 | 2280 | 3015 |
| Heilongjiang | 4 | 4464 | 6983 |
| Henan | 356 | 1795 | 1989 |
| Hong Kong | 0 | 305 | 284 |
| Hubei | 551 | 2666 | 4197 |
| Hunan | 829 | 2681 | 3396 |
| Inner Mongolia | 14 | 2709 | 9633 |
| Jiangsu | 462 | 2336 | 2464 |
| Jiangxi | 289 | 3586 | 4348 |
| Jilin | 603 | 3456 | 4375 |
| Liaoning | 1234 | 5651 | 5896 |
| Ningxia | 0 | 409 | 6803 |
| Qinghai | 18 | 4253 | 6275 |
| Shaanxi | 390 | 2307 | 4926 |
| Shandong | 451 | 1917 | 2402 |
| Shanghai | 445 | 531 | 481 |
| Shanxi | 404 | 2981 | 4550 |
| Sichuan | 286 | 1777 | 3103 |
| Tianjin | 547 | 1622 | 1819 |
| Tibet | 0 | 0 | 0 |
| Xinjiang | 13 | 3041 | 2434 |
| Yunnan | 4 | 1906 | 2415 |
| Zhejiang | 850 | 2143 | 1922 |

Figure S11 The changes of provincial stock Gini index from 2010 to 2035

Figure S12 The Chinese HSR stocks separated by the Hu-line in 2020

- Material flows across Chinese high-speed rail networks

Table S36 The material composition in material production stage

| Material | Cement | Fly ash | Gravel | Sand | Steel | Water | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mass (Mt) | 561.44 | 91.89 | 1773.94 | 895.84 | 133.59 | 206.27 | 18.21 |

Table S37 The material composition in construction stage

| Process | Cement (Mt) | Fly ash (Mt) | Gravel (Mt) | Sand (Mt) | Steel (Mt) | Water (Mt) | Others (Mt) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Roadbed | 58.36 | 6.34 | 125.82 | 78.77 | 54.00 | 19.15 | 1.34 |
| Foundation | 54.60 | 33.12 | 655.94 | 172.30 | 10.59 | 38.71 | 0.87 |
| Ancillary | 195.62 | 19.37 | 391.19 | 244.95 | 10.69 | 59.00 | 10.53 |
| Concrete | 251.40 | 32.18 | 583.16 | 395.23 | 57.08 | 88.38 | 5.45 |
| Framework |  |  |  |  | 1.91 |  |  |
| Temporary | 1.45 | 0.88 | 17.83 | 4.58 |  | 1.03 | 0.02 |

Table S38 The material composition in operation stage

| Process | Cement (Mt) | Fly ash (Mt) | Gravel (Mt) | Sand (Mt) | Steel (Mt) | Water (Mt) | Others (Mt) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Track | 58.36 | 6.34 | 125.82 | 78.77 | 54.00 | 19.15 | 1.34 |
| Subgrade | 177.44 | 19.29 | 776.39 | 219.82 | 2.02 | 55.13 | 5.43 |
| Tunnel | 103.67 | 11.27 | 223.50 | 139.93 | 8.65 | 34.03 | 2.51 |
| Bridge | 186.56 | 52.31 | 563.53 | 406.32 | 58.33 | 87.89 | 3.64 |
| Culvert | 15.77 | 1.71 | 34.00 | 21.29 | 1.41 | 5.18 | 0.35 |
| EPCS systems | 0.71 | 0.08 | 1.53 | 0.96 | 3.97 | 0.23 | 2.30 |
| Station | 17.48 |  | 31.34 | 24.18 | 3.98 | 3.64 | 2.63 |

Table S39 The material composition in recycling & reuse and construction waste stages

|  | Cement (Mt) | Fly ash (Mt) | Gravel (Mt) | Sand (Mt) | Steel (Mt) | Water (Mt) | Others (Mt) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Recycling & reuse |  |  |  |  | 0.68 |  |  |
| Construction waste | 1.45 | 0.88 | 17.83 | 4.58 | 0.56 | 1.03 | 0.02 |

- Material stocks of Chinese road and aviation networks

Figure S13 Development of road material stocks in China from 2008 to 2020

Material stocks (Mt)

Figure S14 The road stock distribution across construction materials in 2020

Figure S16 The aviation stock distribution across construction materials in 2020

Figure S15 Development of aviation material stocks in China from 2008 to 2020

Material stocks (Mt)

| Material efficiency (kg/passenger * km) | HSR | Road | Aviation |
| --- | --- | --- | --- |
| 2008 | 80.44 | 36.92 | 0.91 |
| 2009 | 21.67 | 36.70 | 0.80 |
| 2010 | 11.43 | 35.28 | 0.74 |
| 2011 | 6.26 | 33.25 | 0.68 |
| 2012 | 6.54 | 31.78 | 0.62 |
| 2013 | 5.98 | 54.37 | 0.58 |
| 2014 | 6.49 | 57.86 | 0.57 |
| 2015 | 5.78 | 61.87 | 0.52 |
| 2016 | 5.36 | 67.86 | 0.47 |
| 2017 | 4.50 | 73.18 | 0.43 |
| 2018 | 4.42 | 79.62 | 0.46 |
| 2019 | 4.44 | 87.12 | 0.44 |
| 2020 | 7.54 | 174.50 | 0.85 |

Table S40 The material stock efficiency among Chinese road, aviation, and high-speed rail transport from 2008 to 2020

Figure S17 The carbon replacement value of Chinese road network from 2008 to 2020

Carbon replacement value among Chinese road, aviation, and high-speed rail transport

Figure S18 The share of Chinese road carbon replacement value across materials

Figure S20 The share of Chinese aviation carbon replacement value across materials

Figure S19 The carbon replacement value of Chinese aviation network from 2008 to 2020

Figure S22 The share of Chinese high-speed rail carbon replacement value across materials

Figure S21 The carbon replacement value of Chinese high-speed rail network from 2008 to 2020

| CRV efficiency (kg/passenger * km) | HSR | Road | Aviation |
| --- | --- | --- | --- |
| 2008 | 19.11 | 0.65 | 0.03 |
| 2009 | 5.47 | 0.65 | 0.03 |
| 2010 | 2.98 | 0.62 | 0.02 |
| 2011 | 1.70 | 0.59 | 0.02 |
| 2012 | 1.79 | 0.56 | 0.02 |
| 2013 | 1.63 | 0.96 | 0.02 |
| 2014 | 1.77 | 1.02 | 0.02 |
| 2015 | 1.58 | 1.09 | 0.02 |
| 2016 | 1.46 | 1.20 | 0.02 |
| 2017 | 1.24 | 1.29 | 0.01 |
| 2018 | 1.21 | 1.40 | 0.01 |
| 2019 | 1.22 | 1.54 | 0.01 |
| 2020 | 2.08 | 3.08 | 0.03 |

Table S41 The carbon replacement value efficiency among Chinese road, aviation, and high-speed rail transport from 2008 to 2020

- Operational GHG emissions among Chinese road, aviation, and high-speed rail transport

Table S42 The operational GHG emissions and efficiency among Chinese road, aviation, and high-speed rail transport from 2008 to 2020

| Year | Operational GHG emissions (Mt) |  |  | Operational GHG emissions efficiency (g/pkm) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | High-speed rail | Road | Aviation | High-speed rail | Road | Aviation |
| 2008 | 0.15 | 164.45 | 43.59 | 108.97 | 131.82 | 150.93 |
| 2009 | 1.61 | 173.39 | 48.20 | 106.66 | 128.33 | 142.54 |
| 2010 | 4.59 | 183.57 | 56.26 | 99.09 | 122.21 | 139.04 |
| 2011 | 6.46 | 202.99 | 58.44 | 61.04 | 121.11 | 128.57 |
| 2012 | 9.40 | 217.18 | 66.14 | 65.00 | 117.60 | 131.34 |
| 2013 | 12.20 | 128.14 | 75.24 | 56.98 | 113.90 | 132.74 |
| 2014 | 18.36 | 125.73 | 82.47 | 60.00 | 114.33 | 129.96 |
| 2015 | 22.02 | 114.24 | 94.82 | 56.94 | 106.34 | 129.95 |
| 2016 | 25.06 | 104.88 | 103.19 | 53.95 | 102.54 | 122.94 |
| 2017 | 25.85 | 96.49 | 114.62 | 44.01 | 98.81 | 120.26 |
| 2018 | 30.24 | 88.31 | 126.19 | 44.02 | 95.16 | 117.58 |
| 2019 | 34.09 | 81.06 | 134.74 | 43.98 | 91.51 | 114.89 |
| 2020 | 31.49 | 40.34 | 70.95 | 64.91 | 86.92 | 112.21 |

- Uncertainty analysis

Figure S24 The contributions of the end-use component to the uncertainty of Chinese high-speed rail material stocks

Figure S23 The uncertainty of Chinese high-speed rail material stocks

Figure S25 The contributions of annual construction to the uncertainty of Chinese high-speed rail material stocks

| Year | High-speed rail (Mt) |  |  | Road (Mt) |  |  | Aviation (Mt) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean | Min | Max | Mean | Min | Max | Mean | Min | Max |
| 2008 | 0.15 | 0.14 | 0.20 | 164.45 | 111.67 | 237.91 | 43.59 | 26.22 | 61.97 |
| 2009 | 1.61 | 1.46 | 2.11 | 173.39 | 119.48 | 254.49 | 48.20 | 29.00 | 68.52 |
| 2010 | 4.59 | 4.13 | 5.10 | 183.57 | 128.13 | 272.81 | 56.26 | 33.85 | 79.99 |
| 2011 | 6.46 | 6.04 | 6.87 | 202.99 | 143.04 | 304.48 | 58.44 | 35.16 | 83.08 |
| 2012 | 9.40 | 7.75 | 10.86 | 217.18 | 154.32 | 328.54 | 66.14 | 39.79 | 94.03 |
| 2013 | 12.20 | 11.52 | 13.45 | 128.14 | 91.84 | 195.49 | 75.24 | 45.26 | 106.96 |
| 2014 | 18.36 | 12.52 | 20.14 | 125.73 | 90.13 | 193.26 | 82.47 | 49.62 | 117.25 |
| 2015 | 22.02 | 16.33 | 27.38 | 114.24 | 81.86 | 176.68 | 94.82 | 57.05 | 134.81 |
| 2016 | 25.06 | 17.64 | 33.41 | 104.88 | 75.13 | 163.11 | 103.19 | 62.09 | 146.71 |
| 2017 | 25.85 | 21.68 | 31.13 | 96.49 | 69.10 | 150.59 | 114.62 | 68.96 | 162.95 |
| 2018 | 30.24 | 25.64 | 34.77 | 88.31 | 63.23 | 138.04 | 126.19 | 75.92 | 179.40 |
| 2019 | 34.09 | 28.52 | 40.78 | 81.06 | 58.03 | 126.88 | 134.74 | 81.06 | 191.56 |
| 2020 | 31.49 | 24.17 | 38.54 | 40.34 | 28.88 | 63.21 | 70.95 | 42.69 | 100.87 |

Table S43 The uncertainties of operational GHG emissions among Chinese road, aviation, and high-speed rail transport from 2008 to 2020

- Reference

1.	Chinese Life Cycle Database (CLCD). Chinese Core Life Cycle Database Version 0.8. Available at eBalance 4.7 Software (2021).

2.	Zhang, Y. et al. Environmental footprint of aluminum production in China. J. Clean. Prod. 133, 1242–1251 (2016).

3.	Zhang, H., Chen, H., Xu, S. & Li, P. Evaluation on Energy Consumption and Carbon Emission of Rubber Modified Asphalt Mixture Pavement (in Chinese). Highw. Eng. 46, 154–164 (2021).

4.	Cai, B. et al. China high resolution emission database (CHRED) with point emission sources, gridded emission data, and supplementary socioeconomic data. Resour. Conserv. Recycl. 129, 232–239 (2018).

5.	Zhou, Y., Gan, L. & HAN, F. Accounting of Express Delivery Carbon Footprint Based on Life Cycle Assessment (in Chinese). Logist. Technol. 40, 104-109+142 (2021).

6.	Sun, R. et al. Life Cycle Cost Analysis of Polyethylene Production (in Chinese). ACTA Petrol. Sin. 32, 401–406 (2016).

7.	Liu, H., Fu, J., Liu, S., Xie, X. & Yang, X. Calculation methods and application of carbon dioxide emission during steel-making process (in Chinese). Iron Steel 51, 74–82 (2016).

8.	Gambhir, A., Tse, L. K. C., Tong, D. & Martinez-Botas, R. Reducing China’s road transport sector CO2 emissions to 2050: Technologies, costs and decomposition analysis. Appl. Energy 157, 905–917 (2015).

9.	Chang, Y. et al. The energy use and environmental emissions of high-speed rail transportation in China: A bottom-up modeling. Energy 182, 1193–1201 (2019).

10.	Cui, Z. Carbon Emission Analysis of High-speed Railway in Operation Stage (in Chinese). (ShijiazhuangTiedaoUniversity, 2019). doi:10.27334/d.cnki.gstdy.2019.000502.

11.	Shan, Y. et al. China CO2 emission accounts 1997–2015. Sci. Data 5, 170201 (2018).

12.	National Railway Administration of the People’s Republic of China. Code for Design of High Speed Railway （TB 10621-2014）. Preprint at (2014).