Monitoring of Air Pollution in Agricultural Settings: A Systematic Review Of The Literature

Francesca Borghia*, Andrea Spinazzèa*, Nicholas De Nardisa, Serena Straccinia, Sabrina Rovellia, Giacomo Fantia, Daniele Oxolib, Andrea Cattaneoa, Domenico Maria Cavalloa, Maria Antonia Brovellib.

aDepartment of Science and High Technology, University of Insubria, 22100 Como, Italy

bDepartment of Civil and Environmental Engineering, Politecnico di Milano, 20133, Milan, Italy

FB: francesca.borghi@uninsubria.it; AS: andrea.spinazze@uninsubria.it; NDN: ndenardis@studenti.uninsubria.it; SS: sstraccini@studenti.uninsubria.it; SR: sabrina.rovelli@uninsubria.it; GF: giacomo.fanti@uninsubria.it; DO: daniele.oxoli@polimi.it; AC: andrea.cattaneo@uninsubria.it; DMC: domenico.cavallo@uninsubria.it; MAB: maria.brovelli@polimi.it

Correspondence:

FB: Department of Science and High Technology, University of Insubria, 22100 Como, Italy

Mail: francesca.borghi@uninsubria.it

Phone: +39 031 238 6645

AS: Department of Science and High Technology, University of Insubria, 22100 Como, Italy

Mail: andrea.spinazze@uninsubria.it

Phone: +39 031 238 6629

Supplementary material

Table SM1. Summary of the characteristics of the experimental design adopted in the different studies considered in this review (aim of the study; pollutants investigated; year and period of monitoring/sampling; country; area). S/M: Sampling/Monitoring; W: Winter; SP: Spring; SU: Summer; F: Fall.

| Reference | Aim of the study | Pollutants | Year of S/M | S/M period | Country | Area |
| --- | --- | --- | --- | --- | --- | --- |
| (Ali-Sak et al. 2021) | - Measure the peak expiratory flow rates before and after pesticide application and record upper and lower respiratory tract complaints among children. - Explore the relationship of these data with PM10 and PM2.5 levels. | PM2.5; PM10 | 2016 | SU | Turkey | Villages scattered over an area of 40 km2: Giyimli, Küçük Minareli, Büyük Minareli and Bozyazı. |
| (Barnig et al. 2012) | - Compare indoor endotoxin concentrations in air and dust samples from randomly selected dwellings in an urban and rural environment. | Dust; Endotoxin | - | W | France | - |
| (Blanes-vidal et al. 2012) | - Evaluate the association between NH3 concentrations measured and estimated at non-urban air quality monitoring stations, and prevalence of odor annoyance in local residential communities. - Study the link between seasonal patterns in NH3 measurements and public perception of the seasonal variation on odor pollution. | NH3 | - | W; SP; SU; F | Denimark | Areas with different degree of agricultural activity. |
| (Buczaj et al. 2014) | - Measurements of concentrations of asbestos fibres in ambient air in rural environment. | Asbestos fibres | 2009-2011 | W; SP; SU; F | Poland | Rural areas: Lublin and Wlodawa countries. |
| (Campos-Ramos and Arago 2009) | - Give insight and detailed of the analysis of the morphological characteristics and chemical composition of atmospheric particles at individual level. | PM10 | 2006-2007 | W; SP; SU; F | Mexico | Centro Universitario de Investigaciones en Ciencias del Ambiente (CUICA), located 12 km from the city of Colima, considered as a regional background station. |
| (de Assuncao et al. 2014) | - Measurements of concentrations of PAH and estimated the incremental cancer risk in Araraquara. | PAHs | 2009 | SU; F | Brazil | Three sites: one in the city center and two in rural localities. |
| (Green et al. 1990) | - Characterize, through chemical and mineralogical analysis, the airborne particulates at representative rural sites. - Assess their potential for toxic effects on humans, in terms of risk for pneumoconiosis in exposed farmer populations. | TSP | 1973-1979 | SP; SU | Canada | Six farm sites: south central region (Red Deer), representative of a predominantly grain growing district with minimal use of irrigation; six farm: south eastern region (Medicine Hat), which is mainly forage combined with irrigated crop production. |
| (Jimenez et al. 2006) | - Characterize the air quality in Pullman, WA during the fall 2002 prescribed field burning season. | PM2.5; PM10; CO2; NOx; OC; EC; Levoglucosan | 2002 | F | USA | One central monitoring site located on the roof of a building at Washington State University (WSU) in Pullman, WA (elevation = 770 m, approximately 12 m above street level). |
| (Le Blond et al. 2017) | - To estimate occupational exposure levels to PM10 during the commercial production of sugar from sugarcane. - Assess its potential impacts on human health. | PM10 | 2007-2008 | SU; F | Ecuador; Brazil | Commercial sugarcane-growing estates. |
| (Loftus et al. 2016) | - To investigate associations between PM2.5 and pediatric asthma exacerbations in an agricultural community of Washington State. | PM2.5 | 2010 | SU | USA | Area covering approximately 300 square miles and characterized by a high density of large-scale agricultural operations, including tree fruit orchards and dairy farms. |
| (Maesano et al. 2019) | - Determine the effects on respiratory health from exposure to PM of 4 sizes, TSP and 19 VOCs (from 5 VOC families), objectively measured in dwellings and workplaces, among farmers drawn from the Environmental Factors of Allergic and Respiratory Diseases study (FERMA), conducted in the Auvergne region of France. | UFP; PM1; PM2.5; PM10; TSP; VOCs | 2012 | W; SP | France | French farmers. |
| (Maw et al. 2002) | - Assess the losses of NOx from grass and maize silage, and their potential nutritional, atmospheric and health implications. | NO; NO2 | 1998-2000 | W; SP | - | n.a. |
| (Pavilonis et al. 2013) | - To quantify airborne concentrations of PM10, PM2.5, and endotoxin in an intensely agricultural area. - Compare findings with reported concentrations from urban areas. - Identify factors contributing to rural PM and endotoxin concentrations in both ambient and indoor air in homes. - Evaluate the effect of seasonal variation on PM and endotoxin levels. | PM2.5; PM10; Endotoxin | 2007-2011 | - | USA | Intensely agricultural area: the majority of the land area in the county was devoted to agricultural production (86%), with approximately 318,160 acres considered cropland, pastures, and trees. The primary crops grown in the county were corn and soybeans, accounting for 157 and 57 tonnes harvested in 2009, respectively. |
| (Ramli et al. 2021) | - Determining the compositions of trace elements and the morphological properties of fine particles. - Establish a causal connection between rice straw open burning and ambient air quality in Bagan Serai, Perak, Malaysia. - Investigate the chemical, biological, and morphological properties of fine particles from the emissions and the potential impacts on the air quality in the neighbouring areas. | PM2.5 | 2020 | F | Malaysia | n.a. |
| (Reinhardt et al. 2011) | - Characterize exposure to smoke among a rural population in Rondônia during the peak of the burning season. | CO; HCHO; PM3.5; Acrolein; Benzene | 1995 | SU | Brazil | Northwest of central Theobroma in an open agricultural research field. The nearest road was ~70 m away to the southwest, downwind of the predominant wind direction. |
| (Ryu et al. 2007) | - Investigate the chemical characteristics of biomass burning aerosol and its impact on regional air quality during an agricultural waste burning period in early summer in the rural areas of Korea. | PM2.5; PM10 | 2003 | SU | South Korea | Mobile laboratory trailer at the Gwangju Institute of Science and Technology, which is located in a northern suburb of Gwangju and surrounded by rural areas from north to east of the sampling site. |
| (Sevimoglu and Rogge 2016) | - Gain insight into differences between size-segregated airborne PM concentrations at Belle Glade and Delray Beach during an entire year with special emphasis on PM exposure levels during sugarcane harvesting and growing seasons. - Determine exposure levels of PAHs associated with different particle size fractions. | PM3; PM10; PAHs | 1996-1997 | W; SP; SU; F | USA | Delray Beach is located roughly 60 km east and 36 km south of Belle Glade. This sampling site is typically under the impact of traffic-related and industrial emissions. The major industries include electric power production, construction, aircraft testing, computer and electronics manufacturing, waste incineration, concrete and asphalt production. Belle Glade is located in the western part of Palm Beach County, where sugarcane growing is the major agricultural crop and the largest sugarcane-producing industry in the continental United States. |
| (Swanepoel et al. 2011) | - To determine whether farming activities on a clay soil farm generated time-weighted average respirable quartz concentrations above OELs - Test the hypothesis that the level of exposure to respirable quartz follows a declining gradient from sandy to sandy loam soils with less potential for high exposure from clay soils. | Respirable dust; Quartz | 2006-2009 | W; SP; SU; F | South Africa | Three farms located in the Free State and North West provinces of South Africa: (i) north western part of the Free-State province (being about 1500 ha in size); (ii) southern and eastern part of the North West province. |
| (Wei et al. 2019) | - Determine microbial community structure and abundance between biomass burning events and non-biomass burning events. - Understand the potential eco-physiological function of bacterial and fungal community. - Identify the main environmental variables in shaping community structure and abundance. | PM2.5; CO; SO2; O3; NOx; Inorganic aerosols (NH4+, NO3−, SO42−, Cl−); Mineral cations (Na+, K+, Ca2+, Mg2+) | 2014 | SU | China | Yucheng is located almost in the center of the North China Plain. The sampling site was inside the Yucheng Comprehensive Experiment Station of Chinese Academy of Sciences, which located about 50 km northwest of Jinan and downwind of Jinan in summer. The experimental field was in a highly intensive agricultural area surrounded by arable land and without major pollution sources nearby. |
| (Xing et al. 2020) | - Explain the concentration, composition, source and transmission of PM2.5-bound PAHs at rural, suburban, and urban in central China during summer harvest activity. - Clarifying the potential PAH sources through MDR and PCA–MLR. - Clustering trajectories by HYSPLIT model for potential pollution source areas. - Evaluate the human health risk of PAHs by ILCR model. | PM2.5; PAHs | 2017 | SU | China | Wuhan in the central of China, is a mega city. Pingdingshan and Suizhou located in north of Wuhan, are less developed city/county: (i) the sampling site in Wuhan is the roof of the 8th floor in Hubei Environmental Monitoring Center Station, adjoining Bayi Road. This location is approximately 25 m above the ground with intensive human activities of the large city; (ii) Pingdingshan site is situated on the roof of the residents' committees in Baofeng. This site is located in the central and western of Henan province; (iii) Suizhou site is located the north of Hubei province and is surrounded by hills with no industrial emissions. |
| (You et al. 2008) | - Describe the levels and variability of airborne concentrations for VOC. - Identify factors that determine airborne concentrations of VOC. - Produce statistical models that can be used to gain insight into the impact of major oil and gas facilities on the airborne VOC concentrations in the rural areas of Western Canada. | VOCs | 2001-2002 | W; SP; SU; F | Canada | The monitoring sites were located in the vicinity of approximately 33,000 beef cattle in some 200 herds selected across a geographic area associated with primary oil and gas industry in British Columbia, Alberta, and Saskatchewan. |
| (Zbieranowski and Aherne 2012) | - Assess the spatial and temporal variation in ambient atmospheric gaseous (NH3, NO2 and HNO3) concentrations. - Estimated dry deposition at four sites spanning a transect in major land use types across southern Ontario (intensive agricultural, urban - agricultural transition, urban, and rural background). | NH3; NO2; HNO3 | 2010-2011 | W; SP; SU; F | Belgium; Canada | (i) The intensive agricultural site was located at the road side beside a farm field where cattle occasionally grazed; (ii) he high-density urban site (TOR) was located +/- 250m south of Highway 401 at the Environmental Monitoring and Reporting Branch of the MOE in Etobicoke, part of the Greater Toronto Area; the sample site was located in a large asphalt parking lot surrounded by office buildings, directly adjacent to Highway 401 (to the north) which has an annual average daily traffic volume of 416,400 vehicles day -1; a parkland and a golf course were located to the south; (iii) the urban - agricultural transition site (EGB) was located in Egbert (+/- 80 km north of Toronto) at Environment Canada’s (EC) ‘Centre for Atmospheric Research Experiments’. The sampler site was located on the roof-top deck of the clean air building facility, surrounded by unenclosed grass fields in an agricultural and forested area. The rural background site (DOR) was located at the Dorset Environmental Science Centre. The sampler site was located within a large gated sampling area in an open field surrounded by dense woodland. |
| (Zhuo et al. 2018) | - Develop a large number of agricultural nonpoint source emission scenarios of various sources based on fixed position experiments or surveys to characterize the diversity of agricultural and environmental conditions of China’s arable. systems. - Analyze the status and spatial variation of agricultural nonpoint source pollution loads of nutrient, pesticide, plastic film residue, and crop straw in situ burning. - Analyze the spatial correlation between different agricultural nonpoint source pollution loads using data of land units. | Nutrient loss to water; Pesticide; Agricultural film residual; Straw in situ burning | 2007-2008 | - | China | n.a. |

Table SM2. Summary of the instruments and analyses adopted in the different studies considered in this review. S/M: Sampling/Monitoring; E: Environmental; P: Personal; M: Models. AES: Atomic Emission Spectrometry; FESEM-EDX: Field Emission Scanning Electron Microscopy with Energy Dispersive X-Ray Spectroscopy; HPLC: High Performance Liquid Chromatography; ICP: Inductively Coupled Plasma; ICP-MS: Mass Spectrometry; ICP-OES: Plasma Emission Spectroscopy; SEM-EDS: Scanning Electron Microscopy; XRD: X-Ray Diffraction.

| Reference | N | Pollutant | Type of S/M | Instruments | Sampling substrate | Collection time | Analyses |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (Maesano et al. 2019) | 109 French farmers. | UFP | E | TSI P-track sensor | - | 2 min | - |
| (Maesano et al. 2019) | 109 French farmers. | PM1 | E | AEROCET 531S | - | 2 min | - |
| (Swanepoel et al. 2011) | 298 respirable dust and respirable quartz measurements. | Respirable dust | P | Higgins–Dewell cyclone | 25 mm polyvinyl chloride filters | 8 h | Gravimetric analyses. |
| (Ali-Sak et al. 2021) | 266 volunteers recruited from 4 different villages; the measurements were repeated for 72 of 266 children after pesticide application. | PM2.5 | E | pDR-1500 cyclones (Personal DataRAM Pdr, Thermo Scientific, USA) | - | - | - |
| (Jimenez et al. 2006) | 1872 observations (TEOM-PM2.5); 2788 observations (Neph-PM2.5) |  | E | TEOM monitors (Series 1400a, Thermo Electron Co) | 37-mm Teflon filters (Harvard Impactor) | 30 min; 10 min; 12 h | The PM collected on the Teflon media was analyzed gravimetrically (Mettler-Toledo UMT2 microbalance) |
|  |  |  |  | Light scattering nephelometer (M903, Radiance Research, Seattle, WA) | Quartz filter (Harvard Impactor) |  |  |
|  |  |  |  | Harvard Impactors (Air Diagnostics Inc., Naples, ME) |  |  |  |
|  |  |  |  | DataRAM (Thermo- Andersen, Smyrna, GA) |  |  |  |
| (Loftus et al. 2016) | - |  | E | Nephelometer | - | 24 h | - |
| (Maesano et al. 2019) | 109 French farmers. |  | E | AEROCET 531S | - | 2 min | - |
| (Pavilonis et al. 2013) | 197 rural households. |  | E | Personal Environmental Monitors (SKC, Eighty Four, PA) | 37 mm polytetrafluoroethylene filters with a 0.8-mm pore size (Pall Corporation, Ann Arbor, MI) | 30 min | - |
| (Ryu et al. 2007) | 26 samples. |  | E | URG Versatile Air Pollutant Sampler (VAPS, URG-3000K) with a PM10 cyclone inlet and a Dichotomous PM10 sampler (ASI/GMW series 241) | Pre-baked quartz fiber filters (for carbonaceous species) | 12 h | Gravimetric method using an electronic microbalance |
|  |  |  |  |  | Teflon filters (for mass, ionic and elemental species analyses) |  | HPLC and IC |
|  |  |  |  |  |  |  | ICP – AES |
|  |  |  |  |  |  |  | ICP-MS |
| (Ramli et al. 2021) | 2 sites. |  | E | E-BAM (Met One Instrument Inc., Grants Pass, OR, USA) | Glass-fiber filter tape with a pore size of 2 µm | 12 h | Analysis of: morphology, size of particles, and elemental compositions of individual particles using high-resolution field emission scanning electron microscopy coupled with FESEM-EDX (Quanta FEG 650, Oxford Instrument, Abingdon, UK) |
|  |  |  |  |  |  |  | ICP-OES identifies the concentration of trace elements in the fine particle samples |
| (Wei et al. 2019) | - |  | E | Aerosol sampler (TH-150C-III, Wuhan Tianhong Instrument Co., Ltd., China) | 0.45 μm pore size Teflon filters (PALL Life Sciences) | 23 h | - |
|  |  |  |  | PM2.5 monitor (SHARP 5030, Thermo Fisher Scientific, Massachusetts, USA) |  |  |  |
| (Xing et al. 2020) | 11, 13, and 14 samples were collected from the 3 sites. |  | E | TH-150C intelligent medium flow air total suspended particle sampler (Tianhong Co. Ltd, China) | Quartz fiber filter membrane | 23 h | Samples were weighed by a microbalance |
| (Sevimoglu and Rogge 2016) | 55 samples from Belle Gladeand; 61 samples from Delray Beach. | PM3 | E | Samples were collected using a Graseby high volume sampler (Graseby Andersen, Atlanta, USA) with a PM10 selective inlet (G1200). The high volume cascade impactor (Graseby/GWM, series 235 with five impactor stages plus a backup stage) was added to the sampler | Slotted quartz fiber filters (SAC230QF) and backup quartz fiber filter (2500QAO-UP) were used for size-segregated particle sampling for the analysis of the organic compounds, including PAHs | 23 h | - |
| (Reinhardt et al. 2011) | - | PM3.5 | P | Nylon Dorr-Oliver cyclone (3.5-μm cut-point) | 2.0-μm pore size 37-mm diameter Teflon (DuPont) filter | - | - |
| (Ali-Sak et al. 2021) | 266 volunteers recruited from 4 different villages; the measurements were repeated for 72 of 266 children after pesticide application. | PM10 | E | pDR-1500 cyclones (Personal DataRAM Pdr, Thermo Scientific, USA) | - | - | - |
| (Campos-ramos and Arago 2009) | 96 daily samples; 30 samples were randomly selected according to the distribution of PM10; 3600 particles were analyzed by SEM-EDS. |  | E | PM10 high volume Andersen | Quartz fiber filters | 24 h | SEM Phillips XL30 model coupled with EDS microanalysis EDAX DX4 |
|  |  |  |  |  |  |  | XRD (Rigaku D model MAX-2200 diffractometer) |
| (Jimenez et al. 2006) | 2684 observations (TEOM-PM10) |  | E | TEOM monitors (Series 1400a, Thermo Electron Co) | - | 30 min; 10 min; 12 h | - |
|  |  |  |  | Light scattering nephelometer (M903, Radiance Research, Seattle, WA) |  |  |  |
| (Le Blond et al. 2017) | - |  | E + P | DustTrak™ aerosol monitor (Model 8520, TSI Inc.) | Polycarbonate impaction filters (Millipore Isopore™ polycarbonate filters, 25 mm diameter, 0.4 mm pore size) | 1 min | The elemental composition and morphology of the airborne PM were investigated in a LEO VP1455 SEM with X-ray elemental analysis (Oxford INCA system), in high vacuum mode and backscattered electron (BSE) mode. Images were taken on a Philips FEG SEM in secondary electron mode (WD ¼ 8 mm, EHT 10 kV) |
|  |  |  |  | Sioutas cascade impactor |  |  |  |
| (Maesano et al. 2019) | 109 French farmers. |  | E | AEROCET 531S | - | 2 min | - |
| (Pavilonis et al. 2013) | 197 rural households. |  | E | Personal Environmental Monitors (SKC, Eighty Four, PA) | 37 mm polytetrafluoroethylene filters with a 0.8-mm pore size (Pall Corporation, Ann Arbor, MI) | 30 min | - |
| (Ryu et al. 2007) | 26 samples. |  | E | URG Versatile Air Pollutant Sampler (VAPS, URG-3000K) with a PM10 cyclone inlet and a Dichotomous PM10 sampler (ASI/GMW series 241) | Pre-baked quartz fiber filters (for carbonaceous species) and on Teflon filters (for mass, ionic and elemental species analyses) | 12 h | Gravimetric method using an electronic microbalance |
|  |  |  |  |  |  |  | HPLC and IC |
|  |  |  |  |  |  |  | ICP – AES |
|  |  |  |  |  |  |  | ICP-MS |
| (Sevimoglu and Rogge 2016) | 55 samples from Belle Gladeand; 61 samples from Delray Beach. |  | E | Samples were collected using a Graseby high volume sampler (Graseby Andersen, Atlanta, USA) with a PM10 selective inlet (G1200). The high volume cascade impactor (Graseby/GWM, series 235 with five impactor stages plus a backup stage) was added to the sampler | Slotted glass fiber filters (SAC 230GF) and backup glass fiber filter (Graseby GMW P/N-810) were used for the size-segregated particle sampling. Slotted quartz fiber filters (SAC230QF) and backup quartz fiber filter (2500QAO-UP) were used for size-segregated particle sampling for the analysis of the organic compounds, including PAHs | 23 h | - |
| (Green et al. 1990) | 6 farm sites in the cereal-grain district; 6 farm sites in the forage-crop district. | TSP | E | Bulk samples from tractor air filters were resuspended in an aerosol chamber and analyzed for particle size-distribution using a 6-stage cascade impactor (Sierra Source Cascade Impactor, Model 226) | - | - | SEM and STEM to calculate the particles' aspect ratio and to characterize the particles into groups based upon their major elemental components and net fractional X-ray intensities |
|  |  |  |  |  |  |  | Analysis of bulk dusts (in terms of fibres) by STEM, for the identification of their fibrous morphology and aspect ratios) in combination with energy dispersive X-ray analysis |
|  |  |  |  |  |  |  | Qualitative and semi-quantitative mineralogical analysis using XRD |
|  |  |  |  |  |  |  | Quantitative analysis of quartz by infrared absorption spectro-photometry after ashing in a plasma furnace |
|  |  |  |  |  |  |  | Combustible mass fraction and water soluble fraction of bulk dusts determined using high temperature plasma ashing and water suspension filtration respectively |
| (Maesano et al. 2019) | 109 French farmers. |  | E | AEROCET 531S | - | 2 min | - |
| (Barnig et al. 2012) | 100 dwellings and 50 farmhouses (50 rural nonfarming references). | Dust | E + P | IOM cassette (M 00003700, Millipore, St Quentin, France) associated with a portable Gilair pump (Sensidyne®, Clearwater, FL, USA) | Glass fibre filter (AP 4003705, Millipore) | 8 h | Endotoxins extraction from 100 mg dust in 2 ml of PBS-LAL (Biomerieux, Craponne, France) |
| (Buczaj et al. 2014) | 3 farms; 3 samples on each were simultaneously collected at 3 measuring sites; 216 samples were collected during the monitoring period. | Asbestos fibres | E | Stationary aspiratoris (JHS 16,000) | Cellulose ester membrane filter of 25mm diameter with pores of 0,8um size | 60-80 min | Optical phase contrast microscope with a green filter |
| (Swanepoel et al. 2011) | 298 respirable dust and respirable quartz measurements. | Quartz | P | - | - | 8 h | X-ray diffraction (XRD) as specified in the HSE MDHS 101 method (HSE, 2005) |
| (Wei et al. 2019) | - | CO | E | CO analyzer (Thermo Scientific, model 48C) | - | 23 h | - |
| (Reinhardt et al. 2011) | - |  | E | Inert gas sampling bag | - | - | - |
| (Jimenez et al. 2006) | 2842 observations | CO2 | E | CO2 analyzer (Telaire 1050 Engelhard, Goleta, CA) | - | 10 min | - |
| (Maw et al. 2002) | 4 farms + 1 background. | NO | E | Chemiluminescence analysis (Advanced Pollution Instruments, U.S.A.) | - | - | - |
| (Maw et al. 2002) | 4 farms + 1 background. | NO2 | E | Chemiluminescence analysis (Advanced Pollution Instruments, U.S.A.) | - | - | - |
| (Zbieranowski and Aherne 2012) | - |  | E | Willems badge passive sampler | - | - | UVeVIS Spectrometer (Perkin Elmer Lambda XLS) at a wavelength of 540 nm |
| (Jimenez et al. 2006) | 2857 observations | NOx | E | NOx analyzer (model 42 Thermal Environmental Instruments, Inc.) | - | 10 min | - |
| (Wei et al. 2019) | - |  | E |  | - | 23 h | - |
| (Wei et al. 2019) | - | O3 | E | O3 analyzer (Thermo Scientific, model 49C) | - | 23 h | - |
| (Wei et al. 2019) | - | SO2 | E | SO2 analyzer (Thermo Scientific, model 43C) | - | 23 h | - |
| (Reinhardt et al. 2011) | - | HCHO | P | Chemically treated Sep-Pak sorbent tube | - | - | - |
| (Zbieranowski and Aherne 2012) | - | HNO3 | E | Willems badge passive sampler | - | - | Dionex ICS-1100 Ion Chromatograph in a carbonate eluent through an IonPac AS22 analytical column |
| (Zbieranowski and Aherne 2012) | - | NH3 | E | Willems badge passive sampler | - | - | UVeVIS Spectrometer (PerkineElmer Lambda XLS) at a wavelength of 655 nm |
| (Blanes-vidal et al. 2012) | 5 regions; 470 household. 180 subjects agreed to partecipate. |  | E + M | Semiautomatic filter pack samplers to measure NH3 concentrations | A series of three filters: (i) a particle filter (mixed esters of cellulose MF-Millipore RA, l 2/an, 50 mm); (ii) two filters Whatman 41 (50 mm), one of them impregnated with oxalic acid for NH3 collection | - | Ammonia absorbed on the oxalic acid filter was analyzed as NH4 + according to the continuous flow analysis method described in DS/EN ISO 11732 |
| (Wei et al. 2019) | - | The inorganic aerosols (NH4+, NO3−, SO42−, Cl−, and mineral cations Na+, K+, Ca2+, Mg2+) | E | ADI 2080 online analyzer for Monitoring for AeRosols and GAses (MARGA, Applikon Analytical B.V., the Netherlands) | - | 23 h | - |
| (de Assuncao et al. 2014) | 3 sites (one in the city center and two in rural localities). | PAHs | E | PS1 polyurethane foam (PUF) sampler (Andersen Instruments Inc.; Smyrna, GA, USA), in accordance with EPA method TO-13A (EPA, 1999) | 10.16 cm-diameter quartz microfiber filters (Whatman Inc.; Clifton, NJ, USA), followed by a 6 cm diameter and 7.62 cm length PUF sampler (Tisch Environmental, Inc.; Cleveland, OH, USA) | 24 h | The chemical analyses were performed by gas chromatography–mass spectrometry (GC/MS) for the 16 most toxic PAHs |
| (Sevimoglu and Rogge 2016) | 55 samples from Belle Gladeand; 61 samples from Delray Beach. |  | E | Samples were collected using a Graseby high volume sampler (Graseby Andersen, Atlanta, USA) with a PM10 selective inlet (G1200). The high volume cascade impactor (Graseby/GWM, series 235 with five impactor stages plus a backup stage) was added to the sampler | Slotted glass fiber filters (SAC 230GF) and backup glass fiber filter (Graseby GMW P/N-810) were used for the size-segregated particle sampling. Slotted quartz fiber filters (SAC230QF) and backup quartz fiber filter (2500QAO-UP) were used for size-segregated particle sampling for the analysis of the organic compounds, including PAHs | 23 h | - |
| (Xing et al. 2020) | 11, 13, and 14 samples were collected from the 3 sites. |  | E | - | - | 23 h | The priority list of 16 PAHs recommended by the United States Environmental Protection Agency (US EPA) are detected by gas chromatography–mass by using a spectrometric capillary column |
| (You et al. 2008) | 11,399 air samples (13% of them were replicates); 145 - 1031 samples per month. | VOC | E | Teflon membrane impregnated with charcoal | - | - | VOC were extracted from the sampling media using a carbon disulfide solution spiked with deuterated standards. The extracts were then injected into a gas chromatograph equipped with a mass spectrometer operated in selected ion-monitoring mode that detected three ions per compound. |
| (Maesano et al. 2019) | 109 French farmers. |  | E | Radiello 165 cartridge absorbent with 2,4-dinitrophenylhydrazine (2,4-DNPH) coated Florisil® was used to assess aldehydes | - | 2 min | BTEX and VOCs were extracted through thermodesorption and analyzed by gas phase chromatography equipped with flame ionization detection and/or mass spectrometry |
|  |  |  |  | Radiello 145 cartridge absorbent was used to assess BTEX (benzene, toluene, xylene) and other VOCs |  |  |  |
| (Reinhardt et al. 2011) | - | Benzene | P | Charcoal sorbent tube | - | - | - |
| (Reinhardt et al. 2011) | - | Acrolein | P | Chemically treated Sep-Pak sorbent tube | - | - | - |
| (Jimenez et al. 2006) | - | EC | E | - | - | - | Sections of the quartz filters were analyzed via Thermal Optical Transmittance (Sunset Laboratory, Inc. Tigard, OR) using a modified version of the NIOSH 5040 method |
| (Jimenez et al. 2006) | - | OC | E | - | - | - | Sections of the quartz filters were analyzed via Thermal Optical Transmittance (Sunset Laboratory, Inc. Tigard, OR) using a modified version of the NIOSH 5040 method |
| (Barnig et al. 2012) | 100 dwellings and 50 farmhouses (50 rural nonfarming references). | Endotoxin | E + P | - | - | 8 h | Endotoxins extraction from 100 mg dust in 2 ml of PBS-LAL (Biomerieux, Craponne, France). |
| (Pavilonis et al. 2013) | 197 rural households. |  | E | - | - | 30 min | Endotoxin extracted from the filters was evaluated using the kinetic chromogenic Limulus Amebocyte Lysate assay |

Table SM3. Summary of the additional information acquired by the authors (cultivation type; activity performed; presence and type of farms; use of pesticides and fertilizers).

| Reference | Cultivation type | Activity | Farm | Use of pesticides and fertilizers |
| --- | --- | --- | --- | --- |
| (Ali-Sak et al. 2021) | Cotton | Application of pesticides | - | Yes |
| (Barnig et al. 2012) | - | - | Swineries | - |
| (Blanes-vidal et al. 2012) | - | - | Swine; Cattle | - |
| (Buczaj et al. 2014) | - | - | - | - |
| (Campos-ramos and Arago 2009) | Sugarcane | - | - | - |
| (de Assuncao et al. 2014) | Sugarcane | Agricultural burning | - | - |
| (Green et al. 1990) | Barley; Wheat; Oats; Forage (alfalfa); Potatoes; Corn; Canola | Harvesting | - | Yes |
| (Jimenez et al. 2006) | - | Agricultural burning | - | - |
| (Le Blond et al. 2017) | Sugarcane | Harvesting | - | - |
| (Loftus et al. 2016) | Tree fruit orchards | - | Cattle | - |
| (Maesano et al. 2019) | - | - | - | - |
| (Maw et al. 2002) | Grass; Maize | Silages | - | - |
| (Pavilonis et al. 2013) | - | - | - | - |
| (Ramli et al. 2021) | - | Agricultural burning | - | - |
| (Reinhardt et al. 2011) | - | Agricultural burning | - | - |
| (Ryu et al. 2007) | Barley | Agricultural burning | - | Yes |
| (Sevimoglu and Rogge 2016) | Sugarcane | Agricultural burning | - | - |
| (Swanepoel et al. 2011) | Maize; Sunflowers | Planting; Harvesting | - | - |
| (Wei et al. 2019) | Wheat | Agricultural burning | - | - |
| (Xing et al. 2020) | n.a. | - | - | - |
| (You et al. 2008) | - | - | Cow | Yes |
| (Zbieranowski and Aherne 2012) | - | - | - | - |
| (Zhuo et al.(Zhuo et al. 2018) 2018) | - | - | - | Yes |

Figure SM1. Spatial distribution of the evaluated papers. The number of studies is given by the grayscale.

Acknowledgments

This study was performed within the framework of the D-DUST project (Data-driven moDelling of particUlate with Satellite Technology aid).

This work has been supported by Fondazione Cariplo, grant n° 2020-4022.

References

Ali-Sak, Z. H., Kurtuluş, Ş., Ocaklı, B., Töreyin, Z. N., Bayhan, İ., Yeşilnacar, M. İ., et al. (2021). The relationship between particulate matter and childhood respiratory complaints  and peak expiratory flows in Harran agricultural area. The Turkish journal of pediatrics, 63(2), 263–272. https://doi.org/10.24953/turkjped.2021.02.010

Barnig, C., Reboux, G., Roussel, S., Casset, A., Sohy, C., Dalphin, J., & Blay, F. De. (2012). Indoor dust and air concentrations of endotoxin in urban and rural environments, 161–167. https://doi.org/10.1111/lam.12024

Blanes-vidal, V., Nadimi, E. S., Ellermann, T., Andersen, H. V, & Løfstrøm, P. (2012). Perceived annoyance from environmental odors and association with atmospheric ammonia levels in non-urban residential communities : a cross-sectional study, 1–10.

Buczaj, A., Brzana, W., Tarasińska, J., Buczaj, M., & Choina, P. (2014). Study on the concentration of airbone respirable asbestos fibres in rural areas of the Lublin region in south-east Poland, 21(3), 639–643. https://doi.org/10.5604/12321966.1120617

Campos-Ramos, A., & Arago, A. (2009). Characterization of atmospheric aerosols by SEM in a rural area in the western ´ xico and its relation with different pollution sources part of Me, 43, 6159–6167. https://doi.org/10.1016/j.atmosenv.2009.09.004

de Assuncao, J. V, Pesquero, C. R., Nardocci, A. C., Francisco, A. P., Soares, N. S., & Ribeiro, H. (2014). Airborne polycyclic aromatic hydrocarbons in a medium-sized city affected by preharvest sugarcane burning and inhalation risk for human health. Journal of the Air & Waste Management Association, 64(10), 1130–1139. https://doi.org/10.1080/10962247.2014.928242

Green, F. H. Y., Yoshida, K., Fick, G., Paul, J., Hugh, A., & Green, W. F. (1990). Oeupatinfa l Environmental Health Characterization of airborne mineral dusts associated with farming activities in rural Alberta , Canada, (Mumpton 1978), 423–430.

Jimenez, J., Wu, C., Claiborn, C., Gould, T., Simpson, C. D., Larson, T., & Liu, L. S. (2006). Agricultural burning smoke in eastern Washington — part I : Atmospheric characterization, 40, 639–650. https://doi.org/10.1016/j.atmosenv.2005.09.071

Le Blond, J. S., Woskie, S., Horwell, C. J., & Williamson, B. J. (2017). Particulate matter produced during commercial sugarcane harvesting and processing: A respiratory health hazard? Atmospheric Environment, 149, 34–46. https://doi.org/https://doi.org/10.1016/j.atmosenv.2016.11.012

Loftus, C., Yost, M., Sampson, P., Torres, E., Arias, G., Vasquez, V. B., et al. (2016). HHS Public Access, i(6), 794–801. https://doi.org/10.1097/EDE.0000000000000368.Ambient

Maesano, C. N., Caillaud, D., Youssouf, H., Banerjee, S., Homme, J. P., Audi, C., et al. (2019). Indoor exposure to particulate matter and volatile organic compounds in dwellings and workplaces and respiratory health in French farmers, 3, 1–12.

Maw, S. J., Johnson, C. L., Lewis, A. C., & McQuaid, J. B. (2002). A note on the emission of nitrogen oxides from silage in opened bunker silos. Environmental monitoring and assessment, 74(3), 209–215. https://doi.org/10.1023/a:1014227504369

Pavilonis, B. T., Anthony, T. R., Shaughnessy, P. T. O., Humann, M. J., Merchant, J. A., Moore, G., et al. (2013). Indoor and outdoor particulate matter and endotoxin concentrations in an intensely agricultural county, (January), 299–305. https://doi.org/10.1038/jes.2012.123

Ramli, N. A., Faizah, N., Yusof, F., Zarkasi, K. Z., & Suroto, A. (2021). Chemical , Biological and Morphological Properties of Fine Particles during Local Rice Straw Burning Activities.

Reinhardt, T. E., Ottmar, R. D., Castilla, C., Reinhardt, T. E., Ottmar, R. D., Smoke, C. C., et al. (2011). Smoke Impacts from Agricultural Burning in a Rural Brazilian Town Smoke Impacts from Agricultural Burning in a Rural Brazilian Town, 2247. https://doi.org/10.1080/10473289.2001.10464280

Ryu, S. Y., Kwon, B. G., Kim, Y. J., Kim, H. H., & Chun, K. J. (2007). Characteristics of biomass burning aerosol and its impact on regional air quality in the summer of 2003 at Gwangju , Korea, 84, 362–373. https://doi.org/10.1016/j.atmosres.2006.09.007

Sevimoglu, O., & Rogge, W. F. (2016). Particuology Seasonal size-segregated PM 10 and PAH concentrations in a rural area of sugarcane agriculture versus a coastal urban area in Southeastern. Particuology, 28, 52–59. https://doi.org/10.1016/j.partic.2015.09.013

Swanepoel, A. J., Kromhout, H., Portengen, T., Renton, K., Jinnah, Z. A., Gardiner, K., & Rees, D. (2011). Respirable Dust and Quartz Exposure from Three South African Farms with Sandy , Sandy Loam , and Clay Soils, 55(6), 634–643. https://doi.org/10.1093/annhyg/mer025

Wei, M., Xu, C., Xu, X., Zhu, C., Li, J., & Lv, G. (2019). Science of the Total Environment Characteristics of atmospheric bacterial and fungal communities in PM 2 . 5 following biomass burning disturbance in a rural area of North China Plain. Science of the Total Environment, 651, 2727–2739. https://doi.org/10.1016/j.scitotenv.2018.09.399

Xing, X., Chen, Z., Tian, Q., Mao, Y., Liu, W., & Shi, M. (2020). Ecotoxicology and Environmental Safety Characterization and source identi fi cation of PM 2 . 5 -bound polycyclic aromatic hydrocarbons in urban , suburban , and rural ambient air , central China during summer harvest. Ecotoxicology and Environmental Safety, 191(October 2019), 110219. https://doi.org/10.1016/j.ecoenv.2020.110219

You, X. I., Senthilselvan, A., Cherry, N. M., & Kim, H. (2008). Determinants of airborne concentrations of volatile organic compounds in rural areas of Western Canada Determinants of airborne concentrations of volatile organic compounds in rural areas of Western Canada, (November 2016). https://doi.org/10.1038/sj.jes.7500556

Zbieranowski, A. L., & Aherne, J. (2012). Ambient concentrations of atmospheric ammonia , nitrogen dioxide and nitric acid across a rural e urban e agricultural transect in southern Ontario , Canada. Atmospheric Environment, 62, 481–491. https://doi.org/10.1016/j.atmosenv.2012.08.040

Zhuo, D., Liu, L., Yu, H., & Yuan, C. (2018). RESEARCH ARTICLE A national assessment of the effect of intensive agro-land use practices on nonpoint source pollution using emission scenarios and geo-spatial data, (2), 1683–1705. https://doi.org/10.1007/s11356-017-0118-8