Table 1 Shaanxi Province 2017-2020 CEMS key monitoring enterprise classification

|  | Power generation | Chemical industry | Building materials | Coking | Thermal | Cement | Ceramics | Metallurgy | Other | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2017 | 56 | 54 | 8 | 6 | 34 | 31 | 19 | 38 | 50 | 296 |
| 2018 | 60 | 59 | 11 | 6 | 41 | 31 | 22 | 42 | 68 | 340 |
| 2019 | 65 | 65 | 13 | 6 | 52 | 33 | 23 | 44 | 85 | 386 |
| 2020 | 69 | 69 | 24 | 7 | 47 | 31 | 26 | 46 | 86 | 407 |

Table 2 The annual average EFs of air pollutant from China’s plants in the national survey manuals and different studies

|  | Pollutant | Emission factors (kg/t) | Year | Reference |
| --- | --- | --- | --- | --- |
| Power Generation | SO2 | 0.14 | 2017 | This study |
|  |  | 0.31 | 2010 | (Wang et al., 2022) |
|  | NOx | 0.32 | 2017 | This study |
|  |  | 0.52 | 2010 | (Wang et al., 2022) |
|  | PM | 0.22 | 2017 | This study |
|  |  | 0.61 | 2010 | (Wang et al., 2022) |
| Metallurgical | SO2 | 0.032 | 2017 | This study |
|  |  | 0.028 | 2015 | (Guo et al., 2017) |
|  |  | 2.4 | 2012 | (Wu et al., 2015) |
|  | NOx | 0.050 | 2017 | This study |
|  |  | 0.068 | 2015 | (Guo et al., 2017) |
|  |  | 0.522 | 2012 | (Wu et al., 2015) |
|  | PM | 0.008 | 2017 | This study |
|  |  | 1.0 | 2015 | (Guo et al., 2017) |
|  |  | 0.402 | 2012 | (Wu et al., 2015) |