Table 1. Geology and mining of Jajpur district

| Platinum | Chromite |  | High Magnesia | Iron Ore | Nickel Ore |
| --- | --- | --- | --- | --- | --- |
| Associated with Singhbhum- Orissa crato Comprising high-grade schist and gneiss intruded by layered mafics and ultramafics |  | Occur as differentiated layered igneous complexes Ultramafic Complex | Dunite, Peridotite, serpentinite, and pyroxenites, mostly associated with the iron supergroup | Iron ore bands occur in layered BIF along with volcano-sedimentary rock piles | Occurs as nickeliferous Literite overlaying the serpentinised dunite peridotite bed rock |
| Location: Sukinda Valley |  | Location: Sukinda Valley | Location: Sukinda Valley | Location: Daitary- Tomka | Location: Sukinda Valley |

Table 2. Water quality standard and hydrochemical parameter weight (GWQI model).

| Parameters | Water quality standard a | wj (GWQI)b |
| --- | --- | --- |
| pH | 6.5 | 4 |
| TDS (mg/L) | 500 | 5 |
| EC (µS/cm) | 2250 | 4 |
| DO (mg/L) | 5 | 2 |
| F- (mg/L) | 1 | 5 |
| Cl- (mg/L) | 250 | 1 |
| NO3- (mg/L) | 45 | 5 |
| SO42- (mg/L) | 200 | 3 |
| PO43- (mg/L) | 10 | 1 |
| U (µg/L) | 60 | 5 |
| Total hardness (mg/L) | 200 | 3 |
| Ca hardness (mg/L) | 75 | 2 |
| Mg hardness (mg/L) | 30 | 2 |
| Bicarbonate (mg/L) | 200 | 3 |

a from (BIS, 2012), b adopted from Kawo and Karuppannan (2018)

Table 3. Classification of water quality parameters

| S. No. | WQI Range | Rank | Water class |
| --- | --- | --- | --- |
| 1 | <50 | 1 | Excellent |
| 2 | 50-100 | 2 | Good |
| 3 | 100-150 | 3 | Medium |
| 4 | 150-200 | 4 | Poor |
| 5 | >200 | 5 | Extremely Poor |

Table 4. Physico-chemical parameter statistics for the groundwater samples of Jajpur district.

| Parameters | Min | Max | Mean | Std dev | Skewness | Kurtosis | BIS / WHO limits |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pH | 5.54 | 8.52 | 6.8 | 0.53 | -0.061 | 4.10 | 6.5-8.5 |
| TDS (mg/L) | 15.57 | 1314 | 267.1 | 226.58 | 2.028 | 8.5 | 1000 |
| EC (µS/cm) | 2.57 | 1890 | 494.29 | 370.46 | 1.388 | 5.13 |  |
| DO (mg/L) | 1.59 | 7.44 | 4.96 | 1.36 | -0.347 | 3.00 |  |
| F- (mg/L) | 0.048 | 1.62 | 0.77 | 0.374 | 0.167 | 2.22 | 1 |
| Chloride (mg/L) | 0 | 267.8 | 66.81 | 52.56 | 1.695 | 6.40 | 250 |
| NO3- (mg/L) | 3.31 | 47.19 | 25.57 | 11.546 | 0.042 | 2.548 | 45 |
| SO42- (mg/L) | 0 | 96.56 | 19.80 | 22.29 | 1.80 | 5.96 | 200 |
| PO4 3- (mg/L) | 0 | 2.38 | 0.653 | 0.541 | 1.028 | 3.954 |  |
| U (µg/L) | 0 | 10.5 | 0.474 | 1.64 | 4.29 | 22.77 | 60 µg/L AERB |
| Total hardness (mg/L) | 40 | 200 | 87.68 | 40.12 | 0.745 | 2.662 | 200 |
| Ca hardness (mg/L) | 0 | 140 | 47.26 | 30.1 | 1.100 | 4.109 | 75 |
| Mg hardness (mg/L) | 0 | 120 | 39.52 | 27.40 | 0.785 | 2.979 | 30 |
| Bicarbonate (mg/L) | 20 | 400 | 134.61 | 83.00 | 0.9337 | 3.61 | 200 |

Table 5. Correlation matrix of Jajpur district water quality data. significant correlations (r>0.5) are indicated in boldface.

|  | pH | TDS (mg/L) | EC (µS/cm) | DO (mg/L) | F- (mg/L) | Cl- (mg/L) | NO3- (mg/L) | SO42- (mg/L) | PO43- (mg/L) | U (µg/L) | Total hardness (mg/L) | Ca+2 hardness | Mg+2 hardness | HCO3- (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pH | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDS (mg/L) | 0.316 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| EC (µS/cm) | 0.440 | 0.706 | 1 |  |  |  |  |  |  |  |  |  |  |  |
| DO (mg/L) | -0.159 | -0.424 | -0.241 | 1 |  |  |  |  |  |  |  |  |  |  |
| F- (mg/L) | 0.072 | -0.034 | 0.125 | 0.008 | 1 |  |  |  |  |  |  |  |  |  |
| Cl- (mg/L) | 0.271 | 0.524 | 0.657 | -0.221 | -0.109 | 1 |  |  |  |  |  |  |  |  |
| NO3- (mg/L) | 0.159 | 0.292 | 0.467 | -0.248 | -0.061 | 0.438 | 1 |  |  |  |  |  |  |  |
| SO42- (mg/L) | 0.350 | 0.646 | 0.732 | -0.309 | -0.081 | 0.638 | 0.442 | 1 |  |  |  |  |  |  |
| PO43- (mg/L) | 0.061 | 0.204 | 0.172 | -0.303 | -0.049 | 0.197 | 0.199 | 0.240 | 1 |  |  |  |  |  |
| U (µg/L) | 0.118 | 0.067 | 0.091 | -0.204 | -0.078 | -0.048 | 0.232 | 0.180 | 0.099 | 1 |  |  |  |  |
| Total hardness (mg/L) | 0.085 | 0.460 | 0.516 | -0.203 | 0.100 | 0.201 | 0.377 | 0.342 | 0.243 | 0.168 | 1 |  |  |  |
| Ca+2 hardness | 0.029 | 0.306 | 0.391 | -0.083 | 0.085 | 0.107 | 0.212 | 0.263 | 0.106 | 0.212 | 0.737 | 1 |  |  |
| Mg+2 hardness | 0.115 | 0.333 | 0.317 | -0.222 | 0.076 | 0.203 | 0.304 | 0.212 | 0.254 | 0.022 | 0.645 | -0.019 | 1 |  |
| Bicarbonate (mg/L) | 0.154 | 0.472 | 0.611 | -0.229 | 0.171 | 0.344 | 0.473 | 0.410 | 0.301 | 0.104 | 0.466 | 0.410 | 0.234 | 1 |

Table 6. Principal component loading and rotated component loading for water quality parameters.

|  | Component loading |  |  |  |  |  | Rotated component loading |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Eigenvalue | 4.872 | 1.551 | 1.257 | 1.168 | 1.023 |  | 3.565 | 2.159 | 1.781 | 1.220 | 1.146 |
| Rel. Eigen | 0.494 | 0.157 | 0.127 | 0.118 | 0.104 |  | 0.361 | 0.219 | 0.180 | 0.124 | 0.116 |
| Cumulative (%) | 34.797 | 11.081 | 8.981 | 8.341 | 7.306 |  | 25.462 | 15.418 | 12.722 | 8.717 | 8.188 |
| Variables | PC1 | PC2 | PC3 | PC4 | PC5 |  | F1 | F2 | F3 | F4 | F5 |
| pH | -0.401 | 0.383 | 0.215 | 0.078 | -0.522 |  | -0.564 | 0.211 | 0.033 | 0.289 | -0.431 |
| TDS (mg/L) | -0.785 | 0.176 | 0.076 | -0.039 | 0.016 |  | -0.729 | -0.226 | -0.261 | 0.036 | -0.051 |
| EC (µS/cm) | -0.873 | 0.141 | 0.307 | 0.054 | -0.001 |  | -0.844 | -0.351 | -0.105 | -0.003 | -0.180 |
| DO (mg/L) | 0.457 | -0.112 | 0.463 | 0.127 | 0.366 |  | 0.276 | -0.096 | 0.553 | -0.439 | 0.056 |
| F- (mg/L) | -0.043 | -0.349 | 0.534 | -0.275 | -0.566 |  | 0.085 | -0.145 | -0.007 | -0.091 | -0.877 |
| Cl- (mg/L) | -0.663 | 0.488 | 0.118 | -0.033 | 0.262 |  | -0.831 | -0.006 | -0.110 | -0.155 | 0.186 |
| NO3- (mg/L) | -0.628 | 0.020 | -0.188 | 0.080 | 0.129 |  | -0.471 | -0.297 | -0.286 | 0.160 | 0.189 |
| SO42- (mg/L) | -0.781 | 0.339 | 0.034 | 0.160 | 0.058 |  | -0.829 | -0.155 | -0.117 | 0.148 | 0.094 |
| PO43- (mg/L) | -0.387 | -0.063 | -0.497 | -0.291 | -0.035 |  | -0.114 | -0.055 | -0.655 | 0.157 | 0.131 |
| U (µg/L) | -0.225 | -0.184 | -0.482 | 0.565 | -0.401 |  | 0.008 | -0.200 | -0.049 | 0.865 | 0.073 |
| Total hardness (mg/L) | -0.704 | -0.611 | 0.002 | -0.090 | 0.145 |  | -0.208 | -0.816 | -0.424 | -0.017 | -0.094 |
| Ca+2 hardness | -0.499 | -0.600 | 0.169 | 0.447 | 0.150 |  | -0.149 | -0.886 | 0.125 | 0.190 | -0.017 |
| Mg+2 hardness | -0.490 | -0.232 | -0.187 | -0.648 | 0.016 |  | -0.143 | -0.205 | -0.783 | -0.228 | -0.146 |
| Bicarbonate (mg/L) | -0.701 | -0.205 | 0.134 | 0.017 | 0.013 |  | -0.470 | -0.510 | -0.218 | 0.043 | -0.147 |

Table 7. The hydrological parameter weights from Varimax rotated component matrix

.

| Factor | Relative Eigen value | variable No | Rotated loading | Relative loading on same PC | Weight |
| --- | --- | --- | --- | --- | --- |
| F1 | 0.361 | pH | 0.564 | 0.132 | 0.048 |
|  |  | TDS | 0.729 | 0.171 | 0.062 |
|  |  | EC | 0.844 | 0.198 | 0.071 |
|  |  | Cl- | 0.831 | 0.195 | 0.070 |
|  |  | NO3- | 0.471 | 0.110 | 0.040 |
|  |  | SO42- | 0.829 | 0.194 | 0.070 |
|  |  | Total |  | 1.00 |  |
| 2 | 0.219 | Total hardness | 0.816 | 0.369 | 0.081 |
|  |  | Ca+2 hardness | 0.886 | 0.401 | 0.088 |
|  |  | Bicarbonate | 0.510 | 0.230 | 0.050 |
|  |  | Total |  | 1.00 |  |
| 3 | 0.180 | DO | 0.553 | 0.278 | 0.050 |
|  |  | PO43- | 0.655 | 0.329 | 0.059 |
|  |  | Mg+2 hardness | 0.783 | 0.393 | 0.071 |
|  |  | Total |  | 1.00 |  |
| 4 | 0.124 | U | 0.865 | 1.00 | 0.124 |
| 5 | 0.116 | F- | 0.877 | 1.00 | 0.116 |

Table 8. The water sample's agreement or disagreement between different WQI methods is shown in the confusion matrix.

| Rating of water samples across the water quality indices* |  |  |  | Sample count |
| --- | --- | --- | --- | --- |
| EWQI | GWQI | PWQI-1 | PWQI-2 |  |
| 1 | 1 | 1 | 1 | 46 |
| 2 | 1 | 1 | 1 | 2 |
| 2 | 1 | 2 | 1 | 3 |
| 2 | 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 1 | 2 |
| 2 | 2 | 2 | 2 | 20 |
| 3 | 2 | 3 | 2 | 1 |

* Rating of 1-4 stands for excellent, good, medium, and poor water quality, respectively