// Switch off batch mode to view images during editing--------------------------------------------------

dir1 = getDirectory("source directory");

dir2 = getDirectory("target directory");

list = getFileList(dir1);

setBatchMode(false);

for (k=0; k<list.length; k++) {

showProgress(k+1, list.length);

open(dir1+list[k]);

// scaling--------------------------------------------------------------------------------------------------------------

distance = 506.6825;

known = 7.81;

//run("Set Scale...", "distance=508.0157 known=10 pixel=1 unit=mm global");

// crop

makeRectangle(750, 700, 2000, 2000);

run("Crop");

// write time in results

run("Exif Data...");

selectWindow("EXIF Metadata for "+getTitle());

info = getInfo();

index = indexOf(info, "Date/Time Original");

f (index==-1)

return "Error: Date/Time not available for \"" + getTitle() + "\"";

path = substring(info, index+30, index+39);

//print(path);

selectWindow("EXIF Metadata for "+getTitle());

run("Close");

// Edit left measurement mark and copy to slice 1 ---------------------------------------------------------

run("Color Threshold...");

// Color Thresholder 2.0.0-rc-69/1.52p

// Autogenerated macro, single images only

min=newArray(3);

max=newArray(3);

filter=newArray(3);

a=getTitle();

run("Duplicate...", "title=Duplikat");

run("RGB Stack");

run("Convert Stack to Images");

selectWindow("Red");

rename("0");

selectWindow("Green");

rename("1");

selectWindow("Blue");

rename("2");

min[0]=0;

max[0]=130;

filter[0]="pass";

min[1]=0;

max[1]=255;

filter[1]="pass";

min[2]=0;

max[2]=255;

filter[2]="pass";

for (i=0;i<3;i++){

selectWindow(""+i);

setThreshold(min[i], max[i]);

run("Convert to Mask");

if (filter[i]=="stop")  run("Invert");

}

imageCalculator("AND create", "0","1");

imageCalculator("AND create", "Result of 0","2");

for (i=0;i<3;i++){

selectWindow(""+i);

close();

}

selectWindow("Result of 0");

close();

selectWindow("Result of Result of 0");

makeRectangle(0, round(getHeight()/2)-400, 600, 800);

// Cut again and paste into image

run("Copy");

run("Put Behind [tab]");

run("Restore Selection");

run("Paste");

run("Select None");

selectWindow("Result of Result of 0");

// Close duplicate window

close();

// Edit right measurement mark and copy to slice 1 ---------------------------------------------------------

run("Color Threshold...");

// Color Thresholder 2.0.0-rc-69/1.52p

// Autogenerated macro, single images only!

min=newArray(3);

max=newArray(3);

filter=newArray(3);

a=getTitle();

run("Duplicate...", "title=Duplikat");

run("RGB Stack");

run("Convert Stack to Images");

selectWindow("Red");

rename("0");

selectWindow("Green");

rename("1");

selectWindow("Blue");

rename("2");

min[0]=0;

max[0]=70;

filter[0]="pass";

min[1]=0;

max[1]=255;

filter[1]="pass";

min[2]=0;

max[2]=255;

filter[2]="pass";

for (i=0;i<3;i++){

selectWindow(""+i);

setThreshold(min[i], max[i]);

run("Convert to Mask");

if (filter[i]=="stop")  run("Invert");

}

imageCalculator("AND create", "0","1");

imageCalculator("AND create", "Result of 0","2");

for (i=0;i<3;i++){

selectWindow(""+i);

close();

}

selectWindow("Result of 0");

close();

selectWindow("Result of Result of 0");

makeRectangle(getWidth()-600, round(getHeight()/2)-400, 600, 800);

// Cut again and paste into image

run("Copy");

run("Put Behind [tab]");

run("Restore Selection");

run("Paste");

run("Select None");

selectWindow("Result of Result of 0");

// close duplicate window

close();

// Edit upper measurement mark and copy to slice 1 ---------------------------------------------------------run("Color Threshold...");

// Color Thresholder 2.0.0-rc-69/1.52p

// Autogenerated macro, single images only!

min=newArray(3);

max=newArray(3);

filter=newArray(3);

a=getTitle();

run("Duplicate...", "title=Duplikat");

run("RGB Stack");

run("Convert Stack to Images");

selectWindow("Red");

rename("0");

selectWindow("Green");

rename("1");

selectWindow("Blue");

rename("2");

min[0]=0;

max[0]=130;

filter[0]="pass";

min[1]=0;

max[1]=255;

filter[1]="pass";

min[2]=0;

max[2]=255;

filter[2]="pass";

for (i=0;i<3;i++){

selectWindow(""+i);

setThreshold(min[i], max[i]);

run("Convert to Mask");

if (filter[i]=="stop")  run("Invert");

}

imageCalculator("AND create", "0","1");

imageCalculator("AND create", "Result of 0","2");

for (i=0;i<3;i++){

selectWindow(""+i);

close();

}

selectWindow("Result of 0");

close();

selectWindow("Result of Result of 0");

makeRectangle(round(getWidth()/2)-400, 0, 800, 600);

// Cut again and paste into image

run("Copy");

run("Put Behind [tab]");

run("Restore Selection");

run("Paste");

run("Select None");

selectWindow("Result of Result of 0");

// close duplicate window

close();

// Edit lower measurement mark and copy to slice 1 ---------------------------------------------------------run("Color Threshold...");

// Color Thresholder 2.0.0-rc-69/1.52p

// Autogenerated macro, single images only!

min=newArray(3);

max=newArray(3);

filter=newArray(3);

a=getTitle();

run("Duplicate...", "title=Duplikat");

run("RGB Stack");

run("Convert Stack to Images");

selectWindow("Red");

rename("0");

selectWindow("Green");

rename("1");

selectWindow("Blue");

rename("2");

min[0]=0;

max[0]=90;

filter[0]="pass";

min[1]=0;

max[1]=255;

filter[1]="pass";

min[2]=0;

max[2]=255;

filter[2]="pass";

for (i=0;i<3;i++){

selectWindow(""+i);

setThreshold(min[i], max[i]);

run("Convert to Mask");

if (filter[i]=="stop")  run("Invert");

}

imageCalculator("AND create", "0","1");

imageCalculator("AND create", "Result of 0","2");

for (i=0;i<3;i++){

selectWindow(""+i);

close();

}

selectWindow("Result of 0");

close();

selectWindow("Result of Result of 0");

makeRectangle(round(getWidth()/2)-400, getHeight()-600, 800, 600);

// Cut again and paste into image

run("Copy");

run("Put Behind [tab]");

run("Restore Selection");

run("Paste");

run("Select None");

selectWindow("Result of Result of 0");

// Close duplicate window

close();

// Detect and mark measurement marks --------------------------------------------------------------

// colour

run("RGB Color");

// Determine variables

Ymitte = round(getHeight() / 2)- 101; // 1/2 Height of the image - 101 pixels write in variable

Xmitte = round(getWidth() / 2)- 101;

Links = 0;

Oben = 0;

Rechts = 0;

Unten = 0;

// Detect edge left, change from white to blackLinksArray=newArray(200);

for (j=1;j<201; j++){

for (i=2;i < 999;i++){

flag = true;

for (h=0;h<5;h++){

if((getPixel(i+h,Ymitte+j))&0xff>0){

flag = false;

}

}

for (h=0;h<5;h++){

if((getPixel((i-(1+h)),Ymitte+j)&0xff)==0) {

flag = false;

}

}

if (flag==true) {

setColor(255, 0, 0);

setLineWidth(1);

drawLine(i-5, Ymitte+j, i+5, Ymitte+j);

LinksArray [j-1] = i;

i = 998;

}

}

}

Array.sort(LinksArray);

// calculate median

middle = lengthOf(LinksArray)/2;

if (lengthOf(LinksArray)%2 == 0){

Links = (LinksArray[middle-1]+LinksArray[middle])/2;

} else { (

Links = LinksArray[middle];

}

// Detect edge on the right, change from white to black

RechtsArray=newArray(200);

for (j=1;j<201; j++){

for (i=getWidth()-2;i > 999;i--){

flag = true;

for (h=0;h<5;h++){

if((getPixel(i-h,Ymitte+j))&0xff>0){

flag = false;

}

}

for (h=0;h<5;h++){

if((getPixel((i+(1+h)),Ymitte+j)&0xff)==0) {

flag = false;

}

}

if (flag==true) {

setColor(255, 0, 0);

setLineWidth(1);

drawLine(i-5, Ymitte+j, i+5, Ymitte+j);

RechtsArray [j-1] = i;

i = 998;

}

}

}

Array.sort(RechtsArray);

// calculate median

middle = lengthOf(RechtsArray)/2;

if (lengthOf(RechtsArray)%2 == 0){

Rechts = (RechtsArray[middle-1]+RechtsArray[middle])/2;

} else { (

Rechts = RechtsArray[middle];

}

// Detect upper edge, change from white to black

ObenArray=newArray(200);

for (j=1;j<201; j++){

for (i=2;i < 999;i++){

flag = true;

for (h=0;h<5;h++){

if((getPixel(Xmitte+j,i+h))&0xff>0){

flag = false;

}

}

for (h=0;h<5;h++){

if((getPixel(Xmitte+j,(i-(1+h)))&0xff)==0) {

flag = false;

}

}

if (flag==true) {

setColor(255, 0, 0);

setLineWidth(1);

drawLine(Xmitte+j, i+5, Xmitte+j, i-5);

ObenArray [j-1] = i;

i = 998;

}

}

}

Array.sort(ObenArray);

//calculate median

middle = lengthOf(ObenArray)/2;

if (lengthOf(ObenArray)%2 == 0){

Oben = (ObenArray[middle-1]+ObenArray[middle])/2;

} else { (

Oben = ObenArray[middle];

}

// Detect lower edge, change from white to black

UntenArray=newArray(200);

for (j=1;j<201; j++){

for (i=getHeight()-2;i > 999;i--){

flag = true;

for (h=0;h<5;h++){

if((getPixel(Xmitte+j,i-h))&0xff>0){

flag = false;

}

}

for (h=0;h<5;h++){

if((getPixel(Xmitte+j,(i+(1+h)))&0xff)==0) {

flag = false;

}

}

if (flag==true) {

setColor(255, 0, 0);

setLineWidth(1);

drawLine(Xmitte+j, i+5, Xmitte+j, i-5);

UntenArray [j-1] = i;

i = 998;

}

}

}

Array.sort(UntenArray);

// calculate median

middle = lengthOf(UntenArray)/2;

if (lengthOf(UntenArray)%2 == 0){

Unten = (UntenArray[middle-1]+UntenArray[middle])/2;

} else { (

Unten = UntenArray[middle];

}

// Centre markings on measuring marks for orientation

setColor(0, 255, 0);

setLineWidth(5);

drawLine(Links-100, Ymitte+101, Links+100, Ymitte+101);

drawLine(Rechts-100, Ymitte+101, Rechts+100, Ymitte+101);

drawLine(Xmitte+101, Oben-100, Xmitte+101, Oben+100);

drawLine(Xmitte+101, Unten-100, Xmitte+101,Unten+100);

// Calculated measuring point as marker

setColor(0, 0, 255);

setLineWidth(20);

drawLine(Links, Ymitte+101, Links, Ymitte+101);

drawLine(Rechts, Ymitte+101, Rechts, Ymitte+101);

drawLine(Xmitte+101, Oben, Xmitte+101, Oben);

drawLine(Xmitte+101, Unten, Xmitte+101,Unten);

// Write values in Results

row = nResults;

setResult("Name", row, getTitle());

setResult("Zeit", row, path);

setResult("Höhe_Pix", row, Unten- Oben);

setResult("Breite_Pix", row, Rechts- Links);

setResult("Höhe_mm", row, (Unten- Oben)*known/distance);

setResult("Breite_mm", row, (Rechts- Links)*known/distance);

// REVIEW

// Can be commented out when it is running....

//waitForUser( "Pause","Bestaetige Masken-Einstellung");

//wait(1000);

//Save

saveAs("JPEG", dir2+list[k]); // Speichern im Out Ordner

close();

}