2. Materials and Methods

2.1. Study Area

The purpose study area is Dolakha District, Nepal situated in the north-east of Kathmandu that extends from latitude and longitude are 27°46′42.24′′ N and 86°10′30.72′′ E respectively [32]. Last few years, this district becomes more and more fragile for soil erosion, deforestation, and terrace farming on steep slopes, rapid population pressure on natural resources [33], [34]. Other major contributors are land use and land cover change caused by a human that increases the erosion rate here [35].

2.2. Data Collection

The study used various spatial datasets acquired from different sources are shown in Table 1.

2.3. Methods

The computation of mean annual soil loss for ground slopes is performed by RUSLE model in Geographic Information System (GIS) platform ([36] Figure 2).The RUSLE is expressed by an equation,

A = [ R] * [ K] * [ LS] * [ C] *[ P], 			(1)

where,  A = soil loss (mg ha-1 yr-1),  R = rainfall erosivity factor ( mm ha-1 yr-1),  K = soil erodibility factor (mg/ha.yr),  LS = slope-length and slope steepness factor (dimensionless), C = land management factor (dimensionless), and  P = conservation practice factor (dimensionless).

2.3.1. RUSLE Parameters Computation

(a) Rainfall Erosivity Factor (R)

The rainfall erosivity factor (R) describes the erosivity and intensity of rainfall at a particular location based on the rainfall amount on soil erosion[37], [38]. The rainfall erosivity quantifies an effect of raindrop amount and rate of runoff associate with rainfall and its unit is expressed in mm ha-1 h-1 yr-1. During this study, the rainfall map produced by the National Aeronautics and Space Administration (NASA) was used to generate a rainfall erosivity factor [39]. The rainfall map represents mean annual precipitation over the district, equation integrated to generate the R-factor is given by reference [9].

R = 38.5 + 0.35P,					 (2)

where, R = Rainfall Erosivity Factor, P = Mean Annual Rainfall in mm

(b) Soil Erodibility Factor (K)

The soil erodibility factor (K) measures the susceptibility of different types of soil and its particles to detachment and transport by rainfall and runoff. The K factor is influenced by soil texture, organic matter, soil structure, and permeability of the soil profile [39]. The equation provided by reference was used to estimate the soil erodibility of soil[40].

K = Fcsand*Fsi-cl *Forgc*Fhisand *0.1317,	 	(3)

where,

Fcsand = [0.2 + 0.3 exp ( -0.0256 SAN (1- SIL100 ))],	(4)

Fsi - cl =[SILCLA+SIL]0.3,				 (5)

Forgc =[1.0 -0.25 CC+exp(3.72-2.95 C)  ], 			(6)

Fhisand = =[1.0 -0.70  SN!SN1 + exp( -5.51+ 22.9 SN1)  ], 		(7)

where, SAN, SIL and CLA are % sand, silt and clay, respectively; C is the organic carbon content; and

SN1 is sand content subtracted from 1 and divided by 100.

Fcsand = it gives a low soil erodibility factor for soil with coarse sand and a high value for soil with little sand content.

Fsi-cl = it gives a low soil erodibility factor with high clay to silt ratio.

Forgc = it is the factor that reduces soil erodibility for soil with high organic content.

Fhisand= it is the factor that reduces soil erodibility for soil with extremely high sand content.

(c) Topographic Factor (LS)

The topographic factor (LS) was created from two sub-factors: a slope gradient factor (S) and a slope-length factor (L); both of which are determined from the Digital Elevation Model (DEM). Slope-length and gradient is the important parameter in the soil erosion modeling [22], in calculating the transport capacity of overland flow (Surface runoff). The L and S represent the effect of slope length and steepness respectively on erosion, also when slope increases soil loss per unit area increases. The slopes gradient and slope length factors were calculated from the DEM and combined to result in the topographical factor grid using the following relation[41].

L = (22.13 ) m,			 (8)

where, L = slope length factor, _ = slope length (m), m = slope-length exponent

m =F1+F' , 			(9)

F = sin/0.0896 3 (sin)0.8+0.56 , 		(10)

where, F = Ratio of rill erosion to inter rill erosion,  = slope angle (º)

In ArcGIS, L was calculated as,

L = flowacc + 625 )m+1 – flowaccm+125(m+2) * 22.13m  , (11)

For slope gradient factor,

S = Con((Tan(slope* 0.01745) < 0.09),(10.8 * Sin(slope * 0.01745) + 0.03),

(16.8 *  Sin(slope * 0.01745) - 0.5)),     (12)

Final, LS = L * S, 		       (13)

(d) Cover Management Factor (C)

The cover-management factor (C) is used to reflect the effect of cropping and other management practices on erosion rates [42]. In this study Land Use/Land Cover (LULC) produced by the ICIMOD was used for preparing a C-factor map [43]. The raster map was converted to polygon through raster to polygon tool and the attributes with the same land use type were merged into a single class using ArcGIS 10.5 software, the study used eight types of land use (Table 2). For each land-use type, C values were assigned through reference [44]. The value of C factor ranges from 0 to 1, where lower C means strong cover effect resulting in no loss, while higher indicates no cover effect and significant chances of soil loss [45].

(e) Support Practice Factor (P)

The support practice factor indicates the rate of soil loss according to the various agricultural practice. There are three methods such as contours, cropping, and terrace and it is an important factor that can control erosion [46]. This contouring method is used with P values range from 0 to 1, where the value 0 represents a very good anthropic erosion resistance facility and the value 1 indicates a non-anthropic resistance erosion facility (Table 3 [47]).

2.3.2. Potential Erosion Map

The input data from different sources were processed in ArcGIS for creating five-factor maps: R, K, LS, C, and P (Figure 3). These raster maps were integrated within the ArcGIS environment using the RUSLE relation to generate composite maps of the estimated erosion loss within the study area[41]. Using a zonal statistics tool and calculate geometry, used for computing an area-weighted mean of the potential erosion rates for slopes and LULC were generated to explore the relationship between slope and LULC on erosion [49]. First, the slope map of Dolakha was generated from DEM in ArcGIS and then reclassified into eight classes[50].