Methods

Field Tests

Field tests were conducted from 2016 to 2018 in an experimental vineyard heavily infected with Esca at the JKI Geilweilerhof located in Siebeldingen, Germany (49°21.7470 N, 8°04.6780 E). The vineyard consisted of Vitis vinifera L. cv. ‘Phoenix’ planted in 1996. Rows of 120 m length were oriented in east-west direction. Interrow and grapevine spacing was 1.8 m and 1.3 m, respectively. Grapevines were trained in a VSP-system and pruned as single guyot. The precise location of all plants was acquired using a portable GPS (SPS585, Trimble®, Sunnyvale, CA, USA) providing highly accurate positioning (resolution: 0.02 m).

The acquisition of spectral data was conducted at several time points during the growing season to record vines pre-symptomatically and with different disease severity levels. Therefore, the plot was monitored biweekly – always in parallel to hyper- and multispectral field tests – and showed an average incidence of chronic Esca leaf symptoms of 11 %.

Hyperspectral Imaging

1. Data Acquisition

Hyperspectral data were acquired during the growing seasons 2016, 2017, and 2018. Two rows of the experimental vineyard were recorded in all three years. This results in 129, 114, and 112 monitored plants for the three years, respectively. Due to the persistently high number of Esca symptomatic vines within this vineyard, many vines had to be removed in recent years. In 2017 an additional row with 69 vines was considered. Hyperspectral imaging was performed using the Phenoliner, a novel field phenotyping platform, which was recently introduced by Kicherer et al. [51]. Due to its tunnel structure, it avoids disturbing environmental factors (e.g. sunlight, effects of clouds) and enables the acquisition of field data under almost standardized conditions. Few modifications have been made in comparison to the original design. First, additional light sources were mounted for a better illumination of the recorded area consisting of two 300 W short-wave spotlights (Hedler C12, Hedler Systemlicht, Runkel/Lahn, Germany) with a broad power spectral density. Second, the PTFE spectralon (1 x 1 m) used as a calibration standard was replaced by a smaller one (54.5 x 35 cm) (Sphere Optics, Herrsching, Germany) covered with a borosilicate glass for protection. The calibration standard was reflectance certified by Sphere Optics GmbH (Herrsching, Germany).

Spectral reflectance range from 400 to 2,500 nm was measured with two separate line scanning cameras (HySpex®) supplied by Norsk Elektro Optikk A/S, Skedsmokorset, Norway. During the course of the project, two sets of cameras were used. Spectra in the visible and near-infrared range (VNIR; 400 to 1,000 nm) were recorded in 2016 by a HySpex VNIR 1600 and in 2017 as well as in 2018 with a HySpex VNIR 1800. The spectral acquisition in the short-wave infrared (SWIR; 1,000 to 2,500 nm) was performed in 2016 with a HySpex SWIR 320 m-e and in 2017 as well as in 2018 with a HySpex SWIR 384. In Table 6 an overview of the camera specifications is given.

Table 6: Key specifications of the camera systems used

| Specification | HySpex VNIR 1600 | HySpex VNIR 1800 | HySpex SWIR 320m-e | HySpex SWIR 384 |
| --- | --- | --- | --- | --- |
| Dynamic Range (bit) | 12 | 16 | 14 | 16 |
| Spectral Bands | 160 | 186 | 256 | 288 |
| Spatial Pixel | 1600 | 1800 | 320 | 384 |
| Max. Framerate (Hz) | 100 | 100 | 160 | 400 |
| Spectral Resolution (nm) | 3.70 | 3.26 | 6.00 | 5.45 |

Spectral camera to camera transformation was achieved by nearest wavelength interpolation scheme. As a calibration standard, the PTFE spectralon is recorded constantly in the background of the camera viewport. For the correct mapping of spectral data and vine position, a proprietary image acquisition software developed by Fraunhofer Institute for Factory Operation and Automation IFF was used, which integrates the two hyperspectral sensors and the Phenoliner GPS.

2. Data Calibration and Labeling

Data calibration was performed by calculating the reflectance value from a recorded dark current (while camera shutter is closed) and a recording when the spectralon (white target) is covering the camera viewport. Calibration measurement is performed before and after each grape row recording. Reflectance Rλ per pixel is calculated as

Rλ=Iλ-IλDCIλW-IλDC

where Iλ is the image pixel intensity at wavelength λ, IλDC the intensity when measured with closed shutter (“dark current”) and IλW being the intensity while recording the spectralon device. Based on the reflectance image, a classification model was trained to classify each spectral pixel into either vegetation, grapes, stems and general background. The classification model training was performed with the AutoML platform HawkSpex®Flow developed by the Fraunhofer IFF. The best combination of model and background was selected and used for all segmentation purposes. The winning model was a Multi-Layer Perceptron [52] with SNV normalization [53]. Separate models for the VNIR and SWIR image processing were created.

In order to label leaf material with visual rating data, two different approaches were followed:

a. Vine based GPS labeling: While recording the hyperspectral images, a log with GPS positions was created. GPS positions were corrected for the relative position of GPS antenna and camera position within the vehicle. From a list of manually measured vine locations, a vine ID was associated with an image window centered on the scanning line with the closest distance to the vine GPS position. The window was set to 50 cm in order to avoid overlapping of vine plans. The window was further adjusted by considering the vines arc direction. Labels for symptomatic rating as well as pre-symptomatic rating were associated to the data via the vine ID. Pre-symptomatic plants did not show symptoms at the moment of hyperspectral imaging but within the next two weeks.

b. Manual labeling of Esca symptoms: Field data is a heterogeneous mixture of symptomatic and non-symptomatic leaves; this could potentially influence the performance of a disease detection model. In order to acquire precise ground truth data and simulate best possible conditions, RGB images were reconstructed from the VNIR and SWIR camera separately. Reconstruction was performed using three wavelength reflectance values as RGB values. Esca symptoms were manually labeled. As control vegetation, data from vine labeled as uninfected were extracted using the approach in a).

3. Detection Model Development

The labeled data formed a number of datasets for the subsequent machine learning for creating a detection model. A machine learning model is a generic mathematical formula that calculates an output from an input using a number of free parameters. Best-known methods are Artificial Neural Networks which mimic the information processing of the human brain or method motivated from statistics like Support Vector Machines (SVM) or Bayesian that model data distributions. For each method, an algorithm is defined that finds the optimal parameter of the model by calculating the error between the model output and a target value. In this case, a coded representation of the visual rating class associated with the data point. Coding of -1 for control and +1 for pathogen infection was used (either symptomatic or pre-symptomatic). The optimization algorithm then finds parameters for the model that minimizes the error, either by iteratively applying an optimization strategy like gradient descent (Artificial Neural Networks) or by a closed form mathematical calculation (e.g. Partially Least Squares).

An important aspect of data-driven modeling is the cross-validation scheme. In this study an n-fold cross validation is performed with the dataset being divided into n parts. The model is the one optimized on n-1 folds while tested on the nth-fold. The modeling process is performed with all possible combinations without repetition of folds. As a model performance indicator, the average and standard deviations of the performance value is calculated across modeling runs.

All models where assessed using the following performance criteria:

a. True Positive Rate (TPR): Ratio calculated from the number of samples detected correctly as infected among all possible infected samples. Determines the precision of the detector.

b. False Positive Rate (FPR): Ratio calculated from the number of samples detected incorrectly as infected among all possible control samples. Determines the false alarm rate or 1-specificity.

c. Overall Accuracy (CA): Ratio calculated from the number of samples correctly classified among all possible samples.

A sample in the modeling phase is one spectrum labeled with its respective class. A number of spectral preprocessing methods in combination with machine learning models were tested:

Table 7: Pre-processing methods:

| Method | Formula |
| --- | --- |
| Vector L2 Normalization | RλN=RλλRλ2 |
| Vector SNV Normalization [53] | RλN=Rλ-1NλRλ1N-1λRλ-1NλRλ2 |

Table 8: Model hyper-parameters:

| Method | Citation | Hyper-Parameters |
| --- | --- | --- |
| Multi-Layer Perceptron Network (MLP) | [52, 54] | number of hidden layers: 3 optimization method: scaled conjugate gradient backpropagation neurons per hidden layer: 50, 25, 10 |
| Radial-Basis Function Network with Relevance (rRBF) | [55-57] | number of radial basis functions: 30 optimization method: scaled non-linear conjugate gradient |
| Partially Least Square (PLS) | [58] | number of components: 20 |
| Linear Discriminance Model (LDA) | [59] | no hyper-parameters |

The models were generated on datasets describing a certain detection task with data separately for the measurement years 2016, 2017, and 2018 as well as combined datasets for two years in all possible combinations. These two-year models where then used to detect samples from the third year in order to test their transferability to an independent year. Due to computational demand, only 10,000 spectra per label class were used in the modeling. VNIR und SWIR reflectance data is modeled separately since an alignment of camera images was not successful, partly due to different camera resolutions.

4. Application of Detection Model

Once optimized to the detection task, the best performing model in terms of accuracy (see Additional Files 2 and 3) was then applied back to the hyperspectral image. Each vegetation pixel was labeled using the detector. In order to assess the detection performance of the detector a statistics over all vegetation pixels within the vine window as described above was calculated and the label with the highest occurrence was deemed as the representing label for the whole vine (majority vote).

In Figure 3 the hyperspectral imaging steps are shown, a calibrated reflectance image (a) is semantically segmented for its principal material components (c) and labeled for vine positions from GPS (d). A detector is applied to each pixel and the ratio detection decision within a vine window is calculated (e).

Figure 3: Hyperspectral imaging steps. A calibrated reflectance image (a) is used to reconstruct a RGB-image (b) for manual annotation. The reflectance image (a) is segmented into its main components (c) and vines are labeled based on their GPS position (d). The disease detection model is then applied to the pixels associated with the GPS vine position and the detection ratio is calculated (e).

5. Spectral Relevance and Multispectral Simulation

During the minimization of the classification error, a wavelength specific vector of numerical weights is adjusted by the training algorithm of the Radial Basis Function Network (RBF). This vector is termed relevance profile [56] and indicates the importance of wavelength for the task at hand. Since a multispectral camera system is not measuring certain wavelength at the same precision as a hyperspectral camera, a spectral band is approximated by using a gauss function with 30 nm full-width-half-maximum (FWHM). This a typical specification for optical filters used in multispectral imaging systems.

In order to optimally place the spectral band, an automatic algorithm is applied. For that purpose 100,000 random wavelength values are generated by using the relevance function as probability density function (pdf). Subsequently, the generated values are denser in areas of high relevance profile and less dense in areas of low relevance. On this sample set, a neural gas vector quantization algorithm is trained [60] that places a set number of wavelength values in a way to minimize the quantization error measured by the mean squared error between placed wavelengths and best matching generated wavelengths. Naturally, the neural gas will cover dense areas with more wavelength candidates (or “prototypes”) than less dense areas. For the multispectral simulation, 10 wavelength prototypes are considered. After placing the spectral bands, a multispectral simulation is performed by elementwise multiplication of spectrum and gauss function followed by a summation of the resulting products. Spectral bands are ordered with descending relevance value at mid-wavelength position.

This generated a new dataset with identical ground truth where 2 to 10 input spectral features are selected according to the band ordering. These datasets were modeled with the same set of machine learning methods as above in order to actually assess the change in precision compared to using the full spectral range at native sensor resolution.

Airborne Multispectral Imaging

Multispectral data was acquired by Lilienthal Digitaler-Weinbau GmbH (Wiesbaden, Germany) during the growing seasons of 2017 and 2018. The whole plot was recorded, but for the analysis only selected rows were considered (see Hyperspectral Data Acquisition).

Multispectral images were collected using a UAV platform (Geo XR6; hexacopter) able to fly by remote control for starts and landings as well as autonomously during image acquisition. Flight altitude was set at 35-45 m above ground with a flight speed of approx. 5 m/s. In this study, the MicaSense RedEdge™ camera (MicaSense Inc., Simi Valley, CA, USA) was used capturing five discrete wavelength bands: Blue (455-495 nm), Green (540-580 nm), Red (658-678 nm), Red-Edge (707-727 nm), and Near Infrared (NIR; 800-880 nm). The multispectral sensor was equipped with an integrated GPS system and a downwelling light sensor (DLS) for better calibration. Using a focal length of 5.5 nm and a field of view of 47.2° the MicaSense RedEdge™ allows the acquisition of 12 Bit RAW data with a resolution of 2.5-3 cm/pixel.

Specific flight patterns were flown to enable an optimal photogrammetric processing. Thus an automatic soil extraction could be processed to hide information from intra-row vegetation. Data orthorectification was conducted using Agisoft Software (Agisoft LLC., St. Petersburg, Russia). For the analysis of the spectral data, only an area of 50 cm around the GPS vine position (depending on the guyot pruning) is considered. Plants were grouped according to their disease severity (no symptoms, < 25 %, 25-50 %, 50-75 %, and > 75 % symptomatic) for data analysis.

The reflectance data in the five camera channels added by the NDVI and NDRE calculated with the following formula served as input to the modeling processing using the same setup as described above for the hyperspectral data processing.

NDVI=RNIR-RRedRNIR+RRed

NDRE=RNIR-RRed-EdgeRNIR+RRed-Edge