Table 1. Pyrolysis technology review - feedstock composition

| Pyrolysis plant number and supplier |  | PET | HDPE | PVC | LDPE | PP | PS | Other (7) | Polyamide | Rubber | ABS | Other materials | Additional requirements/ explanations | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P1 T | Envion | -1 | + | +/-2 | + | + |  | + |  |  |  |  | Particle size < 4 cm and melted plastic 1PET in small quantities 2PVC < 10 % < 2% moisture | (American Chemistry Council and RTI International 2012) |
| P2 T | Agilyx | +/- | + | +/- | + | + | + |  |  |  |  |  | Ground plastic PVC + PET < 10% | (The American Chemistry Council 2015), (American Chemistry Council and RTI International 2012) |
| P3 T | Climax | +3 | +3 | +3 | +3 | +3 | +3 | +3 | +3 |  | +3 | -3 | Ground plastic 3100% plastic < 5% moisture | (American Chemistry Council and RTI International 2012) |
| P4 T | Veba |  | + | -4 | + | + |  |  |  |  |  |  | Particle size < 1 cm 4PVC < 4 % < 5% of inorganic material < 1% moisture | (American Chemistry Council and RTI International 2012), (Veba Oil Bottrop Germany 2023) |
| P5 T | BP Chem. |  | + |  | + | + | + |  |  |  |  |  | - | (Thunman et al. 2019) |
| P6 T | BP Chem. | -5 | + | -6 | + | + | +7 |  |  |  |  | -8 | Ground plastic 8Separation of contaminants 5PET < 3-5 % 6PVC < 2-4 % 7PS 15-30 % < 1% moisture | (Arena and Mastellone 2006) |
| P7 T | BP Chem. |  | + | -9 | + | + |  |  |  |  |  | -10 | 9Chlorine < 2,5 % 10Contaminations < 4 % Ash < 4,5 % < 1% moisture | (Perugini et al. 2005), (Anuar Sharuddin et al. 2016) |
| P8 T | Veba |  | + |  | + | + |  |  |  |  |  |  | - | (Perugini et al. 2005) |
| P9 T | Agilyx | + | + | + | + | + | + | + |  |  |  |  | Ground plastic, plastic granulates of pallets | (Tsiamis and Themelis 2013) |
| P10 T | Climax | +11 | +11 | + | + | + | + | + |  |  |  |  | Ground plastic 11Municipal plastic waste (preferred #3-7 per RIC) | (Tsiamis and Themelis 2013), (Edwards et al. 2020), (Closed Loop Partners 2019) |
| P11 T | Pyrolysis | +12 | +12 | +12 | +12 | +12 | +12 | +12 |  |  |  |  | 12Municipal plastic waste < 5% moisture | (Haig et al. 2013) |
| P12 T | Cynar | -13 | + | -14 | + | + | + |  |  |  |  | -15 | Particle size < 1 cm 13PET < 2 % 14PVC ~ 0 % 15Separation of contaminants | (The American Chemistry Council 2015) |
| P13 T | Golden Renew. |  | + | + | + | + | + | + |  |  |  | -16 | 16Cleaned, dried, and ground plastics | (The American Chemistry Council 2015) |
| P14 T | Sustane Tech. | -17 | + | -17 | + | + | + |  |  |  |  | -18 | 17Do not accepts PET and PVC 18Cleaned and ground plastics Particle size 6 mm < 1% moisture | (Strum Consulting 2018), (Singh and Cosic 2018) |
| P15 T | Pyrolysis system |  | + |  | + | + | + |  |  |  |  |  | - | (Fivga and Dimitriou 2018) |
| P16 T | NRP |  | +19 |  | +19 | +20 |  |  |  |  |  |  | 1940 % PE 2060 % PP | (Castaldi et al. 2018) |
| P17 T | Polyflow | + | + | + | + | + | + | + |  |  |  | +/-21 | 21Separation of contaminants Accepts carpet and tire shreds | (Technische Universtität München 2018) |
| P18 T | Nexus Fuels | -22 | + | -23 | + | + | + |  |  |  |  | -24 | 22PET < 2 % 23PVC < 1 % 24Separation of contaminants | (The American Chemistry Council 2015) |
| P19 C | JBI Inc. | -25 | + | -25 | + | + | + | + |  |  |  |  | Ground and softened plastic 25Does not accept PET and PVC < 5% of inorganic material < 10% moisture | (American Chemistry Council and RTI International 2012), (Edwards et al. 2020), [12] |
| P20 C | JBI Inc. | +/-26 | + | -27 | + | + |  |  | -27 |  |  |  | 26PET in small quantities 27Does not accept PVC and nylon | (Tsiamis and Themelis 2013) |
| P21 C | Catalytic | +28 | +28 | +28 | +28 | +28 | +28 | +28 |  |  |  |  | Dried plastic 28Municipal plastic waste < 5% moisture | (Haig et al. 2013) |
| P22 T/C | PK Clean | +29 | + | +29 | + | + | + | + |  |  |  |  | Ground and softened plastic 29PET + PVC < 40 % | (The American Chemistry Council 2015) |
| P23 T | Vadxx | -30 | + | -30 | + | + | + | + | -30 | + |  | +31 | Particle size < 5 cm 30Avoid PET, PVC, and Nylon 31Allowed Metal/Wire contaminant, Tires, EPDM44 , TPO45, Butadiene Rubber, Styrene | (The American Chemistry Council 2015) |
| P24 C | Polyfuel, Pyrocrat Systems LPP | +/-32 | + | -33 | + | + | + | + |  |  | + |  | 32PET < 15 % 33PVC < 1 % < 5% moisture | (Pyrocrat Systems 2020) |
| P25 C | PARC | -34 | + | -35 | + | + | + |  |  |  |  | +36 | 34Separation of PET 35PVC < 5 % 36Oil sludge | (Sun et al. 2020) |
| P27 C | R-ONETM | +/-37 | +38 | +/-37 | +38 | +38 | +38 | +/-37 |  |  | +/-37 |  | 3715 % (ABS+PET+PVC+other) 3885 % (PP+PE+PS) | (Yu et al. 2018) |
| P28 C | MK Aromatics | -39 | + | -39 | + | + | + |  |  |  |  | -39 | 39Separation of PET, PVC and contaminants | (The American Chemistry Council 2015) |
| P29 C | Pyrocrat Systems LLP | -40 | + | -41 | + | + | + |  |  |  |  | -42 | 40PET: <5% 41PVC: <1% Dried and separated 42Contamination: <5% | (The American Chemistry Council 2015) |
| P30 C | Klean Industries | -43 | + | -43 | + | + | + |  | -44 |  | -44 | -44 | 43PVC, PET, K-coated products not accepted 44Semi Suitable: ABS, PA, PUR, EVA Particle size < 10 cm | (The American Chemistry Council 2015) |
| T thermal pyrolysis C catalytic pyrolysis T/C use of catalysis is optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3. Gasification technology review - feedstock composition

| Gasification plant number and supplier |  | HDPE | LDPE | PP | PS | PET | PVC | Other (7) | Other materials | Additional requirements/ explanations | Gasification medium | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 T | Enerkem | + | + | + | + | + | + | + | +/-1 | Sorted, dried, and ground input material 1< 15% of inorganic material < 10% moisture | Air/oxygen | (American Chemistry Council and RTI International 2012) |
| G2 T | Ze-gen | + | + | + | + | + | + | + | +2 | Sorted, dried, and ground input material 2Input material from municipal, construction, and industrial waste 2< 5% of inorganic material < 20% moisture | - | (American Chemistry Council and RTI International 2012) |
| G3 T | Plasco | + | + | + | + | + | + | + | +3 | Unrecycled plastic waste 3Accepts wood waste | - | (American Chemistry Council and RTI International 2012), (Caroline Ducharme et al. 2010) |
| G4 T |  | + | + | + | + | + | +/-4 | + | + | Input material from municipal waste Dried input material 4PVC < 10% < 5% moisture | Oxygen | (Haig et al. 2013) |
| G5 T |  | + | + | + | + | + | +/-5 | + | + | Dried input material 5PVC < 10% < 5% moisture | Oxygen/ steam | (Haig et al. 2013) |
| G6 T |  | + | + | + | + | + | +/-6 | + | + | Dried input material 6PVC < 10% < 5% moisture | Oxygen/ steam | (Haig et al. 2013) |
| G7 T |  | + | + | + | + | + | +/-7 | + | + | Dried input material 7PVC < 10% < 5% moisture | Oxygen/ steam | (Haig et al. 2013) |
| G8 T | Pilot FBG | +8 | +8 |  |  |  |  |  |  | 8Sorted PE from municipal waste < 0.2% moisture | Air/steam | (Ardolino et al. 2018) |
| G9 T | Pilot FBG | +9 | +9 | +9 |  |  |  | + |  | 9Polyolefins from food and beverage packaging < 0.3% moisture | Air/steam | (Ardolino et al. 2018) |
| G10 T | Neolite | + | + | + | + | + | + | + | +10 | 10Mixture of packaging waste with metal inclusions < 0.6% moisture | Air/steam | (Ardolino et al. 2018) |
| G11 T | Plasmix | + | + | + | + | + | + | + | +11 | 11Mixture of packaging waste with metal inclusions < 0.7% moisture | Air/steam | (Ardolino et al. 2018) |
| G12 T | PDF - Pilot FBG | + | + | + | + | + | + | + | +12 | 12Mixture of packaging waste with multilayer packaging < 5.6% moisture | Air/steam | (Ardolino et al. 2018) |
| G13 T | Powerhouse | + | + | + | + | + | +13 | + | +12 | 12Mixed plastic waste 13Can take up to 100% PVC with the addition of natrium hydroxide | - | (Himebaugh et al. 2020) |
| G14 T | Texaco | + | + | + | + | + | +/-14 | +15 |  | Mixed polymer waste Particle size < 10 cm 14PVC < 10% 15< 10% of inorganic material < 5% moisture | Oxygen/ steam | (Tukker et al. 1999) |
| G15 T |  | + | + | + | + | + | -16 | + |  | Mixed polymer waste Particle size > 2 and < 8 cm 16Chlor < 2% | Air/steam | (Tukker et al. 1999) |
| G16 T | Akzo Nobel | + | + | + |  |  | +17 |  | +18 | 17PVC < 40% 18Accepts inorganic filters up to 25% of input material 18< 25% of inorganic material | Nitrogen/ steam | (Tukker et al. 1999) |
| G17 T | Linde KCA |  |  |  |  |  | +19 |  |  | 19Separately collected PVC waste Ground and separated input material (separation of metals) | Oxygen/ steam | (Tukker et al. 1999) |
| G18 P | InEnTec | + | + | + | + | + | + | + | +20 | 20Input material from municipal waste Ground input material | Oxygen/ steam | (Caroline Ducharme et al. 2010) |
| G19 P | Alter NRG/ Westinghouse Plasma Corporation | + | + | + | + | + | + | + | +21 | 21Input material from municipal waste Ground input material | Air | (Caroline Ducharme et al. 2010) |
| G20 P | Europlasma | + | + | + | + | + | + | + | +22 | 22Input material from municipal waste and shredded tires <30% moisture | Air | (Caroline Ducharme et al. 2010) |
| G21 P | Geoplasma | + | + | + | + | + | + | + | +23 | Adding slaked lime and coke to the input material 23Input material from municipal waste, construction waste, and tires | Air | (American Chemistry Council and RTI International 2012) |
| G22 T | Ankur Energy Technology | + | + | + | + | + | + | + | +24 | 24Input material from municipal waste <35% moisture | - | (Asian Development Bank 2020) |
| T thermal gasification P plasma gasification |  |  |  |  |  |  |  |  |  |  |  |  |