# Competing for space? - A multi-criteria scenario framework to model the energy-biodiversity-land nexus for regional renewable energy planning on a German case

Jessica Weber1*, Tim Steinkamp2, Marc Reichenbach3

1Berlin Institute of Technology (TU Berlin), Straße des 17. Juni 135, 10623 Berlin, Germany, j.weber@campus.tu-berlin.de

2ARSU GmbH, Escherweg 1, 26121 Oldenburg, Germany. steinkamp@arsu.de, reichenbach@arsu.de

*Corresponding author

# Supplemental Materials

- Habitat model for bird priority zones and species selection

- PESTEL analysis

- Impact/Uncertainty-Matrix

- Data sources for GIS modeling

- GIS method for scenario preparation

- Excel model for scenario calculation

- Supplemental results

  - Bird priority zones in overlap with other protection areas

  - Bird priority zones as umbrella approach for other species

  - Calamity areas in spruce forests for wind energy use?

  - Comparing the area impact of wind energy and ground-mounted PV

# Habitat model for bird priority zones and species selection

## Species selection

First, we defined for which species bird priority zones should be a promising approach to handle species protection needs in context of further wind energy development. Relevant criteria are the conflict potential (collision risk), the distribution, difficulties in planning, especially generated through a high breeding site variability. For this combination problems during the approval process are highly expected. Widely distributed species (in Brandenburg) with certain difficulties in planning and/or a high collision risk are highlighted in Table 1. Out of these species White-tailed Eagle, Osprey as well as Red and Black Kite provide the conditions for the use of habitat models, since these species show a strong connection to certain habitat parameters, which can be used to define bird priority zones (Table 1).

Table 1: Approach to systematizing species selection for the use of the bird priority zones approaches at the regional planning level. Highlighted species reflect difficulties in planning and/or a high collision risk.

| Criterion species | Particularly high potential for conflict | Wide distribution XX = nationwide X = statewide | Difficulty in planning | High breeding site variability | Good suitability for habitat models | Good knowledge of population |
| --- | --- | --- | --- | --- | --- | --- |
| White-tailed Eagle | x | x |  |  | x | x |
| Osprey | x | x |  |  | x | x |
| Lesser Spotted Eagle | x |  |  |  | x | x |
| Red Kite | x | xx | x | x | x | x |
| White stork |  | x |  |  | x | x |
| Northern Harrier |  |  |  | x |  | x |
| Buzzard |  | xx | x | x |  | x |
| Black Kite |  | xx | x | x | x | x |
| Marsh Harrier |  | X | x | x |  | x |
| Honey Buzzard |  | xx | x | x | x |  |
| Hobby |  | xx | x | x |  |  |

## Data acquisition

For Brandenburg, the Bird Protection Office provided the project with complete presence data in the form of nesting sites for the Osprey and incomplete mapping presence data for the Red Kite. Presence points for the White-tailed Eagle could not be provided according to the responsible staff member and only MTB quadrants (TK10) were provided. Through this request, TK25 data from 2014 DDA ADEBAR mappings for White-Tailed Eagle, Osprey, and Red Kite were obtained. For the Black Kite no data was provided.

## Method

Basically, the identification of potential bird priority zones developed consists of two consecutive work steps. First, the potential range of the target species in the study area is determined, using a logistic regression with presence and ‘pseudo-absence’ locations as the dependent variable and several habitat variables as the explanatory variables. The areas determined are then evaluated using various parameters in order to thus identify the most suitable areas for the designation of focal areas.

### Habitat modelling

#### Distribution data

Red Kite and Osprey presence data could be based on the (mostly) point-recorded breeding locations which were recorded between 2016 and 2021. The amount and the distribution of Osprey breeding pairs in Brandenburg is well known, therefore the set of presence locations is almost complete. In the case of the point data obtained for the Red Kite in Brandenburg, the quality of the data is not optimal, as they come from incomplete or unverifiable mapping. Incomplete means either that the mapped grid cell was not completely searched or also that a large part of the region was not covered with mapping. Non-verifiable means that the points have no or very general background information. Important information such as the time period of the mapping or under which initiative this data was collected is missing, for example. Therefore, all data points were checked for plausibility or duplication. The final analysis contained 1.445 Red kite and 537 Osprey records. The breeding records (presence data) were complemented by an equal number of ‘pseudo-absence’ locations [1]. The distance between these pseudo-absence points and presence points was equal to the average distance between two breeding sites (Red Kite: 1.300m, Osprey: 1.400m).

#### Habitat variables

To identify suitable nest sites, different habitat parameters were consulted as explanatory variables. For the target species, the availability of suitable nest sites (e.g., breeding trees or artificial sites) as well as potential foraging habitats in the surrounding area are of primary importance when selecting a suitable territory [1, 2]. A pre-selection of promising habitat variables was carried out based on already existing habitat models of the Red Kite in Germany [1, 3] as well as information about the ecology of the target species [4, 5].

To map the central demands of the target species, information on different land use classes was mainly used. Information from the European Copernicus Earth Observation Program is available as a database. Copernicus covers up to 44 different land use classes across Europe with a minimum size of 25 ha (Corine Land Cover data). For the land use types grassland, (temporary) water bodies, (temporary) wetlands as well as forest density, significantly higher resolution raster data (20m x 20m) are available. All wooded areas with a crown density of >10 % and more than 0.5ha in size are evaluated as forest.

To further differentiate the quality of individual grassland or open land sites, especially with respect to food availability, additional habitat parameters were used. In this context, the intensity of cultivation represents a measure of the suitability of the areas as field vole habitat. Field mice are the central food source for Red Kites. The occurrence of small linear (hedgerows, tree rows) as well as extensive (>200m - <5,000m²) woody stands was considered as a possible variable that can be used to represent the intensity of use of potential bird habitats. A more frequent subdivision of agriculturally used areas by such structures, indicates a lower intensity of use. Information on the distribution of lakes and the course of high voltage lines were especially gathered for Brandenburg through state geoinformation platforms (Table 2).

Table 2: Data of the environmental variables considered

| Variable | Resolution | Period | Comment | Source |
| --- | --- | --- | --- | --- |
| Corine Land cover | Minimum size of land use classes = 25 ha | 2012 | In total, up to 44 land use types are represented | https://land.copernicus.eu/pan-european/corine-land-cover |
| Grassland | 20 x 20 m | 2015 |  | https://land.copernicus.eu/pan-european/high-resolution-layers/grassland/status-maps |
| Forest | 20 x 20 m | 2015 | All wooded areas with crown density >10 % are taken into account. |  |
| Small woody structures | 5 x 5 m | 2015 | Small linear and flat woody structures | https://land.copernicus.eu/pan-european/high-resolution-layers/small-woody-features |
| Water surfaces | 10 x 10 m | 2015 | Permanent and temporary | https://land.copernicus.eu/pan-european/high-resolution-layers/water-wetness/status-maps/2015 |
| Wetlands | 10 x 10 m | 2015 | Permanent and temporary | https://land.copernicus.eu/pan-european/high-resolution-layers/water-wetness/status-maps/2015 |
| Lakes | > 5 ha | 2011 |  | https://data.geobasis-bb.de/geofachdaten/Wasser/Hydrologie/seen25.zip |
| High voltage lines |  | 2014 |  | https://energieportal-brandenburg.de/cms/inhalte/ ausbaustand/karten/energieinfrastruktur |

The distribution of all relevant landcover types in Brandenburg is displayed in Figure 1.

Figure 1: Distribution of all relevant landcover types in Brandenburg (geodata copyright by CORINE Landcover)

### Radius of main territory

Since the target species are highly mobile raptors that base their breeding site selection on the habitat characteristics of a wide-ranging territory, the habitat potential modeling must not consider only the site conditions at the actual breeding sites. Therefore, the percentage of each land use type within a radius of the presence and pseudo-absence points is determined. The size of the radius is species-specific and is based on the action radii of the target species known from the literature. Analogous to Heuck, Brandl, Albrecht, Gottschalk [1] and Katzenberger [3], a radius of 1.000m was chosen for the Red Kite and the Osprey.

Ospreys mainly hunt at large lake sites which are not necessarily located directly next to the breeding site; therefore, the birds cover distances up to 10km, but prefer territories close to their hunting grounds [4]. For this reason, we included the distance between the (Osprey) nesting sites and larger lakes (>5ha) as an explanatory variable for the Osprey. Potential breeding sites are binary categorial variables which include the information if the certain land use type is suitable for breeding or not. For Red Kites mostly forests and small wooded structures are relevant. However, in Brandenburg, due to a lack of suitable natural nest trees, ospreys breed predominantly on top of high-voltage transmission poles which guarantee an excellent overview.

A grid with a cell size of 200m x 200m was created in order to be able to represent the expression of the individual habitat parameters (percentage share in the 1.000m radius, mean value in the 1.000m radius or distance to the breeding site) over the entire federal state. Based on the centroids of the individual grid cells, the habitat characteristics were determined analogously to the procedure for the presence and pseudo-absence points for the entire grid. The selection and preparation of the various variables were accomplished using Quantum GIS Version 3.14.0 [6] (Table 3).

Table 3: Environmental variables considered for Red Kite habitat potential modeling

| Variable | Type | Value | Comment |
| --- | --- | --- | --- |
| Proportion of grassland within 1.000m radius | Metric | Percent | Determined from the output variable ‘Grassland’ |
| Proportion of arable land within 1.000m radius | Metric | Percent | Determined from Corine land use types of the category "arable land" |
| Proportion of forest area within 1.000m radius | Metric | Percent | Determined from the output variable ‘Forest’ |
| Forest site | Binary | Yes, no | Determined from the output variable ‘Forest’ |
| Proportion of urban areas within a 1000m radius | Metric | Percent | Determined from Corine land use types in the ‘urban’ category |
| Small wooded areas | Binary | Yes, no | Determined from the output variable ‘small woody structures’ |
| Proportion of small wooded areas within 1000m radius | Metric | Percent | Determined from the output variable ‘small woody structures’ |
| Proportion of water areas within 1.000m radius | Metric | Percent | Determined from the output variable ‘water surfaces’ |
| Proportion of temporary wetland areas within a 1.000m radius | Metric | Percent | Determined from the output variable ‘wetlands’ |
| Distance to nearest lake (>5ha) | Metric | Meter |  |
| Distance to nearest high voltage line | Metric | Meter |  |

### Logistic regression

To determine the probability of occurrence of the Red Kite and the Osprey in Brandenburg, habitat models were created. For this purpose, the probability of occurrence, derived from the habitat suitability, was calculated for certain characteristics and constellations of individual or several habitat parameters. Logistic regression is a widely used habitat potential modeling technique for dealing with explanatory variables that can take only two possible values (e.g., presence/absence, breeding success yes/no) [7, 8]. All work was conducted using RStudio 2022.02.3+492 [9].

Univariate logistic regressions were performed for the individual independent variables and a ‘generalised linear model’ (glm) with binominal error distribution and logit link function was generated. The model was then tested for significance (p-value <0.05) and ‘explanatory value’. The proportion of explained variance and thus the usefulness of the model are indicated by the pseudo coefficient of determination R² according to Nagelkerke [10]. R² can take values between 0 and 1, where 1 means that the independent variable (or several variables) predicts 100% of the variance of an occurrence [11].

Since it is not known at the beginning of the investigations whether the relations between the choice of territory and the independent variables are linear (‘sigmoidal’) or whether there is an optimum in the course of the expression (‘unimodal’), both variants were tested. In the present form, the variables correspond to the sigmoidal variant. To test for a possible non-linear dependence, the numerical variables must first be squared [12].

With the help of the regression equation, an occurrence probability can be calculated for each category of a variable or for each value of a metric variable. In addition, the regression coefficient (β) indicates how strongly and in which direction the habitat parameter influences the occurrence. Positive values indicate a positive relationship, i.e., the larger the independent (metric) variable, the greater the probability of occurrence. To better interpret the influence, the odds ratios of the individual variables are calculated, since logistic regression is not a linear function, but a logit function, the natural logarithm of a chance. For this purpose, the coefficients are inserted into the exp() exponential function.

Multivariate models were then created from the pool of significant variables. The use of multiple independent variables which highly correlate with each other, causing multicollinearity and leading to misinterpretation must be prevented. Therefore, we used the car package [13] to calculate the variance inflation factor (VIF). The VIF describes the degree of correlation between variables used in a regression analysis.

To check whether adding another variable significantly improves the model, the models were compared with an ANOVA. A p-value of >0.05 means that expanding the model does not significantly improve the explanation of the total variance [14]. The aim is to generate a model that is as ‘lean’ as possible with a high R² value. In addition, we quantified each model’s prediction accuracy with AUC values using the ROCR package version 1.0-11 [15]. An AUC-Value of 0.5 indicates a random distribution of presence and absence points and a model without any explanatory value. Models with an AUC-Value > 0.7 are considered acceptable [16].

### Identification of the distribution area

The representation of the probability of occurrence (p) in the individual 200 x 200 m grid cells is done with the help of the grid calculator in Quantum GIS. The intercept (intersection of the y-axis) of the regression equation and the determined coefficients (a, b, ...n) of the individual environmental variables (v) are inserted into the following formula.

The values of the environmental variables are stored in the individual grid cells. For each grid cell, the probability of occurrence is between 0 and 1, where 1 represents a certain occurrence based on the model and 0 excludes an occurrence. The values of the quality measures change depending on which threshold value has been set. Several variants are available for this purpose, which are differently suitable for different uses of the model. In addition to R, the program ‘ROC Plotting and AUC Calculation Transferability Test’  in version 1.3 was used for the evaluation [17].

- P_opt: Threshold value with the highest proportion of correct predictions.

- P_fair: Threshold with the most similar values for sensitivity and specificity

Depending on the type of threshold selected, a different classification threshold (P_krit) is obtained. If the occurrence probability of the target species is below this value, the corresponding study area is not considered as potential habitat for this species.

### Identification of the bird priority zones

Out of the calculated potential habitat areas, bird priority zones were determined. As crucial parameters the size and the quality of the habitats were considered. First, all habitats were ranked by their coherent area size. Subsequent, the proportion of highly suitable (>0.75 occurrence probability) habitat area was calculated. Suitable territories attract large numbers of individuals and appropriate abiotic factors (habitat variables) lead to further population growth. By considering the land use types, a high continuity of occurrence can be assumed, so that protection of the identified focal areas can contribute to future conservation of the populations.

Overlapping bird priority zones of different species could be considered as an additional selection criterion, because of the effectiveness of the method. In this case only two target species were considered whose habitats hardly overlap. For other target species like White-tailed eagle and Black Kite overlaps with the distribution of Osprey respectively Red Kite are to be expected. For both target species, Osprey and Red Kite, the most suitable habitat areas were ranked and the Top 10 as well as the Top 5 bird priority zones were used for two different approaches for the multi criteria analysis.

### Results

Out of the tested models the proportion of grassland and the proportion of forest area (unimodal) within the 1.000m radius have the highest R²-values and therefore explain most of the occurrence possibility of the Red Kite in Brandenburg. On the other hand, the occurrence of the Osprey is mostly dependent from the availability of high voltage line poles close to larger lakes (Table 4).

Table 4: Environmental variable and occurrence possibilities for the Red Kite and Osprey in Brandenburg

| Environmental variable | Unit | Odds ratio |  | Nagelkerke R² |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Red Kite | Osprey | Red Kite | Osprey |
| Proportion of Grassland within 1.000m radius | % | 1.94*** | 1.21** | 0.109 | 0.121 |
| Proportion of Arable land within 1.000m radius | % | 1.46*** | 0.97 | 0.088 | 0.007 |
| Proportion of Arable land within 1.000m radius (unimodal) | % | 0.56*** | 0.87 | 0.088 | 0.007 |
| Proportion on Forest area within 1.000m radius | % | 0.74*** | 1.12 | 0.187 | 0.005 |
| Proportion on Forest area within 1.000m radius (unimodal) | % | 0.45*** | 0.92 | 0.187 | 0.005 |
| Forest site | Yes/No | 1.12** | 0.76*** | 0.004 | 0.025 |
| Proportion of Urban area within 1.000m radius | % | 0.74*** | 0.53*** | 0.025 | 0.064 |
| Small wooded area | Yes/No | 2.10*** | not tested | 0.121 | not tested |
| Proportion of Water area within 1.000m radius | % | 1.42*** | 1.56** | 0.011 | 0.014 |
| Proportion of Water area within 1.000m radius (unimodal) | % | 0.95*** | 0.88** | 0.011 | 0.014 |
| Proportion of Temporary water area within 1.000m radius | % | 2.24*** | 1.77*** | 0.064 | 0.031 |
| Proportion of Temporary water area within 1.000m radius (unimodal) | % | 0.75*** | 0.83*** | 0.064 | 0.031 |
| Distance to nearest lake (>5ha) | Meter | 0.79*** | 0.46*** | 0.017 | 0.122 |
| Distance to nearest voltage line | Meter | not tested | 0.50*** | not tested | 0.125 |

The best multivariate model for the red kites contains the variables proportion of grassland, forest area (unimodal), urban area and temporary water area (unimodal) within the 1.000m radius as well as the availability of small wooded areas. The used variables explained 36 % of the variation in the osprey data. The AUC-value of 0.80 of our model indicates a nearly excellent discriminative power [16].

The best multivariate model for the Red Kite contains the variables distance to the nearest lake as well as to the nearest voltage line and the proportion of grassland within the 1.000m radius as well as the availability of small wooded areas. The used variables explained 31% of the variation in the osprey data. The AUC-value of 0.78 of our model indicates a nearly excellent discriminative power [16].

While the Red Kite is widely distributed in Brandenburg, with highly suitable breeding areas around larger grassland concentrations with forest or small wooded areas nearby, the presence of Ospreys is limited by the availability of larger lakes and high voltage line poles in a reasonable distance (Figure 2).

Figure 2: Potential habitats for the Red Kite and Osprey in Brandenburg (geodata copyright by ARSU GmbH)

To designate areas as potential habitats, we calculated habitat suitability thresholds (p_crit) of 0.525 (Red Kite) respectively 0.545 (Osprey). Out of the potential habitat areas the largest and most suitable ones were chosen as ten (five) bird priority zones in Brandenburg (Figure 3).

Figure 3: Bird priority zones for the Red Kite and Osprey based on Top 5 and Top 10 most suitable habitats in Havelland-Fläming region, Brandenburg (geodata copyright by © GeoBasis-DE / BKG (2022), and ARSU GmbH)

Bird priority zones in the region Havelland-Fläming, Brandenburg

Bird priority zones Red Kite Top 10

Bird priority zones Osprey Top 10

All bird priority zones (cumulative)

Overlapping bird priority zones

Bird priority zones Red Kite Top 5

# PESTEL analysis

Table 5: PESTEL analysis for planning criteria employed within regional planning in Havelland-Fläming region, Brandenburg, for which geodata are available

| PESTEL Analysis | Geodata available and used | Legal factors (case-specific) |
| --- | --- | --- |
| Ecological criteria | - Bird priority zones for the Red Kite | - |
|  | - Bird priority zones for the Osprey | - |
|  | - Ground-mounted photovoltaic or vertical ground-mounted photovoltaic for ecological impacts | - |
|  | - Species-specific buffers including protection of species areas | X (Restrictive criterion) |
|  | - Deciduous forest areas | X (Restrictive criterion) |
|  | - Landscape protection areas | X (Soft criterion) |
|  | - Rivers and lakes | X (Restrictive criterion) |
|  | - Forest function areas | X (Soft criterion) |
|  | - Protected forest | X (Restrictive criterion) |
|  | - Nature reserves | X (Hard criterion) |
|  | - SPA | X (Restrictive criterion) |
|  | - FFH areas | X (Restrictive criterion) |
|  | - Nature parks | X (Restrictive criterion) |
|  | - Protected landscape elements | X (Restrictive criterion) |
|  | - Biosphere reserves | - Not available in the case study area |
|  | - National parks | - Not available in the case study area |
|  | - Protected biotopes | X (Restriction criterion) |
| Socio-cultural criteria | - Locations of electricity demand | - |
|  | - Settlement buffers | X (Soft criterion) + 1000m (rounded value for all area categories) X (Hard criterion) + 350m (rounded value for all area categories, area categories can differ) |
|  | - Commercial areas buffers | X (Hard criterion) + 170m |
|  | - Hospitals buffers | X (Soft criterion) + 1800m X (Hard Criterion) + 1000m |
|  | - Military areas | X (Hard criterion) |
|  | - Streets buffers | X (Restrictive criterion) + 20m (rounded value for all street categories) |
|  | - Tracks buffers | X (Restrictive criterion) + 2D (2x149m in this analysis) |
|  | - Airports | X (Hard criterion) |
|  | - Open space areas | X (Hard criterion) |
|  | - Rotary beacon | - Not available in the case study area |
|  | - Water protection areas | X (Restrictive criterion) |
|  | - Flood risk areas | X (Restrictive criterion) |
| Technological criteria | - Installed wind turbines | X (Restrictive criterion) (+ 391 m in this analysis) |
|  | - Installed ground-mounted photovoltaics | X (Hard criterion) |
|  | - Power lines buffers | X (Restrictive criterion) + 400 m |
|  | - Minimum size of areas for wind turbines (25ha) | X (Soft criterion) |
|  | - Maximum size of areas for wind turbines (2.000 ha) | X (Restrictive criterion) |
| Economic criteria | - Repowering of installed wind turbines | - |
|  | - Retrofit of installed wind turbines | - |
|  | - Efficient locations for wind turbines according to the wind speed | - |
|  | - Efficient locations for grid connection and transformer stations | - |
|  | - Raw material extraction areas | X (Soft criterion for priority area) X (Restrictive criterion for reserved area) |
|  | - Agricultural areas | X (Restrictive criterion for priority areas) |
| Political criteria | - Species protection exception in bird priority zones | - |
|  | - Species conservation aid programs to support populations | - |
|  | - Balance between wind turbines and ground-mounted photovoltaic (energy mix) | - |
|  | - Spatial targets and energy targets | Provision by federal wind energy law and state specifications |

Table 6: PESTEL analysis for planning criteria employed within regional planning in Havelland-Fläming region, Brandenburg, for which geodata are not available

| PESTEL Analyse | Geoddata not available *Inconsitent data available, therefore supplemental data has been used | Legal factors (case-specific) |
| --- | --- | --- |
| Ecological criteria | - Natural monuments | X (Restrictive criterion) |
|  | - Wintering and resting areas of disturbance-sensitive migratory birds, bird migration corridors | X (Restrictive criterion) |
|  | - Breeding areas of disturbance-sensitive large bird species, bird migration corridors | X (Restrictive criterion) |
|  | - Breeding areas of endangered and disturbance-sensitive bird species | X (Restrictive criterion) |
|  | - Species-specific buffers | X (Restrictive criterion) |
|  | - Biotope network | X (Restrictive criterion) |
| Socio-cultural criteria | - Residential areas | X (Hard criterion) + 610m X (Soft criterion) + 1.100m |
|  | - Mixed areas | X (Hard criterion) + 350m X (Soft criterion) + 1.110m |
|  | - *Residential areas in the external area, outside settlements | X (Hard criterion) + 350m X (Soft criterion) + 600m |
|  | - Low-flying routes of the German Armed Forces | X (Restrictive criterion) |
|  | - Telecommunications channels | X (Restrictive criterion) |
|  | - Highways | X (Hard criterion) X (Restriction criterion) + 40m |
|  | - Federal and country roads | X (Hard criterion) X (Restriction criterion) + 20m |
|  | - District roads | X (Hard criterion) X (Restriction criterion) + 20m |
|  | - Radars | X (Restrictive criterion) |
|  | - Urban planning (urban land use planning) | X (Hard criterion) |
|  | - Soil and archaeological monuments and surrounding area protection | X (Restrictive criterion) |
|  | - Areas for the preservation of the experiential value of the landscape | X (Restrictive criterion) |
| Technological criteria | - EEG-Basis areas for ground-mounted photovoltaics | - |
|  | - Areas for market-based ground-mounted photovoltaic | - |
| Economic criteria | - | - |
| Political criteria | - Sites species conservation aid programs | - |

# Impact/Uncertainty-Matrix

In order to structure the criteria and make a selection for the scenarios, the detailed impact/uncertainty matrix is shown in Figure 4.

Figure 4: Impact/uncertainty matrix for analyzing criteria as ‘key’ for combining them later in scenarios as profile curves. The selected criteria are referred to as criteria of ‘critical uncertainties’, which are evaluated within the scenario analysis.

# Data sources for GIS modeling

For the analysis, most of the geospatial data could be obtained free of charge from publicly available sources. This made it possible to reproduce the planning concept for wind energy in the region for later variation of the scenarios. Especially in the field of 'ecology', protected areas and biotype types cover most of the criteria, such as forest functions, forest types, nature protection areas, landscape protection areas. Also, for the 'socio-cultural' criteria, protective buffers around settlements, commercial areas and hospitals can be mapped by accessing public Open Street Map data. Geospatial resources on existing wind energy and PV facilities in the region complete the map.

### Data generated externally

Table 7: Data generated externally

| External GIS data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Data set | Original name | Type | Source | Comment | Date |
| Bird priority zones Osprey Top 10 | Bird priority zones osprey Top 10 | Shape | ARSU GmbH | Bird priority zones determined on the basis of a habitat potential model | 2022 |
| Bird priority zones Red Kite Top 10 | Bird priority zones red kite Top 10 | Shape | ARSU GmbH | Bird priority zones determined on the basis of a habitat potential model | 2022 |
| Bird priority zones Red Kite Top 5 | Bird priority zones red kite Top 5 | Shape | ARSU GmbH | Bird priority zones determined on the basis of a habitat potential model | 2022 |

### Geoportals

Table 8: Geoportals referred to for planning criteria in the Havelland-Fläming region, Brandenburg

| Geoportals |  |
| --- | --- |
| Name | Link |
| Geobroker BB | Https://geobroker.geobasis-bb.de/ |
| Metaver BB | Https://metaver.de/datenkataloge?Opennodes=1-2ddb2d82267087ce442c1a4a3d972c06,2-6f76d4508417ca51067bed2cbc7a4f23,3-945db2f8c2a61d947b5a973fb51d0291 |
| Digitale Geodaten Bund | Https://gdz.bkg.bund.de/index.php/default/digitale-geodaten.html |
| Geofabric OSM | Http://download.geofabrik.de/europe/germany.html |

### Geodata for Germany

Table 9: Geodata for Germany referred to for planning criteria in the Havelland-Fläming region, Brandenburg

| Geodata | Germany |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data set | Original name | Type | Source | Link | Age | Download |
| Corine Landcover data (Mixed forest, coniferous forest, deciduous forest, airports) | CORINE Land Cover 5 ha (CLC5 2018) | Shape | Digitale Geodaten Bund | Https://gdz.bkg.bund.de/index.php/default/catalog/product/view/id/1071/s/corine-land-cover-5-ha-stand-2018-clc5-2018/category/8/?___store=default | 2018 | 8.2.22 |
| Administrative boundaries | Verwaltungsgrenzen | Shape | Digitale Geodaten Bund | Https://gdz.bkg.bund.de/index.php/default/digitale-geodaten/verwaltungsgebiete/verwaltungsgebiete-1-250-000-ebenen-stand-01-01-vg250-ebenen-01-01.html | 2021 | 8.2.22 |
| Brandenburg OSM data (Streets, settlements, hospitals, commercial areas, railways, industrial areas) | Brandenburg latest free shape | Shape | Geofabric | Http://download.geofabrik.de/europe/germany/brandenburg.html | 2022 | 9.2.22 |
| Digital Landscape Model (DLM250) (Powerlines) | DLM250 | XML | Digitale Geodaten Bund | Http://gdz.bkg.bund.de/index.php/default/open-data/digitales-landschaftsmodell-1-250-000-kompakt-dlm250-kompakt.html | 2021 | 5.4.22 |

### Geodata sources for the state Brandenburg

Table 10: Geodata for the state of Brandenburg referred to for planning criteria in the Havelland-Fläming region, Brandenburg

| Geodata sources | State Brandenburg |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data set | Original name | Type | Sources | Link | Date | Download |
| Wind turbines | WKA | Shape | Geobroker | Https://geobroker.geobasis-bb.de/gbss.php?MODE=getproductinformation&PRODUCTID=45C506E5-3E9D-4DE2-9073-C3DB636CE7CF | 2021 | 7.2.22 |
| FFH areas | Fauna-Flora-Habitat (FFH)-Gebiete des Landes Brandenburg | Shape | Geobroker | Https://geobroker.geobasis-bb.de/gbss.php?MODE=getproductinformation&PRODUCTID=7DE3A549-769C-4F01-A5E6-B3E25D40975E | 2017 | 7.2.22 |
| SPA | Vogelschutzgebiete (SPA) des Landes Brandenburg | Shape | Geobroker | Https://geobroker.geobasis-bb.de/gbss.php?MODE=getproductinformation&PRODUCTID=F88F1BEB-FD2C-41AE-B3A4-94711747DA7D | 2014 | 7.2.22 |
| Protection areas | Schutzgebiete nach Naturschutzrecht des Landes Brandenburg | Shape | Geobroker | Https://geobroker.geobasis-bb.de/gbss.php?MODE=getproductinformation&PRODUCTID=AB2F53A4-A68E-413F-84C4-A972D2A2DA0B | 2020 | 7.2.22 |
| Biotope types 2009 (transformator stations) | CIR-Biotoptypen 2009 (Luftbildinterpretation) - Flächendeckende Biotop- und Landnutzungskartierung im Land Brandenburg (BTLN) | Shape | Geobroker | Https://geobroker.geobasis-bb.de/gbss.php?MODE=getproductinformation&PRODUCTID=B57B9F35-AFFF-49F2-BA32-618D1A1CD412 | 2009 | 7.2.22 |
| Biotopes, protected biotopes | Biotope, geschützte Biotope (§ 30 Bnatschg und § 18 BBgnatschg) und FFH-Lebensraumtypen im Land Brandenburg | Shape | Geobroker | Https://geobroker.geobasis-bb.de/gbss.php?MODE=getproductinformation&PRODUCTID=A061BB02-70AC-4422-BB58-4A49F585D7F2 | 2021 | 7.2.22 |
| Water protection areas | Wasserschutzgebiete des Landes Brandenburg (wsg.shp) | Shape | Geobroker | Https://geobroker.geobasis-bb.de/gbss.php?MODE=getproductinformation&PRODUCTID=657B712B-9009-49C0-8C91-A373AA87291A | 2022 | 7.2.22 |
| Water network | Gewässernetz des Landes Brandenburg | Shape | Geobroker | Https://geobroker.geobasis-bb.de/gbss.php?MODE=getproductinformation&PRODUCTID=B9D461F1-99A1-4C10-97B4-9C36C0BD40B9 | 3.11.16 | 7.2.22 |
| Lakes | Seen im Land Brandenburg | Shape | Geobroker | Https://geobroker.geobasis-bb.de/gbss.php?MODE=getproductinformation&PRODUCTID=D9C4E283-00C3-42A2-9F1F-15BFD6A40B55 | 2021 | 7.2.22 |
| Sensitive peatlands | Sensible Moore des Landes Brandenburg | Shape | Geobroker | Https://geobroker.geobasis-bb.de/gbss.php?MODE=getproductinformation&PRODUCTID=F1C8BE78-6BB4-4D13-9C29-F523E690209B | 2008 | 7.2.22 |
| Forest functions | Waldfunktionen | WFS | MLUK | Https://mluk.brandenburg.de/mluk/de/service/geoinformationen/geodaten-fachbereiche/# | 2011 | 8.2.22 |
| Flood risk areas | Festgesetzte Überschwemmungsgebiete des Landes Brandenburg | Shape | Geobroker/Metaver | Https://metaver.de/search/dls/#?Serviceid=05EC61E6-C81E-4616-ACE6-2DC3D5E67E24&datasetid=05EC61E6-C81E-4616-ACE6-2DC3D5E67E24%3fdatasetuuid%3D830D525C-66F1-4112-9EF5-E8341685CC9E | 08.03.22 | 27.04.22 |
| Site designations in Regional Plan 3.0 Havelland-Fläming (Reserved areas for settlements, open space network, suitable areas for wind energy use, raw material extraction, reserved areas for raw material extraction, priority areas for agriculture) | Auslegungsunterlagen zum Regionalplan Havelland-Fläming 3.0 | WFS | Havelland-Flaeming.de | Https://havelland-flaeming.de/regionalplan/regionalplan-3-0/beteiligungsverfahren/ | 05.10.21 | 28.04.22 |
| Solar energy | DLM250 | Shape | Digitale Geodaten Bund |  |  |  |
| Meadow breeders | Wiesenbrüter | Shape | Metaver | https://metaver.de/trefferanzeige?cmd=doShowDocument&docuuid=10685606-1EE8-49F2-A31C-A40FF86E720F&plugid=/ingrid-group:ige-iplug-bb |  |  |
| Birds Landscape Program 2009 (Meadow breeders) | Vögel LaPro BB | Shape | Oeko Log | http://www.oeko-log.com/Materialien17.html |  |  |
| Landscape Program 2009 | Landschaftsprogramm des Landes Brandenburg | Shape | Metaver | https://metaver.de/trefferanzeige?cmd=doShowDocument&docuuid=0F633D35-7CDE-4A64-BE2F-01156C39180C&plugid=/ingrid-group:ige-iplug-bb |  |  |

# GIS method for scenario preparation

1 Data pre-processing

- Target: Data are prepared by clipping shapes regarding the individual land uses to the Havelland-Fläming region and by calculating the area sizes in km2

- Steps

  - Displaying administrative boundaries shapes

  - Data not specific to the area are selected (data of Brandenburg state)

  - Geometry tool clip for clipping the shapes for area uses to the Havelland-Fläming region

  - Field calculator for calculating the area size in km2 ($Area/1000000) for polygon layers

  - Field calculator for calculating the sum of the area in km2 for the shape file/land use in total (sum($Area/1000000)

- Target: Calculating a buffer zone around wind turbines installed, approved and planned, for analyzing installed wind turbines in the region

- Steps

  - Determining the size of a buffer zone around wind turbines using reference to a general estimate of 4,5 D in all directions [18]

  - Determining the general rotor diameter of wind turbines already installed in the region

    - Field calculator for selecting wind turbines bigger than 3,5 MW

    - Field calculator for calculating the mean of the rotor diameter (mean(ROTORDURCH))

  - Determining the general buffer estimate using 4,5 D in every direction

    - Field calculator (4,5*mean(ROTORDURCH)) (86,823 m)

  - Buffering already installed, approved and planned wind turbines for the region

    - Buffer with 390,702 m as radius (rounded 391)

- Target: Clipping and calculating bird priority zones to the Havelland-Fläming region

- Steps

  - Displaying bird priority zones shapes

  - Geometry tool clip for clipping the shapes for area uses to the Havelland-Fläming region

  - Field calculator for calculating the area size in km2 ($Area/1000000)

  - Field calculator for calculating the sum of the area in km2 for the shape file/land use in total (sum($Area/1000000))

- Target: Generating cumulative and overlapping bird priority zones approaches

- Steps

Geometry tool merge for merging the shapes of Red Kite Top 5 and 10 bird priority zones and Osprey Top 10 bird priority zones

  - Field calculator for calculating the sum of the area in km2 for the shape file/land use in total (sum($Area/1000000))

Gemetry tool clip for identifying overlapping bird priority zones for the shapes of Red Kite Top 5 and 10 bird priority zones and Osprey Top 10 bird priority zones

  - Field calculator for calculating the sum of the area in km2 for the shape file/land use in total (sum($Area/1000000))

- Target: Buffering criteria according to state requirements of Brandenburg

- Steps

  - Buffering shapes when there are state requirements towards potential buffer zones (settlements, hospitals, streets, tracks, water protection areas, commercial areas, power lines) [19]

  - Using buffer values according to state requirements

  - Clipping the buffers to the area of the Havelland-Fläming region

  - Calculating the area size of buffers using the field calculator (sum($Area/1000000))

- Target: Identifying restrictions on data and supplemental data

  - Shape Vorbehaltsgebiete settlements only includes residential areas, areas for commerce are not included. Also Potsdam city is not included.

  - Therefore OMS data for residential areas is used. Distinctive uses, such as castle Sanssouci in Potsdam city is not included in this data set, as well as parking lots. Certain areas for commerce are also not included.

  - An additional shape for areas for commerce is employed, having different requirements for buffer zones.

2 Data processing scenarios for more complex calculations, see graphical model for all model calculations

- Target: Scenario 1: Wind energy in BB state according to business-as-usual

- Steps

  - Criteria are excluded according to the states requirements (see PESTEL analysis)

  - Merging shapes of planning specifications

  - Difference between the region layer and the new layer merging planning specifications

  - Splitting layer to individual shapes for calculation of area sizes

  - Calculating the areas size of the remaining areas using field calculator

- Target: Scenario 2: Wind energy in bird priority zones

- Steps

  - Criteria are excluded according to the states requirements (see PESTEL analysis) and bird priority zones shapes

  - Merging shapes of planning specifications

  - Difference between the region layer and the new layer merging planning specifications

  - Splitting layer to individual shapes for calculation of area sizes

  - Calculating the areas size of the remaining areas using field calculator

  - Selecting areas with a minimum area of 2500m2 using select function

- Target: Scenario 3 and 4: Repowering and Retrofitting in bird priority zones

- Steps

  - Using table of contents to identify installed wind energy in bird priority zones and attributed capacity

- Target: Scenario 9: Efficient areas for wind energy

- Steps

  - Identifying areas of wind speed with >5.5 m/sec within the wind speed shape using table of contents

  - Selecting areas of higher wind speed as shape layer

  - Clipping shape layer to bird priority zones layer

  - Field calculator for calculating the area size in km2 ($Area/1000000)

  - Field calculator for calculating the sum of the area in km2 for the shape file/land use in total (sum($Area/1000000))

- Target: Scenario 10: Higher electricity demand

- Steps

  - Buffering settlements layer to 1000m

  - Buffering settlements layer to 800m

  - Clipping settlement buffer of 1000m layer to bird priority zones layers

  - Geometry tool difference between clipped settlement buffer of 1000m and 800m buffer to settlements

  - Splitting layer to individual shapes for calculation of area sizes

  - Calculating the areas size of the remaining areas using field calculator

  - Selecting areas with a minimum area of 2500m2 using select function

- Target: Scenario 11: Energy efficiency

- Steps

  - Clipping transformer stations file from bird priority zones files

  - Splitting layer to individual shapes for calculation of area sizes

  - Calculating the areas size of the remaining areas using field calculator

  - Selecting areas with a minimum area of 2500m2 using select function

- Target: Scenario 12: Ground-mounted PV

- Steps

  - Using the grid function to generate a grid with a diameter of 5km

  - Clipping the grid to the region size

  - Using table of contents to identify number of grid cells in the region (70 cells)

  - Using field calculator to calculate an equal distribution of power capacity according to [20] (3807 MWp on subsidizes EEG areas for PV / grid cells = 54.3857 MWp per grid cell) (4969 MWp on agricultural areas for PV / grid cells = 70.9857 MWp per grid cell)

  - Using field calculator to attribute calculated power capacity to each grid cell

  - Clipping grid cells to bird priority zones layers

  - Using field calculator to calculate the power potential of cut grid cells in km2

  - Dividing the result by 2 due to edges

Figure 5: Pre-Processing of data (planning criteria) for Havelland-Fläming region, Brandenburg using graphical modelling in QGIS, part 1

Figure 6: Pre-Processing of data (planning criteria) for Havelland-Fläming region, Brandenburg using graphical modelling in QGIS, part 2

Figure 7: Scenario models and re-check of area usability using planning criteria within the Havelland-Fläming region, Brandenburg

# Excel model for scenario calculation

Please refer to the Excel sheet provided.

# Supplemental results

## Bird priority zones in overlap with other protection areas

### Method

For supplemental results, GIS was also employed to determine how bird priority zones approaches in the region overlapped with existing categories of protected areas, such as Special Protection Areas (SPAs) and nature reserves, to determine whether the protection categories are spatially complementary. For this purpose, the areas were overlaid in GIS using the difference function.

### Results

The bird priority zones approaches overlap in the north of the region in part with SPA areas to 61.72%. For nature reserves, the overlapped area is very small and heterogeneous in the north with Red Kite Top 5 areas, but otherwise no clear overlapping areas occur, making bird priority zones rather complementary as a protected area category (Figure 8).

a)

b)

Figure 8: Overlap of bird priority zone with SPA areas (a), and nature reserves (b) (geodata copyright by © GeoBasis-DE / BKG 2022, ARSU GmbH 2022, Regional Planning Authority Havelland-Fläming 2022 (RPG 2022), OSM 2022, Geoportal LGB 2022)

## Bird priority zones as umbrella approach for other species

### Method

It was investigated whether the bird priority zones approaches in the region could provide an umbrella effect for other species. Additional bird occurrence datasets in the region were examined to identify overlaps with the bird priority zones approaches. Since bird data are generally not publicly available and often only available as raster data, this overlay analysis could be performed for the white-tailed eagle. For other species, public geodata on land use are available based on the 2009 Landscape Program. While acknowledging that these datasets are dated, the proposed bird priority zones approaches were overlayed for the following bird areas: wintering and resting areas, large bird areas, core and connecting areas for the great bustard, and areas for grassland nesting birds.

### Results

Whether bird priority zones can also develop potential welfare effects for other species that are not explicitly addressed as target species, the overlay analysis yields different results. The overlay with raster data for the Sea eagle finds a large proportion of White-tailed eagle point data in bird priority zones. However, since these are raster data with different numbers of breeding occurrences per raster, all breeding occurrences were counted. In the Havelland-Fläming region there are 31 out of 45 point data in grid cells in bird priority zones. This corresponds to 68.89%. Since breeding sites can also be located outside bird priority zones due to split grid cells at the edge of bird priority zones, it is assumed that about 60% of the Sea eagle point data could be located in bird priority zones. This equates to 18.6 possible points. In total, this represents approximately 41.33% of the total White-tailed eagle points in grids in the region (Figure 9).

Figure 9: Overlay analysis of bird priority zones for the Red Kite and Osprey with raster cells of breeding occurrences for the White-tailed eagle (geodata copyright by © GeoBasis-DE / BKG 2022, ARSU GmbH 2022, Regional Planning Authority Havelland-Fläming 2022 (RPG 2022), LfU 2022)

For other area categories of species occurrence, there are mainly region-specific overlaps in the North of the region, affecting the Red Kite Top 5 and Osprey Top 10 bird priority zones approaches. Large bird areas according to the Landscape Program are scattered throughout the country and thus partially coincide with all bird priority zones, but not explicitly in form. No correlation can be found here. Wintering and resting bird areas are also distributed across the region and are found in the west, where there are no bird priority zones. Areas are also found in the bird priority zones, but not explicitly. For Great Bustard core and connecting areas, there is overlap in the north with Red Kite Top 5 and Osprey Top10 bird priority zones, presumably due to the greater habitat suitability of these areas. Otherwise, there is a concentration to the west and thus no unilateral overlap. For grassland nesting birds, there are also overlaps in the north with the Red Kite Top 5 bird priority zones and partly with the Osprey Top 10 bird priority zones in the south. Therefore, the Red Kite Top 10 bird priority zones are mostly not affected (Figure 10).

a)

b)

c)

d)

Figure 10: Overlay analysis of bird priority zones for the Red Kite and Osprey with a) connecting areas for the Great Bustard, b) large bird areas, c) grass land nesting bird areas, and d) wintering and resting bird areas (geodata copyright by © GeoBasis-DE / BKG 2022, ARSU GmbH 2022, Regional Planning Authority Havelland-Fläming 2022 (RPG 2022), LfU 2009, LfU 2022)

## Calamity areas in spruce forests for wind energy use?

As a result of climate change, forests, especially spruce forests, have been damaged and are being considered as calamity areas for developing wind energy [21–23]. To meet the 2.2% wind energy spatial target, an additional 4% development in coniferous forests that have already been degraded could be envisaged in this case study. However, the share of spruce forests and calamity areas in Brandenburg is with 80 km2 [24], or with an even distribution in the region Havelland-Fläming with 1.33% too low to realize this necessary share of area in pre-damaged forest areas.

## Comparing the area impact of wind energy and ground-mounted PV

For wind energy and PV in the energy mix, a different land footprint arises for the respective energy sources [18, 25–27]. Although the energy targets can be achieved for PV and it can also substitute for the still missing development dimensions for wind energy in this case, the impact on landscape is larger due to module coverage [28]. Likewise land uses are still possible under wind turbines, such as agriculture [29, 30]. As the scenarios for PV are based on the assumption of an even distribution according to the state area analysis in this case [20], this may result in additional interlinkages regarding the location of the bird priority zones.

# References

- 1.	Heuck C, Brandl R, Albrecht J et al. (2013) The potential distribution of the Red Kite in Germany. J Ornithol 154:911–921. https://doi.org/10.1007/s10336-013-0955-2

- 2.	Gelpke C, Hormann M (2010) Artenhilfskonzept Rotmilan (Milvus milvus) in Hessen: Gutachten im Auftrag der Staatlichen Vogelschutzwarte

- 3.	Katzenberger J (2019) Verbreitungsbestimmende Faktoren und Habitateignung für den Rotmilan Milvus milvus in Deutschland. Die Vogelwelt 139

- 4.	Bauer HG, Betzel E, Fiedler W (2005) Das Kompendium der Vögel Mitteleuropas: Alles über Biologie, Gefährdung und Schutz - Nonpasseriformes - Nichtsperlingsvögel. Aula-Verlag

- 5.	Stiftung Vogelmonitoring Deutschland; Dachverband Deutscher Avifaunisten (2014) Atlas Deutscher Brutvogelarten: Atlas of German breeding birds, 1. Aufl. Stiftung Vogelmonitoring Deutschland; Dachverband Deutscher Avifaunisten, Münster, Westf., Münster

- 6.	QGIS Development Team (2023) QGIS Geopraphic Information System. Open Source Geospatioal Foundation Project. https://www.qgis.org/en/site/

- 7.	Schröder B, Reineking B (2004) Modellierung der Art-Habitat-Beziehung - ein Überblick über die Verfahren der Habitatmodellierung

- 8.	Kuhn W, Kleyer M (2000) A statistical habitat model for the blue winged grasshopper (Oedipoda caerulescens) considering the habitat connectivity. Zeitschrift für Ökologie und Naturschutz 8:207–2018

- 9.	RStudio Team (2023) RStudio: Integrated Developent for R., Boston. https://posit.co/

- 10.	Mangiafico S (2016) Summary and analysis of extension program evaluation in R, version 1.18. 1. Rutgers Cooperative Extension

- 11.	Bruin J (2006) Stata Programs for Data Analysis: Newtest: command to compute new test.

- 12.	Leyer I, Wesche K (2008) Multivariate Statistik in der Ökologie: Eine Einführung, Korr. Nachdr. Springer-Lehrbuch. Springer, Berlin, Heidelberg

- 13.	Fox J, Weisberg S (2011) An R companion to applied regression. Sage Publications

- 14.	Dormann CF, Frund J, Bluthgen N et al. (2009) Indices, Graphs and Null Models: Analyzing Bipartite Ecological Networks. TOECOLJ 2:7–24. https://doi.org/10.2174/1874213000902010007

- 15.	Sing T, Sander O, Beerenwinkel N et al. (2005) ROCR: visualizing classifier performance in R. Bioinformatics 21:3940–3941. https://doi.org/10.1093/bioinformatics/bti623

- 16.	Hosmer DW, Lemeshow S, Sturdivant RX (2013) Applied logistic regression, 3. ed. Wiley series in probability and statistics. Wiley, Hoboken, NJ

- 17.	Schröder B (2004) ROC Plotting and AUC Calculation Transferability Test. Version 1.3

- 18.	Forschungsstelle für Energiewirtschaft (2022) 2 % der Landesfläche für Windenergie: ein geeignetes Maß?: FfE Discussion Paper 2022-01

- 19.	Regionale Planungsgemeinschaft Havelland-Fläming (2019) Planungskonzept zur Steuerung der Windenergienutzung im Regionalplan Havelland-Fläming 3.0: Die Anwendung der Siedlungsabstände und der Tierökologischen Ab- standskriterien und ihre Auswirkungen auf Bestandsanlagen und die räumliche Verteilung von Potenzialflächen für die Windenergienutzung

- 20.	Energieagentur Brandenburg (2022) Solaratlas Brandenburg: Freiflächen. https://solaratlas-brandenburg.de/cms/inhalte/meine-kommune/dachflaechen/position/52.473,13.229,8

- 21.	Fachagentur Windenergie an Land (2022) Entwicklung der Windenergie im Wald: Ausbau, planerische Vorgaben und Empfehlungen für Windenergiestandorte auf Waldflächen in den Bundesländern

- 22.	Bundesministerium für Ernährung und Landwirtschaft (2021) Waldbericht der Bundesregierung 2021

- 23.	Burger R, Lucka S (2022) Im Auge des Energiewendesturms. https://www.faz.net/aktuell/politik/inland/windkraft-warum-vogelschuetzer-in-nrw-windraeder-verhindern-wollen-18348658.html

- 24.	Ministerium für Landwirtschaft, Umwelt und Klimaschutz Brandenburg (2021) Waldzustandsbericht 2021 des Landes Brandenburg

- 25.	Wirth H (2022) Aktuelle Fakten zur Photovoltaik in Deutschland

- 26.	Wirth H, Kost C, Kramer K et al. (2022) Solaroffensive für Deutschland: Wie wir mit Sonnenenergie einen Wirtschaftsboom entfesseln und das Klima schützen

- 27.	Chiabrando R, Fabrizio E, Garnero G (2009) The territorial and landscape impacts of photovoltaic systems: Definition of impacts and assessment of the glare risk. Renewable and Sustainable Energy Reviews 13:2441–2451. https://doi.org/10.1016/j.rser.2009.06.008

- 28.	Kim JY, Koide D, Ishihama F et al. (2021) Current site planning of medium to large solar power systems accelerates the loss of the remaining semi-natural and agricultural habitats. Sci Total Environ 779:146475. https://doi.org/10.1016/j.scitotenv.2021.146475

- 29.	Amaducci S, Yin X, Colauzzi M (2018) Agrivoltaic systems to optimise land use for electric energy production. Applied Energy 220:545–561. https://doi.org/10.1016/j.apenergy.2018.03.081

- 30.	Efremov C, Sanduleac M, Braga D (2022) Challenges and Opportunities of Agri-PV systems in a Clean Energy Transition for Rural Areas. In: IEEE (ed) 2022 International Conference and Exposition on Electrical And Power Engineering (EPE). IEEE, pp 556–561