Supporting Information

Vapor-Liquid and Chemical Equilibrium for Esterification of Acetic Acid + Isopropanol + Ionic Liquid [HSO3-bmim][HSO4] at 101.33 kPa

Yimin Guo, Yun Zou, Kelei Huang, Qing Li, Zhangfa Tong*

Guangxi Key Laboratory of Petrochemical Resource Processing and Process Intensification Technology, School of Chemistry and Chemical Engineering, Guangxi University, Nanning 530004, China

*Corresponding author: zftong@gxu.edu.cn (Z. Tong)

Fig. S1. Effect of 0.01mol [HSO3-bmim][HSO4] on the reaction rate of HAc at 353.15K

Table S1. Antoine parameters for pure components

| Component | A | B | C | references |
| --- | --- | --- | --- | --- |
| water | 7.07400 | 1657.459 | 46.130 | [29] |
| acetic acid | 6.57290 | 1572.320 | 46.777 | [29] |
| isopropanol | 6.86100 | 1357.427 | 75.814 | [11] |
| isopropyl acetate | 6.46612 | 1436.530 | 39.485 | [11] |

Table S2. Experimental VLE data of water (1) + acetic acid (2) at 101.33kPaa

| NO. | T/K | x1 | y1 | γ1 | γ2 |
| --- | --- | --- | --- | --- | --- |
| 1 | 373.08 | 1.0000 | 1.0000 | - | - |
| 2 | 373.57 | 0.9762 | 0.9819 | 0.9911 | 1.3344 |
| 3 | 373.98 | 0.9420 | 0.9582 | 0.9878 | 1.2471 |
| 4 | 374.52 | 0.9000 | 0.9296 | 0.984 | 1.1962 |
| 5 | 374.98 | 0.8605 | 0.9024 | 0.983 | 1.1705 |
| 6 | 375.50 | 0.8160 | 0.8750 | 0.9868 | 1.1169 |
| 7 | 375.98 | 0.7724 | 0.8443 | 0.9891 | 1.1068 |
| 8 | 376.53 | 0.7193 | 0.8077 | 0.9967 | 1.0882 |
| 9 | 377.18 | 0.6647 | 0.7671 | 1.0014 | 1.0798 |
| 10 | 377.99 | 0.6125 | 0.7286 | 1.0036 | 1.06 |
| 11 | 378.66 | 0.5554 | 0.6859 | 1.0173 | 1.0593 |
| 12 | 379.39 | 0.4919 | 0.6296 | 1.0289 | 1.0537 |
| 13 | 380.13 | 0.4401 | 0.5816 | 1.0357 | 1.0543 |
| 14 | 380.92 | 0.3784 | 0.5227 | 1.0538 | 1.0559 |
| 15 | 382.35 | 0.2948 | 0.4421 | 1.0659 | 1.0388 |
| 16 | 383.11 | 0.2580 | 0.3950 | 1.0846 | 1.0449 |
| 17 | 384.19 | 0.1974 | 0.3254 | 1.1063 | 1.0406 |
| 18 | 385.56 | 0.1397 | 0.2411 | 1.1266 | 1.0457 |
| 19 | 386.99 | 0.0917 | 0.1690 | 1.1476 | 1.0369 |
| 20 | 388.52 | 0.0419 | 0.0883 | 1.2483 | 1.0283 |
| 21 | 391.10 | 0.0000 | 0.0000 | - | - |

a Standard uncertainties: u(T) = 0.01 K, u(P) = 0.01 kPa, u (x1) =0.001, u(y1) = 0.001

Table S3. Experimental VLE data of isopropanol (1) + water (2) at 101.33kPaa

| NO. | T/K | x1 | y1 | γ1 | γ2 |
| --- | --- | --- | --- | --- | --- |
| 1 | 373.08 | 0.0000 | 0.0000 | - | - |
| 2 | 371.23 | 0.0033 | 0.0511 | 8.5034 | 1.0202 |
| 3 | 368.86 | 0.0076 | 0.1129 | 8.3379 | 1.0446 |
| 4 | 366.52 | 0.0108 | 0.1817 | 8.1121 | 1.0575 |
| 5 | 363.56 | 0.0185 | 0.2547 | 7.8243 | 1.0863 |
| 6 | 360.97 | 0.0265 | 0.3224 | 7.5091 | 1.1025 |
| 7 | 359.10 | 0.0398 | 0.3793 | 7.3138 | 1.0975 |
| 8 | 357.90 | 0.0541 | 0.4273 | 6.6585 | 1.0763 |
| 9 | 356.98 | 0.0731 | 0.4618 | 5.9306 | 1.0657 |
| 10 | 356.44 | 0.0923 | 0.4802 | 4.9897 | 1.0738 |
| 11 | 356.18 | 0.1202 | 0.4871 | 3.9269 | 1.1044 |
| 12 | 355.88 | 0.1493 | 0.4967 | 3.2626 | 1.1343 |
| 13 | 355.55 | 0.1824 | 0.5030 | 2.7402 | 1.1809 |
| 14 | 355.27 | 0.2338 | 0.5097 | 2.1907 | 1.2570 |
| 15 | 354.93 | 0.2631 | 0.5168 | 2.0009 | 1.3057 |
| 16 | 354.56 | 0.3423 | 0.5406 | 1.6329 | 1.4117 |
| 17 | 354.26 | 0.4198 | 0.5591 | 1.3937 | 1.5544 |
| 18 | 353.92 | 0.5369 | 0.6066 | 1.1987 | 1.7616 |
| 19 | 353.74 | 0.6800 | 0.6793 | 1.0676 | 2.0934 |
| 20 | 354.05 | 0.8442 | 0.8060 | 1.0076 | 2.5687 |
| 21 | 355.59 | 1.0000 | 1.0000 | - | - |

a Standard uncertainties: u(T) = 0.01 K, u(P) = 0.01 kPa, u (x1) =0.001, u(y1) = 0.001.

Table S4. Experimental VLE data of acetic acid (1) + isopropyl acetate (2) at 101.33kPaa

| NO. | T/K | x1 | y1 | γ1 | γ2 |
| --- | --- | --- | --- | --- | --- |
| 1 | 391.10 | 1.0000 | 1.0000 | - | - |
| 2 | 388.70 | 0.9599 | 0.9189 | 1.0288 | 0.9101 |
| 3 | 386.57 | 0.9207 | 0.8443 | 1.0531 | 0.9364 |
| 4 | 384.31 | 0.8734 | 0.7599 | 1.0731 | 0.9628 |
| 5 | 382.88 | 0.8289 | 0.6857 | 1.0679 | 0.9705 |
| 6 | 381.18 | 0.7823 | 0.6153 | 1.0725 | 0.9795 |
| 7 | 379.40 | 0.7304 | 0.5426 | 1.0734 | 0.9893 |
| 8 | 377.97 | 0.6857 | 0.4836 | 1.0681 | 0.9982 |
| 9 | 376.46 | 0.6338 | 0.4223 | 1.0609 | 1.0014 |
| 10 | 375.05 | 0.5843 | 0.3682 | 1.0518 | 1.0054 |
| 11 | 373.52 | 0.5213 | 0.3052 | 1.0290 | 1.0045 |
| 12 | 371.71 | 0.4710 | 0.2561 | 1.0165 | 1.0271 |
| 13 | 370.42 | 0.4115 | 0.2066 | 0.9812 | 1.0237 |
| 14 | 369.20 | 0.3517 | 0.1630 | 0.9449 | 1.0173 |
| 15 | 367.90 | 0.3018 | 0.1311 | 0.9269 | 1.0203 |
| 16 | 366.89 | 0.2426 | 0.0985 | 0.8977 | 1.0066 |
| 17 | 365.87 | 0.1913 | 0.0720 | 0.8628 | 1.0016 |
| 18 | 364.88 | 0.1315 | 0.0452 | 0.8163 | 0.9896 |
| 19 | 363.68 | 0.0778 | 0.0237 | 0.7553 | 0.9895 |
| 20 | 362.57 | 0.0380 | 0.0106 | 0.7199 | 0.9956 |
| 21 | 361.31 | 0.0000 | 0.0000 | - | - |

a Standard uncertainties: u(T) = 0.01 K, u(P) = 0.01 kPa, u (x1) =0.001, u(y1) = 0.001.

Table S5. Maximum absolute deviations and mean absolute deviations of vapor- phase mole fraction for ternary systems

| System |  | Maximum absolute deviations |  |  | Mean absolute deviations |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Δmaxy1 |  |
| Water (1) + acetic acid (2) + [HSO3-bmim][HSO4] (3) | x3=0 |  | 0.0110 |  | 0.0037 |
|  | x3=0.01 |  | 0.0137 |  | 0.0050 |
|  | x3=0.02 |  | 0.0049 |  | 0.0020 |
|  | x3=0.03 |  | 0.0071 |  | 0.0062 |
| Isopropanol (1) + water (2) + [HSO3-bmim][HSO4] (3) | x3=0 |  | 0.0032 |  | 0.0008 |
|  | x3=0.01 |  | 0.0047 |  | 0.0010 |
|  | x3=0.02 |  | 0.0040 |  | 0.0011 |
|  | x3=0.03 |  | 0.0060 |  | 0.0013 |
| Acetic acid (1) + isopropyl acetate (2) +[HSO3-bmim][HSO4] (3) | x3=0 |  | 0.0052 |  | 0.0014 |
|  | x3=0.01 |  | 0.0048 |  | 0.0012 |
|  | x3=0.02 |  | 0.0023 |  | 0.0010 |
|  | x3=0.03 |  | 0.0038 |  | 0.0009 |

Table S6. Mean absolute deviations of vapor- phase mole fraction for CPE systems

| Mean absolute deviations | acetic acid (1) + isopropanol (2) + isopropyl acetate (3) + water (4) | acetic acid (1) + isopropanol (2) + isopropyl acetate (3) + water (4) (0.01mol [HSO3-bmim][HSO4]) |
| --- | --- | --- |
| Δy1 | 0.0005 | 0.0001 |
| Δy2 | 0.0014 | 0.0043 |
| Δy3 | 0.0006 | 0.0001 |
| Δy4 | 0.0010 | 0.0010 |