Table 1 The pseud-first-order rate constants kobs for BPA photodegradation under different conditions

| Conditions |  |  | Kinetic equations | k (min-1) | t1/2 (min) | Contribution rate R (%) | Regression coefficient R2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pure water | Without quenching agent | air atmosphere | ln(Ct/C0) = -0.02127t+0.23810 | 0.02127 | 32.6 | - | 0.9731 |
|  |  | N2-sparging | ln(Ct/C0) = -0.02508t+0.01389 | 0.02508 | 27.6 | -17.9(DO) | 0.9849 |
|  |  | O2-sparging | ln(Ct/C0) = -0.01485t-0.16700 | 0.01485 | 46.7 | 30.2 | 0.9734 |
|  | different quenching agent (air atmosphere) | IPA | ln(Ct/C0) = -0.01842t-0.05154 | 0.01842 | 37.6 | 13.4(·OH) | 0.9758 |
|  |  | NaN3 | ln(Ct/C0) = -0.01679t+0.00930 | 0.01679 | 41.3 | 7.7(1O2) | 0.9538 |
|  |  | BQ | ln(Ct/C0) = -0.01639t+0.26930 | 0.01639 | 42.3 | 22.9( O2.-) | 0.9469 |
|  |  | SA | ln(Ct/C0) = -0.00945t+0.02531 | 0.00945 | 73.3 | 47.9(BPA*) | 0.9723 |
| HA (11.2 mg·C·L-1) | Without quenching agent |  | ln(Ct/C0) = -0.00896t+0.00904 | 0.00896 | 77.4 | - | 0.9433 |
|  | different quenching agent | IPA | ln(Ct/C0) = -0.00851t-0.30726 | 0.00851 | 81.5 | 5.0(·OH) | 0.9286 |
|  |  | NaN3 | ln(Ct/C0) = -0.00820t-0.00031 | 0.00820 | 84.5 | 3.5(1O2) | 0.9286 |
| NO- 3 (1 mM) | Without quenching agent |  | ln(Ct/C0) = -0.01542t+0.10320 | 0.01542 | 45.0 | - | 0.9805 |
|  | different quenching agent | IPA | ln(Ct/C0) = -0.01425t-0.12265 | 0.01425 | 48.6 | 7.6(·OH) | 0.9465 |
|  |  | SA | ln(Ct/C0) = -0.0093t-0.07365 | 0.0093 | 74.5 | 19.7(BPA*) | 0.9841 |
|  |  | NaN3 | ln(Ct/C0) = -0.01116t-0.00124 | 0.01116 | 62.1 | 20.0(1O2) | 0.9391 |
| NO2- (1 mM) | Without quenching agent |  | ln(Ct/C0) = -0.01747t+0.00844 | 0.01747 | 39.8 | - | 0.9947 |
|  | different quenching agent | IPA | ln(Ct/C0) = -0.01703t-0.05981 | 0.01703 | 40.7 | 2.5(·OH) | 0.9402 |
|  |  | SA | ln(Ct/C0) = -0.00429t-0.11194 | 0.00429 | 161.6 | 44.4(BPA*) | 0.8216 |
|  |  | NaN3 | ln(Ct/C0) = -0.01161t-0.2949 | 0.01161 | 59.7 | 31.0(1O2) | 0.9029 |
| Fe(III)/Fe(II) (1 mM) | Without quenching agent | Fe(III) | ln(Ct/C0) = 0.05011t+0.1918 | 0.05011 | 13.83 | - | 0.9547 |
|  |  | Fe(II) | ln(Ct/C0) = -0.07000t-0.1580 | 0.07000 | 9.90 | - | 0.9720 |
|  | quenching agent | IPA+Fe(III) | ln(Ct/C0) = -0.02818t-0.02893 | 0.02818 | 24.60 | 43.7(·OH) | 0.9608 |
|  |  | IPA+Fe(II) | ln(Ct/C0) = -0.03154t-0.06679 | 0.03154 | 21.98 | 54.9(·OH) | 0.9583 |
| Cl- (0.1 mM) | Without quenching agent |  | ln(Ct/C0) = -0.01639t-0.24741 | 0.01639 | 42.3 | - | 0.9383 |
|  | different quenching agent | IPA | ln(Ct/C0) = -0.01592t-0.22585 | 0.01592 | 43.5 | 2.9(·OH) | 0.9363 |
|  |  | NaN3 | ln(Ct/C0) = -0.01331t-0.14993 | 0.01331 | 52.1 | 15.9(1O2) | 0.9347 |
|  |  | SA | ln(Ct/C0) = -0.00878t+0.0295 | 0.00878 | 79.0 | 30.5(BPA*) | 0.9939 |
|  |  | TB | ln(Ct/C0) = -0.00936t-0.11322 | 0.00936 | 74.1 | 40.0(RHS) | 0.9480 |
| Cl- (200 mM) | Without quenching agent |  | ln(Ct/C0) = -0.02807t-0.00500 | 0.02807 | 24.7 | - | 0.9469 |
|  | different quenching agent | IPA | ln(Ct/C0) = -0.02022t+0.12636 | 0.02022 | 34.3 | 28.0(·OH) | 0.9508 |
|  |  | NaN3 | ln(Ct/C0) = -0.01821t+0.09079 | 0.01821 | 38.1 | 7.2(1O2) | 0.9763 |
|  |  | SA | ln(Ct/C0) = -0.01258t-0.008913 | 0.01258 | 55.1 | 48.0(BPA*) | 0.9636 |
|  |  | TB | ln(Ct/C0) = -0.01401t+0.09571 | 0.01401 | 49.5 | 22.1(RHS) | 0.8961 |