Supplement 1. Physical meaning of Fresnel Zone Plate (FZP)

The sound pressure pr at a specific point radiated by a circular planar piston source can be obtained by the following integral equation1.

pr, θ,t=jρ0ckU02πS1r'ejωt-kr'dS       		              (s1-1)

where r is the displacement from the center of the surface of the circular piston to the point of interest, θ is the angle between the radial axis and the displacement vector, and t is time. ρ0 and c on the right side of the equation are the density and sound velocity of the propagation medium, respectively. The angular frequency and wave number are represented ω and k, respectively. And U0dS is the source strength. j=-1 and r' are imaginary symbol and the displacement from the surface element dS to the point of interest, respectively.

For simplicity, assuming that the radius of the circular piston is a, ignoring the time dependence, and using axial symmetry, the equation becomes:

pz=jρ0ckU02π0aexp-jkz2+σ2z2+σ2dS           	             (s1-2)

where σ is the displacement from the origin to the area element which corresponds to dS=2πσdσ.  The integrand pσz=jρ0ckU02πexp-jkz2+σ2z2+σ2 is a complex, and its phase at the axial distance z depends on σ.

Φz,σ = ∠jρ0ckU02πexp-jkz2+σ2z2+σ2          	              (s1-3)

Assuming that the frequency is 1.1 MHz and the point of interest on the acoustic axis, that is, the focal length is 75 mm, calculated the σ-dependence of the phase from the origin to 60 mm is shown in the graph of Fig. S1-1. The two blue lines are the phase boundaries that determine the ring radii of classical FZP. As seen in the graph of Fig. S1-1, ϕ0=0.5151 rad and ϕ1=-2.6265 rad, therefore the phase difference between ϕ0 (dotted line) and ϕ1 (dashed line) is π rad (ϕ1 is the phase at σ=0). The radii of FZP determined by these phase boundaries and Eq. (s1-3) are indicated by red circular markers on the graph, and the corresponding radius values are displayed in the 2nd and 3rd columns of the table in Fig. S1-1. This table compares the inner- and outer- radii of the rings composing the FZP determined by Eq. (s1-3) and Eq. (1) in the main text. The dimensions of all rings are almost same. This implies that acoustic waves having the same phase in the π range are superposed at the Fresnel distance, and as a result, an acoustic focus is formed at the point.

Figure S1-1. The inner- and outer- radii of the rings composing the FZP: (a) graph of the pressure phase Φz=75 mm,σ at 1.1 MHz, the phase boundaries (ϕ0 and ϕ1), and the radii of FZP obtained therefrom, (b) table comparing the radii of FZP determined by Eq. (s1-3) of the Supplement 1 and Eq. (1) of the main text.

Supplementary References

- Kinsler, L. E., Frey, A. R., Coppens, A. B., & Sanders, J. V. Fundamentals of acoustics. (John wiley & sons, 2000)