| Location | Zone class | Major events history | human and material damage |
| --- | --- | --- | --- |
| El Biar landslide | Urban area | From 1785-to 2020 (after each period of heavy rainfall) | Destruction and deformation of infrastructures (Swedish Embassy building, houses, Telemly aqueducts, Bougara road,..), the collapse of the molassic cliff, rockfall. |
|  |  | 2021 | Upstream of the landslide, the deformation of the building on the Saint Raphael cliff balcony (cracked stairs, subsidence, and fracturing of the courtyard and the balcony wall). Downstream of the landslide, rockfall, subsided road. |
| Al Achour | Urban area | 1989 | The single crack observed at the beginning evolved with time and revealed other fracture surfaces |
|  |  | 1992 | The escarpment of 4 m, accompanied by cracks on the slipped mass and displacement of the trees of about thirty meters downwards |
|  |  | Reactivates at different winter periods | Cracked buildings, tilted trees. |
| Bologhine | Urban area | 2017 | House partially invaded by land. |
|  |  | 2021 | 3 deaths and 2 injuries, 2 houses destroyed, and another partially invaded by land, cracked and collapsed buildings, roads blocked causing major traffic disruption, vehicle invaded by sliding lands. |
| Rais hamidou | Urban area | 2021 | 01 death, 07 houses destroyed, roads blocked for days due to land moved downwards, cracked and collapsed buildings, tilting of fences and retaining walls, rockfall. |
| Frais Vallon (Bouzaréah) | Urban area (highway) | 2011 | Highway blocked for hours due to land moved downwards causing major traffic disruption |
|  |  | 2021 | Highway blocked for hours due to land moved downwards causing major traffic disruption |
| El Madania | Urban area (highway) | 2019 | Closure of the road for a few days, the collapse of the molasse cliff. |
| Ouled Fayet (South plateau) | Urban development area | 2019 | Soil slid under the building which led to the denudation of the foundations and the subsidence of the land. |
| Souidania (Palmiers city) | Urban development area | Since 2008 (reactivates at different winter periods) | Land moved to the bottom of the hillside, trees moved about 50 m downwards, soil movement away from the foundation, and the main scarp of about 13m under the houses. |
| Al Achour | Agricole land | Since 1991 (reactivates at different winter periods) | Land subsidence and cracking, tilting of utility poles, and trees. |
| Bainem forest (19th June road) | forest | 2009 | Road destroyed by the landslide, uprooted trees, loss of land. |
|  |  | 2018 | Subsided and cracked road, trees swept down the hillside by the landslide, uprooted trees. |

Table 1

Table 2

| Thematic layers | Source of data and properties |
| --- | --- |
| Landslide inventory | Historical record, Google Earth satellite imagery (years 2002-2020), OLI 8 visual observations, and field surveys (observations, GPS, years 2016-2022) |
| ### Lithology ### Distance to lithological boundaries | Geological maps (N◦ 20, 21, 41, and 42, at 1:50,000 scale), and geological field observations |
| Soil permeability | Hydrogeological map from the National Agency of Meteorology and Hydrology (ANRH) |
| Precipitation | Precipitation data from the National Agency of Meteorology and Hydrology (ANRH) |
| ### Slope, exposure, altitude, TWI, plan curve, profile curve, | ALOS PALSAR Digital Elevation Model (DEM) resolution12.5 m then resampling to 10m (https://search.asf.alaska.edu/) |
| ### Distance to rivers | Google Earth satellite imagery |
| NDVI | Landsat 8 OLI sensor satellite data (resolution 30m for spectral bands, and 15m for panchromatic), acquired on September 19, 2020 (https://earthexplorer.usgs.gov/) |
| Distance to roads | ## OpenStreetMap (https://www.openstreetmap.org/#map=13/36.7972/3.0188) |

Table 3

| Lithology unit |  | Lithological class | Age |
| --- | --- | --- | --- |
| Sands and dunes |  | L1 | Actual |
| Colluvium deposits | L2 |  | Actual |
| Alluvium deposits |  | L3 | Actual |
| Clayey sands |  | L4 | Villafranchian |
| Marine terraces |  | L5 | Pleistocene |
| Stony and clay |  | L6 | Pleistocene |
| Marls and gravels |  | L7 | Pleistocene |
| Sandstone facies |  | L8 | Astian |
| Molassic limestone |  | L9 | Astian |
| Sandy marls |  | L10 | Plaisancian |
| Blue marls |  | L11 | Plaisancian |
| Sandstone and conglomerates |  | L12 | Miocene |
| Schists |  | L13 | Precambrian |
| Crystalline limestone |  | L14 | Precambrian |
| Gneiss |  | L15 | Precambrian |

Table 4

| Classes | Pixels in classes | Pixels in classes (%) | Landslide pixels in classes | landslides Pixels in classes (%) | FR |
| --- | --- | --- | --- | --- | --- |
| Lithology |  |  |  |  |  |
| L1 | 99251 | 2.448 | 0 | 0 | 0 |
| L2 | 33878 | 0.836 | 690 | 6.25 | 7.485 |
| L3 | 378568 | 9.337 | 0 | 0.00 | 0.000 |
| L4 | 602918 | 14.871 | 90 | 0.82 | 0.055 |
| L5 | 384094 | 9.473 | 0 | 0.00 | 0.000 |
| L6 | 255404 | 6.299 | 254 | 2.30 | 0.365 |
| L7 | 171077 | 4.220 | 25 | 0.23 | 0.054 |
| L8 | 489144 | 12.064 | 91 | 0.82 | 0.068 |
| L9 | 248627 | 6.132 | 569 | 5.16 | 0.841 |
| L10 | 281266 | 6.937 | 580 | 5.26 | 0.758 |
| L11 | 773880 | 19.087 | 8310 | 75.31 | 3.945 |
| L12 | 14242 | 0.351 | 14 | 0.13 | 0.361 |
| L13 | 276637 | 6.823 | 412 | 3.73 | 0.547 |
| L14 | 12724 | 0.314 | 0 | 0 | 0 |
| L15 | 32702 | 0.807 | 0 | 0 | 0 |
| Distance to lithological Boundaries (m) |  |  |  |  |  |
| 0-50 | 1188900 | 29.324 | 4090 | 37.06 | 1.264 |
| 50-100 | 812449 | 20.039 | 2692 | 24.40 | 1.217 |
| 100-150 | 543314 | 13.401 | 1458 | 13.21 | 0.986 |
| 150-200 | 376535 | 9.287 | 828 | 7.50 | 0.808 |
| >200 | 1133214 | 27.950 | 1967 | 17.83 | 0.638 |
| Soil permeability |  |  |  |  |  |
| Null | 322063 | 7.944 | 412 | 3.73 | 0.470 |
| Very low | 1069161 | 26.370 | 8875 | 80.43 | 3.050 |
| Low | 773995 | 19.090 | 115 | 1.04 | 0.055 |
| Moderate | 1027281 | 25.337 | 1633 | 14.80 | 0.584 |
| High | 861912 | 21.259 | 0 | 0.00 | 0 |
| Slope (%) |  |  |  |  |  |
| 0-5 | 555705 | 13.706 | 4 | 0.04 | 0.003 |
| 05-10 | 1510555 | 37.257 | 21 | 0.19 | 0.005 |
| 10-15 | 749969 | 18.498 | 1479 | 13.40 | 0.725 |
| 15-20 | 524832 | 12.945 | 3175 | 28.77 | 2.223 |
| 20-25 | 308703 | 7.614 | 2412 | 21.86 | 2.871 |
| 25-30 | 165064 | 4.071 | 1799 | 16.30 | 4.005 |
| 30-35 | 101282 | 2.498 | 1170 | 10.60 | 4.238 |
| >35 | 138302 | 3.411 | 975 | 8.84 | 2.591 |
| Exposure |  |  |  |  |  |
| East | 393642 | 9.709 | 972 | 8.81 | 0.907 |
| South-east | 511809 | 12.624 | 1804 | 16.35 | 1.295 |
| North-west | 533727 | 13.164 | 2569 | 23.28 | 1.769 |
| Flat | 247721 | 6.110 | 1 | 0.01 | 0.001 |
| North | 458542 | 11.310 | 2661 | 24.11 | 2.131 |
| North-east | 434219 | 10.710 | 2087 | 18.91 | 1.766 |
| West | 444369 | 10.960 | 759 | 6.88 | 0.628 |
| South-west | 526619 | 12.989 | 89 | 0.81 | 0.062 |
| South | 503764 | 12.425 | 93 | 0.84 | 0.068 |
| Altitude (m) |  |  |  |  |  |
| 0-100 | 835080 | 20.597 | 196 | 1.78 | 0.086 |
| 100-200 | 1828591 | 45.101 | 4765 | 43.18 | 0.957 |
| 200-300 | 1022217 | 25.212 | 5067 | 45.92 | 1.822 |
| 300-400 | 358791 | 8.849 | 995 | 9.02 | 1.019 |
| 400-450 | 9733 | 0.240 | 12 | 0.11 | 0.453 |
| Profile curvature |  |  |  |  |  |
| Convex | 1446738 | 35.683 | 4266 | 38.66 | 1.084 |
| Flat | 1137687 | 28.060 | 1736 | 15.73 | 0.561 |
| Concave | 1469987 | 36.256 | 5031 | 45.59 | 1.258 |
| Plan curvature |  |  |  |  |  |
| Concave | 1124463 | 27.734 | 4574 | 41.45 | 1.495 |
| Plat | 1781517 | 43.940 | 2441 | 22.12 | 0.504 |
| Convex | 1148432 | 28.325 | 4020 | 36.43 | 1.286 |
| Precipitation (mm/year) |  |  |  |  |  |
| 500-550 | 292744 | 7.220 | 0 | 0 | 0 |
| 550-600 | 1656302 | 40.852 | 2563 | 23.23 | 0.568 |
| 600-700 | 1996263 | 49.237 | 8172 | 74.06 | 1.504 |
| 700-800 | 109103 | 2.691 | 300 | 2.72 | 1.010 |
| TWI |  |  |  |  |  |
| TWI1 | 1617660 | 39.899 | 5533 | 50.14 | 1.257 |
| TWI2 | 1561823 | 38.522 | 3956 | 35.85 | 0.931 |
| TWI3 | 643404 | 15.869 | 1285 | 11.64 | 0.733 |
| TW4 | 231525 | 5.710 | 261 | 2.37 | 0.414 |
| Distance to rivers (m) |  |  |  |  |  |
| 0-50 | 292495 | 7.214 | 1299 | 11.77 | 1.630 |
| 50-100 | 288515 | 7.116 | 1295 | 11.74 | 1.649 |
| 100-150 | 281233 | 6.936 | 1084 | 9.82 | 1.416 |
| 150-200 | 270139 | 6.663 | 883 | 8.00 | 1.201 |
| >200 | 2922030 | 72.070 | 6474 | 58.67 | 0.814 |
| NDVI |  |  |  |  |  |
| NDVI1 | 13202 | 0.326 | 0 | 0 | 0 |
| NDVI2 | 1382968 | 34.110 | 3271 | 29.64 | 0.869 |
| NDVI3 | 659256 | 16.260 | 4611 | 41.79 | 2.570 |
| NDVI4 | 323356 | 7.975 | 215 | 1.95 | 0.244 |
| NDVI5 | 1316565 | 32.472 | 2925 | 26.51 | 0.816 |
| NDVI6 | 359065 | 8.856 | 13 | 0.12 | 0.013 |
| Distance to the road (m) |  |  |  |  |  |
| 0-50 | 173447 | 4.278 | 2180 | 19.76 | 4.615 |
| 50-100 | 163049 | 4.022 | 1322 | 11.98 | 2.980 |
| 100-150 | 155998 | 3.848 | 1064 | 9.64 | 2.506 |
| 150-200 | 151118 | 3.727 | 994 | 9.01 | 2.417 |
| >200 | 3410800 | 84.126 | 5475 | 49.61 | 0.590 |

Table 5

| Classes | Pixels in classes | Landslide pixels in classes | W+ | W- | C |
| --- | --- | --- | --- | --- | --- |
| Lithology |  |  |  |  |  |
| L1 | 99251 | 0 | 0.000 | 0.025 | 0.000 |
| L2 | 33878 | 690 | 2.031 | -0.056 | 2.087 |
| L3 | 378568 | 0 | 0.000 | 0.098 | 0.000 |
| L4 | 602918 | 90 | -2.906 | 0.153 | -3.059 |
| L5 | 384094 | 0 | 0.000 | 0.100 | 0.000 |
| L6 | 255404 | 254 | -1.008 | 0.042 | -1.050 |
| L7 | 171077 | 25 | -2.927 | 0.041 | -2.968 |
| L8 | 489144 | 91 | -2.685 | 0.121 | -2.806 |
| L9 | 248627 | 569 | -0.174 | 0.010 | -0.184 |
| L10 | 281266 | 580 | -0.278 | 0.018 | -0.296 |
| L11 | 773880 | 8310 | 1.381 | -1.189 | 2.569 |
| L12 | 14242 | 14 | -1.020 | 0.002 | -1.022 |
| L13 | 276637 | 412 | -0.604 | 0.033 | -0.637 |
| L14 | 12724 | 0 | 0.000 | 0.003 | 0.000 |
| L15 | 32702 | 0 | 0.000 | 0.008 | 0.000 |
| Distance to lithological Boundaries (m) |  |  |  |  |  |
| 0-50 | 1188900 | 4090 | 0.235 | -0.116 | 0.352 |
| 50-100 | 812449 | 2692 | 0.197 | -0.056 | 0.253 |
| 100-150 | 543314 | 1458 | -0.014 | 0.002 | -0.016 |
| 150-200 | 376535 | 828 | -0.214 | 0.020 | -0.233 |
| >200 | 1133214 | 1967 | -0.451 | 0.132 | -0.582 |
| Soil permeability |  |  |  |  |  |
| Null | 322063 | 412 | -0.756 | 0.045 | -0.801 |
| Very low | 1069161 | 8875 | 1.121 | -1.327 | 2.447 |
| Low | 773995 | 115 | -2.910 | 0.202 | -3.112 |
| Moderate | 1027281 | 1633 | -0.539 | 0.132 | -0.671 |
| High | 861912 | 0 | 0.000 | 0.240 | 0.000 |
| Slope (%) |  |  |  |  |  |
| 0-5 | 555705 | 4 | -5.938 | 0.147 | -6.085 |
| 05-10 | 1510555 | 21 | -0.323 | 0.061 | -0.384 |
| 10-15 | 749969 | 1479 | -5.280 | 0.466 | -5.745 |
| 15-20 | 524832 | 3175 | 0.802 | -0.201 | 1.003 |
| 20-25 | 308703 | 2412 | 1.060 | -0.168 | 1.228 |
| 25-30 | 165064 | 1799 | 1.396 | -0.137 | 1.533 |
| 30-35 | 101282 | 1170 | 1.453 | -0.087 | 1.540 |
| >35 | 138302 | 975 | 0.956 | -0.058 | 1.014 |
| Exposure |  |  |  |  |  |
| East | 393642 | 972 | -0.097 | 0.010 | -0.107 |
| South-east | 511809 | 1804 | 0.260 | -0.044 | 0.303 |
| North-west | 533727 | 2569 | 0.572 | -0.124 | 0.697 |
| Flat | 247721 | 1 | -6.516 | 0.063 | -6.579 |
| North | 458542 | 2661 | 0.760 | -0.156 | 0.916 |
| North-east | 434219 | 2087 | 0.571 | -0.097 | 0.668 |
| West | 444369 | 759 | -0.467 | 0.045 | -0.512 |
| South-west | 526619 | 89 | -2.781 | 0.131 | -2.913 |
| South | 503764 | 93 | -2.693 | 0.125 | -2.818 |
| Altitude (m) |  |  |  |  |  |
| 0-100 | 835080 | 196 | -2.453 | 0.213 | -2.666 |
| 100-200 | 1828591 | 4765 | -0.044 | 0.035 | -0.079 |
| 200-300 | 1022217 | 5067 | 0.602 | -0.325 | 0.927 |
| 300-400 | 358791 | 995 | 0.019 | -0.002 | 0.021 |
| 400-450 | 9733 | 12 | -0.793 | 0.001 | -0.795 |
| Profile curvature |  |  |  |  |  |
| Convex | 1446738 | 4266 | 0.081 | -0.048 | 0.128 |
| Flat | 1137687 | 1736 | -0.580 | 0.159 | -0.738 |
| Concave | 1469987 | 5031 | 0.230 | -0.159 | 0.389 |
| Plan curvature |  |  |  |  |  |
| Concave | 1124463 | 4574 | -4.702 | -0.211 | -4.491 |
| Plat | 1781517 | 2441 | -4.705 | 0.330 | -5.035 |
| Convex | 1148432 | 4020 | -4.703 | -0.120 | -4.583 |
| Precipitation (mm/year) |  |  |  |  |  |
| 500-550 | 292744 | 0 | 0.000 | 0.075 | 0.000 |
| 550-600 | 1656302 | 2563 | -0.566 | 0.262 | -0.828 |
| 600-700 | 1996263 | 8172 | 0.410 | -0.673 | 1.083 |
| 700-800 | 109103 | 300 | 0.010 | 0.000 | 0.011 |
| TWI |  |  |  |  |  |
| TWI1 | 1617660 | 5533 | 0.229 | -0.187 | 0.417 |
| TWI2 | 1561823 | 3956 | -0.072 | 0.043 | -0.114 |
| TWI3 | 643404 | 1285 | -0.312 | 0.049 | -0.361 |
| TW4 | 231525 | 261 | -0.883 | 0.035 | -0.918 |
| Distance to rivers (m) |  |  |  |  |  |
| 0-50 | 292495 | 1299 | 0.490 | -0.050 | 0.540 |
| 50-100 | 288515 | 1295 | 0.502 | -0.051 | 0.553 |
| 100-150 | 281233 | 1084 | 0.349 | -0.032 | 0.381 |
| 150-200 | 270139 | 883 | 0.184 | -0.015 | 0.198 |
| >200 | 2922030 | 6474 | -0.206 | 0.393 | -0.599 |
| NDVI |  |  |  |  |  |
| NDVI1 | 13202 | 0 | 0.000 | 0.003 | 0.000 |
| NDVI2 | 1382968 | 3271 | -0.141 | 0.066 | -0.206 |
| NDVI3 | 659256 | 4611 | 0.948 | -0.365 | 1.313 |
| NDVI4 | 323356 | 215 | -1.411 | 0.064 | -1.475 |
| NDVI5 | 1316565 | 2925 | -0.204 | 0.085 | -0.289 |
| NDVI6 | 359065 | 13 | -4.322 | 0.092 | -4.414 |
| Distance to road (m) |  |  |  |  |  |
| 0-50 | 173447 | 2180 | 1.539 | -0.177 | 1.716 |
| 50-100 | 163049 | 1322 | 1.097 | -0.087 | 1.184 |
| 100-150 | 155998 | 1064 | 0.923 | -0.062 | 0.985 |
| 150-200 | 151118 | 994 | 0.886 | -0.057 | 0.943 |
| >200 | 3410800 | 5475 | -0.529 | 1.161 | -1.690 |

Table 6

| Factors / Classes | Pixels in classes | Landslide pixels in classes | Bel | Dis | Unc | Pls |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology |  |  |  |  |  |  |
| L1 | 99251 | 0 | 0.000 | 0.069 | 0.931 | 0.931 |
| L2 | 33878 | 690 | 0.520 | 0.063 | 0.416 | 0.937 |
| L3 | 378568 | 0 | 0.000 | 0.074 | 0.926 | 0.926 |
| L4 | 602918 | 90 | 0.004 | 0.078 | 0.918 | 0.922 |
| L5 | 384094 | 0 | 0.000 | 0.074 | 0.926 | 0.926 |
| L6 | 255404 | 254 | 0.025 | 0.070 | 0.905 | 0.930 |
| L7 | 171077 | 25 | 0.004 | 0.070 | 0.927 | 0.930 |
| L8 | 489144 | 91 | 0.005 | 0.076 | 0.920 | 0.924 |
| L9 | 248627 | 569 | 0.057 | 0.068 | 0.875 | 0.932 |
| L10 | 281266 | 580 | 0.052 | 0.068 | 0.880 | 0.932 |
| L11 | 773880 | 8310 | 0.272 | 0.020 | 0.708 | 0.980 |
| L12 | 14242 | 14 | 0.025 | 0.067 | 0.908 | 0.933 |
| L13 | 276637 | 412 | 0.037 | 0.069 | 0.893 | 0.931 |
| L14 | 12724 | 0 | 0.000 | 0.067 | 0.933 | 0.933 |
| L15 | 32702 | 0 | 0.000 | 0.067 | 0.933 | 0.933 |
| Distance to lithological Boundaries (m) |  |  |  |  |  |  |
| 0-50 | 1188900 | 4090 | 0.257 | 0.060 | 0.683 | 0.940 |
| 50-100 | 812449 | 2692 | 0.248 | 0.063 | 0.689 | 0.937 |
| 100-150 | 543314 | 1458 | 0.201 | 0.067 | 0.732 | 0.933 |
| 150-200 | 376535 | 828 | 0.164 | 0.068 | 0.767 | 0.932 |
| >200 | 1133214 | 1967 | 0.130 | 0.076 | 0.794 | 0.924 |
| Soil permeability |  |  |  |  |  |  |
| Null | 322063 | 412 | 0.112 | 0.070 | 0.818 | 0.930 |
| Very low | 1069161 | 8875 | 0.735 | 0.018 | 0.248 | 0.982 |
| Low | 773995 | 115 | 0.013 | 0.082 | 0.905 | 0.918 |
| Moderate | 1027281 | 1633 | 0.140 | 0.076 | 0.784 | 0.924 |
| High | 861912 | 0 | 0.000 | 0.085 | 0.915 | 0.915 |
| Slope (%) |  |  |  |  |  |  |
| 0-5 | 555705 | 4 | 0.000 | 0.078 | 0.922 | 0.922 |
| 05-10 | 1510555 | 21 | 0.000 | 0.107 | 0.893 | 0.893 |
| 10-15 | 749969 | 1479 | 0.043 | 0.071 | 0.886 | 0.929 |
| 15-20 | 524832 | 3175 | 0.133 | 0.055 | 0.812 | 0.945 |
| 20-25 | 308703 | 2412 | 0.172 | 0.057 | 0.771 | 0.943 |
| 25-30 | 165064 | 1799 | 0.241 | 0.058 | 0.701 | 0.942 |
| 30-35 | 101282 | 1170 | 0.255 | 0.061 | 0.684 | 0.939 |
| >35 | 138302 | 975 | 0.155 | 0.063 | 0.782 | 0.937 |
| Exposure |  |  |  |  |  |  |
| East | 393642 | 972 | 0.105 | 0.068 | 0.827 | 0.932 |
| South-east | 511809 | 1804 | 0.150 | 0.064 | 0.786 | 0.936 |
| North-west | 533727 | 2569 | 0.205 | 0.059 | 0.736 | 0.941 |
| Flat | 247721 | 1 | 0.000 | 0.071 | 0.929 | 0.929 |
| North | 458542 | 2661 | 0.247 | 0.057 | 0.695 | 0.943 |
| North-east | 434219 | 2087 | 0.205 | 0.061 | 0.734 | 0.939 |
| West | 444369 | 759 | 0.073 | 0.070 | 0.857 | 0.930 |
| South-west | 526619 | 89 | 0.007 | 0.076 | 0.916 | 0.924 |
| South | 503764 | 93 | 0.008 | 0.076 | 0.916 | 0.924 |
| Altitude (m) |  |  |  |  |  |  |
| 0-101 | 835080 | 196 | 0.020 | 0.083 | 0.897 | 0.917 |
| 100-201 | 1828591 | 4765 | 0.221 | 0.069 | 0.710 | 0.931 |
| 200-301 | 1022217 | 5067 | 0.421 | 0.048 | 0.531 | 0.952 |
| 300-401 | 358791 | 995 | 0.235 | 0.067 | 0.698 | 0.933 |
| 400-451 | 9733 | 12 | 0.104 | 0.067 | 0.829 | 0.933 |
| Profile curvature |  |  |  |  |  |  |
| Convex | 1446738 | 4266 | 0.373 | 0.064 | 0.563 | 0.936 |
| Flat | 1137687 | 1736 | 0.193 | 0.078 | 0.729 | 0.922 |
| Concave | 1469987 | 5031 | 0.434 | 0.057 | 0.509 | 0.943 |
| Plan curvature |  |  |  |  |  |  |
| Concave | 1124463 | 4574 | 0.455 | 0.054 | 0.490 | 0.946 |
| Plat | 1781517 | 2441 | 0.153 | 0.093 | 0.754 | 0.907 |
| Convex | 1148432 | 4020 | 0.392 | 0.059 | 0.549 | 0.941 |
| Precipitation (mm/year) |  |  |  |  |  |  |
| 500-551 | 292744 | 0 | 0.000 | 0.072 | 0.928 | 0.928 |
| 550-601 | 1656302 | 2563 | 0.184 | 0.087 | 0.729 | 0.913 |
| 600-701 | 1996263 | 8172 | 0.488 | 0.034 | 0.477 | 0.966 |
| 700-801 | 109103 | 300 | 0.328 | 0.067 | 0.605 | 0.933 |
|  |  | TWI |  |  |  |  |
| TWI1 | 1617660 | 5533 | 0.377 | 0.056 | 0.567 | 0.944 |
| TWI2 | 1561823 | 3956 | 0.279 | 0.070 | 0.651 | 0.930 |
| TWI3 | 643404 | 1285 | 0.220 | 0.070 | 0.710 | 0.930 |
| TWI4 | 231525 | 261 | 0.124 | 0.069 | 0.807 | 0.931 |
| Distance to rivers (m) |  |  |  |  |  |  |
| 0-51 | 292495 | 1299 | 0.243 | 0.064 | 0.693 | 0.936 |
| 50-101 | 288515 | 1295 | 0.246 | 0.064 | 0.690 | 0.936 |
| 100-151 | 281233 | 1084 | 0.211 | 0.065 | 0.724 | 0.935 |
| 150-201 | 270139 | 883 | 0.179 | 0.066 | 0.755 | 0.934 |
| >201 | 2922030 | 6474 | 0.121 | 0.099 | 0.780 | 0.901 |
|  |  | NDVI |  |  |  |  |
| NDVI1 | 13202 | 0 | 0.000 | 0.067 | 0.933 | 0.933 |
| NDVI2 | 1382968 | 3271 | 0.192 | 0.071 | 0.736 | 0.929 |
| NDVI3 | 659256 | 4611 | 0.571 | 0.046 | 0.383 | 0.954 |
| NDVI4 | 323356 | 215 | 0.054 | 0.071 | 0.875 | 0.929 |
| NDVI5 | 1316565 | 2925 | 0.180 | 0.073 | 0.747 | 0.927 |
| NDVI6 | 359065 | 13 | 0.003 | 0.073 | 0.924 | 0.927 |
| Distance to the road (m) |  |  |  |  |  |  |
| 0-51 | 173447 | 2180 | 0.353 | 0.056 | 0.591 | 0.944 |
| 50-101 | 163049 | 1322 | 0.227 | 0.061 | 0.711 | 0.939 |
| 100-151 | 155998 | 1064 | 0.191 | 0.063 | 0.746 | 0.937 |
| 150-201 | 151118 | 994 | 0.184 | 0.063 | 0.753 | 0.937 |
| >201 | 3410800 | 5475 | 0.045 | 0.214 | 0.742 | 0.786 |

Table 7

| Statistics | Value |
| --- | --- |
| The Hosmer and Lemeshow test (chi-square) | 84.54 |
| 2log (likelihood) of the initial step | 30595.517 |
| 2log (likelihood) of the last step | 14369.472 |
| Pseudo R Square | 0.530 |
| R-square value of Cox and Snell R2 | 0.521 |
| Nagelkerke R2 | 0.694 |
| ROC | 0.865 |

Table 8

| Factors | βk | α0 |
| --- | --- | --- |
| Lithology | 0.430 | 3.08 |
| Distance to lithological boundaries | -0.003 |  |
| Soil permeability | 0.469 |  |
| Slope | 0.627 |  |
| Exposure | -0.193 |  |
| Altitude | 0.481 |  |
| Profile curvature | 0.031 |  |
| Plan curvature | -0.070 |  |
| Precipitation | 0.191 |  |
| TWI | 0.046 |  |
| Distance to rivers | -0.286 |  |
| NDVI | -0.286 |  |
| Distance to road | -0.648 |  |

Table 9

| Preference factor | Degree of preference | Explanation |
| --- | --- | --- |
| 1 | Equal importance | Two activities contribute equally to the objective |
| 3 | Moderate importance | Experience and judgment strongly favor one activity over another |
| 5 | Strong importance | The evidence favoring one activity over another is of the highest possible order of affirmation |
| 7 | Very strong or demonstrated importance | An activity is favored very strongly over another, its dominance demonstrated in practice |
| 9 | Extreme importance | The evidence favoring one activity over another is of the highest possible order of affirmation |
| 2, 4, 6, 2, 4, 6, 8 | Intermediate values between adjacent scale values | When compromise is needed |
| Reciprocals | Opposites | Used for inverse comparison |

Table 10

| Factors | Pair-wise comparison matrix |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (1) | (2) | (3) | (4) | (5) | (6) | (7) | (8) | (9) | (10) | (11) | (12) | (13) | Weight AHP |
| Permeability (1) | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 0.042 |
| Lithology (2) | 3 | 1 |  |  |  |  |  |  |  |  |  |  |  | 0.200 |
| Precipitation (3) | 5 | 1/2 | 1 |  |  |  |  |  |  |  |  |  |  | 0.168 |
| Distance to lithological boundaries (4) | 3 | 1/3 | 1/2 | 1 |  |  |  |  |  |  |  |  |  | 0.107 |
| Slope (5) | 5 | 1/2 | 1/2 | 3 | 1 |  |  |  |  |  |  |  |  | 0.171 |
| Exposure (6) | 3 | 1/3 | 1/3 | 1/2 | 1/5 | 1 |  |  |  |  |  |  |  | 0.083 |
| Distance to rivers (7) | 3 | 1/6 | 1/4 | 1/3 | 1/6 | 1/3 | 1 |  |  |  |  |  |  | 0.053 |
| Profile curvature (8) | 1/5 | 1/9 | 1/9 | 1/7 | 1/9 | 1/7 | 1/6 | 1 |  |  |  |  |  | 0.013 |
| Plan curvature (9) | 1/5 | 1/9 | 1/9 | 1/7 | 1/9 | 1/7 | 1/6 | 1/2 | 1 |  |  |  |  | 0.011 |
| TWI (10) | 1/5 | 1/7 | 1/7 | 1/6 | 1/8 | 1/5 | 1/3 | 3 | 3 | 1 |  |  |  | 0.020 |
| NDVI (11) | 2 | 1/7 | 1/6 | 1/5 | 1/6 | 1/3 | 1/3 | 3 | 5 | 3 | 1 |  |  | 0.038 |
| Distance to the road (12) | 5 | 1/4 | 1/4 | 1/3 | 1/4 | 1/3 | 3 | 5 | 5 | 3 | 2 | 1 |  | 0.059 |
| Altitude (13) | 1/2 | 1/8 | 1/8 | 1/7 | 1/7 | 1/4 | 1/3 | 3 | 4 | 2 | 1/3 | 3 | 1 | 0.033 |
| CI= 0.144. λmax =14.72. CR=0.092 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 11

| Factors classes | (1) | (2) | (3) | (4) | (5) | (6) | (7) | (8) | (9) | (10) | (11) | (12) | (13) | (14) | (15) | Weight |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lithology |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) L2 | 1 | 5 | 3 | 3 | 5 | 5 | 5 | 5 | 7 | 7 | 9 | 9 | 9 | 9 | 9 | 0,209 |
| (2) L11 | 1/2 | 1 | 3 | 3 | 3 | 5 | 5 | 5 | 7 | 7 | 9 | 9 | 9 | 9 | 9 | 0,179 |
| (3) L9 | 1/3 | 1/3 | 1 | 1/2 | 3 | 3 | 3 | 3 | 5 | 5 | 7 | 7 | 7 | 7 | 7 | 0,108 |
| (4) L10 | 1/3 | 1/3 | 2 | 1 | 1/2 | 3 | 2 | 3 | 5 | 5 | 7 | 7 | 7 | 7 | 7 | 0,103 |
| (5) L13 | 1/5 | 1/3 | 1/3 | 2 | 1 | 3 | 2 | 2 | 5 | 5 | 7 | 7 | 7 | 7 | 7 | 0,096 |
| (6) L6 | 1/5 | 1/5 | 1/3 | 1/3 | 1/3 | 1 | 2 | 2 | 2 | 2 | 5 | 5 | 5 | 5 | 5 | 0,056 |
| (7) L4 | 1/5 | 1/5 | 1/3 | 1/2 | 1/2 | 1/2 | 1 | 2 | 2 | 2 | 5 | 5 | 5 | 5 | 5 | 0,054 |
| (8) L8 | 1/5 | 1/5 | 1/3 | 1/3 | 1/2 | 1/2 | 1/2 | 1 | 2 | 2 | 5 | 5 | 5 | 5 | 5 | 0,048 |
| (9) L12 | 1/7 | 1/7 | 1/5 | 1/5 | 1/5 | 1/2 | 1/2 | 1/2 | 1 | 1/2 | 5 | 5 | 5 | 5 | 5 | 0,038 |
| (10) L7 | 1/7 | 1/7 | 1/5 | 1/5 | 1/5 | 1/2 | 1/2 | 1/2 | 2 | 1 | 5 | 5 | 5 | 5 | 5 | 0,041 |
| (11) L1 | 1/9 | 1/9 | 1/7 | 1/7 | 1/7 | 1/5 | 1/5 | 1/5 | 1/5 | 1/5 | 1 | 2 | 2 | 2 | 2 | 0,017 |
| (12) L15 | 1/9 | 1/9 | 1/7 | 1/7 | 1/7 | 1/5 | 1/5 | 1/5 | 1/5 | 1/5 | 1/2 | 1 | 2 | 2 | 2 | 0,015 |
| (13) L5 | 1/9 | 1/9 | 1/7 | 1/7 | 1/7 | 1/5 | 1/5 | 1/5 | 1/5 | 1/5 | 1/2 | 1/2 | 1 | 2 | 2 | 0,014 |
| (14) L3 | 1/9 | 1/9 | 1/7 | 1/7 | 1/7 | 1/5 | 1/5 | 1/5 | 1/5 | 1/5 | 1/2 | 1/2 | 1/2 | 1 | 2 | 0,012 |
| (15) L14 | 1/9 | 1/9 | 1/7 | 1/7 | 1/7 | 1/5 | 1/5 | 1/5 | 1/5 | 1/5 | 1/2 | 1/2 | 1/2 | 1/2 | 1 | 0,011 |
| Consistency ratio = 0.066 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distance to lithological Boundaries (m) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) 0-50 | 1 | 3 | 5 | 7 | 9 |  |  |  |  |  |  |  |  |  |  | 0.529 |
| (2) 50-100 | 1/3 | 1 | 3 | 5 | 7 |  |  |  |  |  |  |  |  |  |  | 0.255 |
| (3) 100-150 | 1/5 | 1/3 | 1 | 3 | 5 |  |  |  |  |  |  |  |  |  |  | 0.126 |
| (4) 150-200 | 1/7 | 1/5 | 1/3 | 1 | 1 |  |  |  |  |  |  |  |  |  |  | 0.050 |
| (5) >200 | 1/9 | 1/7 | 1/5 | 1/3 | 1 |  |  |  |  |  |  |  |  |  |  | 0.039 |
| Consistency ratio = 0.055 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Soil permeability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) Moderate | 1 | 1 | 1/5 | 5 | 3 |  |  |  |  |  |  |  |  |  |  | 0.205 |
| (2) High | 1/5 | 1 | 1/9 | 1/3 | 1/5 |  |  |  |  |  |  |  |  |  |  | 0.036 |
| (3) Very low | 5 | 9 | 1 | 9 | 7 |  |  |  |  |  |  |  |  |  |  | 0.574 |
| (4) Low | 1/5 | 3 | 1/9 | 1 | 1/3 |  |  |  |  |  |  |  |  |  |  | 0.063 |
| (5) Null | 1/3 | 5 | 1/7 | 3 | 1 |  |  |  |  |  |  |  |  |  |  | 0.122 |
| Consistency ratio = 0.091 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Slope(%) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) 0-05 | 1 | 3 | 1/3 | 1/7 | 1/7 | 1/9 | 1/9 | 1/9 |  |  |  |  |  |  |  | 0,018 |
| (2) 05-10 | 2 | 1 | 1/3 | 1/7 | 1/7 | 1/9 | 1/9 | 1/9 |  |  |  |  |  |  |  | 0,022 |
| (3) 10-15 | 3 | 3 | 1 | 0.2 | 1/7 | 1/7 | 1/7 | 1/7 |  |  |  |  |  |  |  | 0,036 |
| (4) 15-20 | 7 | 7 | 5 | 1 | 1/2 | 1/3 | 1/3 | 1/3 |  |  |  |  |  |  |  | 0,101 |
| (5) 20-25 | 7 | 7 | 7 | 2 | 1 | 1/3 | 1/3 | 1/3 |  |  |  |  |  |  |  | 0,123 |
| (6) 25-30 | 9 | 9 | 7 | 3 | 3 | 1 | 0.5 | 1/3 |  |  |  |  |  |  |  | 0,183 |
| (7) 30-35 | 9 | 9 | 7 | 3 | 3 | 2 | 1 | 1/3 |  |  |  |  |  |  |  | 0,212 |
| (8) >35 | 9 | 9 | 7 | 3 | 3 | 3 | 3 | 1 |  |  |  |  |  |  |  | 0,306 |
| Consistency ratio = 0.070 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Exposure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) Plat | 1/9 | 1/9 | 1/5 | 1/7 | 1/3 | 1/3 | 1/3 | 1/9 |  |  |  |  |  |  |  | 0.017 |
| (2) North | 1 | 3 | 5 | 5 | 5 | 5 | 5 | 3 |  |  |  |  |  |  |  | 0.285 |
| (3) North-east | 1/3 | 1 | 2 | 5 | 5 | 5 | 5 | 1/3 |  |  |  |  |  |  |  | 0.155 |
| (4) East | 1/5 | 1/2 | 1 | 1/3 | 5 | 5 | 5 | 1/5 |  |  |  |  |  |  |  | 0.099 |
| (5) South-east | 1/5 | 1/5 | 3 | 1 | 2 | 2 | 2 | 1/7 |  |  |  |  |  |  |  | 0.079 |
| (6) South | 1/5 | 1/5 | 1/5 | 1/2 | 1 | 2 | 2 | 1/7 |  |  |  |  |  |  |  | 0.044 |
| (7) South-west | 1/5 | 1/5 | 1/5 | 1/2 | 1/2 | 1 | 2 | 1/7 |  |  |  |  |  |  |  | 0.038 |
| (8) West | 1/5 | 1/5 | 1/5 | 1/2 | 1/2 | 1/2 | 1 | 1/7 |  |  |  |  |  |  |  | 0.032 |
| (9) North-west | 1/3 | 3 | 5 | 7 | 7 | 7 | 7 | 1 |  |  |  |  |  |  |  | 0.251 |
| Consistency ratio = 0.099 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Altitude (m) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) 0-100 | 1 | 3 | 1/7 | 1/7 | 1/3 |  |  |  |  |  |  |  |  |  |  | 0.039 |
| (2) 100-200 | 5 | 1 | 1/5 | 1/5 | 3 |  |  |  |  |  |  |  |  |  |  | 0.134 |
| (3) 200-300 | 7 | 5 | 1 | 2 | 5 |  |  |  |  |  |  |  |  |  |  | 0.428 |
| (4) 300-400 | 7 | 5 | 1/2 | 1 | 5 |  |  |  |  |  |  |  |  |  |  | 0.323 |
| (5) 400-450 | 3 | 1/3 | 1/5 | 1/5 | 1 |  |  |  |  |  |  |  |  |  |  | 0.077 |
| Consistency ratio = 0.082 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Profile curvature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) Concave | 1 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  | 0.633 |
| (2) Convex | 1/3 | 1 | 3 |  |  |  |  |  |  |  |  |  |  |  |  | 0.260 |
| (3) Flat | 1/5 | 1/3 | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 0.106 |
| Consistency ratio = 0.016 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plan curvature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) Concave | 1 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  | 0.633 |
| (2) Convex | 1/3 | 1 | 3 |  |  |  |  |  |  |  |  |  |  |  |  | 0.260 |
| (3) Flat | 1/5 | 1/3 | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 0.106 |
| Consistency ratio = 0.016 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Precipitation (mm/year) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) 500-550 | 1 | 1/5 | 1/7 | 1/5 |  |  |  |  |  |  |  |  |  |  |  | 0.047 |
| (2) 550-600 | 5 | 1 | 1/3 | 1/3 |  |  |  |  |  |  |  |  |  |  |  | 0.170 |
| (3) 600-700 | 7 | 3 | 1 | 2 |  |  |  |  |  |  |  |  |  |  |  | 0.457 |
| (4) 700-800 | 5 | 3 | 1/2 | 1 |  |  |  |  |  |  |  |  |  |  |  | 0.326 |
| Consistency ratio = 0.045 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) TWI1 | 1 | 3 | 5 | 7 |  |  |  |  |  |  |  |  |  |  |  | 0.575 |
| (2) TWI2 | 1/3 | 1 | 5 | 5 |  |  |  |  |  |  |  |  |  |  |  | 0.251 |
| (3) TWI3 | 1/5 | 1/3 | 1 | 2 |  |  |  |  |  |  |  |  |  |  |  | 0.109 |
| (4) TWI4 | 1/7 | 1/5 | 1/2 | 1 |  |  |  |  |  |  |  |  |  |  |  | 0.065 |
| Consistency ratio = 0.079 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distance to rivers (m) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) 0-50 | 1 | 3 | 5 | 7 |  |  |  |  |  |  |  |  |  |  |  | 0.255 |
| (2) 50-100 | 1/3 | 1 | 5 | 5 |  |  |  |  |  |  |  |  |  |  |  | 0.529 |
| (3) 100-150 | 1/5 | 1/3 | 1 | 2 |  |  |  |  |  |  |  |  |  |  |  | 0.126 |
| (4) 150-200 | 1/7 | 1/5 | 1/2 | 1 |  |  |  |  |  |  |  |  |  |  |  | 0.051 |
| (5) >200 | 1/7 | 1/9 | 1/5 | 1/3 |  |  |  |  |  |  |  |  |  |  |  | 0.039 |
| Consistency ratio = 0.048 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NDVI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) NDVI1 | 1 | 3 | 1/9 | 1/5 | 1/7 | 1/5 |  |  |  |  |  |  |  |  |  | 0.026 |
| (2) NDVI2 | 7 | 1 | 1/2 | 3 | 2 | 5 |  |  |  |  |  |  |  |  |  | 0.240 |
| (3) NDVI3 | 9 | 2 | 1 | 5 | 3 | 7 |  |  |  |  |  |  |  |  |  | 0.392 |
| (4) NDVI4 | 5 | 1/3 | 1/5 | 1 | 1/5 | 2 |  |  |  |  |  |  |  |  |  | 0.087 |
| (5) NDVI5 | 7 | 1/2 | 1/3 | 5 | 1 | 3 |  |  |  |  |  |  |  |  |  | 0.188 |
| (6) NDVI6 | 5 | 1/5 | 1/7 | 1/2 | 1/3 | 1 |  |  |  |  |  |  |  |  |  | 0.066 |
| Consistency ratio = 0.066 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distance to the road (m) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) 0-50 | 1 | 3 | 7 | 7 | 9 |  |  |  |  |  |  |  |  |  |  | 0.509 |
| (2) 50-100 | 1/3 | 1 | 5 | 5 | 7 |  |  |  |  |  |  |  |  |  |  | 0.274 |
| (3) 100-150 | 1/7 | 1/5 | 1 | 3 | 5 |  |  |  |  |  |  |  |  |  |  | 0.117 |
| (4) 150-200 | 1/7 | 1/5 | 1/3 | 1 | 3 |  |  |  |  |  |  |  |  |  |  | 0.066 |
| (5) >200 | 1/9 | 1/7 | 1/5 | 1/3 | 1 |  |  |  |  |  |  |  |  |  |  | 0.034 |
| Consistency ratio = 0.090 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |