Concentrations of PCDDs/PCDFs and dl-PCBs in ambient air in Hanoi, Vietnam,

between 2017 and 2021, and health risk assessment

Trinh Khac Saua,*

aInstitute of Tropical Medicine, Joint Vietnam-Russia Tropical Science and Technology Research Center, 63 Nguyen Van Huyen Street, Cau Giay district, Hanoi, Vietnam

*Corresponding author

E-mail address: sau_tk@yahoo.com

Telephone: +84-912 206 942

Fax: +84-243 756 2390

Table S1 PAS sample identifications and periods

| CG site | QM site | BV site | Sampling period | Duration (day) |
| --- | --- | --- | --- | --- |
| CG-50 | QM-49 | BV-51 | 14 Nov 2017 - 26 Feb 2018 | 104 |
| CG-53 | QM-52 | BV-54 | 26 Feb 2018 - 18 May 2018 | 81 |
| CG-56 | QM-55 | BV-57 | 18 May 2018 - 15 Aug 2018 | 89 |
| CG-59 | QM-58 | BV-60 | 15 Aug 2018 - 14 Nov 2018 | 91 |
| CG-62 | QM-61 | BV-63 | 14 Nov 2018 - 21 Feb 2019 | 99 |
| CG-65 | QM-64 | BV-66, BV-66D* | 21 Feb 2019 - 10 May 2019 | 78 |
| CG-68 | QM-67 | BV-69, BV-69D | 10 May 2019 - 22 Aug 2019 | 104 |
| CG-71 | QM-70 | BV-72, BV-72D | 22 Aug 2019 - 28 Nov 2019 | 98 |
| CG-74 | QM-73 | BV-75, BV-75D | 28 Nov 2019 - 18 Feb 2020 | 82 |
| CG-77 | QM-76 | BV-78, BV-78D | 18 Feb 2020 - 13 May 2020 | 85 |
| CG-80 | QM-79 | BV-81, BV-81D | 13 May 2020 - 11 Aug 2020 | 90 |
| CG-83 | QM-82 | BV-84, BV-84D | 11 Aug 2020 - 13 Nov 2020 | 94 |
| CG-86 | QM-85 | BV-87, BV-87D | 13 Nov 2020 - 22 Feb 2021 | 101 |
| CG-89 | QM-88 | BV-90, BV-90D | 22 Feb 2021 - 18 May 2021 | 85 |
| CG-92 | QM-91 | BV-93, BV-93D | 18 May 2021 - 07 Sep 2021 | 112 |

(*) D field duplicate PAS samples at the BV site

Table S2 HRGC and HRMS parameters for the determination of PCDDs/PCDFs and dl-PCBs

| Oven temperature program | Ratio (°C/min) | Temperature (°C) | Hold time (min.) |
| --- | --- | --- | --- |
| Initial |  | 150 | 2 |
|  | 20 | 220 | 16 |
|  | 5 | 320 | 15 |
| Carrier gas | Helium |  |  |
| Carrier gas flow | 1.0 mL/min. |  |  |
| Injection volume | 2 µL splitless |  |  |
| Column type | DB-5MS Capillary Column |  |  |
| Column length | 60 m |  |  |
| Column inner diameter | 0.25 mm |  |  |
| Film thickness | 0.25 µm |  |  |
| Injector temperature | 280°C |  |  |
| Source temperature | 290°C |  |  |
| Detector type | HRMS |  |  |
| Ionization mode | Positive electron ionization (EI+) |  |  |
| Acquisition mode | Voltage SIR |  |  |
| Resolution | ≥ 10,000 with a 10% valley |  |  |

Table S3 Concentrations of PCDDs/PCDFs, dl-PCBs (fg/PUF/day) in ambient air in residential areas in Hanoi from the winter of 2017 to the summer of 2021

| PAS site | CG (n = 15) |  |  | QM (n = 15) |  |  | BV (n = 15) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Congener | Min. | Ave. | Max. | Min. | Ave. | Max. | Min. | Ave. | Max. |
| 2,3,7,8-TCDD | 20 | 39 | 74 | 29 | 54 | 107 | ND | 20 | 60 |
| 1,2,3,7,8-PeCDD | 70 | 107 | 190 | 55 | 112 | 233 | 18 | 44 | 72 |
| 1,2,3,4,7,8-HxCDD | 32 | 47 | 70 | 26 | 50 | 88 | 7 | 21 | 38 |
| 1,2,3,6,7,8-HxCDD | 74 | 107 | 147 | 53 | 114 | 266 | 21 | 49 | 79 |
| 1,2,3,7,8,9-HxCDD | 51 | 69 | 84 | 43 | 85 | 246 | 8 | 36 | 63 |
| 1,2,3,4,6,7,8-HpCDD | 208 | 299 | 379 | 157 | 306 | 512 | 73 | 164 | 255 |
| OCDD | 251 | 477 | 792 | 194 | 579 | 2358 | 134 | 334 | 654 |
| 2,3,7,8-TCDF | 250 | 359 | 508 | 230 | 374 | 645 | 84 | 198 | 385 |
| 1,2,3,7,8-PeCDF | 194 | 241 | 298 | 147 | 242 | 358 | 72 | 120 | 222 |
| 2,3,4,7,8-PeCDF | 290 | 401 | 530 | 211 | 425 | 822 | 101 | 213 | 427 |
| 1,2,3,4,7,8-HxCDF | 228 | 304 | 395 | 154 | 282 | 469 | 82 | 141 | 212 |
| 1,2,3,6,7,8-HxCDF | 170 | 226 | 301 | 113 | 210 | 347 | 63 | 117 | 184 |
| 2,3,4,6,7,8-HxCDF | 199 | 250 | 337 | 121 | 234 | 367 | 72 | 129 | 194 |
| 1,2,3,7,8,9-HxCDF | 64 | 113 | 206 | 48 | 108 | 221 | 22 | 55 | 135 |
| 1,2,3,4,6,7,8-HpCDF | 444 | 648 | 954 | 268 | 529 | 913 | 233 | 318 | 475 |
| 1,2,3,4,7,8,9-HpCDF | 43 | 100 | 154 | 30 | 89 | 156 | 27 | 55 | 125 |
| OCDF | 54 | 117 | 227 | 44 | 115 | 223 | 28 | 53 | 127 |
| 3,4,4´,5-TCB (#81) | 162 | 738 | 1565 | 228 | 667 | 1229 | 259 | 415 | 945 |
| 3,3´,4,4´-TCB (#77) | 3048 | 3989 | 6155 | 1316 | 2870 | 4536 | 1187 | 1883 | 3676 |
| 2´,3,4,4´,5- PeCB (#123) | 352 | 1225 | 2582 | 161 | 694 | 2184 | 142 | 565 | 2530 |
| 2,3´,4,4´,5-PeCB (#118) | 9958 | 29650 | 74850 | 4480 | 11710 | 25820 | 2978 | 9514 | 23380 |
| 2,3,4,4´,5- PeCB (#114) | 431 | 1309 | 2752 | 222 | 634 | 1113 | 165 | 468 | 1214 |
| 2,3,3´,4,4´-PeCB (#105) | 4109 | 12710 | 32050 | 1775 | 5466 | 13870 | 1218 | 4199 | 10390 |
| 3,3´,4,4´,5-PeCB (#126) | 355 | 600 | 1162 | 221 | 522 | 766 | 175 | 291 | 501 |
| 2,3´,4,4´,5,5´-HxCB (#167) | 213 | 692 | 1321 | 209 | 421 | 641 | 57 | 300 | 575 |
| 2,3,3´,4,4´,5-HxCB (#156) | 964 | 1702 | 2782 | 458 | 886 | 1441 | 341 | 615 | 1117 |
| 2,3,3´,4,4´,5´-HxCB (#157) | 283 | 598 | 2100 | 138 | 280 | 417 | 93 | 290 | 1305 |
| 3,3´,4,4´,5,5´-HxCB (#169) | 49 | 141 | 338 | 51 | 124 | 263 | 14 | 63 | 243 |
| 2,3,3´,4,4´,5,5´-HpCB(#189) | 113 | 213 | 395 | 69 | 186 | 338 | 32 | 118 | 340 |
| ∑PCDDs | 748 | 1145 | 1586 | 591 | 1300 | 3533 | 267 | 669 | 1010 |
| ∑PCDFs | 2209 | 2759 | 3306 | 1574 | 2608 | 3774 | 868 | 1400 | 2306 |
| ∑dl-PCBs | 20920 | 53570 | 123800 | 9500 | 24460 | 49480 | 7110 | 18720 | 44630 |
| ∑TEQD/F | 351 | 418 | 499 | 278 | 452 | 670 | 102 | 201 | 361 |
| ∑TEQdl | 21 | 64 | 128 | 13 | 54 | 84 | 14 | 30 | 53 |
| ∑TEQD/F&dl | 372 | 482 | 615 | 317 | 506 | 752 | 121 | 231 | 414 |
| PeCDD/TCDD | 1430 | 3100 | 6810 | 1170 | 2160 | 3530 | 1020 | 2270 | 4250 |
| ∑PCDFs/∑PCDDs | 3570 | 2380 | 1560 | 2940 | 2040 | 1010 | 3230 | 2130 | 1410 |
| % TCDD/∑TEQD/F&dl | 4,5 | 8,2 | 15,1 | 7,1 | 10,8 | 18,1 | 3,9 | 9,1 | 14,5 |
| %∑TEQD/F/∑TEQD/F&dl | 79,1 | 87,2 | 94,3 | 84,8 | 89,2 | 96,8 | 82,7 | 86,8 | 94,9 |

ND not detected or value below the detection limit, min. minimum value, ave. average value, max. maximum value

Table S4 Annual average ∑TEQD/F&dl (fg/PUF/day) concentration in Hanoi from November 2017 to September 2021

| ∑TEQD/F&dl | CG urban area |  |  | QM industrial-suburban area |  |  | BV rural area |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ave. | Min. | Max. | Ave. | Min. | Max. | Ave. | Min. | Max. |
| Year 2018 | 399 | 372 | 424 | 442 | 340 | 628 | 243 | 200 | 277 |
| Year 2019 | 521 | 489 | 579 | 540 | 330 | 752 | 285 | 149 | 414 |
| Year 2020 | 500 | 445 | 551 | 492 | 331 | 625 | 195 | 154 | 231 |
| Year 2021 | 459 | 289 | 615 | 555 | 445 | 740 | 162 | 79 | 235 |

Table S5 Seasonal variation of ∑TEQD/F&dl (fg/PUF/day) in Hanoi during Winter 2017 and Summer 2021

| ∑TEQD/F&dl | CG urban area |  |  | QM industrial-suburban area |  |  | BV rural area |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ave. | Min. | Max. | Ave. | Min. | Max. | Ave. | Min. | Max. |
| Spring (n = 4) | 472 | 377 | 551 | 635 | 578 | 740 | 280 | 213 | 414 |
| Summer (n = 4) | 452 | 372 | 524 | 559 | 413 | 752 | 208 | 121 | 281 |
| Autumn (n = 3) | 465 | 421 | 489 | 344 | 317 | 386 | 182 | 149 | 219 |
| Winter (n = 4) | 534 | 424 | 615 | 446 | 340 | 520 | 241 | 200 | 299 |

ADD calculation:

The ADD (fg TEQ/kg BW/day) through inhalation was calculated using the following equation (United States Environmental Protection Agency 2018):

ADD = Cair × InhR × ET × EF × ED/BW × AT

Where:

Cair is the ∑TEQD/F&dl in ambient air (fg/m3),

InhR is the inhalation rate (m3/day),

ET is the exposure time (hours/day),

EF is the exposure frequency (days/year),

ED is the exposure duration (years),

BW is the body weight (kg),

AT is the average time (days).

Since people, including adults and children, reside and work in the three study areas, we assumed that the ET was 24 h per day. The EF was 365 days per year, and the ED was 3 years and 10 months with a total of 1,393 days from 14 November 2017 to 07 September 2021. The BW varied according to the age.

According to the Vietnam National Institute of Occupational Safety and Health (VNNIOSH 2019), the average BW for Vietnamese men aged 16-59 years is 59.2 ± 8.9 kg, and the value for women is 50.8 ± 6.6 kg. Therefore, this study used the average BW (*) of 55 kg for adults (both sexes) aged 16 years and above. According to statistics (April 1, 2021), the average life expectancy for Vietnamese people of both sexes is 73.6 years (General Statistics Office 2022). The AT over the lifetime is usually 26,864 days.

The ∑TEQD/F&dl values (fg/PUF/day) at each PAS site in Table S3 were converted into the concentration per air volume (fg/m3) at an average sampling rate of 3.5 m3/day (Gouin et al. 2005; Li et al. 2011; Mari et al. 2008; Trinh Khac Sau et al. 2021).

Table S6 The ADDs (fg TEQ/kg BW/day) for the residents and children between Winter 2017 and Summer 2021

| Age group | Birth to <1 year | 2 to < 3 years | 3 to < 6 years | 6 to < 11 years | 11 to < 16 years | 16 to < 21 years | 21 to < 31 years | 31 to < 41 years | 41 to < 51 years | 51 to < 61 years | 61 to < 71 years | 71 to < 81 years | ≥ 81 years |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cair at CG area (fg/m3): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Average | 138 | 138 | 138 | 138 | 138 | 138 | 138 | 138 | 138 | 138 | 138 | 138 | 138 |
| Min | 106 | 106 | 106 | 106 | 106 | 106 | 106 | 106 | 106 | 106 | 106 | 106 | 106 |
| Max | 176 | 176 | 176 | 176 | 176 | 176 | 176 | 176 | 176 | 176 | 176 | 176 | 176 |
| Cair at QM area (fg/m3): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Average | 145 | 145 | 145 | 145 | 145 | 145 | 145 | 145 | 145 | 145 | 145 | 145 | 145 |
| Min | 90,6 | 90,6 | 90,6 | 90,6 | 90,6 | 90,6 | 90,6 | 90,6 | 90,6 | 90,6 | 90,6 | 90,6 | 90,6 |
| Max | 215 | 215 | 215 | 215 | 215 | 215 | 215 | 215 | 215 | 215 | 215 | 215 | 215 |
| Cair at BV area (fg/m3): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Average | 66,0 | 66,0 | 66,0 | 66,0 | 66,0 | 66,0 | 66,0 | 66,0 | 66,0 | 66,0 | 66,0 | 66,0 | 66,0 |
| Min | 34,6 | 34,6 | 34,6 | 34,6 | 34,6 | 34,6 | 34,6 | 34,6 | 34,6 | 34,6 | 34,6 | 34,6 | 34,6 |
| Max | 118 | 118 | 118 | 118 | 118 | 118 | 118 | 118 | 118 | 118 | 118 | 118 | 118 |
| InhR (m3/day, 95th percentile) | 12,8 | 13,7 | 13,8 | 16,6 | 21,9 | 24,6 | 21,3 | 21,4 | 21,2 | 21,3 | 18,1 | 16,6 | 15,7 |
| ED (days/year) | 365 | 365 | 365 | 365 | 365 | 365 | 365 | 365 | 365 | 365 | 365 | 365 | 365 |
| EF (3 years and 10 months, total of 1393 days from 14 November 2017 to 07 September 2021) | 3,82 | 3,82 | 3,82 | 3,82 | 3,82 | 3,82 | 3,82 | 3,82 | 3,82 | 3,82 | 3,82 | 3,82 | 3,82 |
| BW (kg) * | 11,3 | 13,3 | 17,5 | 28,7 | 50,2 | 55 | 55 | 55 | 55 | 55 | 55 | 55 | 55 |
| AT (days) | 1393 | 1393 | 1393 | 1393 | 1393 | 1393 | 1393 | 1393 | 1393 | 1393 | 1393 | 1393 | 1393 |
| ADD for the residents living in CG area: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Average | 156 | 142 | 109 | 79,7 | 60,1 | 61,7 | 53,4 | 53,6 | 53,1 | 53,4 | 45,4 | 41,6 | 39,3 |
| Min | 121 | 110 | 83,9 | 61,5 | 46,4 | 47,6 | 41,2 | 41,4 | 41,0 | 41,2 | 35,0 | 32,1 | 30,4 |
| Max | 199 | 181 | 139 | 102 | 76,7 | 78,7 | 68,1 | 68,4 | 67,8 | 68,1 | 57,9 | 53,1 | 50,2 |
| ADD for the residents living in QM area: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Average | 164 | 149 | 114 | 83,7 | 63,1 | 64,7 | 56,0 | 56,3 | 55,8 | 56,0 | 47,6 | 43,7 | 41,3 |
| Min | 103 | 93,4 | 71,5 | 52,4 | 39,5 | 40,5 | 35,1 | 35,3 | 34,9 | 35,1 | 29,8 | 27,4 | 25,9 |
| Max | 244 | 222 | 170 | 124 | 93,8 | 96,2 | 83,3 | 83,7 | 82,9 | 83,3 | 70,8 | 64,9 | 61,4 |
| ADD for the residents living in BV area: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Average | 74,8 | 68,0 | 52,1 | 38,2 | 28,8 | 29,5 | 25,6 | 25,7 | 25,5 | 25,6 | 21,7 | 19,9 | 18,9 |
| Min | 39,2 | 35,6 | 27,3 | 20,0 | 15,1 | 15,5 | 13,4 | 13,5 | 13,3 | 13,4 | 11,4 | 10,4 | 9,9 |
| Max | 134 | 122 | 93,4 | 68,5 | 51,7 | 53,0 | 45,9 | 46,1 | 45,6 | 45,9 | 39,0 | 35,7 | 33,8 |