Noise Resilient Exceptional-Point Sensing based on Neuromorphic functionalities

Arunn Suntharalingam1, Lucas Fernández-Alcázar2, Rodion Kononchuck1, Tsampikos Kottos1

1Wave Transport in Complex Systems Lab, Department of Physics, Wesleyan University, Middletown CT, USA

2Instituto de Modelado e Innovación Tecnológica (CONICET-UNNE) and Facultad de Ciencias Exactas, Naturales y Agrimensura, Universidad Nacional del Nordeste, W3404AAS Corrientes, Argentina

Supplementary Information

Coupled Mode Theory modeling of the electronic circuit

The nonlinear electronic circuit of Fig. 1A of the main text can be described by the temporal coupled mode theory (TCMT), see Eq. (1) of the main text. Apart from the simplicity and the physical intuition that a TCMT modeling can offer, it also allows us to extrapolate our findings to a broader class of systems described by such TCMT. We start the derivation of Eq. (1) by expressing the energy stored in the LC resonator as ψ2 where the complex modal amplitude of a single LC resonator ψt is

ψt=C2 Vt-iω0 V(t) .            (S1)

Above V(t) is the node voltage and ω0=2π f0=1/LC the resonant frequency of the resonator in the absence of dissipation (amplification). Using is representation, we rewrite the circuit equation of the LC resonator, i.e., d2dt2V+ω02 V=0, as a set of two (uncoupled) first order differential equations i.e., dψdt=i ω0 ψ and its complex conjugate. The latter relation emphasizes the time dependence of the modal amplitude ψ∼ eiω0t and ψ*∼ e-iω0t.

Next, we incorporate in our circuit, an energy dissipation (enhancement) associated with a nonlinear damping (antidamping) channel. We consider nonlinear components that are characterized by the I-V curve I(V)=α V/R0+b V3 where α=1 (α=-1) represents loss (gain) and b≈7⋅10-4[AV3]. The corresponding Kirchoff’s equation for the individual RLC resonator collapse to the following expression

V(t) +α Γ0 V(t) +β V2(t) V(t) +ω02 V(t) =0,         (S2)

where Γ(0)=R0 C-1 is the relaxation rate of the individual (linear) LRC oscillator, and β=3b/C. Equation (S2) can be written in terms of ψ and its conjugate ψ* as

2ψ*+2i ω0ψ*-(α Γ0+β2Cψ2)ψ-ψ*-β2Cψ3-ψ*3=0.     (S3)

By invoking a rotating wave approximation and eliminating the fast-oscillating terms, the above equation can be further simplified as

iψ*≈ω0 ψ* -iα2 Γ0 ψ* - iχ ψ*2ψ*,      (S4)

where χ=β/(4C)=3b/(4C2).

We proceed by developing a TCMT associated to the two coupled RLC tanks. From Kirchoff’s laws we obtain the equations for the voltage Vn in resonator n=1,2

1+κ V1-κ V2-Γ10 V1+β V12 V1 +ω02 V1=01+κ V2-κ V1+Γ20V2 +β V22 V2 +ω02 V2=0 ,     (S5)

where Γn(0)=Rn0C-1is the relaxation rate of each resonator (below we will assume the high-Q limit, i.e.,  Γn0ω0≪1), Cκ=κ C is the (voltage controlled) coupling capacitance, with κ≪1 being the coupling strength coefficient. Using the complex-mode representation for each resonator n, we rewrite the circuit equations Eq. (S5) as

iψ1*ψ2*=ω0  1-κ2+ iΓ1(0)2-iχ ψ12 ω0 κ/2ω0 κ/2ω0  1-κ2- iΓ2(0)2-iχ ψ22 ψ1*ψ2*;   (S6)

where we have invoked, as in the case of single RLC resonator, the rotating wave approximation, together with the weak coupling limit κ≪ 1 and high-Q Γn0ω0≪1 approximations. We further simplify Eq. (S6) by defining the rescaled complex field an=χ/ω0 ψn* and time τ=ω0 t,

iddτa1a2= 1-κ2+ iγ1(0)-a12  κ/2κ/2 1-κ2- iγ2(0)+a22 a1a2;  (S7)

where γn(0)=Γn0/(2ω0)=1/(2 Rn0 C ω0).

Coupling of the circuit to transmission lines

The effect of a weak coupling of the RLC resonators to a transmission line (TL) via a coupling capacitance Ce=εC, ε≪1, can be also modeled using CMT. At the node connecting the TL with the coupling capacitance, the voltage and current flowing toward the n-th (n=1,2) RLC resonator can be written as a superposition of forward and backward propagating voltage waves, VTL,n(+) and VTL,n(-)

VTL,n=VTL,n++VTL,n-;    ITL,n=ITL,n++ITL,n-=1z0VTL,n+-VTL,n-,       (S8)

where z0=50Ω is the TL’s characteristic impedance. In turn, the voltages can be represented by complex wave amplitudes VTL,n(±)=z02Sn(±)+Sn±*, where Sn(±)=Sn(±)e-iω0t, being Sn(±)  a slowly varying amplitudes. For a single RLC resonator which is weakly coupled to a TL, we have

ITL,n=CeVTL,n-Vn;   -1CITL,n+Vn+Γn(0)Vn+ω02Vn=0,       (S9)

where Vn represents the voltage at the n-th RLC resonator with characteristic frequency ω0 and decay rate Γn(0). Under the assumptions of impedance matching z0/Z0~O(1) and weak coupling ε→0, we rewrite Eqs. (S8) using the complex mode amplitude of the resonator ψ* and the input/output wave amplitude S(±)

idψn*dt≈ω01-η2Z0z0ψn*-ηω0ψn*-i2ω0ηSn(+)Sn(-)≈Sn(+)-i2ω0ηψn*,      (S10)

where we also invoked the rotating wave approximation, and we introduced the TL-RLC coupling coefficient η≡(z0/Z0)ε2/2. Using the transformations an=χ/ω0 ψn* and τ=ω0t, and combining Eqs. (S7) and (S10) we arrive to Eq. (1) of the main text which describes the whole system of coupled RLC resonators and TLs. Let us finally point out that when the input wave Sn(+)=0, the output power emitted from the n-th node takes a simple form

Pn=Sn-2≈2ω0ηψn*2=2χηan2,      (S11)

Equations of motion in polar form

It is convenient for our analysis, to rewrite Eq. (1) of the main text in polar representation. To this end, we express the complex amplitudes as anτ=Anτeiφnτe-ifτ/f0 where the magnitudes An≥ 0 and the phases φn of the fields in resonator n=1,2 are real numbers. Substitution of these expressions back to Eq. (1) leads to the following set of coupled differential equations

A1 = γ1(0)-A12-ηA1+κ2A2 sinφ;    A2=-γ2(0) +A22 +ηA2-κ2A1sinφA1φ1 = ff0 - νkA1-κ2A2 cosφ;      A2φ2 = ff0 - νkA2-κ2A1 cosφ,    (S12)

where φ≡φ2-φ1 is the relative phase. If A1,A2≠0, the last two equations can be combined in a compact form

φ  =κ2(A2A1 -A1A2) cosφ,     (S13)

Figure S1: Nonlinear Super-modes and effective gain/loss coefficient: (A) The structure of the NS in the (κ/γ2(0), γ10/γ20) parameter space. Blue (green) surface indicates the value of the field amplitude in the first A1 (second A2) RLC resonator. Temporal evolution of the effective gain (loss) coefficients γ1(τ) (γ2(τ)) for linear gain/loss parameter values  γ1(0)≈0.18, γ20≈0.12 and (B) κ=κNLEPD+0.1 in the AD regime where the system is protected by a parity-time symmetry; (C) κ=κNLEPD-0.1 in the OD regime where the system is explicitly violates the parity-time symmetry. (D) Overview of the asymptotic gain/loss coefficients versus the coupling variation from the NLEPD. The coupling to the TL is taken η=0.01 while κNLEPD≈0.3.

Finally, in this polar representation the emitted power spectrum Eq. (S10) takes the form 
Pnω=η⋅An2431bZ02.

Nonlinear supermodes

The nonlinear supermodes correspond to the fixed points of Eqs. (S12,S13). We find the fixed points by requiring constant magnitudes and relative phase, i.e., A1=A2=0, and φ=0. The latter condition leads to three possible scenarios: (1) A1=A2≠0, (2) cosφ=0, and (3) a variant of the first case where A1=A2=0. We refer to this last case as the trivial scenario and it is not relevant for hypersensitive sensing schemes. Therefore, we will not analyze it further. A panorama of the nodal amplitudes (A1,A2) is shown in Fig. S1A versus the relative gain γ10/γ2(0) and the relative capacitive coupling κ/γ2(0). Below, we analyze in detail each one of the first two cases.

AD Supermodes

We consider first the case of identical field amplitude in both resonators, i.e., A1=A2=A. This corresponds to the AD domain, characterized by an exact parity-time symmetry. From the first set of Eqs. (S12), we find that the field amplitude and the relative phase are given by the expressions

A =γ1(0)-γ2(0)-2η2,         sinφ=-γ1(0)+γ2(0)κ,        (S14)

which, in turn, establishes bounds for the parameters where such a solution exists. From the second of these equations, we conclude that the two supermodes differ in their relative phase i.e. φ±=-1±12π∓sin-1γ10+γ20κ while the coupling between resonators is bounded by κ≥γ10+γ2(0). This condition determines the boundary between the AD and the OD domains (regions II and I, respectively in Fig. 1B). On the other hand, the physical requirement that A∈R>0 which guarantees the existence of a non-trivial steady state, leads to the constraint that the gain must be strong enough to overcome the total loss of the system, i.e. γ1(0)≥ γ2(0)+2η. This condition determines the boundary between the AD domain and region III (see Fig. 1 B).

The nonlinear eigenfrequencies f± associated with the NS of Eq. (S14) can be found by imposing in either of the second set of Eqs. (S12), the fixed-point condition φ1,2=0 together with the expression for cosφ±=±1-γ1(0)+γ2(0)κ2derived from the second equation in Eq. (S14). We have for the corresponding frequencies

f±=f0⋅(νκ±12κ2-γ1(0)+γ2(0)2),     (S15)

which indicate the existence of a NLEPD at κNLEPD=γ1(0)+γ2(0) corresponding to the transition between AD and OD domain, where not only the eigenfrequencies but also the eigenvectors coalesce.

OD Supermodes

Next, we discuss the fixed points of Eqs. (S12,S13) when cosφ=0, i.e. φ=±π/2. From the first set of Eqs. (S12), we conclude that only the relative phase φ=-π2 ensures a solution with A22≥ 0. In this case the NS have an asymmetric field amplitude, i.e., A1≠ A2 and the corresponding field intensities at each resonator are

A12=γ1(0)-η-κ2 ρ ;  A22 = -γ20-η +κ2  ρ,     (S16)

where, ρ=A2/A1>0 represents the relative field amplitude and is a solution of the following quartic equation

0= 1 - 2ρ γ2(0)+ηκ -2ρ3  γ1(0)-ηκ +ρ4.     (S17)

Out of the four roots of Eq. (S17), one has to select the ones that satisfy (a) ρ∈R>0, , bρ≤2γ1(0)-ηκ such that A12≥0, and (c) ρ≤κ2γ2(0)+η such that A22≥0 (see Eq. (S16)). It turns out from our extensive numerical analysis that the requirement for the stability of the fixed point in the OD domain, results in  ρ≤ 1 for the relative field amplitude of the nonlinear supermode Eq. (S16).  This output is consistent with the intuition that the intensity at the lossy resonator is smaller than the intensity at the gain resonator at the OD domain where parity-time symmetry is explicitly violated.

The corresponding nonlinear eigenfrequency f can be found by substituting in the last Eq. (S12) the value of the relative phase φ=-π2. In this case, we get

f=f0⋅νκ.        (S18)

Symmetry phases of the dimer system of Eq. (1)

It is finally instructive to evaluate the effective gain and loss coefficients γ1t and γ2(t) as a function of time in the AD and OD regimes. In Fig. S1B we show their temporal behavior for a typical set of parameters γ1(0)≈0.18, γ2(0)≈0.12 and κ=κNLEPD+0.1 for which the system of Eq. (1) is in the AD domain (κNLEPD≈0.3). We see that in the asymptotic time limit, γ1(∞)=γ2(∞), indicating that the electronic dimer is in the exact parity-time symmetric phase. Similarly, in Fig. S1C we show the temporal behavior of γ1t and γ2(t) for the same values of parameters γ1(0), γ2(0) as previously and κ=κNLEPD-0.1 for which the system of Eq. (1) is in the OD domain. In this case, the asymptotic values of the effective gain and loss parameters, differ from one-another, i.e., γ1(∞)≠γ2(∞), indicating that the system is in an explicitly broken parity-time symmetric phase. A panorama of the asymptotic values γ1(∞),γ2(∞) versus the coupling constant κ for a fixed relative gain γ10γ20=1.5 is shown in Fig. S1D.

Jacobian matrix and stability analysis in the parameter space

The system of Eqs. (S12,S13) can be written in the form dudτ=f(u), where  u=A1 , A2, φ T. The NSs of the previous section u0 are obtained by the fixed-point condition du0dτ= f(u0)=0. Linearizing the equations of motion around u0 we get fu0+δu≈ J(u0) δu,  where J(u0) is the Jacobian matrix evaluated at u0. Subsequently the linearized equations of motion read as,

dδudτ= J(u0) δu;   Ju=γ1(0)-η-3A12κ2sinφκ2A2cosφ-κ2sinφγ2(0)-η-3A22-κ2A1cosφ-κ2cosφ(A2A12+1A2)κ2cosφ(1A1+A1A2)κ2sinφ(A1A2-A2A1).    (S19)

The first Lyapunov criterion allow us to characterize the stability of each nonlinear supermode by analyzing the eigenvalues of the Jacobian matrices evaluated at the NSs u0. The fixed-point solution u0 is stable if all the eigenvalues λn;n=1,2,3 of the Jacobian (evaluated at the supermode) have negative real part, indicating that the solution is an attractor. If, on the other hand, one of the eigenvalues of the Jacobian has positive real part, the solution u0 is unstable. In both cases the associated fixed points are classified as hyperbolic equilibria. These equilibria points are further classified as nodes or foci depending on if the corresponding Jacobian eigenvalues are purely real or have also an imaginary component. In case that one of the eigenvalues of the Jacobian has a zero real part, then the fixed point is characterized as non-hyperbolic. Non-hyperbolic equilibria are not robust to small perturbations (i.e., the system is not structurally stable) and their stability is determined via direct dynamical simulations with the initial dynamical equations.

Figure S2: Parameter space characterization and stability of fixed points. (A) A characterization of the parameter space (κ/γ20,γ10/γ20) of the nonlinear dimer described by the TCMT Eq. (S12,S13) in terms of the nature and number of its (stable) fixed points. Domain I is identified with the OD phase and supports one nontrivial stable fixed point; Domain II is identified with the AD phase and supports two non-trivial stable fixed points; Domain III supports only trivial stable fixed points (i.e., A1=A2=0). The white dashed lines indicate the borders of various domains (see main text). The red dashed line indicates NLEPDs associated with the coalescence of unstable fixed points and occur in the domain III. The red solid line indicates NLEPD associated with the coalescence of stable fixed points and occur at the transition between OD and AD. The inset indicates the parameter domain for which we have analyze the stability (and number) of steady-state solutions of the actual electronic circuit using an NGSPICE simulator. The blue highlighted area indicates the OD domain while the yellow highlighted area indicates the AD domain. The red line between the two domains matches with the one found from the analysis of the TCMT. (B) Dynamical simulations with Eqs. (S12,S13) with κ=κNLEPD indicate that various initial conditions (black circles) in the phase space of the system, converge asymptotically to the corresponding fixed point (violet circle).

A panorama of the (κ,γ10γ20) parameter space and its partition to various domains, according to the nature (stable nontrivial versus stable trivial) and number (one, two or none) of stable fixed points, is shown in Fig. S2A. This map has been created by analyzing the stability of the TCMT Eq. (1) of the main text (Eqs. (S12,S13) of the Supplementary Information) together with the evaluation of the eigenmodes of the Jacobian matrix Eq. (S19). In the inset of Fig. S2A, we show the results of the numerical analysis using NGSPICE. We have simulated the evolution of several initial conditions for various (κ, γ10γ20) values in the domain between AD and OD (dashed square) and analyzed the emitted power spectrum associated with the voltage of the gain resonator. The number of extracted frequencies peak(s) and the values of the amplitudes A1,A2 have been used to identify the parameter domain.

Let us discuss in more detail the stability of the NLEPD at the transition between AD and OD domains, see Fig. S2A. This fixed point turns out to be non-hyperbolic (i.e. one eigenvalue of the Jacobian matrix has zero real part). For an analysis of its stability, we cannot rely on the first Lyapunov criterion. Instead, we have performed direct dynamical simulations for a large ensemble of initial conditions in the phase space of the system and analyze their long-time dynamics. We have found that in all cases, the trajectories are attracted to a final state that is the NLEPD, see Fig. S2B.

For completeness of the discussion, we would also like to comment on the unstable non-trivial solutions that exists both in the AD domain and OD domain (see dashed lines in Fig. 2A). These are fixed points of Eq. (S12) corresponding to cosφ=0 and A1,A2≠0,0 and exists both in the AD and in the OD domains. The analysis of these fixed points follows the lines of the “OD Supermode” section and correspond to roots of Eq. (S17) with ρ>1. The corresponding frequency of these solutions is given by the either of the second set of Eq. (S12) and are funstable=f0⋅νk.