Additional File 1: Tidal parameters for the superconducting gravimeters TT70 #007 and #016 at Mizusawa

In this file, we present tidal parameters at Mizusawa obtained by applying the software BAYTAP-G (Tamura et al., 1991). These are shown for the convenience of potential users of the data from the superconducting gravimeters TT70 #007 and #016 installed at Mizusawa. BAYTAP-G decomposes the input data into a combination of a tidal term, a response term, a smooth trend term and an irregular term. Smoothness of the trend term is controlled with a hyperparameter. The optimal value of the hyperparameter is selected by Akaike Bayesian Information Criterion (ABIC).

Table A1-1 and Table A1-2 show the results for #007 and #016, respectively. BAYTAP-G uses grouping of the tidal waves derived from the tidal potential of Tamura et al. (1991). In the results shown below, a total of 909 short-period tidal waves are divided into 20 groups. Each group corresponds to a particular tidal wave, represented by the symbol shown in the column “Symbol”. “Factor” is the non-dimensional delta factor. “Phase” is in degree, with positive values indicating advance. The delay due to the antialiasing filter (GGP1 filter) has not been corrected. “Amplitude” is in µGal.

The results of tidal analysis for the two instruments differ slightly. The difference lies mainly in the magnitude (delta factor and amplitude) of the tidal waves. Comparing the results for the M2, O1, K1 and S2 waves suggests that the magnitude from #007 is systematically larger than that from #016 by about 0.1–0.2 %. This is likely due to the error in the sensitivity calibration. This error in the sensitivity does not have a serious effect on the main conclusion of the present paper.

Table A1-1: Tidal parameters at Mizusawa for the superconducting gravimeter TT70 #007 from January 1, 2014 through October 24, 2017. The optimized hyperparameter is 1.4142. Atmospheric admittance is –0.322 µGal hPa–1.

| Group | Number | Symbol | Factor | Phase | Amplitude |
| --- | --- | --- | --- | --- | --- |
| 1 | 1–143 | Q1 | 1.21149 ± 0.00281 | 1.836 ± 0.133 | 7.052 ± 0.016 |
| 2 | 144–201 | O1 | 1.21340 ± 0.00052 | 1.187 ± 0.024 | 36.891 ± 0.016 |
| 3 | 202–249 | M1 | 1.21416 ± 0.00501 | 1.569 ± 0.236 | 2.903 ± 0.012 |
| 4 | 250–256 | 1 | 1.20137 ± 0.01366 | 0.719 ± 0.651 | 0.994 ± 0.011 |
| 5 | 257–266 | P1 | 1.20644 ± 0.00079 | –0.022 ± 0.038 | 17.067 ± 0.011 |
| 6 | 267–270 | S1 | 1.74153 ± 0.04707 | 27.406 ± 1.565 | 0.583 ± 0.016 |
| 7 | 271–288 | K1 | 1.19243 ± 0.00029 | 0.142 ± 0.014 | 50.986 ± 0.013 |
| 8 | 289–292 | 1 | 1.34221 ± 0.03348 | 0.059 ± 1.429 | 0.449 ± 0.011 |
| 9 | 293–305 | 1 | 1.20644 ± 0.01795 | 1.336 ± 0.853 | 0.735 ± 0.011 |
| 10 | 306–345 | J1 | 1.21064 ± 0.00502 | –0.725 ± 0.238 | 2.895 ± 0.012 |
| 11 | 346–450 | OO1 | 1.19144 ± 0.01159 | –0.913 ± 0.557 | 1.559 ± 0.015 |
| 12 | 451–549 | 2N2 | 1.19012 ± 0.00180 | 0.297 ± 0.087 | 1.362 ± 0.002 |
| 13 | 550–599 | N2 | 1.17298 ± 0.00036 | 0.493 ± 0.018 | 10.148 ± 0.003 |
| 14 | 600–655 | M2 | 1.18640 ± 0.00007 | 1.418 ± 0.003 | 53.609 ± 0.003 |
| 15 | 656–663 | 2 | 1.19030 ± 0.00905 | 1.606 ± 0.435 | 0.397 ± 0.003 |
| 16 | 664–681 | L2 | 1.20143 ± 0.00237 | 1.539 ± 0.113 | 1.534 ± 0.003 |
| 17 | 682–701 | T2 | 1.19948 ± 0.00250 | 0.373 ± 0.119 | 1.474 ± 0.003 |
| 18 | 702–710 | S2 | 1.21093 ± 0.00016 | 0.290 ± 0.009 | 25.458 ± 0.003 |
| 19 | 711–827 | K2 | 1.20748 ± 0.00070 | 0.662 ± 0.033 | 6.901 ± 0.004 |
| 20 | 828–909 | M3 | 1.09338 ± 0.00233 | 2.003 ± 0.122 | 0.753 ± 0.002 |

Table A1-2: Tidal parameters at Mizusawa for the superconducting gravimeter TT70 #016 from October 29, 2017 through July 14, 2021. The optimized hyperparameter is 2.0000. Atmospheric admittance is –0.345 µGal hPa–1.

| Group | Number | Symbol | Factor | Phase | Amplitude |
| --- | --- | --- | --- | --- | --- |
| 1 | 1–143 | Q1 | 1.21289 ± 0.00092 | 1.785 ± 0.043 | 7.060 ± 0.005 |
| 2 | 144–201 | O1 | 1.21189 ± 0.00017 | 1.190 ± 0.008 | 36.845 ± 0.005 |
| 3 | 202–249 | M1 | 1.20833 ± 0.00177 | 0.206 ± 0.084 | 2.889 ± 0.004 |
| 4 | 250–256 | 1 | 1.19672 ± 0.00536 | –0.420 ± 0.257 | 0.990 ± 0.004 |
| 5 | 257–266 | P1 | 1.20288 ± 0.00031 | 0.243 ± 0.015 | 17.016 ± 0.004 |
| 6 | 267–270 | S1 | 1.51453 ± 0.01851 | 8.099 ± 0.712 | 0.507 ± 0.006 |
| 7 | 271–288 | K1 | 1.19129 ± 0.00010 | 0.220 ± 0.005 | 50.937 ± 0.004 |
| 8 | 289–292 | 1 | 1.29347 ± 0.01296 | 0.114 ± 0.574 | 0.433 ± 0.004 |
| 9 | 293–305 | 1 | 1.22304 ± 0.00698 | –0.007 ± 0.327 | 0.745 ± 0.004 |
| 10 | 306–345 | J1 | 1.20392 ± 0.00166 | –0.661 ± 0.079 | 2.879 ± 0.004 |
| 11 | 346–450 | OO1 | 1.19331 ± 0.00275 | –1.291 ± 0.132 | 1.561 ± 0.004 |
| 12 | 451–549 | 2N2 | 1.18882 ± 0.00073 | 0.485 ± 0.035 | 1.361 ± 0.001 |
| 13 | 550–599 | N2 | 1.17361 ± 0.00015 | 0.630 ± 0.007 | 10.153 ± 0.001 |
| 14 | 600–655 | M2 | 1.18476 ± 0.00003 | 1.396 ± 0.001 | 53.535 ± 0.001 |
| 15 | 656–663 | 2 | 1.17551 ± 0.00385 | 1.470 ± 0.187 | 0.392 ± 0.001 |
| 16 | 664–681 | L2 | 1.19720 ± 0.00110 | 2.005 ± 0.053 | 1.529 ± 0.001 |
| 17 | 682–701 | T2 | 1.21986 ± 0.00103 | 0.103 ± 0.049 | 1.499 ± 0.001 |
| 18 | 702–710 | S2 | 1.20861 ± 0.00007 | 0.294 ± 0.004 | 25.409 ± 0.001 |
| 19 | 711–827 | K2 | 1.20968 ± 0.00023 | 0.769 ± 0.011 | 6.913 ± 0.001 |
| 20 | 828–909 | M3 | 1.09227 ± 0.00111 | 1.797 ± 0.058 | 0.752 ± 0.001 |