Supplementary Information

CFD Modeling of a Multichannel Fischer–Tropsch Reactor Module with Microscale Cooling Channels: Effects of Mirrored Structure Cooling Layers

# Yesol Woo1, Da Bin Oh2, Jae Eun Park2, Seung Ju Han3, Yun-Jo Lee3, Myung-June Park1,2,*

1Department of Energy Systems Research, Ajou University, Suwon 16499, Republic of Korea

2Department of Chemical Engineering, Ajou University, Suwon 16499, Republic of Korea

3C1 Gas & Carbon Convergent Research Center, Korea Research Institute of Chemical Technology (KRICT), Daejeon 34114, Republic of Korea

*Corresponding author: mjpark@ajou.ac.kr

Table S1. CFD simulation results for the FTS reactor with the mirror-structured cooling microchannels presented in this study

|  | Feed/coolant temperature [℃] | Space Velocity [mL/gcat/h] | Conversion [%] | C5+ Selectivity [%] | Maximum temperature [℃] | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 215 | 5000 | 80.35 | 85.88 | 228.01 | Effects of temperature and space velocity |
| 2 | 215 | 6600 | 73.1 | 85.62 | 228.29 |  |
| 3 | 215 | 8000 | 67.33 | 85.42 | 228.89 |  |
| 4 | 215 | 10000 | 60.18 | 85.22 | 229.50 |  |
| 5 | 220 | 5000 | 85.83 | 82.00 | 228.01 |  |
| 6 | 220 | 6600 | 80.87 | 81.78 | 238.30 |  |
| 7 | 220 | 8000 | 76.67 | 81.39 | 238.96 |  |
| 8 | 220 | 10000 | 71.10 | 80.84 | 240.22 |  |
| 9 | 225 | 5000 | 89.14 | 73.32 | 265.58 |  |
| 10 | 225 | 6600 | 84.57 | 76.25 | 253.61 |  |
| 11 | 225 | 8000 | 83.69 | 75.87 | 253.27 |  |
| 12 | 225 | 10000 | 80.44 | 74.71 | 254.94 |  |
| 13 | 230 | 8000 | 88.87 | 65.63 | 292.99 |  |
| 14 | 230 | 10000 | 87.97 | 65.24 | 285.81 |  |
|  | Feed/coolant temperature [℃] | Pressure [bar] | Conversion [%] | C5+ Selectivity [%] | Maximum temperature [℃] | Remarks |
| 15 | 215 | 20 | 73.1 | 85.62 | 228.29 | Effects of temperature and pressure |
| 16 | 220 | 20 | 80.87 | 81.78 | 238.35 |  |
| 17 | 220 | 22 | 84.83 | 80.65 | 243.16 |  |
| 18 | 225 | 15 | 73.03 | 80.14 | 238.75 |  |
| 19 | 225 | 20 | 84.57 | 76.25 | 253.6 |  |
| 20 | 225 | 22 | 88.74 | 72.21 | 272.28 |  |
| 21 | 230 | 15 | 80.00 | 75.33 | 249.08 |  |

Table S2. Fitting polynomials and the fitted values of the parameters for the effects of operating conditions

| Independent variables | Output variable | Parameters (95% confidence interval) | R2 |
| --- | --- | --- | --- |
| Temperature (x) and space velocity (y) | Conversion = p00 + p10*x + p01*y + p20*x2 + p11*x*y + p02*y2 | p00 = -3617 (-1.092e+04, 3686), p10 = 27.12 (-33.61, 87.86), p01 = 54.89 (-20.38, 130.2), p20 = -0.04979 (-0.1764, 0.07682), p11 = -0.1974 (-0.5286, 0.1337), p02 = -0.2228 (-0.37, -0.07553) | 0.9978 |
|  | Selectivity = p00 + p10*x + p01*y + p20*x2 + p11*x*y + p02*y2 | p00 = -2497 (-3470, -1523), p10 = 24.65 (15.73, 33.56), p01 = -0.004306 (-0.0164, 0.007787), p20 = -0.0589 (-0.07937, -0.03842), p11 = 2.781e-05 (-2.779e-05, 8.341e-05), p02 = -1.246e-07 (-2.81e-07, 3.177e-08) | 0.9935 |
|  | Tmax = p00 + p10*x + p01*y + p20*x2 + p11*x*y + p30*x3 + p21*x2*y | p00 = -1.917e+05 (-1.393e+06, 1.01e+06), p10 = 2698 (-1.364e+04, 1.904e+04), p01 = -1.574 (-14.63, 11.48), p20 = -12.64 (-86.78, 61.5), p11 = 0.01441 (-0.1038, 0.1327), p30 = 0.01975 (-0.09245, 0.1319), p21= -3.297e-05 (-0.0003006, 0.0002347) | 0.7968 |
| Temperature (x) and pressure (y) | Conversion = p00 + p10*x + p01*y + p20*x2 + p11*x*y + p02*y2 | p00 = -3006 (-2.288e+04, 1.687e+04), p10 = 22.86 (-140.2, 185.9), p01 = 41.5 (-195.3, 278.3), p20 = -0.04207 (-0.379, 0.2948), p11 = -0.1577 (-1.103, 0.7878), p02 = -0.1034 (-1.362, 1.155) | 0.9956 |
|  | Selectivity = p00 + p10*x + p01*y + p11*x*y + p02*y2 | p00 = 33.16 (-1658, 1724), p10 = 0.1852 (-6.914, 7.284), p01 = 15.6 (-75.89, 107.1), p11 = -0.06418 (-0.4261, 0.2977), p02 = -0.0557 (-0.6968, 0.5854) | 0.9603 |
|  | Tmax = p00 + p10*x + p01*y + p20*x2 + p11*x*y | p00 = 9952 (-2.292e+04, 4.283e+04), p10 = -79.7 (-354.1, 194.7), p01 = -130.2 (-476.2, 215.8), p20 = 0.161 (-0.412, 0.734), p11 = 0.5986 (-0.9426, 2.14) | 0.9545 |