## SI8 Decarbonisation Assumptions

Decarbonisation of the forestry value chain is highly dependent on the development of zero emissions mobile machinery and heavy goods vehicles (HGVs), as well as electricity and gas grid decarbonisation.  A summary of the decarbonisation projections used in the study are summarised in Table 1 and described below. Calculations are also contained in S3 LCA – BAU.

Table 1 – Industrial decarbonisation projection assumptions for greenhouse gas (GHG) emissions from forestry operations, wood processing and transport, and from production of substituted products (concrete and fossil fuels). For clarity, 100% indicates zero GHG emissions.

|  | Time period |  |  |  | Source |
| --- | --- | --- | --- | --- | --- |
| Sub-sector | 2020-2029 | 2030-2039 | 2040-2049 | 2050-2059 |  |
| Forestry operations | 13% | 33% | 67% | 100% | CCC Sixth Carbon budget report1 |
| Sawmills & woodfuel production | 39% | 62% | 82% | 92% | (Wood fuel assumed to be similar to sawmills). |
| Wood panel production | 25% | 60% | 70% | 79% | SLR Consulting Ltd.2 |
| Energy supply |  |  |  |  |  |
| Gas grid | 5% | 10% | 20% | 30% | SLR Consulting Ltd2 |
| Electricity grid | 50% | 77% | 95% | 100% | CCC Sixth Carbon budget report1 |
| Substitution credits |  |  |  |  |  |
| Concrete production | 5% | 10% | 20% | 33% | WSP Global Inc. & DNV GL3 |

Forestry operations

Projections for forestry operations are taken from the Committee for Climate Change Sixth Carbon Budget1 ‘balanced net zero pathway’ for decarbonising heavy goods vehicles (HGVs) (optimistically, given the remote and arduous nature of operations); agricultural machinery has similar decarbonisation projections in the CCC report.

Sawmills & wood fuel production

Projections for decarbonising sawmills are calculated using the projections for forestry operations (mobile plant), the electricity grid and gas grid; and apportioned according to the breakdown of energy sources and tonnage (averaged across the sector) from primary UK sawmill data (2020)4.

Woodfuel production decarbonisation is assumed to be similar to sawmills since the process is also highly electrified and employs mobile plant for moving material around site.

Wood panel production

The Wood Panel Industry Federation (WPIF) commissioned a report2 (by SLR Consulting Ltd) in 2022 to outline a decarbonisation pathway for the wood panel industry in the UK and Ireland. The ‘realistic’ decarbonisation pathway proposed in this report is used to project decarbonisation of the wood panel production in this study.

Electricity grid

The projection for electricity grid decarbonisation is taken from the Committee for Climate Change Sixth Carbon Budget ‘balanced net zero pathway’ for electricity generation1.

Gas grid

Gas decarbonisation is highly uncertain and will be regionally dependent1,5 (CCC report and National grid report). Therefore, we use the assumptions proposed in the WPIF report regarding gas decarbonisation, which takes into account industry specific barriers, including remote location.

Concrete production

The decarbonisation projection used for cement production is the ‘maximum technical pathway – without carbon capture and storage’ for decarbonisation of cement production, taken from the ‘Industrial Decarbonisation & Energy Efficiency Roadmaps to 2050 – Cement’ report, prepared in 2015 for the UK government Department of Energy and Climate Change and the Department for Business, Innovation and Skills3. This pathway assumes decarbonisation of 33% by 2050.

References

1 Committee on Climate Change. The Sixth Carbon Budget - The UK’s path to Net Zero. Presented to the Secretary of State pursuant to section 34 of the Climate Change Act 2008. (2020)

2 SLR Consulting Ltd. Decarbonisation of UK and Ireland wood panel manufacturing sector. Prepared for Wood Panel Industries Federation. (2022)

3 WSP Global Inc. Industrial Decarbonisation & Energy Efficiency Roadmaps to 2050 – Cement. Prepared for UK Department of Energy and Climate Change and the Department of Business, Innovation and Skills (2015)

4 UK Sawmill primary data. Quantity and source of consumed energy for 20 UK sawmills. Provided confidentially. (2020)

5 National Grid Electricity System Operator Limited. Future Energy Scenarios. Strategic Insights (2019)