Table1. Proudaction teueqnecs used for prepare the layers

| Parameters | Source | Techniques | Reference |
| --- | --- | --- | --- |
| Content of clay (%) | Geological Survey of Iran | Digitization process | (Gong et al., 2009) |
| Content of sand (%) | Geological Survey of Iran | Digitization process | (Shi et al., 2017) |
| Groundwater withdraw | RWO of Semnan Province | Kriging interpolation method | (Rahmati et al., 2019a) |
| TWI | Geological survey | TWI=lnActanβ where Ac is the upper slope area and β slope of the drainage slopes affecting the hydraulic slope of the drainage path | (Abdollahi et al., 2019) |
| Plan curvature | Geological survey | Digital elevation model (DEM) | (Pourghasemi and Saravi, 2019) |
| Distance to road | Survey of Iran | Euclidian distance Buffering | (Mohammady et al., 2019) |
| Elevation | Geological survey | Digital elevation model (DEM) | (Ghorbanzadeh et al., 2018b) |
| Slope | Geological survey | Tanθ=(Nxi636.6) N= No. of contour cutting; contour interval | (Oh et al., 2019) |
| Distance to stream | Survey of Iran | Euclidian distance Buffering | (Zamanirad et al., 2019) |
| Drainage density | Survey of Iran | Kernel density | (Arabameri et al., 2020a) |
| Litho logy | Geological survey of Iran | Digitization process | (Rahmati et al., 2019b) |
| LU/LC | Landsat 8 OLI/TIRS | Maximum likelihood classification | (Arabameri et al., 2020a) |

Table2. Description of litho logical units in the study area

| Unit | Description |
| --- | --- |
| Edvt | Rhyolitic to rhyodacitic volcanic tuff |
| Qft2 | Low level piedment fan and vally terrace deposits |
| Eav | Andesitic volcanics |
| E2c | Conglomerate and sandstone |
| E2m | Pale red marl, gypsiferous marl and limestone |
| Qcf | Clay flat |
| EOgy | Gypsum ( salt plug ) |
| Mur | Red marl, gypsiferous marl, sandstone and conglomerate (Upper red Fm.) |
| PlQc | Fluvial conglomerate, Piedmont conglomerate and sandstone. |
| Qft1 | High level piedmont fan and vally terrace deposits |
| Olgy | Gypsum |

Table3. Statistical validation relationships

| Statistical measures | Formula | References |
| --- | --- | --- |
| ACC (%) |  | (Pham et al., 2020) |
| SST (%) | SST= | (Nhu et al., 2020c) |
| SPF (%) | SPF= | (Lee et al., 2020) |
| RMSE | RMSE= i=1m(Xest-Xobs)2n | (Ghorbanzadeh et al., 2019) |
| Kappa | K= Pa- Pest 1- Pest Pa=(A+C) Pest=A+D×A+D+ B+C×(D+C) | (Arabameri et al., 2020a) |

Table4. Multi-collinearity statistics for LSCF

| Factors | Collinearity Statistics |  |
| --- | --- | --- |
|  | Tolerance | VIF |
| Content of clay (%) | 0.228 | 4.395 |
| Content of sand (%) | 0.245 | 4.086 |
| Elevation | 0.265 | 3.773 |
| Drainage density | 0.402 | 2.491 |
| Distance to road | 0.875 | 1.143 |
| Distance to stream | 0.379 | 2.639 |
| Groundwater withdraw | 0.429 | 2.329 |
| Land use/land cover | 0.331 | 3.022 |
| Litho logy | 0.407 | 2.456 |
| Plan curvature | 0.673 | 1.486 |
| Slope | 0.700 | 1.429 |
| TWI | 0.606 | 1.650 |

Table5. Percentage of each susceptibility zones in different models

| Model | Class | Count of pixels | No of land subsidence | % of pixels | % of land subsidence |
| --- | --- | --- | --- | --- | --- |
| DSC | Very Low | 280659 | 4 | 43.94 | 4.26 |
|  | Low | 147958 | 5 | 23.16 | 5.32 |
|  | Moderate | 70772 | 5 | 11.08 | 5.32 |
|  | High | 62045 | 6 | 9.71 | 6.38 |
|  | Very High | 77347 | 74 | 12.11 | 78.72 |
| DSC-NBTree | Very Low | 300716 | 3 | 53.10 | 3.19 |
|  | Low | 96672 | 2 | 17.07 | 2.13 |
|  | Moderate | 57687 | 4 | 10.19 | 4.26 |
|  | High | 27867 | 5 | 4.92 | 5.32 |
|  | Very High | 83354 | 80 | 14.72 | 85.11 |
| DSC-J48 | Very Low | 252745 | 3 | 39.70 | 3.19 |
|  | Low | 274603 | 10 | 43.14 | 10.64 |
|  | Moderate | 16817 | 3 | 2.64 | 3.19 |
|  | High | 23825 | 5 | 3.74 | 5.32 |
|  | Very High | 68621 | 73 | 10.78 | 77.66 |
| DSC-SVM | Very Low | 250125 | 2 | 39.16 | 2.13 |
|  | Low | 168345 | 4 | 26.35 | 4.26 |
|  | Moderate | 75316 | 6 | 11.79 | 6.38 |
|  | High | 66885 | 9 | 10.47 | 9.57 |
|  | Very High | 78110 | 73 | 12.23 | 77.66 |
| DSC-LMT | Very Low | 343903 | 3 | 53.84 | 3.19 |
|  | Low | 119468 | 7 | 18.70 | 7.45 |
|  | Moderate | 59998 | 2 | 9.39 | 2.13 |
|  | High | 44225 | 5 | 6.92 | 5.32 |
|  | Very High | 71187 | 77 | 11.14 | 81.91 |
| DSC-AD Tree | Very Low | 371641 | 4 | 60.27 | 4.26 |
|  | Low | 109864 | 3 | 17.82 | 3.19 |
|  | Moderate | 44409 | 5 | 7.20 | 5.32 |
|  | High | 45442 | 6 | 7.37 | 6.38 |
|  | Very High | 45252 | 76 | 7.34 | 80.85 |