Supplementary Table 2

| Procedure | Advised Table |
| --- | --- |
| The choice of TAMs mold | The scannable area of Hyperion imaging mass cytometer is 60mm long and 16mm wide. Normal tissue section is 75mm long and 25mm wide. Therefore, it is important to select an appropriate mold of TAMs so that the sample is within the scannable area. |
| Marker stain of TAMs | 1. We baked TAMs slide in 65°C for only 30minutes, otherwise the tissue would have fell off easily. 2. Dewaxing and hydration process was the same as standard IHC protocol. 3. Antigen retrieval process was the same as standard IHC protocol. Because different antibodies might differ in antigen retrieval conditions, we should confirm that all antibodies we chose in panel have the same antigen retrieval condition. 4. Blocking was the same as standard IHC protocol. 5. For antibodies incubation, antibodies were diluted with 0.5% BSA in DPBS and incubated at 4℃ overnight. Recycled antibodies mixture after incubation cannot be used because some antibodies were depleted during the first incubation. 6. We used 0.1% Triton-X in DPBS to wash sections for 8 min twice with slow agitation, and then washed sections with DPBS for 8 min twice with slow agitation. 7. We stained sections for 5 min at RT with Cell-ID™ Intercalator-Ir (Fluidigm, San Francisco, USA, Cat# 201192A) which contains natural-abundance iridium (191Ir and 193Ir). The best concentration was 1:200. 8. We raised sections three times in ddH2O, for 10 min each time. 9. We dried sections at RT for at least 20 min. |
| Preparation before scanning | 1. During the operation of the Hyperion imaging mass cytometer, the humidity of the room should be maintained at 50%. 2. The TAMs should be completely dry. 3. Argon and helium should be sufficient. |
| IMC scanning | 1. We operated the machine in strict accordance with user instructions. 2. The TAMs should be scanned continuously. Because of the large amount of data, other operations were forbidden on the computer during the scanning to avoid crashes. 3. Because the voltage value of the laser and detector would naturally lose during use, we should pay attention to check and replace it when it is lower than the threshold. |