# Appendix A: OSeMOSYS Input Data

Sarah Golobish (sarah.golobish21@imperial.ac.uk)

This modelling has been run using the open-source OSeMOSYS model – a linear optimisation simulation of the cost-optimal energy solution for a nation. The modelling builds off the “Starter Data Kit” for Nigeria which is described in Allington et al. 2021. The OSeMOSYS model was run with 8 time slices to reduce computation time.

This document outlines the methodology behind the input data used in this modelling. The data can be found in the “Nigeria_OSeMOSYS_Input_Data.xlsx” file and a SAND file with this data and no scenario constraints is included as “Nigeria_Base_SAND_UpdatedCapacity.xlsm.” Historic generation data is available in the “Nigeria_Generatio_Data.xlsx” file.

## Technoeconomic Assumptions

Technology costs, lifetimes and efficiencies used in this analysis come from Allington et al (2021). Table 1 shows some of this cost data.

Table 1. Selected technoeconomic input data

Source: Allington et al. 2021

| Technology | Capital Cost ($/kW in 2020) | Fixed Cost ($/kW/yr in 2020) | Operational Life (years) | Efficiency | Average Capacity Factor |
| --- | --- | --- | --- | --- | --- |
| Biomass Power Plant | 2500.0 | 75.0 | 30 | 0.35 | 0.5 |
| Coal Power Plant | 2500.0 | 78.0 | 35 | 0.37 | 0.85 |
| Geothermal Power Plant | 4000.0 | 120.0 | 25 | 0.8 | 0.79 |
| Light Fuel Oil Power Plant | 1200.0 | 35.0 | 25 | 0.35 | 0.8 |
| Oil Fired Gas Turbine (SCGT) | 1450.0 | 45.0 | 25 | 0.35 | 0.8 |
| Gas Power Plant (CCGT) | 1200.0 | 35.0 | 30 | 0.48 | 0.85 |
| Gas Power Plant (SCGT) | 700.0 | 20.0 | 25 | 0.3 | 0.85 |
| Solar PV (Utility) | 1378.0 | 17.91 | 24 | 1.0 | 0.34 |
| CSP without Storage | 4058.0 | 40.58 | 30 | 1.0 | 0.45 |
| CSP with Storage | 5797.0 | 57.97 | 30 | 1.0 | 0.45 |
| Large Hydropower Plant (Dam) (>100MW) | 3000.0 | 90.0 | 50 | 1.0 | 0.36 |
| Medium Hydropower Plant (10-100MW) | 2500.0 | 75.0 | 50 | 1.0 | 0.36 |
| Small Hydropower Plant (<10MW) | 3000.0 | 90.0 | 50 | 1.0 | 0.36 |
| Onshore Wind | 1489.0 | 59.56 | 25 | 1.0 | 0.15 |
| Offshore Wind | 3972.4 | 158.9 | 25 | 1.0 | 0.37 |
| Nuclear Power Plant | 6137.0 | 184.11 | 50 | 0.33 | 0.85 |
| Light Fuel Oil Standalone Generator (1kW) | 750.0 | 23.0 | 10 | 0.16 | 0.3 |
| Solar PV (Distributed with Storage) | 4320.0 | 86.4 | 24 | 1.0 | 0.34 |

Renewable energy potentials and fuel reserves were also taken from Allington et al 2021 with the exception of hydropower potential, as the capacity used there representants a far more conservative estimate (5,650MW) which only includes operational and planned hydropower projects. This conservative estimate was replaced with an estimated technical potential of 14,120MW for large hydropower projects which is consistent with the potentials used in other studies surveyed (Pappis et al., 2019).

The emission factors for each of the fossil fuels were also updated with factors from Staffell (2011) to include impacts of CH4 and N2O as used in the NDCs. These emissions factors are shown in Table 2.

Table 2. Updated emissions factors

Source: Staffell 2011

| OSeMOSYS Technology | Description | g CO2e/MJ |
| --- | --- | --- |
| IMPOIL | Imported Oil | 73.6 |
| MINOIL | Domestic Oil | 73.6 |
| IMPBIO | Imported Biomass | 121.5 |
| MINBIO | Domestic Biomass | 121.5 |
| IMPCOA | Imported Coal | 95.6 |
| MINCOA | Domestic Coal | 95.6 |
| IMPLFO | Light Fuel Oil | 74.4 |
| IMPHFO | Heavy Fuel Oil | 77.6 |
| IMPNGS | Imported Natural Gas | 56.2 |
| MINNGS | Domestic Natural Gas | 56.2 |

## Residual and Available Capacity

The residual capacity used in this modelling was compiled from historic power sector data from the Nigerian Electricity Regulatory Commission (Nigerian Electricity Regulatory Commission, 2021, 2020).

Table 3. Installed power plant capacity as of 2022

Source: Nigerian Electricity Regulatory Commission 2021

| Name | Year Completed | Fuel | Type | Installed Capacity (MW) |
| --- | --- | --- | --- | --- |
| Afam VI | 2009 | Gas | Gas (CCGT) | 685 |
| Alaoji NIPP | 2015 | Gas | Gas (CCGT) | 1074 |
| Asco | 2021 | Gas | Gas (CCGT) | 22 |
| Egbin | 1985 | Gas/Steam | Gas (CCGT) | 1320 |
| Okpai | 2005 | Gas | Gas (CCGT) | 480 |
| Olorunsogo NIPP | 2012 | Gas | Gas (CCGT) | 675 |
| Sapele | 1978 | Gas/Steam | Gas (CCGT) | 504 |
| AES Gas | 2001 | Gas | Gas (OCGT) | 290 |
| Afam IV-V | 1982 | Gas | Gas (OCGT) | 724 |
| Azura-Edo | 2018 | Gas | Gas (OCGT) | 450 |
| Delta (Transcorp Ughelli) | 1990 | Gas | Gas (OCGT) | 900 |
| Gbarain | 2016 | Gas | Gas (OCGT) | 225 |
| Geregu Gas | 2007 | Gas | Gas (CCGT) | 435 |
| Geregu NIPP | 2012 | Gas | Gas (OCGT) | 435 |
| Ibom Power | 2009 | Gas | Gas (OCGT) | 190 |
| Ihovbor NIPP | 2012 | Gas | Gas (OCGT) | 450 |
| Odukpani NIPP | 2013 | Gas | Gas (OCGT) | 563 |
| Olorunsogo Gas | 2007 | Gas | Gas (OCGT) | 336 |
| Omoku | 2005 | Gas | Gas (OCGT) | 150 |
| Omotosho Gas | 2005 | Gas | Gas (OCGT) | 336 |
| Omotosho NIPP | 2012 | Gas | Gas (OCGT) | 450 |
| Paras Energy | 2016 | Gas | Gas (OCGT) | 65 |
| Rivers IPP | 2009 | Gas | Gas (OCGT) | 136 |
| Sapele NIPP | 2012 | Gas | Gas (OCGT) | 450 |
| Trans Amadi | 2010 | Gas | Gas (OCGT) | 136 |
| Dadinkowa | 2019 | Hydro | Hydro | 60 |
| Jebba | 1986 | Hydro | Hydro | 570 |
| Kainji | 1968 | Hydro | Hydro | 760 |
| Shiroro | 1989 | Hydro | Hydro | 600 |

Finally, the residual capacity was also updated to include generation projects currently in construction. These projects were found using the Global Energy Monitor (2022). Projects without updates in the last 5 years and projects that are in planning stages were not included.

Table 4. Pending on-grid capacity in Nigeria

Source: Global Energy Monitor 2022

| Plant Name | Status | Technology | Capacity (MW) | Expected Completion |
| --- | --- | --- | --- | --- |
| Abuja/Gwagwalada | Funded | CCGT | 1350 | 2037 |
| Zungeru | Construction | Hydropower | 700 | 2022 |
| Kashimbila | Construction | Hydropower | 40 | 2021 |
| Mambilla | Construction | Hydropower | 3,050 | 2030 |
| Egbema | Construction | OCGT | 339 | 2024 |
| Maiduguri | Construction | OCGT | 50 | 2022 |
| Katsina solar farm | Development | Solar | 80 | 2025 |
| Katsina | Construction | Wind | 10 | 2022 |

## Additional Constraints

For any new capacity built in the model, there were constraints added on the earliest time a plant of a particular type could start generating power. This was added to account for the fact that construction of new plants typically takes more than a year and this fact is not included in the OSeMOSYS model. These lead times for each type of plant are presented in Table 5 and were sourced from Allington (Allington, 2020).

Table 5. Lead time by technology

Source: Allington 2020

|  | Lead Time | Start Year |
| --- | --- | --- |
| Biomass | 3 | 2025 |
| Coal | 4 | 2026 |
| Centralized Diesel | 2 | 2024 |
| CCGT | 3 | 2025 |
| SCGT | 2 | 2024 |
| Utility Scale PV | 1 | 2023 |
| CSP | 3 | 2025 |
| CSP with Storage | 3 | 2025 |
| Large and Medium Hydropower | 4 | 2026 |
| Small Hydropower | 4 | 2026 |
| Onshore Wind | 2 | 2024 |
| Offshore Wind | 2 | 2024 |
| Nuclear Power | 7 | 2029 |
| Utility Scale PV with storage | 1 | 2023 |
| Onshore Wind with storage | 2 | 2024 |
| Distributed PV | 1 | 2023 |
| Micro Hydro | 4 | 2026 |

## Plant Availability

Given the focus of this modelling, the installed capacity numbers needed to be adjusted with the actual available capacity. The actual available capacity for each on-grid plant in Nigeria in 2019 is shown in Table 6.

Additional variation was introduced across scenarios and is further explained in the scenario brief document.

Table 6. 2019 Available power plant capacity

Source: Nigerian Electricity Regulatory Commission 2021

| Plant | Type | Installed Capacity | 2019 Available Capacity | 2019 Average Generation |
| --- | --- | --- | --- | --- |
| AES | Gas (OCGT) | 290 | 0.00 | 0.00 |
| AFAM IV-V | Gas (OCGT) | 724 | 75.13 | 33.13 |
| AZURA-EDO | Gas (OCGT) | 450 | 450.67 | 259.16 |
| DELTA | Gas (OCGT) | 900 | 458.36 | 314.44 |
| GBARAIN | Gas (OCGT) | 225 | 67.66 | 30.26 |
| GEREGU NIPP | Gas (OCGT) | 435 | 217.36 | 138.62 |
| IBOM POWER | Gas (OCGT) | 190 | 90.12 | 56.08 |
| IHOVBOR NIPP | Gas (OCGT) | 450 | 120.52 | 80.05 |
| OLORUNSOGO | Gas (OCGT) | 336 | 274.64 | 159.08 |
| OMOKU | Gas (OCGT) | 150 | 80.77 | 57.66 |
| OMOTOSHO | Gas (OCGT) | 336 | 175.16 | 118.47 |
| OMOTOSHO NIPP | Gas (OCGT) | 450 | 133.32 | 92.15 |
| PARAS ENERGY | Gas (OCGT) | 65 | 66.10 | 55.24 |
| RIVERS IPP | Gas (OCGT) | 136 | 139.77 | 94.78 |
| SAPELE NIPP | Gas (OCGT) | 450 | 93.54 | 67.64 |
| TRANS AMADI | Gas (OCGT) | 136 | 56.64 | 37.36 |
| ODUKPANI | Gas (OCGT) | 563 | 470.25 | 223.02 |
| ASCO | Gas (CCGT) | 22 | 0.00 | 0.00 |
| AFAM VI | Gas (CCGT) | 685 | 437.11 | 166.88 |
| ALAOJI NIPP | Gas (CCGT) | 1074 | 70.44 | 24.23 |
| OLORUNSOGO NIPP | Gas (CCGT) | 675 | 37.10 | 24.72 |
| GEREGU | Gas (CCGT) | 435 | 378.96 | 217.84 |
| OKPAI | Gas (CCGT) | 480 | 266.23 | 154.79 |
| EGBIN | Gas (CCGT) | 1320 | 773.57 | 433.60 |
| SAPELE | Gas (CCGT) | 504 | 86.61 | 33.10 |
| DADIN KOWA | Hydro | 60 | 0.00 | 0.00 |
| JEBBA | Hydro | 570 | 435.12 | 327.09 |
| SHIRORO | Hydro | 600 | 500.96 | 298.12 |
| KAINJI | Hydro | 760 | 415.74 | 339.49 |

## Additional Sources

Allington, L., 2020. Long-Term Modelling of Nigeria’s Power System using OSeMOSYS (Open-Source Energy Modelling System).

Allington, L., Cannone, C., Pappis, I., Barron, K.C., Usher, W., Pye, S., Brown, E., Howells, M., Walker, M.Z., Ahsan, A., Charbonnier, F., Halloran, C., Hirmer, S., Taliotis, C., Sundin, C., Sridharan, V., Ramos, E., Ramos, E., Deane, P., Gritsevskyi, A., Moura, G., Rouget, A., Wogan, D., Barcelona, E., Rogner, H., 2021. Selected ‘Starter Kit’ energy system modelling data for Nigeria (#CCG) 13. https://doi.org/10.21203/rs.3.rs-480085/v2

Global Energy Monitor, 2022. Global Power Plant Tracker.

Nigerian Electricity Regulatory Commission, 2021. Key Operational & Financial Data of NESI for Jan 2019 to Sept 2021.

Nigerian Electricity Regulatory Commission, 2020. Nigeria Electricity Regulatory Commission Quarterly Report: Fourth Quarter 2020. Nigeria Electricity Regulatory Commission.

Pappis, I., Howells, M., Sridharan, V., Usher, W., Shivakumar, A., Gardumi, F., Ramos, E., 2019. Energy projections for African countries. European Commission. Joint Research Centre., LU.

Staffell, I., 2011. The Energy and Fuel Data Sheet 11.