# Appendix C: FlexTool Modelling

Sarah Golobish (sarah.golobish21@imperial.ac.uk)

This modelling explored pathways to meeting Nigerian electricity demand with particular focus on the role of improved power plant availability and energy efficiency to reduce reliance on diesel self-generation. The different constraints used which differentiate scenarios are outlined below. The key takeaways from the modelling are:

- Improved plant availability can dramatically reduce CO2 emissions from the power sector even under a business-as-usual scenario. However, increasing the pace of that improvement has a smaller impact on emissions.

- Nigeria can meet its 2030 Unconditional NDC without significant changes to its generation mix if plant availability is improved.  Near-term investments in energy efficiency will help to further ensure that targets are met.

- It is not possible to meet long-term decarbonization goals without energy efficiency and plant improvement. Runs without plant improvement and low efficiency investment were not able to meet demand.

The FlexTool portion of this modelling was used to further evaluate and validate the OSeMOSYS results.

## Modelling Assumptions

Modelling used a single-node FlexTool model with primary inputs derived from the OSeMOSYS modelling. As FlexTool explores the flexibility of a system over a single year, the flexibility analysis presented here focuses on model year 2040. Exploring model results in 2040 exposes the impacts of a range of different availability factors – the runs with 2030 improvement will be at 100% while the 2050 runs will be higher than the baseline but not quite at 100%, while the BAU runs will be at baseline. Under the UNDC and CNDC constraints 2040 also represents a challenging year where demand continues to grow despite emissions caps.

Key inputs were held constant across OSeMOSYS and FlexTool modelling:

- Reserve margin of 15%.

- Costs of technologies (both CAPEX and OPEX).

- Efficiencies of technologies.

- Demand in scenarios without energy efficiency.

See the OSeMOSYS data note for additional rationale behind the choice of these metrics.

As the FlexTool modelling was run for the year 2040, results of the OSeMOSYS modelling were also used as inputs. For the year 2040 OSeMOSYS outputs were used to support:

- Demand in scenarios where investment in energy efficiency was enabled (least cost, unconditional NDC, conditional NDC).

- Installed capacity of each on-grid technology in 2040.

Only the available capacity for each model run was added to FlexTool and diesel capacity was not included as FlexTool focuses on-grid generation and diesel capacity is assumed to be used for backup generation. These choices aim to focus the flexibility modelling on

FlexTool requires additional capacity factors and load profiles which were PLEXOS-World 2015 dataset (Brinkerink and Deane, 2020).

The value of lost load used was $10,000 USD per MWh which is consistent with the value used by Daggash and Mac Dowell (2021) and typical values used in FlexTool (Taibi et al., 2018).

Ramp rates, and other key inputs are not Nigeria-specific and are listed below.

Table 1. Key flexibility input data

|  | Min Load (%) | Ramp Up | Ramp Down | Inertia Constant | Min Uptime (h) | Min Downtime (h) |
| --- | --- | --- | --- | --- | --- | --- |
| Biomass Power Plant | 0.252 | 0.00542 | 0.00542 | 32 | 0 | 0 2 |
| Coal Power Plant | 0.33 | 0.073 | 0.073 | 2.63 2 | 3 | 3 3 |
| Gas Power Plant (CCGT) | 0.23 | 0.083 | 0.083 | 4.69 2 | 2 | 2 3 |
| Gas Power Plant (SCGT) | 0.153 | 0.23 | 0.23 | 4.982 | 0.16 | 0.16 3 |
| Solar PV (Utility) |  | 1 | 1 | 02 |  |  |
| Large Hydropower Plant (Dam) (>100MW) | 0.053 | 0.153 | 0.153 |  | 1 | 13 |
| Onshore Wind |  | 1 | 1 | 02 |  |  |
| Offshore Wind |  | 1 | 1 | 02 |  |  |
| Nuclear Power Plant | 0.5 1,3 | 0.0051,3 | 0.0051,3 | 4.07 1 | 10 | 10 1 |
| Utility-scale PV with 2 hour storage |  | 1 | 1 | 02 |  |  |
| Onshore Wind power plant with storage |  | 1 | 1 | 02 |  |  |

Sources:

- Deloitte (2019) Assessing the flexibility of coal-fired power plants for the integration of renewable energy in Germany. https://www2.deloitte.com/content/dam/Deloitte/fr/Documents/financial-advisory/economicadvisory/deloitte_assessing-flexibility-coal-fired-power-plants.pdf.

- Gonzalez-Salazar, M.A. (2018) Review of the operational flexibility and emissions of gas- and coal-fired power plants in a future with growing renewables. Renewable and Sustainable Energy Reviews. 17.

- Ramirez-Meyers, K., Neal Mann, W., Deetjen, T.A., Johnson, S.C., Rhodes, J.D. & Webber, M.E. (2021) How different power plant types contribute to electric grid reliability, resilience, and vulnerability: a comparative analytical framework. Progress in Energy. 3 (3), 033001. doi:10.1088/2516-1083/abf636.

Table 2. Flexibility modelling penalties

| Parameter | Value |
| --- | --- |
| CO2 cost | 2 USD/Tonne 1 |
| Loss of load penalty | 10,000 USD/MWh2 |
| Loss of reserves penalty | 2,000 USD/MWh1 |
| Lack of inertia penalty | 3,000 USD/MWh1 |
| Curtailment penalty | 8 USD/MWh1 |
| Lack of capacity penalty | 5001 |
| Time modelled | 1.000 year |

Sources:

- Assumption, one tenth of default value to adjust for national context.

- Daggash, H.A. & Mac Dowell, N. (2021) Delivering low-carbon electricity systems in sub-Saharan Africa: insights from Nigeria. Energy & Environmental Science. 14 (7), 4018–4037. doi:10.1039/D1EE00746G.

- Qi Ye, Lu Jiaqi, & Zhu Mengye (n.d.) Wind Curtailment in China and Lessons from the United States. https://www.brookings.edu/wp-content/uploads/2018/03/wind-curtailment-in-china-and-lessons-from-the-united-states1.pdf.

## Scenarios

The scenarios explored in the FlexTool modelling mirror the scenarios used in the OSeMOSYS modelling. Table 3 provides an outline of what was modelled in OSeMOSYS but further detail on the constraints used in each scenario can also be found in the OSeMOSYS scenario brief.

Table 3. Scenario overview

| Scenario | Description and Constraints |
| --- | --- |
| BAU | On-grid investment is constrained to only natural gas and hydropower. No investment in EE. |
| Least Cost | Minimal constraints to find least-cost solution. No investment in nuclear power. Gradual investment in EE. |
| Unconditional NDC | Emissions capped at 20% below BAU emissions in 2030 (85.3 MT) from 2030 onwards. Gradual investment in EE. |
| Conditional NDC | Emissions capped at 47% below BAU emissions in 2030 (56.5 MT) from 2030 onwards. All SCGT are phased out by 2030. 13 GW off-Grid renewables, 3.5 GW of small hydro, and 12GW of new large hydro capacity added by 2030. Improved T&D losses by 2030. Gradual investment in EE. |

As in the OSeMOSYS modelling, each of the above was run with three different scenarios for plant availability – improvement to 2030, 2050 and no improvement. For each of these runs availability reaches default availability in the target year. The availability used for the model runs (in year 2040) are given in Table 4 below.

Table 4. Availability factor by technology in 2040 used in FlexTool modelling

|  | 2030 Improvement | 2050 Improvement | No Improvement |
| --- | --- | --- | --- |
| PWRNGS001 (CCGT) | 100% | 78.39% | 39.5% |
| PWRNGS002 (OCGT) | 100% | 81.14% | 47.2% |
| Other Fossil | 100% | 82.14% | 50% |
| PWRHYD001 | 100% | 88.53% | 67.9% |
| Other Renewables | 100% | 92.85% | 80% |

### Additional Sources

Brinkerink, M., Deane, P., 2020. PLEXOS-World 2015. https://doi.org/10.7910/DVN/CBYXBY

Daggash, H.A., Mac Dowell, N., 2021. Delivering low-carbon electricity systems in sub-Saharan Africa: insights from Nigeria. Energy Environ. Sci. 14, 4018–4037. https://doi.org/10.1039/D1EE00746G

Taibi, E., Nikolakakis, T., Gutierrez, L., Fernandez, C., Kiviluoma, J., Rissanen, S., Lindroos, T.J., 2018. Power system flexibility for the energy transition: Part 2, IRENA FlexTool methodology.