# Appendix B: OSeMOSYS Scenario Brief

Sarah Golobish (sarah.golobish21@imperial.ac.uk)

This modelling explored pathways to meeting Nigerian electricity demand with particular focus on the role of improved power plant availability and energy efficiency to reduce reliance on diesel self-generation. The different constraints used which differentiate scenarios are outlined below. The key takeaways from the modelling are:

- Improved plant availability can dramatically reduce CO2 emissions from the power sector even under a business-as-usual scenario. However, increasing the pace of that improvement has a smaller impact on emissions.

- Nigeria can meet its 2030 Unconditional NDC without significant changes to its generation mix if plant availability is improved.  Near-term investments in energy efficiency will help to further ensure that targets are met.

- It is not possible to meet long-term decarbonization goals without energy efficiency and plant improvement. Runs without plant improvement and low efficiency investment were not able to meet demand.

# Modelling Assumptions

A few core assumptions used throughout scenarios:

- Discount rate of 10% and reserve margin of 15% used throughout modelling. This is consistent with other modelling of the Nigerian energy system (Daggash and Mac Dowell, 2021; Dioha and Kumar, 2020).

- To minimize flexibility issues on the already fragile grid, utility-scale VRE generation was limited across all scenarios to 30% of demand each year with 20% coming from wind and 10% coming from solar. This cap is in line with IRENA modelling of the West African Power Pool which used the same penetration caps to ensure system reliability across scenarios (IRENA, 2013).

- Transport, heating and cooking demands are not included in this modelling as these demands are not expected to be electrified in the short to medium term.

Nuclear build rates were also constrained across scenarios to an added capacity of 0.5 GW per year (UNCD and CNDC model runs). This build rate is more ambitious than the build rate in the Vision30:30:30 targets which set a goal of 1GW by 2025 and a further 1GW by 2030 (an average of 0.2GW added per year from 2025-2030) (Federal Government of Nigeria, 2016) . A build rate of 0.5GW is however consistent with pervious work by Daggash & Mac Dowell (2021) who used this rate across their modelling to capture the existing issues with expanding grid capacity while allowing the model to invest as necessary to roughly meet demand.

Information on the costs, residual capacities and other input data that remain constant throughout the modelling can be found in the corresponding data note.

# Scenario Descriptions

As shown in Figure 1, this modelling explores variations on four core scenarios. The business as usual scenario captures what might happen if Nigeria’s energy system were to continue to grow and develop as it is now. The Least cost scenario allows the model to invest in the cost-optimal solution to meet demand. Finally, the conditional and unconditional NDC scenarios explore the country’s commitments under the Paris Agreement.

As the carbon budget for the NDC had yet to be published as the time of writing, the emissions ceilings were calculated using the BAU model runs found to have 2030 emissions closest to those projected in NDC modelling (108.6MT). The BAU run with plant availability improving to 2050 was selected as it projects emissions of 107 MT of emissions in 2030.The resulting UNDC cap was calculated to be 85 MT of CO2 annually and the CNDC cap was calculated at 56 MT.

Nigeria’s 2021 NDC sets additional targets in the Conditional NDC for the power sector – the replacement of all SCGTs, increased off-grid renewables, improved T&D losses, and 15.5 GW of new hydropower (Federal Government of Nigeria, 2021). These milestones were included in the CNDC scenarios, but the UNDC model was left to meet emissions targets without any capacity milestones.

Figure 1. Scenario framework overview

## Business as Usual

This scenario captures the current power system investment trends and projects them to 2065.

- On-grid investment is constrained to only gas (SCGT and CCGT) and hydropower from 2022 onwards.

- Off-grid investment is mostly unconstrained, distributed solar is limited by projections of off-grid demand from the Global Electrification Platform.

- No investment in energy efficiency is permitted.

## Least Cost

The cost-optimised pathway for meeting Nigeria’s electricity demand.

- No investment in nuclear power is permitted.

- On-grid solar and wind constrained to only meet 30% of demand annually (10% met by solar; 20% met by wind). Biomass can only meet 30% of demand.

- Investment in distributed solar is limited by projections of off-grid demand from the Global Electrification Platform.

- Investment in energy efficiency is enabled with gradual investment constraints (see section on energy efficiency).

## Unconditional NDC

Annual emissions are capped at 20% below BAU emissions in 2030 and remain at that level to the end of the model period.

- Annual emissions are capped at 85 MT from 2030 onwards.

- On-grid solar and wind constrained to only meet 30% of demand annually (10% met by solar; 20% met by wind). Biomass can only meet 30% of demand.

- Investment in energy efficiency is enabled with gradual investment constraints (see section on energy efficiency).

## Conditional NDC

Annual emissions are capped at 47% below BAU emissions in 2030 and remain at that level to the end of the model period. Additional power sector goals (outlined below) are also included to direct model investment.

- Annual emissions are capped at 56 MT from 2030 onwards.

- T&D losses reduced to 8%, down from 15% in 2018 (distribution efficiency increased as transmission efficiency is high in base model).

- SCGTs phased out of the power system by 2030 (upper activity limit for SCGT set to 0 after 2030).

- Additional investment in 12 GW large hydro by 2030.

- Additional investment in 3.5 GW small hydro by 2030.

- Invest in 6.5 GW of utility-scale PV by 2030.

- Invest in 3.2 GW of wind by 2030.

- Invest in 13GW of off-grid renewables by 2030 (small-scale PV and hydro).

- On-grid solar and wind constrained to only meet 30% of demand annually (10% met by solar; 20% met by wind). Biomass can only meet 30% of demand.

- Investment in distributed solar is limited by projections of off-grid demand from the Global Electrification Platform.

- Investment in energy efficiency is enabled with gradual investment constraints (see section on energy efficiency).

- For scenarios with energy efficiency, the model can invest in energy efficiency at cost-optimal levels. Constraints added to create gradual investment allowing investment in up to 5% of 2050 capacity of runs without EE constraints. This method follows guidance in Cannone et al (2023).

## Variation within Scenarios

Each scenario was run at three levels of assumed plant availability improvement. Plant availability only applies to on-grid generation and excludes nuclear power.

To explore the impact of improved plant availability, each of the four core scenarios is run under three different plant availability scenarios – no improvement/BAU, gradual improvement to 2050, and gradual improvement to 2030 (where plant availability meets typical developed country standards in the target year). The baseline for plant availability was calculated using the 2019 average available capacity for each type of plant (hydro, CCGT, SCGT) (Nigerian Electricity Regulatory Commission, 2021). The baseline availability for other on-grid generating technologies was set slightly higher than the observed numbers for gas and hydro to conservatively account for the fact that any new plants added to the grid are likely to experience similar management and maintenance issues. Table 3 shows the baseline assumed and observed availabilities for all technology types. In instances where plant availability is assumed to improve, availability grows linearly until it hits 100% in the target year. No availability constraints were added to decentralised technologies to reinforce their use as back-up and alternative generation sources.

### Plant Availability

Each scenario was run with full plant availability to determine optimal plant investment mix and capacity. For subsequent runs with reduced plant availability the installed capacity from this base run was frozen so that the model could not invest in additional on-grid capacity to fill the deficit caused by reduced availability. The model could only use diesel generation to fill the gap, much as the current supply gap is met by diesel generation.

Plant Availability: Improvement to 2030

- On-grid power plant availability improves linearly (see Table 1) until it hits standard/developed levels in 2030.

Plant Availability: Improvement to 2050

- On-grid power plant availability improves linearly (see Table 1) until it hits standard/developed levels in 2050.

Plant Availability: BAU/No Improvement

- On-grid power plant availability remains at baseline levels throughout the model period.

Table 1. Availability Factors

Data calculated using NERC (2021, 2018).

|  | Baseline Availability in 2021 | Annual Improvement to 2030 | Annual Improvement to 2050 |
| --- | --- | --- | --- |
| Combined Cycle Gas Turbine | 40% | 8% | 2% |
| Single Cycle Gas Turbine | 47% | 7% | 2% |
| Other Fossil | 50% | 6% | 2% |
| On-Grid Hydropower | 68% | 4% | 1% |
| Other Renewables | 80% | 3% | 1% |

Table 2. Files associated with each model run and scenario.

| Scenario | Improvement | File Name |
| --- | --- | --- |
| BAU | 2030 | NG_BAU_2030 |
|  | 2050 | NG_BAU_2050 |
|  | Never | NG_BAU_Never |
| Least Cost | 2030 | NG_LeastCost_2030 |
|  | 2050 | NG_LeastCost_2050 |
|  | Never | NG_LeastCost_Never |
| CNDC | 2030 | NG_CNDC_2030 |
|  | 2050 | NG_CNDC_2050 |
|  | Never | NG_CNDC_Never |
| UNDC | 2030 | NG_UNDC_2030 |
|  | 2050 | NG_UNDC_2050 |
|  | Never | NG_UNDC_Never |

### Additional Sources

Cannone, C., Allington, L., Barron, K.C., Charbonnier, F., Walker, M.Z., Halloran, C., Yeganyan, R., Tan, N., Cullen, J.M., Harrison, J., To, L.S., Howells, M., 2023. Designing a zero-order energy transition model: How to create a new Starter Data Kit. MethodsX 10, 102120. https://doi.org/10.1016/j.mex.2023.102120

Daggash, H.A., Mac Dowell, N., 2021. Delivering low-carbon electricity systems in sub-Saharan Africa: insights from Nigeria. Energy Environ. Sci. 14, 4018–4037. https://doi.org/10.1039/D1EE00746G

Dioha, M.O., Kumar, A., 2020. Exploring the energy system impacts of Nigeria’s Nationally Determined Contributions and low-carbon transition to mid-century. Energy Policy 144, 111703. https://doi.org/10.1016/j.enpol.2020.111703

Federal Government of Nigeria, 2021. Nigeria’s Nationally Determined Contribution – 2021 Update. Federal Government of Nigeria, Abuja.

Federal Government of Nigeria, 2016. Sustainable energy for all action agenda. Federal Government of Nigeria.

IRENA, 2013. West African Power Pool: Planning and Prospects for Renewable Energy. International Renewable Energy Agency.

Nigerian Electricity Regulatory Commission, 2021. Key Operational & Financial Data of NESI for Jan 2019 to Sept 2021.

Nigerian Electricity Regulatory Commission, 2018. NESI Generation statistics.