Supporting Information

FeCl3-Promoted Photocatalytic Cleavage of Cα–Cβ Bond in Lignin Model Compounds and Native Lignin

XinweiLiu*, Liguo Wang, LianjingZhai, Fengxia Sun*, Yujuan Sun, Zhanzhan Liu, Jianing Liu

Table of Contents

1. General	2

1.1 Materials	2

1.2 Instrumentation	2

2. Experimental procedures	3

2.1 Synthesis of lignin models	3

2.2 Photocatalytic cleavage of lignin model compounds	3

3. Additional screening of reaction conditions	5

4. Mechanism study	6

5. Degradation of real lignin	7

6. 1H NMR and 13C NMR spectra of compounds	9

7. References	27

# 1. General

## 1.1 Materials

All reagents and solvents were purchased from Accela, Adamas, Innochem, Psaitong and Aladdin. Unless otherwise noted, materials obtained from commercial suppliers were used without further purification. Blue LED lamps, radiators, fan, regulated DC power supplies and quartz reaction tubes used in the photocatalytic reaction were purchased from Shenzhen Xindaxin Photoelectric Technology Co., Ltd.

## 1.2 Instrumentation

Products were purified by flash chromatography on silica gel. Analysis of crude reaction mixture was performed on an Agilent 7820AGC System with a HP-INNOWAX capillary column (30 m×0.25 mm×0.32 μm) and an FID detector. The following GC temperature program was used: 60 °C is maintained for 2 minutes, rises to 150 °C at 10 °C/min, and finally rises to 300°C at a rate of 20°C/min, and hold for 4 minutes.

Nitrogen was used as a carrier gas. The injector temperature was held at 250 °C. GC-MS analysis was carried out on a SHIMADZU GC-MSQP2010 with a DB-5 capillary column (30 m×0.25 mm×0.32 μm).

1H NMR spectra were recorded in CDCl3 or DMSO using internal reference (the residue proton peaks of CDCl3 at 7.26 ppm and DMSO at 2.5 ppm) on Bruker 400 spectrometer. Liquid 13C NMR was recorded at 100.6 MHz in CDCl3 using residual CDCl3 as internal reference (the residue proton peaks of CDCl3 at 77.02 ppm and DMSO at 40.03 ppm).

# 2. Experimental procedures

## 2.1 Synthesis of lignin models

Scheme S1 synthetic ketone lignin model.

The synthesis reaction was carried out in a 150 mL round glass flask containing a condenser. 2-phenoxyacetophenone were synthesized by the reaction of the corresponding phenol with 2-bromoacetophenone according to the reported procedure.1 Typically, 2-bromoacetophenone (5 mmol) is dissolved in a solution of K2CO3(7.5 mmol, 1.036 g) and phenol (5 mmol) in acetone (50 mL) and loaded in a reactor. The reaction mixture is then stirred at reflux temperature for 5 hours, filtered and vacuumized. The residue was purified by column chromatography with hexane: ethyl acetate a stirring. For the other methoxy substituted 2-phenoxy-1-phenylethanone, the preparation procedure is the same as described above, except of using different stating materials.

Scheme S2 synthetic 3-hydroxy-2-phenoxy-1-phenylpropan-1-one.

To a stirring suspension of K2CO3 (0.6 g, 4.3 mmol) in ethanol/acetone (v/v = 1/1, 20 mL) and 2-phenoxyacetophenones (0.78 g, 4 mmol) at rt, a water solution of formaldehyde (36.5~38wt%, 0.6 mL, 7.3 mmol) was added. After 4 h, the reaction mixture was filtered to remove K2CO3 and concentrated in vacuo to get a solid product. The crude product was purified with petroleum ether: ethyl acetate (3:1) to obtain the required 2-phenoxyacetophenone, on silica gel to obtain 3-hydroxy-1,2-diphenylpropan-1-one in 90% yield.

Scheme S3 synthetic lignin model compounds.

The resulting compound (3.5 mmol, 0.847 g) was dissolved in the mixture of THF:H2O (5:1) (total 25 mL), and sodium borohydride (7 mmol, 0.26 g) was added portionwise to maintain a gentle evolution of gas. Then, the mixture was stirred for 6 h at room temperature. The reaction mixture was quenched with saturated aqueous NH4Cl (50 mL) and diluted with 30 mL water. The aqueous portion was extracted with ethyl acetate (3×30mL). The organic parts were combined, dried over MgSO4, filtered and concentrated under vacuum. The residue was purified by column chromatography with hexane:ethyl acetate (2:1).

## 2.2 Photocatalytic cleavage of lignin model compounds

Scheme S4.Cleavage of lignin model compound.

The general procedure of the reaction: the photocatalytic equipment is composed of two 400-410 nm LED series. The constant voltage steady current power supply voltage is 32 V, the electric current is 1.25 A, and the power is 40 W. Feed was added to a 10mL quartz reaction tube and an oxygen balloon was mounted and stirred on a photocatalytic device while the device was cooled by a fan. After 12 hours, the reaction solution was diluted with dilute hydrochloric acid and extracted with dichloromethane. The yield of the obtained product was determined by GC with dodecane as internal standard.

# 3. Additional screening of reaction conditions

Table S1exploratory experiments.

| Entry | Additive | Yield% |
| --- | --- | --- |
| 1 | Aliquant-336 | -- |
| 2 | HCHO | -- |
| 3 | HCOOH | -- |
| 4 | tBuOK | -- |
| 5 | Ca(OH)2 | 35 |
| 6 | CaO | 28 |

Standard condition: 0.1 mmol substrate, 0.2 eq FeCl3, 0.2 eqTBACl, additive 1 eq, 2.0 mL acetone, air atmosphere, RT, 12 h, 20 W blue LED. The yield of guaiacol, measured by GC.

# 4. Mechanism study

4.1 Quenching experiments

Table S2Quenching experiments.

| Trapping agents | Quenched material | Yield of benzoic acid % | Yield ofphenol % |
| --- | --- | --- | --- |
| L-tryptophan 1 eq | 1O2 | 35 | 40 |
| Isopropyl alcohol 1 eq | .OH | 51 | 69 |
| 1,4-benzoquinone 1 eq | .O2- | 61 | 67 |

Standard conditions: substrate (0.1 mmol), FeCl3 (0.2 eq), TBACl (0.2 eq), 2.0 mL acetone, trapping agents 1 eq, O2 atmosphere, RT, 12 h, 20 W blue LED.

4.2 Detection of CO2

To detect the CO2 evolution, we used Ca(OH)2 solution to capture CO2. A 10 mL Schlenk tube 1 was added 0.1mmol FeCl3, 0.1mmol TBACl, substrate (0.5 mmol), and 8 mL acetone. The ice bath 2 is used to collect acetone volatilized during the reaction. O2 is introduced into the closed system and air is discharged. Inject 0.1M clear Ca(OH)2 aqueous solution into tube 3 with syringe to capture CO2. The beaker 4 with silicone oil can prevent CO2 in the air from entering the system. During the reaction the flow rate of oxygen was kept at 10 mL·min-1. After 15 hours of reaction, it was observed that the solution in the test tube 3 became turbid, and it was speculated that CO2 was generated during the reaction.

Figure S1. Experimental device diagram of CO2 collection.

# 5. Degradation of real lignin

5.1 Extraction of lignin

We charged a round bottom flask with 10.3 g poplar sawdust, 50 mL 1,4-dioxane, and 1.75 mL HCl (37wt%), and heated to reflux at 85 °C in oil bath for 3 h. After cooling to rt, the mixture was added with 3.36 g sodium bicarbonate (NaHCO3), stirred for another 30 min, after which it was filtered and washed with 10 mL of dioxane. Then the solution was concentrated at 40 °C under reduced pressure. The resulting dark-brown oil was diluted with 30 mL ethyl acetate (EtOAc) and added dropwise to 500 mL of hexane to precipitate the lignin. After filtration, the collected lignin was washed with hexane (50 mL). The recovered lignin was dried overnight at rt in a desiccator to afford 1.36 g poplar lignin.2

5.2 Depolymerization of lignin

Figure S2.Photocatalytic depolymerization of natural lignin.

Put 100mg poplar lignin, 12mg FeCl3, 27mg TBACI into four test tubes respectively, add 2mL acetone to each test tube, and irradiate the reaction system under two 400-410nm LEDs for 12 hours. After the reaction, acidify with HCl, remove the flocs with a sand core funnel, extract the reaction products with CH2Cl2, detect the products by GC-MS, and conduct quantitative analysis by GC.

5.3 GC-MS analysis

Figure S3 GC-MS analysis of natural lignin depolymerization.

# 6. 1H NMR and 13C NMR spectra of compounds

Figure S4 1H (top) and 13C (bottom) NMR spectra of 2-Phenoxy-1-phenyl-ethanol.

Figure S5 1H (top) and 13C (bottom) NMR spectra of 2-(2-methoxyphenoxy)-1-phenylethan-1-ol.

Figure S6 1H (top) and 13C (bottom) NMR spectra of 2-(4-chlorophenoxy)-1-phenylethan-1-ol.

Figure S7 1H (top) and 13C (bottom) NMR spectra of 1,2-diphenylethan-1-ol.

Figure S8 1H (top) and 13C (bottom) NMR spectra of 1-(4-methoxyphenyl)-2-phenoxyethan-1-ol.

Figure S9 1H (top) and 13C (bottom) NMR spectra of 2-(2-methoxyphenoxy)-1-(4-methoxyphenyl) ethan-1-ol.

Figure S10 1H (top) and 13C (bottom) NMR spectra of 2-(2,6-dimethoxyphenoxy)-1-(4-methoxyphenyl) ethan-1-ol.

Figure S11 1H (top) and 13C (bottom) NMR spectra of 2-phenoxy-1-phenylpropane-1,3-diol.

Figure S12 1H (top) and 13C (bottom) NMR spectra of 2-(2-methoxyphenoxy)-1-phenylpropane-1,3-diol.

Figure S13 1H (top) and 13C (bottom) NMR spectra of 2-(2,6-dimethoxyphenoxy)-1-phenylpropane-1,3-diol.

Figure S14 1H (top) and 13C (bottom) NMR spectra of 1-(4-methoxyphenyl)-2-phenoxypropane-1,3-diol.

Figure S15 1H (top) and 13C (bottom) NMR spectra of 2-(2-methoxyphenoxy)-1-(4-methoxyphenyl) propane-1,3-diol.

Figure S16 1H (top) and 13C (bottom) NMR spectra ofbenzoic acid.

Figure S17 1H (top) and 13C (bottom) NMR spectra of4-methoxybenzoic acid.

Figure S18 1H (top) and 13C (bottom) NMR spectra of phenol.

Figure S19 1H (top) and 13C (bottom) NMR spectra of 2-methoxyphenol.

Figure S201H (top) and 13C (bottom) NMR spectra of 2,6-dimethoxyphenol.

# 7. References

[1]	S. A. Kim, S. E. Kim, Y. K. Kim and H. Y. Jang, ACS Omega, 2020, 5, 31684-31691.

[2]	Y. Wang, J. He and Y. Zhang, CCS Chemistry, 2020, 2, 107-117.