Supplementary Material #2

to the paper

Removal of Methylene Blue Dye Using Crushed Walnut Shell Adsorbent –Determining Comminution Energy and Optimal Operational Conditions

Manual calculations to extract the relationships used in depicting Figs 2, and 3

1- Extracting the relationships between ξ1, ξ2, X1, X2 and W, and C0

ξ1 = Adsorbent amount (g)Initial adsorbate amount (g) = W1000C01000×0.200 = 5 WC0

X1 = ξ1 - 2515         ⟹   X1 = W3C0 -  53

ξ2 = Adsorbent concentration (g/L) = W10000.200 =  W200

X2 = ξ2 - 2.51.5         ⟹   X2 = W300 -  53

2- Rewring Eqs 7 - 9 based on W, and C0

YType I = 86.66949 + 17.01530X1- 4.82050X2 - 12.20588X12+8.19610X1X2

⟹    YType I = 86.66949 + 17.01530 ×W3C0 -  53- 4.82050 ×W300 -  53 - 12.20588 ×W3C0 -  532+8.19610×W3C0 -  53×W300 -  53

YType II = 96.48167 + 14.85884X1 - 14.50059X12

⟹    YType II = 96.48167 + 14.85884 ×W3C0 -  53- 14.50059 ×W3C0 -  532

YType III = 98.81498 + 4.11230X1- 3.24546X12

⟹    YType III = 98.81498 + 4.11230 ×W3C0 -  53- 3.24546 ×W3C0 -  532

3- Extracting the relationships between (En/CA) and Y, W, and C0

Y = Dye Removal Efficiency % = C0 - CeC0 × 100

⟹    C0 Y100 = C0 - Ce

En/CA Jmg-dye adsorbed = WB × 3.6 ×  W 1000C0 - Ce × V= WB × 3.6200 × WC0 - Ce

or                  En/CA = WB × 3.6200 × 100 × WC0 ×Y

⟹    En/CAType I = 48.33 × 1.8YType I × WC0

⟹    En/CAType II = 137.92 × 1.8YType II × WC0

⟹    En/CAType III = 435.67 × 1.8YType III × WC0

4- Extracting the relationships between (qe) and Y, W, and C0

qe = C0 - Ce × VW × 1000 = 200 × C0 Y100W

⟹    qeType I = 2 × YType I × C0 W

⟹    qeType II = 2 × YType II × C0 W

⟹    qeType III = 2 × YType III × C0 W