# Trade-offs between hydropower and irrigation in transboundary river systems: the implications of further development on the Blue Nile in Ethiopia: Supplementary Information

## Contents

Supplementary Tables	2

Supplementary Figures	3

Supplementary note 1: History of the Idea of the Blue Nile Cascade	4

Supplementary note 2: ENRM schematic	6

Supplementary note 3: Modelling assumptions	7

A) Scenario labels	7

B) Reservoir assumptions	7

C) Evaporation	7

Supplementary note 4: Cascade assumptions	9

A) Flood space elevation	9

B) Target powers	9

Supplementary note 5: Additional results	17

A) Average annual hydropower generation	17

B) Average annual evaporation losses	18

C) Average annual reservoir outflow	19

D) Annual average HAD releases	20

E) HAD reliability	21

Supplementary note 6: Breakeven analysis	23

A) Worked example	23

References	24

## Supplementary Tables

Table 1. Scenarios Modelled assuming HAD Target Release.

Table 2. Modelling assumptions of reservoirs along the Blue and Main Nile in the ENRM.

Table 3. Monthly evaporation, precipitation and net evaporation values for the GERD. Note these values do not represent the absolute net evaporation, but are used alongside simulated reservoir levels in the ENRM to calculate monthly evaporation at the reservoir. Rates based on values used in the River Basin Simulation software (RIBASIM; Deltares).

Table 4. Monthly evaporation, precipitation and net evaporation values for Upper Mendaya Dam. Note these values do not represent the absolute net evaporation, but are used alongside simulated reservoir levels in the ENRM to calculate monthly evaporation at the reservoir. Rates based on values used in the River Basin Simulation software (RIBASIM; Deltares).

Table 5. Monthly evaporation, precipitation and net evaporation values for Beko Abo High Dam. Note these values do not represent the absolute net evaporation, but are used alongside simulated reservoir levels in the ENRM to calculate monthly evaporation at the reservoir. Rates based on values used in the River Basin Simulation software (RIBASIM; Deltares).

Table 6. Key values used to calculate flood space elevation for the Upper Mendaya and Beko Abo High dams.

Table 7. Known and unknown variables of the reservoirs simulated in Baseline case.

Table 8. GERD 90% power reliability values for varying target power.

Table 9. Known and unknown variables of the reservoirs simulated in Scenario 2: GERD + Upper Mendaya.

Table 10. Upper Mendaya 90% power reliability values for varying target power.

Table 11. GERD 90% power reliability values for varying target power.

Table 12. Known and unknown variables of the reservoirs simulated in Scenario 3: GERD + two dams.

Table 13. Beko Abo High 90% power reliability values for varying target power.

Table 14. Upper Mendaya 90% power reliability values for varying target power.

Table 15. GERD 90% power reliability values for varying target power.

Table 16. Reservoir variables used in cascade scenario analysis.

Table 17. Average annual hydropower generation for the GERD only scenario (one-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to the nearest 100 GWh.

Table 18. Average annual hydropower generation for the GERD and Upper Mendaya scenario (two-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to the nearest 100 GWh.

Table 19. Average annual hydropower generation for the GERD, Upper Mendaya and Beko Abo High scenario (three-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to the nearest 100 GWh.

Table 20. Evaporation losses for the Baseline scenario (GERD only) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to one tenth bcm.

Table 21. Evaporation losses for the GERD and Upper Mendaya scenario (two-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to one tenth bcm.

Table 22. Evaporation losses for the GERD, Upper Mendaya and Beko Abo High scenario (three-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to one tenth bcm.

Table 23. Average annual reservoir outflow for the Baseline scenario (GERD only) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to one tenth bcm.

Table 24. Average annual reservoir outflow for the GERD and Upper Mendaya scenario (two-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to one tenth bcm.

Table 25. Average annual reservoir outflow for the GERD, Upper Mendaya and Beko Abo High scenario (three-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to one tenth bcm.

Table 26. HAD reliability for Baseline scenario (GERD only).

Table 27. HAD reliability for GERD + Upper Mendaya scenario (two-dam cascade).

Table 28. HAD reliability for GERD + Upper Mendaya + Beko Abo High scenario (three-dam cascade).

Table 29. HAD reliability for the one-, two- and three-dam scenario with a 55.5 bcm annual release target.

## Supplementary Figures

Figure 1. Schematic of the Eastern Nile RiverWare Model, with existing and proposed reservoirs, inflows, evaporation and channel losses, and water withdrawals. Annual average values presented are derived from the three-dam cascade scenario with no new irrigation withdrawals (Scenario C0).

Figure 2. GERD target power vs 90% power reliability.

Figure 3. Upper Mendaya target power vs 90% power reliability.

Figure 4. GERD target power vs 90% power reliability.

Figure 5. Beko Abo High target power vs 90% power reliability.

Figure 6. Upper Mendaya target power vs 90% power reliability.

Figure 7. GERD target power vs 90% power reliability.

Figure 9. Average annual HAD releases for a range of HAD target release volumes (51 bcm/yr to 60 bcm/yr) with increasing annual Ethiopian withdrawals upstream of a three-dam cascade (GERD + Upper Mendaya + Beko Abo High).

## Supplementary note 1: History of the Idea of the Blue Nile Cascade

From its headwaters in Lake Tana, the Blue Nile descends through the canyons of the Blue Nile gorge, offering several potential sites for hydropower generation upstream of the GERD. Various dam sites with the potential for high heads and low surface-to-volume ratios have long attracted the attention of civil engineers and energy planners. In 1964 the US Bureau of Reclamation published the first detailed plan for the development of hydropower dams in the Blue Nile gorge (USBR, 1964). This plan included a cascade with four main hydropower dams: 1) Border, 2) Mendaya, 3) Mabil, and 4) Karadobi. Almost five decades later Ethiopian water resource and hydropower development planners are still looking at the same sites or variations of them.

There have been a few new developments in the thinking about a Blue Nile cascade since the USBR report. During the early years of the 21st century, consultants working for the Ethiopian Ministry of Water Resources updated and expanded the US Bureau of Reclamations plan. Their work has been published in a series of studies commissioned by the Nile Basin Initiative (NBI, 2014). When Ethiopia announced in 2011 that the GERD was selected as the first dam to be built with a significantly higher storage capacity than initially envisaged, consultants had to rework their plans. After filling the GERD reservoir is completed, the reservoir will retain 74 billion cubic meters at the peak of the flood season and reach nearly 200 kilometres upstream, flooding the original USBR Mendaya dam site. Norwegian consultants working with the Ethiopian Ministry of Water Resources identified a new Mendaya dam site (“Upper Mendaya”) approximately 30 kilometres upstream of the old site. However, a dam constructed at this Upper Mandaya site would flood the original USBR Mabil site. An attractive new site was then identified at Beko Abo between Mabil and Karadobi (Norconsult, 2007) to replace the Mabil site. Depending on the size of the Beko Abo site, the Karadobi dam site might in turn be inundated.

The USBR study reported that all four sites in the original cascade had benefit-cost ratios between 3:1 and 4:1 based solely on estimates of benefits and costs to Ethiopia, ignoring the system-wide consequences of filling and operating these hydropower dams. Subsequent analyses have re-enforced this initial USBR conclusion that investments in a Blue Nile cascade would be economically attractive, but full economic analysis of the Blue Nile cascade has never been conducted. Nor have the cost estimates been updated in the light of the cost of constructing the GERD, which is not yet final. To the best of our knowledge, none of the pre-feasibility and feasibility studies conducted for the Ethiopian Ministry of Water Resources analyzed how irrigation withdrawals upstream of the cascade would affect system-wide hydropower generation or how management of the cascade could impact downstream riparians.

In a “Scoping Study” report to the Eastern Nile Council of Ministers, Blackmore and Whittington (2008) presented estimates of the additional system-wide hydropower generation that would result from a 4-dam cascade (Border, Mendaya, Mabil, and Karadobi) compared to an infrastructure baseline that included the Merowe Dam in Sudan (completed in 2009), the Tana-Beles link in Ethiopia, and other infrastructure outside the Blue Nile gorge. They showed that the Blue Nile cascade would almost double hydropower generation in the basin, from 26,800 to 45,000 GWh per year.

Jeuland and Whittington (2014) analyzed the economics of different combinations and sizes of hydropower dams in a Blue Nile cascade. Assuming the GERD would be completed as planned, their analyses suggested that the most economically attractive option would be to a two-dam cascade consisting of the GERD and a second large hydropower dam at Beko Abo.

In 2011, Norplan in collaboration with other consultant published a report to the Ethiopian Ministry of Water Resources entitled, “Alternatives to the Mandaya Project – Cascading Options”. This report reviewed the options for dam sites in the Blue Nile gorge. The report focused largely on the technical and physical aspects of alternative dam sites, and did not include any financial or economic analyses. The consultants recommended that the Government of Ethiopia move forward with the preparation of full feasibility studies for dam projects at both the Karadobi and Beko Abo sites, and to conduct a pre-feasibility study at a newly identified “Upper Mandaya” site that would not be inundated by the current GERD configuration.

There is no consensus in the literature on the optimal Blue Nile cascade. For the purposes of this paper, we assume that the complete cascade consists of the GERD, Upper Mendaya and Beko Abo High. A two-dam cascade with only the GERD and Upper Mendaya is also simulated.

## Supplementary note 2: ENRM schematic

Figure 1. Schematic of the Eastern Nile RiverWare Model, with existing and proposed reservoirs, inflows, evaporation and channel losses, and water withdrawals. Annual average values presented are derived from the three-dam cascade scenario with no new irrigation withdrawals (Scenario C0).

## Supplementary note 3: Modelling assumptions

### A) Scenario labels

| Table 1. Scenarios Modelled assuming HAD Target Release. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ethiopian Blue Nile Reservoir Configuration |  | Additional Irrigation Withdrawal |  |  |  |  |  |  |
|  |  | 0 BCM | 1 BCM | 2 BCM | 3 BCM | 4 BCM | 5 BCM |
| A | GERD | A0 - Baseline | A1 | A2 | A3 | A4 | A5 |
| B | GERD + Upper Mendaya | B0 | B1 | B2 | B3 | B4 | B5 |
| C | GERD + Upper Mendaya + Beko Abo High Dam | C0 | C1 | C2 | C3 | C4 | C5 |

### B) Reservoir assumptions

| Table 2. Modelling assumptions of reservoirs along the Blue and Main Nile in the ENRM. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Reservoir | Minimum active pool elevation (m) | Maximum pool elevation (m) | Flood space elevation (m) | Target power (MW) |
| High Aswan Dam | 147 | 178 | 175 | -- |
| Merowe | 285 | 300 | -- | 625 |
| Sennar | 417.2 | 422 | -- | -- |
| Roseires | 467 | 490 | -- | -- |
| Grand Ethiopian Renaissance Dam | 595 | 640 | 625 | See cascade assumptions |
| Upper Mendaya Dam | 770 | 800 | 788* | See cascade assumptions |
| Beko Abo High Dam | 1010 | 1062 | 1042* | See cascade assumptions |
| * Method to define Upper Mendaya and Beko Abo High dam flood space elevation described below in cascade assumptions. |  |  |  |  |

### C) Evaporation

| Table 3. Monthly evaporation, precipitation and net evaporation values for the GERD. Note these values do not represent the absolute net evaporation, but are used alongside simulated reservoir levels in the ENRM to calculate monthly evaporation at the reservoir. Rates based on values used in the River Basin Simulation software (RIBASIM; Deltares). |  |  |  |
| --- | --- | --- | --- |
| Month | Evaporation (mm/month) | Precipitation (mm/month) | Net Evaporation (mm/month) |
| Jan | 187.3 | 0.0 | 187.3 |
| Feb | 185.5 | 0.2 | 185.4 |
| Mar | 187.3 | 2.7 | 184.7 |
| Apr | 212.8 | 17.9 | 194.9 |
| May | 167.0 | 80.2 | 86.7 |
| June | 130.0 | 168.7 | -38.6 |
| July | 126.2 | 237.2 | -110.9 |
| Aug | 110.0 | 241.6 | -131.6 |
| Sept | 141.9 | 163.5 | -21.6 |
| Oct | 154.7 | 76.3 | 78.5 |
| Nov | 149.8 | 6.8 | 142.9 |
| Dec | 171.0 | 1.0 | 170.0 |
| Total | 1923.6 | 996.0 | 927.6 |
| Table 4. Monthly evaporation, precipitation and net evaporation values for Upper Mendaya Dam. Note these values do not represent the absolute net evaporation, but are used alongside simulated reservoir levels in the ENRM to calculate monthly evaporation at the reservoir. Rates based on values used in the River Basin Simulation software (RIBASIM; Deltares). |  |  |  |
| Month | Evaporation (mm/month) | Precipitation (mm/month) | Net Evaporation (mm/month) |
| Jan | 176.4 | 0.9 | 175.4 |
| Feb | 185.5 | 2.7 | 182.8 |
| Mar | 167.4 | 14.4 | 152.9 |
| Apr | 210.1 | 47.7 | 162.4 |
| May | 131.2 | 148.4 | -17.2 |
| June | 126.9 | 212.5 | -85.5 |
| July | 122.1 | 279.9 | -157.8 |
| Aug | 108.6 | 272.5 | -164.0 |
| Sept | 144.4 | 196.0 | -51.6 |
| Oct | 162.8 | 107.1 | 55.7 |
| Nov | 153.2 | 23.3 | 129.9 |
| Dec | 167.4 | 3.5 | 163.8 |
| Total | 1855.9 | 1308.9 | 547.1 |

| Table 5. Monthly evaporation, precipitation and net evaporation values for Beko Abo High Dam. Note these values do not represent the absolute net evaporation, but are used alongside simulated reservoir levels in the ENRM to calculate monthly evaporation at the reservoir. Rates based on values used in the River Basin Simulation software (RIBASIM; Deltares). |  |  |  |
| --- | --- | --- | --- |
| Month | Evaporation (mm/month) | Precipitation (mm/month) | Net Evaporation (mm/month) |
| Jan | 155.3 | 6.5 | 148.8 |
| Feb | 158.7 | 8.2 | 150.4 |
| Mar | 174.1 | 27.2 | 147.0 |
| Apr | 195.8 | 49.1 | 146.7 |
| May | 164.7 | 126.8 | 37.9 |
| June | 132.1 | 188.8 | -56.8 |
| July | 108.2 | 316.7 | -208.4 |
| Aug | 103.5 | 296.7 | -193.2 |
| Sept | 132.1 | 189.1 | -57.0 |
| Oct | 160.0 | 92.5 | 67.5 |
| Nov | 145.7 | 22.2 | 123.5 |
| Dec | 150.6 | 6.1 | 144.5 |
| Total | 1780.7 | 1329.9 | 450.8 |

## Supplementary note 4: Cascade assumptions

### A) Flood space elevation

| Table 6. Key values used to calculate flood space elevation for the Upper Mendaya and Beko Abo High dams. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dam | (A) Minimum active pool elevation (m) | (B) Total volume of reservoir at minimum active pool elevation (MCM) | (C) Maximum active pool elevation (m) | (D) Total volume of reservoir at maximum active pool elevation (MCM) | (E) Total active storage (MCM) | (F) 55.6% of total active storage (MCM) | (B) + (F) | (G) Flood space elevation (m) |
| GERD | 595 | 18400 | 640 | 74000 | 55600 | 30900 | 49300 | 625 |
| Upper Mendaya | 770 | 15827 | 800 | 27500 | 11673 | 6487 | 22314 | 788 |
| Beko Abo High | 1010 | 14707 | 1062 | 31692 | 16985 | 9440 | 24147 | 1042 |

### B) Target powers

The method to define target powers for the three cascade scenarios is described below:

- For the most upstream reservoir in the cascade, for a given infrastructure scenario, perform a sensitivity test on the reservoir target power (MW).

- Select the target power which achieves the maximum 90% power reliability.

- Repeat for the downstream cascade reservoirs (if relevant for the infrastructure scenario).

Results from the target power sensitivity analyses are presented below.

#### Baseline scenario: one-dam (GERD only, no Ethiopian irrigation withdrawals)

Table 7 presents the known and unknown variables for the reservoirs in the cascade in the GERD only scenario. In this scenario there are no new developments upstream of the GERD. Table 8 presents the results from the GERD target power sensitivity analysis with a fixed flood elevation of 625m; Figure 2 visualises the data presented in this table. The optimal target power is highlighted in green.

| Table 7. Known and unknown variables of the reservoirs simulated in Baseline case. |  |  |
| --- | --- | --- |
| Reservoir | Target Power (MW) | Flood space elevation (m) |
| GERD | Unknown | 625 |

| Table 8. GERD 90% power reliability values for varying target power. |  |  |  |
| --- | --- | --- | --- |
| Reservoir | Target Power (MW) | Flood space elevation (m) | 90% reliability (MW) |
| GERD | 1000 | 625 | 1000 |
|  | 1100 | 625 | 1100 |
|  | 1200 | 625 | 1200 |
|  | 1300 | 625 | 1300 |
|  | 1400 | 625 | 1400 |
|  | 1425 | 625 | 1425 |
|  | 1450 | 625 | 1450 |
|  | 1475 | 625 | 1474 |
|  | 1500 | 625 | 1494 |
|  | 1525 | 625 | 1509 |
|  | 1550 | 625 | 977 |
|  | 1575 | 625 | 604 |
|  | 1600 | 625 | 445 |
|  | 1700 | 625 | 182 |
|  | 1800 | 625 | 138 |

Figure 2. GERD target power vs 90% power reliability.

#### Scenario 2: two-dams (GERD and Upper Mendaya, no Ethiopian irrigation withdrawals)

Table 9 presents the known and unknown variables for the reservoirs in the two-dam cascade. In this scenario there is one new development upstream of the GERD: the Upper Mendaya Dam which has a flood space elevation of 788m. Table 10 presents the results from the Upper Mendaya target power sensitivity analysis; Figure 3 visualises the data presented in this table. The optimal target power is highlighted in green.

| Table 9. Known and unknown variables of the reservoirs simulated in Scenario 2: GERD + Upper Mendaya. |  |  |
| --- | --- | --- |
| Reservoir | Target Power (MW) | Flood space elevation (m) |
| GERD | Unknown | 625 |
| Upper Mendaya | Unknown | 788 |

| Table 10. Upper Mendaya 90% power reliability values for varying target power. |  |  |  |
| --- | --- | --- | --- |
| Reservoir | Target Power (MW) | Flood space elevation (m) | 90% reliability (MW) |
| Upper Mendaya | 700 | 788 | 700 |
|  | 750 | 788 | 750 |
|  | 800 | 788 | 800 |
|  | 825 | 788 | 825 |
|  | 850 | 788 | 849 |
|  | 875 | 788 | 803 |
|  | 900 | 788 | 640 |
|  | 925 | 788 | 488 |
|  | 950 | 788 | 394 |
|  | 1000 | 788 | 290 |

Figure 3. Upper Mendaya target power vs 90% power reliability.

The sensitivity analysis for the GERD reservoir is repeated for the two-dam scenario, with the target power for the Upper Mendaya Dam fixed at its optimal power for this infrastructure configuration (850MW). Table 11 presents the results from the Upper Mendaya target power sensitivity analysis; Figure 4 shows the data presented in this table. The optimal target power is highlighted in green.

| Table 11. GERD 90% power reliability values for varying target power. |  |  |  |
| --- | --- | --- | --- |
| Reservoir | Target Power (MW) | Flood space elevation (m) | 90% reliability (MW) |
| GERD | 1000 | 625 | 1000 |
|  | 1100 | 625 | 1100 |
|  | 1200 | 625 | 1200 |
|  | 1300 | 625 | 1300 |
|  | 1400 | 625 | 1400 |
|  | 1425 | 625 | 1423 |
|  | 1450 | 625 | 1444 |
|  | 1475 | 625 | 1184 |
|  | 1500 | 625 | 773 |
|  | 1525 | 625 | 633 |
|  | 1550 | 625 | 604 |
|  | 1575 | 625 | 588 |
|  | 1600 | 625 | 583 |
|  | 1700 | 625 | 573 |
|  | 1800 | 625 | 571 |

Figure 4. GERD target power vs 90% power reliability.

#### Scenario 3: three-dams (GERD + Upper Mendaya + Beko Abo High, no Ethiopian irrigation withdrawals)

Table 12 presents the known and unknown variables for the reservoirs in the three-dam cascade. In this scenario there are two new developments upstream of the GERD: the Upper Mendaya Dam which has a flood space elevation of 788m and the Beko Abo High Dam which has a flood space elevation of 1042m. Table 13 presents the results from the Beko Abo High target power sensitivity analysis; Figure 5 shows the data presented in this table. The optimal target power is highlighted in green.

| Table 12. Known and unknown variables of the reservoirs simulated in Scenario 3: GERD + two dams. |  |  |
| --- | --- | --- |
| Reservoir | Target Power (MW) | Flood space elevation (m) |
| GERD | Unknown | 625 |
| Upper Mendaya | Unknown | 788 |
| Beko Abo High | Unknown | 1042 |

| Table 13. Beko Abo High 90% power reliability values for varying target power. |  |  |  |
| --- | --- | --- | --- |
| Reservoir | Target Power (MW) | Flood space elevation (m) | 90% reliability (MW) |
| Beko Abo High | 700 | 1042 | 700 |
|  | 750 | 1042 | 750 |
|  | 800 | 1042 | 800 |
|  | 825 | 1042 | 825 |
|  | 850 | 1042 | 671 |
|  | 875 | 1042 | 370 |
|  | 900 | 1042 | 249 |
|  | 925 | 1042 | 158 |
|  | 950 | 1042 | 120 |
|  | 975 | 1042 | 92 |
|  | 1000 | 1042 | 72 |

Figure 5. Beko Abo High target power vs 90% power reliability.

The sensitivity analysis for the Upper Mendaya reservoir is repeated for the three-dam scenario, with the target power for the Beko Abo High Dam fixed at its optimal power for this infrastructure configuration (825MW). Table 14 presents the results from the Upper Mendaya target power sensitivity analysis; Figure 6 shows the data presented in this table. The optimal target power is highlighted in green.

| Table 14. Upper Mendaya 90% power reliability values for varying target power. |  |  |  |
| --- | --- | --- | --- |
| Reservoir | Target Power (MW) | Flood space elevation (m) | 90% reliability (MW) |
| Upper Mendaya | 700 | 788 | 700 |
|  | 750 | 788 | 750 |
|  | 800 | 788 | 800 |
|  | 825 | 788 | 825 |
|  | 850 | 788 | 850 |
|  | 875 | 788 | 875 |
|  | 900 | 788 | 900 |
|  | 925 | 788 | 923 |
|  | 950 | 788 | 949 |
|  | 975 | 788 | 972 |
|  | 1000 | 788 | 796 |

Figure 6. Upper Mendaya target power vs 90% power reliability.

The sensitivity analysis for the GERD reservoir is repeated for the three-dam scenario, with the target power for the Upper Mendaya Dam (975MW) and Beko Abo High (825MW) fixed at their optimal power for this infrastructure configuration. Table 15 presents the results from the GERD target power sensitivity analysis; Figure 7 shows the data presented in this table. The optimal target power is highlighted in green.

| Table 15. GERD 90% power reliability values for varying target power. |  |  |  |
| --- | --- | --- | --- |
| Reservoir | Target Power (MW) | Flood space elevation (m) | 90% reliability (MW) |
| GERD | 1000 | 625 | 1000 |
|  | 1100 | 625 | 1100 |
|  | 1200 | 625 | 1200 |
|  | 1300 | 625 | 1300 |
|  | 1400 | 625 | 1400 |
|  | 1425 | 625 | 1419 |
|  | 1450 | 625 | 1346 |
|  | 1475 | 625 | 1097 |
|  | 1500 | 625 | 834 |
|  | 1525 | 625 | 683 |
|  | 1550 | 625 | 659 |
|  | 1575 | 625 | 644 |
|  | 1600 | 625 | 639 |
|  | 1700 | 625 | 635 |
|  | 1800 | 625 | 635 |

Figure 7. GERD target power vs 90% power reliability.

#### Summary of cascade settings

The sensitivity analysis reveals the optimal target powers for each dam in the one-, two- and 3-dam configurations. Table 16 presents a synthesis of the sensitivity analysis and target powers used in the scenario analysis presented in the main results.

| Table 16. Reservoir variables used in cascade scenario analysis. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Scenario | Variable | GERD | Upper Mendaya | Beko Abo High |
| One-dam (Baseline, GERD only, no irrigation) | Flood space elevation (m) | 625 | - | - |
|  | Target Power (MW) | 1525 | - | - |
| Two-dams (GERD + Upper Mendaya, no irrigation) | Flood space elevation (m) | 625 | 788 | - |
|  | Target Power (MW) | 1450 | 850 | - |
| Three-dams (GERD + Upper Mendaya + Beko Abo High, no irrigation) | Flood space elevation (m) | 625 | 788 | 1042 |
|  | Target Power (MW) | 1425 | 975 | 825 |

## Supplementary note 5: Additional results

### A) Average annual hydropower generation

| Table 17. Average annual hydropower generation for the GERD only scenario (one-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to the nearest 100 GWh. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reservoir | Average Annual Energy (GWh/yr) |  |  |  |  |  |
|  | 0 bcm/yr | + 1 bcm/yr | + 2 bcm/yr | + 3 bcm/yr | + 4 bcm/yr | + 5 bcm/yr |
| Beko Abo High | - | - | - | - | - | - |
| Upper Mendaya | - | - | - | - | - | - |
| GERD | 13000 | 12300 | 11700 | 11300 | 10900 | 10600 |
| Roseires | 2400 | 2400 | 2400 | 2300 | 2300 | 2300 |
| Sennar | 100 | 100 | 100 | 100 | 100 | 100 |
| Merowe | 7500 | 7500 | 7500 | 7400 | 7400 | 7300 |
| HAD | 7200 | 7100 | 6900 | 6800 | 6700 | 6600 |
| Total | 30200 | 29400 | 28600 | 27900 | 27400 | 26900 |

| Table 18. Average annual hydropower generation for the GERD and Upper Mendaya scenario (two-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to the nearest 100 GWh. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reservoir | Average Annual Energy (GWh/yr) |  |  |  |  |  |
|  | 0 bcm/yr | + 1 bcm/yr | + 2 bcm/yr | + 3 bcm/yr | + 4 bcm/yr | + 5 bcm/yr |
| Beko Abo High | - | - | - | - | - | - |
| Upper Mendaya | 8400 | 8100 | 7800 | 7400 | 7100 | 6700 |
| GERD | 12900 | 12100 | 11100 | 10700 | 10300 | 10000 |
| Roseires | 2400 | 2400 | 2400 | 2400 | 2400 | 2300 |
| Sennar | 100 | 100 | 100 | 100 | 100 | 100 |
| Merowe | 7300 | 7400 | 7400 | 7400 | 7300 | 7200 |
| HAD | 7100 | 7000 | 6900 | 6800 | 6600 | 6500 |
| Total | 38200 | 37100 | 35700 | 34800 | 33800 | 32800 |

| Table 19. Average annual hydropower generation for the GERD, Upper Mendaya and Beko Abo High scenario (three-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to the nearest 100 GWh. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reservoir | Average Annual Energy (GWh/yr) |  |  |  |  |  |
|  | 0 bcm/yr | + 1 bcm/yr | + 2 bcm/yr | + 3 bcm/yr | + 4 bcm/yr | + 5 bcm/yr |
| Beko Abo High | 7100 | 6700 | 6400 | 6000 | 5700 | 5300 |
| Upper Mendaya | 8400 | 8000 | 7600 | 7200 | 6900 | 6600 |
| GERD | 12100 | 11300 | 10500 | 10300 | 10000 | 9800 |
| Roseires | 2400 | 2400 | 2400 | 2400 | 2300 | 2300 |
| Sennar | 100 | 100 | 100 | 100 | 100 | 100 |
| Merowe | 7400 | 7500 | 7500 | 7400 | 7400 | 7300 |
| HAD | 7200 | 7000 | 6900 | 6800 | 6700 | 6500 |
| Total | 44700 | 43000 | 41400 | 40200 | 39100 | 37900 |

### B) Average annual evaporation losses

| Table 20. Evaporation losses for the Baseline scenario (GERD only) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to one tenth bcm. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reservoir | Average Annual Evaporation (bcm/yr) |  |  |  |  |  |
|  | 0 bcm/yr | + 1 bcm/yr | + 2 bcm/yr | + 3 bcm/yr | + 4 bcm/yr | + 5 bcm/yr |
| Beko Abo High | - | - | - | - | - | - |
| Upper Mendaya | - | - | - | - | - | - |
| GERD | 1.4 | 1.3 | 1.2 | 1.1 | 1.1 | 1.0 |
| Roseires | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| Sennar | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
| Merowe | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.8 |
| HAD | 11.4 | 10.9 | 10.3 | 9.8 | 9.4 | 9.1 |
| Total | 16.1 | 15.5 | 14.8 | 14.2 | 13.8 | 13.3 |

| Table 21. Evaporation losses for the GERD and Upper Mendaya scenario (two-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to one tenth bcm. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reservoir | Average Annual Evaporation (bcm/yr) |  |  |  |  |  |
|  | 0 bcm/yr | + 1 bcm/yr | + 2 bcm/yr | + 3 bcm/yr | + 4 bcm/yr | + 5 bcm/yr |
| Beko Abo High | - | - | - | - | - | - |
| Upper Mendaya | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 |
| GERD | 1.4 | 1.2 | 1.0 | 1.0 | 1.0 | 0.9 |
| Roseires | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| Sennar | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
| Merowe | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.8 |
| HAD | 11.0 | 10.6 | 10.2 | 9.7 | 9.3 | 8.9 |
| Total | 16.0 | 15.4 | 14.8 | 14.3 | 13.9 | 13.3 |

| Table 22. Evaporation losses for the GERD, Upper Mendaya and Beko Abo High scenario (three-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to one tenth bcm. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reservoir | Average Annual Evaporation (bcm/yr) |  |  |  |  |  |
|  | 0 bcm/yr | + 1 bcm/yr | + 2 bcm/yr | + 3 bcm/yr | + 4 bcm/yr | + 5 bcm/yr |
| Beko Abo High | 0.2 | 0.2 | 0.2 | 0.1 | 0.1 | 0.1 |
| Upper Mendaya | 0.3 | 0.3 | 0.3 | 0.3 | 0.2 | 0.2 |
| GERD | 1.2 | 1.0 | 0.9 | 0.9 | 0.9 | 0.9 |
| Roseires | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| Sennar | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
| Merowe | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 |
| HAD | 11.2 | 10.7 | 10.2 | 9.7 | 9.3 | 9.0 |
| Total | 16.2 | 15.5 | 14.9 | 14.3 | 13.8 | 13.5 |

### C) Average annual reservoir outflow

| Table 23. Average annual reservoir outflow for the Baseline scenario (GERD only) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to one tenth bcm. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reservoir | Average Annual Reservoir Outflow (bcm/yr) |  |  |  |  |  |
|  | 0 bcm/yr | + 1 bcm/yr | + 2 bcm/yr | + 3 bcm/yr | + 4 bcm/yr | + 5 bcm/yr |
| Beko Abo High | - | - | - | - | - | - |
| Upper Mendaya | - | - | - | - | - | - |
| GERD | 46.8 | 45.9 | 45 | 44.1 | 43.1 | 42.2 |
| Roseires | 47.8 | 46.9 | 46 | 45.1 | 44.2 | 43.2 |
| Sennar | 39.7 | 38.8 | 37.9 | 37.0 | 36.1 | 35.1 |
| Merowe | 66.9 | 66.1 | 65.3 | 64.4 | 63.6 | 62.7 |
| HAD | 51.5 | 51.4 | 51.3 | 51.2 | 51.0 | 50.8 |

| Table 24. Average annual reservoir outflow for the GERD and Upper Mendaya scenario (two-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to one tenth bcm. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reservoir | Average Annual Reservoir Outflow (bcm/yr) |  |  |  |  |  |
|  | 0 bcm/yr | + 1 bcm/yr | + 2 bcm/yr | + 3 bcm/yr | + 4 bcm/yr | + 5 bcm/yr |
| Beko Abo High | - | - | - | - | - | - |
| Upper Mendaya | 24.8 | 23.8 | 22.9 | 21.9 | 20.9 | 19.9 |
| GERD | 46.4 | 45.6 | 44.8 | 43.8 | 42.9 | 41.9 |
| Roseires | 47.4 | 46.6 | 45.8 | 44.9 | 43.9 | 43.0 |
| Sennar | 39.3 | 38.5 | 37.7 | 36.8 | 35.8 | 34.9 |
| Merowe | 66.6 | 65.8 | 65.1 | 64.2 | 63.3 | 62.5 |
| HAD | 51.5 | 51.4 | 51.3 | 51.1 | 50.9 | 50.7 |

| Table 25. Average annual reservoir outflow for the GERD, Upper Mendaya and Beko Abo High scenario (three-dam) with annual irrigation expansion from 0 to 5 bcm. Numbers have been rounded to one tenth bcm. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reservoir | Average Annual Reservoir Outflow (bcm/yr) |  |  |  |  |  |
|  | 0 bcm/yr | + 1 bcm/yr | + 2 bcm/yr | + 3 bcm/yr | + 4 bcm/yr | + 5 bcm/yr |
| Beko Abo High | 20.6 | 19.6 | 18.6 | 17.6 | 16.6 | 15.6 |
| Upper Mendaya | 24.7 | 23.7 | 22.7 | 21.8 | 20.8 | 19.8 |
| GERD | 46.5 | 45.7 | 44.8 | 43.8 | 42.8 | 41.9 |
| Roseires | 47.5 | 46.7 | 45.8 | 44.9 | 43.9 | 42.9 |
| Sennar | 39.4 | 38.6 | 37.7 | 36.8 | 35.8 | 34.8 |
| Merowe | 66.6 | 65.9 | 65.1 | 64.2 | 63.3 | 62.5 |
| HAD | 51.5 | 51.4 | 51.3 | 51.1 | 50.9 | 50.7 |

### D) Annual average HAD releases

Figure 8. Average annual HAD releases for a range of HAD target release volumes (51 bcm/yr to 60 bcm/yr) with increasing annual Ethiopian withdrawals upstream of a three-dam cascade (GERD + Upper Mendaya + Beko Abo High).

### E) HAD reliability

| Table 26. HAD reliability for Baseline scenario (GERD only). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ethiopian Diversion (bcm/yr) Actual average diversion (bcm/yr) |  |  |  |  |  |
| Annual HAD release target (bcm/yr) | 0 (0) | 1 (1) | 2 (2) | 3 (3) | 4 (3.9) | 5 (4.9) |
| 51 | 98% | 96% | 95% | 92% | 92% | 89% |
| 52 | 97% | 95% | 92% | 92% | 89% | 88% |
| 53 | 95% | 93% | 91% | 89% | 87% | 81% |
| 54 | 93% | 91% | 89% | 87% | 80% | 72% |
| 55 | 91% | 89% | 86% | 80% | 72% | 65% |
| 56 | 91% | 87% | 80% | 73% | 65% | 55% |
| 57 | 87% | 81% | 74% | 63% | 53% | 49% |
| 58 | 77% | 74% | 61% | 53% | 47% | 38% |
| 59 | 66% | 59% | 51% | 46% | 38% | 35% |
| 60 | 52% | 45% | 43% | 37% | 33% | 26% |

| Table 27. HAD reliability for GERD + Upper Mendaya scenario (two-dam cascade). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ethiopian Diversion Request (bcm/yr) Actual average diversion (bcm/yr) |  |  |  |  |  |
| Annual HAD release target (bcm/yr) | 0 (0) | 1 (1) | 2 (2) | 3 (3) | 4 (4) | 5 (5) |
| 51 | 99% | 97% | 95% | 93% | 92% | 89% |
| 52 | 98% | 95% | 93% | 92% | 89% | 87% |
| 53 | 96% | 93% | 92% | 89% | 86% | 80% |
| 54 | 94% | 92% | 89% | 86% | 79% | 71% |
| 55 | 91% | 90% | 86% | 80% | 71% | 61% |
| 56 | 87% | 86% | 80% | 72% | 62% | 53% |
| 57 | 82% | 78% | 72% | 61% | 52% | 47% |
| 58 | 71% | 64% | 60% | 51% | 45% | 37% |
| 59 | 59% | 51% | 48% | 44% | 37% | 34% |
| 60 | 43% | 43% | 37% | 33% | 32% | 26% |

| Table 28. HAD reliability for GERD + Upper Mendaya + Beko Abo High scenario (three-dam cascade). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ethiopian Diversion Request (bcm/yr) Actual average diversion (bcm/yr) |  |  |  |  |  |
| Annual HAD release target (bcm/yr) | 0 (0) | 1 (1) | 2 (2) | 3 (3) | 4 (4) | 5 (5) |
| 51 | 99% | 95% | 95% | 92% | 90% | 89% |
| 52 | 98% | 95% | 92% | 90% | 89% | 85% |
| 53 | 97% | 92% | 90% | 89% | 85% | 79% |
| 54 | 93% | 91% | 89% | 84% | 78% | 70% |
| 55 | 92% | 89% | 84% | 77% | 70% | 61% |
| 56 | 90% | 86% | 79% | 69% | 60% | 52% |
| 57 | 88% | 80% | 66% | 57% | 51% | 47% |
| 58 | 76% | 66% | 56% | 50% | 47% | 37% |
| 59 | 64% | 53% | 51% | 44% | 36% | 34% |
| 60 | 47% | 47% | 41% | 36% | 33% | 26% |

| Table 29. HAD reliability for the one-, two- and three-dam scenario with a 55.5 bcm annual release target. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ethiopian Diversion Request (bcm/yr) |  |  |  |  |  |
| Scenario | 0 | 1 | 2 | 3 | 4 | 5 |
| GERD only | 91% | 89% | 84% | 77% | 68% | 58% |
| GERD + Upper Mendaya | 90% | 89% | 83% | 76% | 66% | 58% |
| GERD + Upper Mendaya + Beko Abo High | 91% | 89% | 81% | 73% | 64% | 55% |

## Supplementary note 6: Breakeven analysis

### A) Worked example

A comparison between GERD + two dams in the cascade and GERD + two Dams with 5 bcm upstream irrigation withdrawal.

The annual 5 BCM withdrawn upstream of the cascade causes an annual reduction in the hydropower generation downstream of 6800000000 KWh/year. If we value this lost energy at US$0.10 KWH, the money loss would be:

[6800000000 KWh] [US$0.10 per KWH] = US$680000000

The average opportunity cost of a 1 m3 withdrawal upstream of the cascade in terms of lost hydropower generation would be:

US$680000000/ 5 BCM = US$0.136 per m3

Based on the reduction in outflows at the HAD when 5BCM is withdrawn above the cascade, we can assume that 1 m3 withdrawn upstream of the cascade results in a 0.03 m3 reduction in the reliable target release from the HAD. The breakeven analysis is:

Value of one m3 withdrawn for irrigation upstream of the cascade [US$ x per m3] =

[Value of the lost hydropower generation] + [Value of the reduction in the target release from HAD]

With the example figures, the value of one m3 withdrawn for irrigation upstream of the cascade:

[US$ x per m3] =

[US$0.136 per m3] + 0.03 [Value of a m3 reduction in reliable target release from the HAD]

If the value of water in irrigation Egypt was US$0.05 m3, then the breakeven value of water in irrigation in Ethiopia is:

Value of a m3 withdrawn for irrigation upstream of the cascade in Ethiopia =

[US$0.136 per m3] + 0.03 m3 [US$0.05]

Therefore, the value of one m3 withdrawn for irrigation upstream of the cascade in Ethiopia = US$0.138.

In summary, for irrigation in Ethiopia to overcome the opportunity cost of downstream reductions in hydropower generation and reduced water for irrigation in Egypt, the value of water in Ethiopian irrigation would have to be greater than or equal to US$0.138 per m3.

## References

Basheer, M., Nechifor, V., Calzadilla, A., Siddig, K., Whittington, D., Hulme, D., Harou, J. J.  Collaborative management of the Grand Ethiopian Renaissance Dam increases economic benefits and resilience. Nature Communications. 12: 5622 (2021). https://doi.org/10.1038/s41467-021-25877-w

Basheer, M., Nechifor, V., Calzadilla, A. et al. Cooperative adaptive management of the Nile River with climate and socio-economic uncertainties. Nat. Clim. Chang. (2023). https://doi.org/10.1038/s41558-022-01556-6

Blackmore, D. & Whittington, D. Opportunities for Cooperative Water Resources Development on the Eastern Nile: Risks and Rewards. Final Report of the Scoping Study Team to the Eastern Nile Council of Ministers. October. 85 pages. Washington D.C.: World Bank (2008).

Block, P.J. & Strzepek, K. Economic analysis of large-scale upstream river basin development on the Blue Nile in Ethiopia considering transient conditions, climate variability, and climate change. Journal of Water Resources Planning and Management, 136, 156-166 (2010).

Borgomeo, E., Farmer, C. L., & Hall, J. W. Numerical rivers: A synthetic streamflow generator for water resources vulnerability assessments. Water Resources Research. 51, 5382–5405 (2015). https://doi.org/10.1002/2014WR016827

Eastern Nile Technical Office. Roadmap for Coordinated Operation of Transboundary Cascade Dams in Eastern Nile. Addis Ababa. Nov. 30. 156 pages (2016).

Geressu, R.T. & Harou, J.J. Screening reservoir systems by considering the efficient trade-offs—informing infrastructure investment decisions on the Blue Nile. Environmental Research Letters, 10, 125008 (2015).

Goor, Q., Halleux, C., Mohamed, Y. & Tilmant, A., Optimal operation of a multipurpose multireservoir system in the Eastern Nile River Basin. Hydrology and Earth System Sciences, 14, 1895-1908 (2010).

Helal, M. & and Bekhit, H. Hydropower or Hydro-Hegemony? An Egyptian Perspective on the Negotiations on the Grand Ethiopian Renaissance Dam. Working paper, Ohio State University. On file with the author. (2022).

Jeuland, M. & Whittington, D. Water resources planning under climate change: Assessing the robustness of real options for the Blue Nile. Water Resources Research, 50, 2086-2107 (2014).

Jeuland, M., Wu, X. & Whittington, D. Infrastructure development and the economics of cooperation in the Eastern Nile. Water International. 42:2, 121-141 (2017). https://doi.org/10.1080/02508060.2017.1278577

Mulat, A.G., Moges, S.A. & Moges, M.A. Evaluation of multi-storage hydropower development in the upper Blue Nile River (Ethiopia): regional perspective. Journal of Hydrology: Regional Studies, 16, 1-14 (2018).

Nile Basin Initiative (NBI). Nile Basin decision support system DSS: Modeling tools training module. Entebbe (2014).

Ouarda, T.B.M.J., Labadie, J.W. & Fontane, D.G., Indexed Sequential Hydrologic Modeling for Hydropower Capacity Estimation. Journal of the American Water Resources Association, 33, 1337-1349 (1997). https://doi.org/10.1111/j.1752-1688.1997.tb03557.x

Sadoff, C. W., Whittington, D. & Grey, D. Africa's International Rivers: An Economic Perspective. Directions in Development. Washington, DC: World Bank (2002).

https://openknowledge.worldbank.org/handle/10986/15175

Sridharan, V. et al. Resilience of the Eastern African electricity sector to climate driven changes in hydropower generation. Nature Communications. 10, 302 (2019).  https://doi.org/10.1038/s41467-018-08275-7

Sterl, S., Fadly, D., Liersch, S., Koch, H. & Thiery, W. Linking solar and wind power in eastern Africa with operation of the Grand Ethiopian Renaissance Dam. Nature Energy, 6, 407-418 (2021). https://doi.org/10.1038/s41560-021-00799-5

Sudan MoIWR. Gage Flows at Dongola Station.  Sudan Ministry of Irrigation and Water Resources (2019).

UNSC Letter S/2020/566 19 June 2020

United States Bureau of Reclamation. Land and Water Resources of the Blue Nile Basin: Ethiopia, Main Report and Appendices I-V. US Department of Interior, Government Printing Office, Washington D.C. (1964).

van der Krogt, W. & Ogink, H. Development of the Eastern Nile Water Simulation Model. Report No. 1206020-000-VEB-0010, Deltares, Delft (2013).

Wheeler, K.G., Basheer, M., Mekonnen, Z.T., Eltoum, S.O., Mersha, A., Abdo, G.M., Zagona, E.A., Hall, J.W. & Dadson, S.J. Cooperative filling approaches for the Grand Ethiopian Renaissance Dam. Water International, 41, 611-634 (2016). https://doi.org/10.1080/02508060.2016.1177698

Wheeler, K.G., Hall, J.W., Abdo, G.M., Dadson, S.J., Kasprzyk, J.R., Smith, R. & Zagona, E.A., Exploring Cooperative Transboundary River Management Strategies for the Eastern Nile Basin. Water Resources Research, 54, 9224-9254 (2018).

Wheeler, K.G., Jeuland, M., Hall, J.W., Zagona, E. & Whittington, D. Understanding and managing new risks on the Nile with the Grand Ethiopian Renaissance Dam. Nature Communications, 11, 5222 (2020). https://doi.org/10.1038/s41467-020-19089-x

Wheeler, K., Jeuland, M., Strzepek, K., Hall, J., Zagona, E., Abdo, G., Basson, T., Blackmore, D., Block, P. & Whittington, D., Comment on ‘Egypt’s water budget deficit and suggested mitigation policies for the Grand Ethiopian Renaissance Dam filling scenarios’. Environmental Research Letters, 17, 088003 (2022).

Whittington, D., Waterbury, J. & Jeuland, M. The Grand Renaissance Dam and Prospects for Cooperation on the Eastern Nile. Water Policy, 16, 595–608 (2014). https://doi.org/10.2166/wp.2014.011b

Whittington, D., Wu, X. & Sadoff, C. W. Water Resources Management in the Nile Basin: The Economic Value of Cooperation. Water Policy 7, 227-252 (2005). https://doi.org/10.2166/wp.2005.0015

Whittington, D. Contested Baselines and Transboundary Natural Resources Management, with Illustrations from the Nile Basin. Water International, 47: 6, 934-951 (2022). https://doi.org/10.1080/02508060.2022.2123611

The Border dam in the USBR report was near the current location of the GERD, but it was envisaged to be much smaller than the GERD. The majority of storage proposed in the USBR plan was in the three large dams upstream (Mendaya, Mabil, and Karadobi).

This calculation also assumed that Ethiopia’s irrigation withdrawals increased 3 bcm (from 2.3 to 5.3 bcm). If these irrigation withdrawals had not been included in their “full Blue Nile development” simulation, system-wide hydropower generation would have been even higher.