- Supplementary Materials for:

Setback distance between coastal cities and wetlands to protect waterbirds

Rong-Bo Luo and Hong-Gang Ni*

School of Urban Planning and Design, Peking University Shenzhen Graduate School, Shenzhen 518055, China

# Corresponding Author

Phone: +86-755-26033017. Fax: +86-755-26033254. E-mail: nihg@pkusz.edu.cn

- Calculation of setback distance

Studies have shown that tidal flats with vegetation cover can purify sewage 19133.1 t/(hm2*a). Coastal wetland area S (classified area of various types) of 48 coastal cities in China (hm2) is derived from the second Wetland Survey Report of China (2013). The annual sewage discharge data X (10000 t) of 48 coastal cities are obtained from the statistical yearbooks of provinces and cities in 2013, as shown in Table S3. The Monte Carlo simulation method is used to predict the setback distance with Eq. (S1):

X*Cdischarge*(1-3.46%)d=(WEC+Lplant)*Swater+196133.3*CV*Stidal flat  (S1)

where X is the annual sewage discharge data of 48 coastal cities; Cdischarge is the national sewage plant discharge standard (TN concentration is 15 mg/L); CV is the Grade V Water Quality Standard of China (TN concentration is 2 mg/L); Swater and Stidal flat are the area of water body and the area of tidal flat wetland of 48 coastal cities (hm2), respectively; 3.46% indicates that TN presents an average decrease of 3.46% for every 1000 m of sewage flowing in the pipelines (Ren, 2015). Therefore, the setback distance can be calculated using Eq. (S2):

d=LN(WEC+Lplant)*Swater+196133.3*CV*Stidal flat-LN(X*Cdischarge)LN(1-3.46%)  (S2)

Crystal Ball was used to conduct the Monte Carlo simulation, and the obtained results are not consistent with the actual observations (Fig. 2). The reasons are provided below:

(1) The statistical calibration of the area data for coastal wetlands in 48 coastal cities was performed on a large scale, and the water depth and water area data cannot be confirmed;

(2) The statistical methods applied for the sewage discharge data obtained from the statistical yearbook differ, and the discharge standard may not be completely in accordance with the national standard, which may have led to large deviations in the data;

(3) The influence of rainfall, river transportation and other factors was not considered, and the sewage discharged by coastal cities was not completely purified by coastal wetlands.

Therefore, based on the improvement of the above research methods, the calculation equation and method in the next section can be obtained by improving Eq. (S2) The pollutant treatment proportion of coastal wetlands to the pollution discharge of urban sewage was considered. In addition, the content of other typical pollutants in sewage, such as the chemical oxygen demand (COD), biochemical oxygen demand (BOD) and suspended solids (SS), can be considered to ensure that the calculated setback distance of coastal constructions is more comprehensive.

Table S1 Range of noise produced by different human activities.

| Type | Activity | Secondary classification | Noise (decibel) |  |
| --- | --- | --- | --- | --- |
|  |  |  | min | max |
| Traffic | Walk |  | 20 | 65 |
|  | Run |  | 20 | 65 |
|  | Bike |  | 20 | 65 |
|  | Electric bicycle |  |  | 62 |
|  | Boat | Diesel engine | 100 | 100 |
|  |  | Pedal boat | 20 | 65 |
|  |  | Electric boat | 50 | 75 |
|  |  | Rowboat | 20 | 65 |
|  | Car |  | 80 | 100 |
| Explore & Experience | Angling |  | 10 | 20 |
|  | Exhibitions |  | 20 | 65 |
|  | Catching fish (shrimp & crab) |  | 20 | 65 |
|  | Aquaculture |  | 20 | 65 |
| Leisure | Setting up exercise |  | 20 | 65 |
|  | Singing |  | 60 | 70 |
|  | Square dancing | Megaphone | 70 | 90 |
|  | Dragon boat race | Noise during the race | 70 | 110 |
|  |  | Cheers from the audience | 120 | 120 |
|  | Catering |  | 20 | 65 |
|  | Shopping |  | 70 | 80 |
|  | Light exhibition |  | 20 | 65 |
|  | Children playing loudly |  | 90 | 90 |
| Management & maintenance | Fitness |  | 70 | 80 |
|  | Construction |  | 70 | 80 |
|  | Construction site |  | 80 | 110 |

Table S2. Simulation results for the setback distance (km) assume that the proportion (W/C) is a normal distribution of 0%-100% and 60% - 100%. Mode Value is the value that occurs most frequently in the probability distribution (0%-100%); and Min and Max mean the minimum and maximum in the probability distribution (60%-100%).

| Pollutant Index | 0% - 100% | 60% - 100% |  |  |
| --- | --- | --- | --- | --- |
|  | Mode Value |  | Min | Max |
| COD | 3.06 |  | 0.35 | 18.1 |
| BOD | 0.19 |  | 0.02 | 1.13 |
| SS | 0.47 |  | 0.05 | 2.83 |
| TN | 2.52 |  | 0.29 | 14.9 |

Table S3. Coastal wetland area S (classified area of various types) of 48 coastal cities (hm2) and annual sewage discharge data X (10000 t) of 48 coastal cities (2013).

| Province | Coastal city | Swater (including estuarine waters and coastal salt water lake) (hm2) | Stidal flat (including muddy beach, intertidal brine marshes and delta) (hm2) | X (annual sewage discharge) (10000 t) |
| --- | --- | --- | --- | --- |
| Liaoning | Dalian | 9108 | 4129 | 57494 |
|  | Dandong | 26202 |  | 11010 |
|  | Jinzhou | 12042 | 863 | 16222 |
|  | Yingkou | 894 | 958 | 10339 |
|  | Panjin | 36313 | 16240 | 9706 |
|  | Huludao | 1720 | 455 | 8872 |
| Hebei | Tangshan | 8914 | 940 | 51008 |
|  | Qinhuangdao | 872 | 202 | 17693 |
|  | Cangzhou |  |  | 12585 |
| Shandong | Qingdao | 15041 | 9120 | 51557 |
|  | Dongying | 74651 | 5090 | 20695 |
|  | Yantai | 5482 | 2610 | 32667 |
|  | Weifang | 20241 | 8237 | 54107 |
|  | Weihai | 4821 | 5628 | 11474 |
|  | Rizhao | 1580 | 368 | 15557 |
|  | Binzhou | 13306 | 2669 | 27315 |
|  | Tianjin | 11474 | 1681 | 84210 |
| Jiangsu | Lianyungang | 19027 | 6062 | 22287 |
|  | Nantong | 165886 | 73331 | 44478 |
|  | Yancheng | 308786 | 7650 | 45270 |
|  | Shanghai | 74848 | 308483 | 222636 |
| Zhejianga (including Ningbo, Wenzhou, Jiaxing, Taizhou) |  | 175165 | 102581 | 182058 |
| Fujian | Fuzhou | 40337 | 35505 | 37010 |
|  | Xiamen | 8578 | 834 | 50212 |
|  | Putian | 15574 | 1463 | 11603 |
|  | Quanzhou | 12268 | 5998 | 55625 |
|  | Zhangzhou | 18281 | 11156 | 42290 |
|  | Ningde | 37314 | 5608 | 10204 |
| Guangdong | Zhanjiang | 25718 | 11610 | 24600 |
|  | Zhuhai | 9337 | 85359 | 23500 |
|  | Jiangmen | 6779 | 4464 | 38100 |
|  | Yangjiang | 6440 | 1554 | 9900 |
|  | Guangzhou | 671 | 24129 | 156700 |
|  | Maoming | 977 | 3238 | 20000 |
|  | Shanwei | 7359 | 9865 | 12700 |
|  | Huizhou | 637 | 2912 | 39600 |
|  | Zhongshan | 217 | 15935 | 40600 |
|  | Shantou | 3112 | 10316 | 25600 |
|  | Shenzhen | 426 | 31940 | 155400 |
|  | Chaozhou | 1101 | 2831 | 13000 |
|  | Dongguan | 80 | 6456 | 103700 |
|  | Jieyang | 558 | 744 | 24700 |
| Guangxi | Beihai | 12633 | 5009 | 7380 |
|  | Fangchenggang | 4407 | 4326 | 7902 |
|  | Qinzhou | 7097 | 6294 | 9768 |

a Because the statistical data of Zhejiang Province are only obtained at the province scale, this part will include the cities overall.

Fig. S1. Distribution of bird FID and MAD data.

Fig. S2. Histogram showing the distribution (statistical analysis) of setback distances (m) calculated by noise.

Reference：

Ren W.A. The study of migration characteristics and transforming discipline of Nitrogen compounds in the transportation and treatment process of urban wastewater (in Chinese) Municipal Engineerin. Ph.D. Xi’an University of Architecture and Technology, Xian, 2015, pp. 123.