Supporting Information

# Measurement and modelling of speed of sound in binary mixtures of methyl oleate with n-hexane or n-decane under pressure

Jean-Luc Daridon*, Jean-Patrick Bazile, Djamel Nasri

Universite de Pau et des Pays de l’Adour, E2S UPPA, CNRS, TotalEnergies, LFCR UMR5150, Pau, France.

*Corresponding author : jean-luc.daridon@univ-pau.fr

The density (ρ) of pure n-hexane and n-decane was measured as a function of pressure and temperature using a vibrating U-tube densimeter (Anton Paar HPM), following the same method described in a previous study [1]. The obtained values were then used to determine the isothermal compressibility (κT ) of the pure liquids with respect to pressure. For this purpose, a curve-fitting method [2] was employed, which involved using three different equations of state [3-5] to represent the change in liquid density as a function of pressure. Finally, to obtain the isobaric expansion αp,  the density was derived with respect to temperature using a curve linearization method [6].

Only the results for temperature T = 303.15 K were reported in Tables S1 to S3, which list the values for density, isothermal compressibility, and isobaric expansion, respectively. The tables also include the values for pure methyl oleate, which were obtained from a previous study [1].

# Table S1

Density   and its expended uncertainty U (level of confidence = 0.95, k = 2) for pure hexane (n-C6), decane (n-C10) and methyl oleate (MC18:1) at temperature T=303.15 K and pressures p

| T/K | P/MPa | n-C6±U / kg.m-3 |  | n-C10±U / kg.m-3 |  | MC18:1±U / kg.m-3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 303.15 | 0.1 | 650.5 | 0.7 | 722.7 | 0.6 | 867.3 | 0.6 |
| 303.15 | 10 | 661.1 | 0.7 | 730.3 | 0.6 | 873.6 | 0.6 |
| 303.15 | 20 | 670.3 | 0.7 | 737.4 | 0.6 | 879.2 | 0.6 |
| 303.15 | 30 | 678.6 | 0.7 | 743.8 | 0.6 | 884.5 | 0.6 |
| 303.15 | 40 | 685.7 | 0.7 | 749.8 | 0.6 | 889.5 | 0.6 |
| 303.15 | 50 | 692.4 | 0.7 | 755.3 | 0.6 | 894.3 | 0.6 |
| 303.15 | 60 | 698.6 | 0.7 | 760.6 | 0.6 | 898.8 | 0.6 |
| 303.15 | 70 | 704.5 | 0.7 | 765.5 | 0.6 | 903.1 | 0.6 |

a Standard uncertainties u are uT=0.05 K, up=0.01 MPa.

Table S2

Isothermal compressibility κT   and its expended uncertainty UκT level of confidence = 0.95, k = 2) for pure hexane (n-C6), decane (n-C10) and methyl oleate (MC18:1) at temperature T=303.15 K and pressures p

| T/K | P/MPa | κT,n-C6±UκT / GPa-1 |  | κT,n-C10±UκT / GPa-1 |  | κT,MC18:1±UκT / GPa-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 303.15 | 0.1 | 1.770 | 0.074 | 1.124 | 0.054 | 0.734 | 0.027 |
| 303.15 | 10 | 1.491 | 0.034 | 1.006 | 0.029 | 0.677 | 0.015 |
| 303.15 | 20 | 1.290 | 0.016 | 0.911 | 0.015 | 0.628 | 0.008 |
| 303.15 | 30 | 1.138 | 0.012 | 0.833 | 0.010 | 0.585 | 0.006 |
| 303.15 | 40 | 1.020 | 0.014 | 0.769 | 0.012 | 0.549 | 0.009 |
| 303.15 | 50 | 0.926 | 0.018 | 0.714 | 0.017 | 0.517 | 0.012 |
| 303.15 | 60 | 0.848 | 0.021 | 0.666 | 0.020 | 0.489 | 0.015 |
| 303.15 | 70 | 0.782 | 0.024 | 0.625 | 0.023 | 0.464 | 0.017 |

# Table S3

Isobaric thermal expansion αP and its expended uncertainty UαP (level of confidence = 0.95, k = 2) for pure hexane (n-C6), decane (n-C10) and methyl oleate (MC18:1) at temperature T=303.15 K and pressures p

| T/K | P/MPa | αP,n-C6±UαP / 10-3 K-1 |  | αP,n-C10±UαP / 10-3 K-1 |  | αP,MC18:1±UαP / 10-3 K-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 303.15 | 0.1 | 1.416 | 0.019 | 1.039 | 0.011 | 0.855 | 0.010 |
| 303.15 | 10 | 1.267 | 0.018 | 0.961 | 0.011 | 0.808 | 0.011 |
| 303.15 | 20 | 1.164 | 0.018 | 0.900 | 0.011 | 0.774 | 0.011 |
| 303.15 | 30 | 1.076 | 0.017 | 0.852 | 0.011 | 0.743 | 0.011 |
| 303.15 | 40 | 1.017 | 0.017 | 0.815 | 0.011 | 0.715 | 0.011 |
| 303.15 | 50 | 0.971 | 0.017 | 0.782 | 0.011 | 0.692 | 0.011 |
| 303.15 | 60 | 0.909 | 0.017 | 0.750 | 0.011 | 0.675 | 0.010 |
| 303.15 | 70 | 0.872 | 0.017 | 0.719 | 0.016 | 0.656 | 0.010 |

References

- J.L. Daridon, J.P. Bazile, D. Nasri, Int. J. Thermophys, 43, 23 (2022)

- J.L. Daridon, J.P. Bazile, J. Chem. Eng. Data 63, 2162 (2018)

- L.J. Hudleston, Trans. Faraday Soc. 33, 97 (1937)

- F.D. Murnaghan, Proc Natl Acad Sci U S A. 30, 244 (1944)

- G. Tammann, Z. Phys. Chem. 1895, 17, 620-636.

- J.L. Daridon, J.P. Bazile, J. Chem. Eng. Data 66, 3961 (2021)