Supplementary Table 1. The Box–Behnken experimental design with three independent variables.

| Design points | Factor levels |  |  |  |  |  | Response |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Run | A: % H2CO2 solution |  | B: Flow rate (mL min-1) |  | C: Column oven temperature (°C) |  | Rs1,2 |  |
|  | A | Coded | B | Coded | C | Coded | Actual | Predicted |
| 1 | 0.3 | +1 | 0.4 | -1 | 35 | 0 | -0.2131 | -0.2128 |
| 2 | 0.15a | 0 | 0.5 | 0 | 35 | 0 | 0.3240 | 0.3216 |
| 3 | 0 | -1 | 0.6 | +1 | 35 | 0 | 0.9650 | 0.9647 |
| 4 | 0 | -1 | 0.4 | -1 | 35 | 0 | 1.15 | 1.15 |
| 5 | 0.15a | 0 | 0.5 | 0 | 35 | 0 | 0.3260 | 0.3216 |
| 6 | 0 | -1 | 0.5 | 0 | 30 | -1 | 0.9780 | 0.9805 |
| 7 | 0.3 | +1 | 0.5 | 0 | 30 | -1 | -0.2600 | -0.2627 |
| 8 | 0.15 | 0 | 0.4 | -1 | 30 | -1 | 0.3340 | 0.3364 |
| 9 | 0.15 | 0 | 0.6 | +1 | 40 | +1 | 0.2694 | 0.2670 |
| 10 | 0.3 | +1 | 0.6 | +1 | 35 | 0 | -0.1124 | -0.1076 |
| 11 | 0.15 | 0 | 0.6 | +1 | 30 | -1 | 0.3156 | 0.3134 |
| 12 | 0.15a | 0 | 0.5 | 0 | 35 | 0 | 0.3230 | 0.3216 |
| 13 | 0.15 | 0 | 0.5 | 0 | 35 | 0 | 0.3230 | 0.3216 |
| 14 | 0 | -1 | 0.5 | 0 | 40 | +1 | 0.9180 | 0.9207 |
| 15 | 0.15 | 0 | 0.4 | -1 | 40 | +1 | 0.3170 | 0.3192 |
| 16 | 0.3 | +1 | 0.5 | 0 | 40 | +1 | -0.2640 | -0.2665 |
| 17 | 0.15 | 0 | 0.5 | 0 | 35 | 0 | 0.3120 | 0.3126 |

a The center point was replicated three times to estimate the experimental errors Rs1,2 (PT,4-HIL).

Supplementary Table 2. Statistical parameters obtained from ANOVA.

| Parameters | Statistics |
| --- | --- |
| Response | Rs1,2 = +0.3216 -0.6076 x A -0.0188 x B -0.0159 x C +0.0714 x AB + 0.0140 AC – 0.0073 x BC + 0.0799 A2 + 0.0459 B2 – 0.0585 C2 |
| Adjusted R2 | 0.9999 |
| P value | < 0.0001 |
| % C.V. | 1.57 |
| Adequate precision | 331.5825 |

A: % H2CO2 Solution, B: Flow rate (mL min-1), C: Column oven temperature (ºC).

Supplementary Table 3. Precision and accuracy data for PT, 4-HIL and RFO were obtained by the developed method.

| Compound | Precision and accuracy of data | Nominal concentrations (mg mL-1) (n=5) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0.9375 (LQC) | 1.875 (MQC) | 3.75 (HQC) |
| PT | Mean | 0.91 | 1.87 | 3.91 |
|  | S.D. | 0.04 | 0.09 | 0.17 |
|  | %R.S.D (± %) | 4.63 | 5.04 | 4.37 |
|  | %Bias | 3.26 | 0.45 | -4.32 |
| 4-HIL | Mean | 0.87 | 1.80 | 3.92 |
|  | S.D. | 0.06 | 0.08 | 0.10 |
|  | R.S.D (± %) | 6.91 | 4.32 | 2.44 |
|  | %Bias | 7.65 | 4.06 | -4.66 |
| RFO | Mean | 0.90 | 1.85 | 3.92 |
|  | S.D. | 0.05 | 0.07 | 0.16 |
|  | R.S.D (± %) | 5.34 | 3.56 | 4.04 |
|  | %Bias | 4.09 | 1.10 | -4.60 |

Supplementary Table 4. Results of the recovery analysis of PT, 4-HIL and RFO.

| Compound | Wt. spiked (mg) | Wt. recovered (mg) | Recovery (%) | RSD (± %) (n=3) |
| --- | --- | --- | --- | --- |
| PT | 1.875 | 1.90 ± 0.03 | 101.27 | 1.50 |
| 4-HIL | 1.875 | 1.92 ± 0.06 | 102.20 | 3.02 |
| RFO | 1.875 | 1.84 ± 0.05 | 98.33 | 2.83 |

Supplementary Table 5. Robustness parameter of PT, 4-HIL and RFO.

| Parameter | Level | Amount of added (mg) |  |  | Amount of detected (mg) (mean± SD) |  |  | RSD (± %) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PT | 4-HIL | RFO | PT | 4-HIL | RFO | PT | 4-HIL | RFO |
| Flow Rate | 0.4 mL min-1 | 1.875 | 1.875 | 1.875 | 1.88 ± 0.03 | 1.84 ± 0.02 | 1.88 ± 0.10 | 1.80 | 1.18 | 5.06 |
|  | 0.5 mL min-1 | 1.875 | 1.875 | 1.875 | 1.87 ± 0.04 | 1.85 ± 0.14 | 1.88 ± 0.12 | 2.23 | 7.49 | 5.50 |
|  | 0.6 mL min-1 | 1.875 | 1.875 | 1.875 | 1.89 ± 0.02 | 1.86 ± 0.08 | 1.90 ± 0.04 | 0.97 | 4.17 | 2.12 |
| Column oven temp. | 30 °C | 1.875 | 1.875 | 1.875 | 1.91 ± 0.04 | 1.85 ± 0.06 | 1.89 ± 0.09 | 1.90 | 3.01 | 5.02 |
|  | 35 °C | 1.875 | 1.875 | 1.875 | 1.90 ± 0.06 | 1.77 ± 0.17 | 1.91 ± 0.03 | 3.18 | 9.67 | 1.78 |
|  | 40 °C | 1.875 | 1.875 | 1.875 | 1.87 ± 0.01 | 1.82 ± 0.05 | 1.83 ± 0.16 | 0.49 | 2.74 | 8.55 |
| pH | 0 | 1.875 | 1.875 | 1.875 | 1.86 ± 0.05 | 1.78 ± 0.17 | 1.81 ± 0.11 | 0.05 | 9.28 | 6.08 |
|  | 0.15 | 1.875 | 1.875 | 1.875 | 1.90 ± 0.14 | 1.94 ± 0.06 | 1.93 ± 0.05 | 7.29 | 2.87 | 2.39 |
|  | 0.3 | 1.875 | 1.875 | 1.875 | 1.88 ± 0.05 | 1.93 ± 0.05 | 1.64 ± 0 .03 | 2.88 | 2.39 | 1.58 |