Supplementary Materials

Extraction and Proteolysis of Sludge Protein Using Ultrasound Synergistic Enzymatic Hydrolysis

Fan Liua,Yixin Yana,*, Jianlei Gaoa, Lei Qinb, Yajing Zhanga, Junfeng Wana, c

a School of Ecology and Environment, Zhengzhou University, Zhengzhou, 450001, Henan, China

b Central Plains Environment Protection Co., LTD., Zhengzhou, 450001, Henan, China

c Henan International Joint Laboratory of Environment and Resources, Zhengzhou University, Zhengzhou 450001, PR China

Calculations S1 Protein extraction efficiency

RP(%)=C1V1C0V0×100%                   (1)

where C1 and C0 are the protein contents (mg/mL) in the supernatant and raw sludge, respectively, and V1 and V0 are the liquid volumes of the supernatant and sludge sample (mL), respectively.

Calculations S2 EPS influence index

The EPS influence index (EI) was calculated according to the protein contents after separate hydrolysis and integrated hydrolysis. The EI index reflects the influence of the EPS barrier effect on different hydrolysis methods. A greater index indicates a more significant barrier effect of EPS on the hydrolysis method.

(1) Integral hydrolysis protein content (Pi): Protein content obtained through the direct hydrolysis of excess sludge.

(2) Separate hydrolysis protein content (Ps):

Ps=Psi+Pse  (2)

where Psi and Pse are the intracellular protein content and the extracellular protein content (mg/mL), respectively.

(3) Intracellular protein content obtained through integral hydrolysis (Pii): Assuming that all EPS proteins were effectively extracted using integral hydrolysis, the remaining proteins could be regarded as intracellular proteins obtained during the integral hydrolysis of sludge.

Pii=Pi−Pse  (3)

where Pii, Pi and Pse are the intracellular protein content obtained from integral hydrolysis, the integral hydrolysable protein content and the extracellular protein content (mg/mL), respectively.

(4) EI: When the sludge cells remaining after the removal of EPS were hydrolysed using a certain method, the amount of intracellular protein (Psi) extracted was assumed to be the maximum amount of intracellular protein extracted using this hydrolysis method. The intracellular protein (Pii) extracted from the integral sludge was hydrolysed using the same method. The ratio of Psi to Pii is the EI, as shown in formula (4).

EI=PsiPii  (4)