ONLINE METHODS

Patient sampling

Biological samples to be tested by RT-PCR were obtained and prepared as follows. Nasopharyngeal and vaginal swabs were obtained following US Center for Disease Control and Prevention guidelines.1,2 Placenta was crushed in 400 mL of RNAase-DNAase-free water; 1 mL of blood and swabs were placed in Virocult® viral transport media (Sigma, St. Louis, MI – USA). Non-bronchoscopic bronchoalveolar lavage (BAL) was performed using a standardized procedure:3 the neonate was placed supine with the head turned to the right so that the left lung would be predominantly sampled. Normal saline (1 mL/Kg, 37°C) was instilled into the endotracheal tube through a Y-piece. After three ventilator cycles, the suction catheter was gently inserted 0.5 cm beyond the tube tip, and the airway fluid was aspirated into a sterile specimen trap (BALF Trap; Vigon, Ecouen, France) with 50 mmHg of negative pressure. This procedure was repeated with the head turned to the left, so that the right lung would be predominantly sampled. This procedure respects European Respiratory Society advices for pediatric and neonatal BAL.4 During the procedure, the patient was never disconnected from the ventilator, the inspired oxygen fraction was 0.25 and there was no desaturation or bradycardia. All specimens were kept at +4°C and tested within 24 hours.

Real-time polymerase chain reaction (RT-PCR)

Viral RNA was extracted from 200 µL clinical samples with the NucliSENS® easyMag® (BioMérieux, Craponne, France) and eluted in 100 µL. The RealStar® SARS-CoV-2 RT-PCR Kit 1.0 (Altona Diagnostics GmbH, Hamburg Germany) targeting the E gene (specific for lineage B-betacoronavirus) and the S gene (specific for SARS-CoV-2) was used following the manufacturer's recommendations. The assay includes a heterologous amplification system (internal positive control) to identify possible RT-PCR inhibition and to confirm the integrity of the reagents of the kit. Thermal cycling was performed at 55 °C for 20 min for reverse transcription, followed by 95 °C for 2 min and then 45 cycles of 95 °C for 15 s, 55 °C for 45s, 72 °C for 15s with an Applied Biosystems ViiA7 instrument (Applied Biosystems, Thermo Fisher, Waltham – MA, USA). A cycle threshold value less than 40 is interpreted as positive for SARS-CoV-2 RNA.

Placental examination

Placental sampling, gross and microscopic examination were performed according to the Amsterdam consensus statement.5 The placenta was fixed in 4% buffered formalin and samples were paraffin embedded. Staining methods performed on 3-5µm thick sections were : haemalun eosin saffran, periodic acid schiff and Gomori-Grocott stains. Immunohistochemistry with peroxydase detection and hemalun counterstain was performed in a Leica Bond III automat using the Bond Polymer Refine Detection kit (Leica DS9800) after heat pretreatment at pH6 or 9 depending on the monoclonal antibodies tested : CD68 (Dako PG-M1, 1:200), CD163 (Leica 10D6, 1:200), CD20 (Dako L26, 1:400), CD3 (Dako F7.2.38, 1:50), CD5 (Novocastra 4C7, 1:50), CMV (Dako CCH2+DDG9, 1:1), Parvo virus (AbcVs, Abc10-P038).

ONLINE METHODS REFERENCES

1.	https://www.cdc.gov/coronavirus/2019-ncov/hcp/inpatient-obstetric-healthcare-guidance.html.

2.	https://www.cdc.gov/groupbstrep/downloads/gbs_swab_sheet21.pdf.

3.	De Luca, D. et al. Role of distinct phospholipases A2 and their modulators in meconium aspiration syndrome in human neonates. Intensive Care Med 37, 1158–1165 (2011).

4.	De Blic, J. et al. Bronchoalveolar lavage in children. ERS Task Force on bronchoalveolar lavage in children. European Respiratory Society. Eur Resp J 15, 217–231 (2000).

5. Khong, T.Y. et al. Sampling and definition of placental lesions. Amsterdam placental workshop group consensus statement. Arch Pathol Lab Med 140, 698-713 (2016).