Additional file 1 of “Automated radiosynthesis and in vivo evaluation of [18F]ADPM06 as a photosensitizer for photodynamic therapy”

Supplemtary information

Fig. S1

Fig. S1. Typical semi-preparative HPLC choromatogram of the [18F]ADPM06 fraction using a radio-HPLC. The semi-preparative HPLC conditions were as follows: InertSustainSwift C18 column (5 μm, 10 mm i.d. × 250 mm length), acetonitrile/0.1% formic acid aqueous solution (95:5, vol./vol.) as the mobile phase, a flow rate of 5 mL/min, UV-VIS detection at 600 nm, and NaI(Tl) scintilattion detection.

Fig. S2

Fig. S2. Typical analytical HPLC choromatogram of the [18F]ADPM06 injection using a radio-HPLC. The analytical HPLC conditions were as follows: CAPCELL PAK C18 ACR column (3 μm, 4.6 mm i.d. × 100 mm length), acetonitrile and 0.1% formic acid aqueous solution (90:10, vol./vol.) as the mobile phase, a flow rate of 1 mL/min, and UV-VIS detection at 650 nm.

Fig. S3

Fig. S2. Typical radio-HPLC chromatogram for metabolite analysis of plasma in BALB/c-nu/nu mice at 30 min after the [18F]ADPM06 injection. The HPLC condition were as follows: XSelect CSH C18 column (5 μm, 10 mm i.d. × 100 mm), a mixture of acetonitrile and 0.1% formic acid solution (90:10, vol./vol.) as a mobile phase, 4.0 mL/min flow rate, and bismuth germanate scintilation detection.