Appendix

Common Background

Regardless of type, each sensor has a coordinate frame rigidly attached to the sensor, denoted as frame A and frame FA for the arm and forearm sensors, respectively, that defines its 3D orientation. There are numerous ways to parameterize 3D orientations, such as the roll-pitch-yaw Euler angles, and many of these parameterizations, their operations and conversions, and their advantages and disadvantages can be found in reference [51]. We utilized several different parameterizations, starting with the quaternion. The quaternion is defined as the following unit length 4D vector

qba=q0,q1,q2,q3T=cosα2,nxsinα2,nysinα2,nzsinα2T #1

where qba describes the rotation from frame a to frame b, n=nx,ny,nzT is the axis of rotation as a unit vector, and α is the scalar amount of rotation about n. Since we placed a sensor on the arm and forearm, we estimated the orientation of each limb by computing the quaternions qAw and qFAw, which describe the arm and forearm sensor frames with respect to the world frame w (origin at the shoulder), respectively. We then computed the relative orientation between the arm and forearm sensors, qFAA, using

qFAA=qwA∘qFAw=qAw*∘qFAw #2

where ∘ represents quaternion multiplication, and * represents the quaternion conjugate.

We then used these quantities, as well as the measured limb lengths, to construct a series of 4x4 homogenous transformation matrices that modeled the UE as a serial kinematic chain model. We multiplied these transformation matrices to compute the net transformation matrix, Twriw, which contains the position vector from the shoulder to the wrist expressed in the world frame, denoted as pwriw. Obtaining qAw and qFAw and creating the kinematic model differed between methods and are discussed below.

IMU-Based Tracking

To estimate an IMU sensor orientation qsw, we fused the accelerometer, magnetometer, and gyroscope measurements after calibrating the raw accelerometer and magnetometer measurements. Let ωs=ωx, ωy,ωzTbe the gyroscopic angular velocity expressed in a sensor frame, either the arm or the forearm. The quaternion rate is computed by the following expression:

qsw=12qsw∘0ωs #3

This quantity can then be numerically integrated to yield qsw using

qswk+1=qswk+qswkTs #4

where Ts is the sampling period. This computation yields an accurate and smooth short-term estimation of qsw.  However, this estimation deviates considerably from the true value of qsw after a short period of time due to integration drift, making it unsuitable for elbow angle tracking beyond a few seconds. Solutions to Wahba’s problem[52] show that vector observations of at least two reference vectors are required to fully determine the 3D orientation of body[53,54].  The accelerometer measurements as=ax, ay,azTof the gravity vector and the magnetometer measurements ms=mx, my,mzTof the Earth’s magnetic field can serve as these vector observations. Since gravity and the magnetic field vectors remain constant in the North-East-Down world frame, the orientation can be determined without use of integration. This yields a stable, drift-free estimation of qsw. However, this estimation is noisy and becomes inaccurate as soon as measurement distortions are present, such as large non-gravity accelerations or nearby magnetic disturbances.

Fusing the accelerometer, gyroscope, and magnetometer measurements is therefore critical to obtain an accurate and stable estimate of qsw. To do so, we implemented the Improved Explicit Complementary Filter described in[28] with slight modifications from[24]. Our implementation of this filter can be summarized by the following equations:

qswk+1=qswk+qswCF, kTs #5

qswCF=12qsw∘0ωs-ωbias+Kaqea+Kmqem #6

ωbias,  k+1=ωbias,  k+Kaωea+Kmωem #7

ea=asas×Rwsg #8

em=as×msas×ms×Rwsg×mwg×mw  #9

Here, qswCF, k is the corrected quaternion rate at sample k; ωbias is the gyroscope bias estimation; Kaq, Kmq, Kaω, and Kmω are gains; ea and em are the accelerometer and magnetometer errors, respectively; Rws is the rotation matrix representation of qws (i.e. the quaternion conjugate of qsw); g is the gravity vector in the world frame (i.e. 0, 0, gT); and mw is the Earth’s magnetic field expressed in the world frame, determined using the World Magnetic Model[55].  We generally set the gains to be as low as possible to maximize the gyroscope’s influence on the estimation of qsw without observing drift, instability (e.g. oscillations), or nonconvergence, as the gyroscope is much less sensitive to distortions than the accelerometer and magnetometer[24]. In general, lower gains require more time for initial convergence. Additionally, we fed the initial magnetometer and accelerometer measurements into the Optimal Quaternion Estimation algorithm[54] to obtain a close initial quaternion for faster convergence. This allowed for lower gain selection, which helped minimize effects of accelerometer and magnetic distortion when necessary.

When we initially placed the sensors on the limbs, there was a static baseline orientation difference between the two sensors. We removed this baseline orientation to ensure that only FE and PS about the elbow joint contributed to differences in the relative orientation between the two sensors[24]. Before performing any motion, the initial estimations of the relative orientation, (qFAA)initial, equaled the baseline orientation difference. Thus, the quaternion from the world frame to the arm sensor frame after baseline orientation removal, qAiw, is defined as

qAiw=qAw∘(qFAA)initial #10

where frame Ai represents a virtual, intermediate arm frame that initially has the same orientation as frame FA during baseline orientation removal. Note that this operation removes any initial orientation difference between the two sensors. With Equation (10), we computed subsequent relative orientations qFAAi, representing only motion about the elbow joint, by modifying Equation (2) to the following:

qFAAi=qwAi∘qFAw=qAiw*∘qFAw #11

After obtaining qFAAi, the next step was to determine the FE axis of rotation j in the sensor frames. If calibration motion to determine jFA was performed at the same PS angle as when baseline orientation was removed, then j=jAi=jFA. We determined j using the method described in [31]. This method utilizes the fact that the direction of ωs is parallel to j during pure FE motion of the elbow. Thus, the following statements are always true during pure FE motion for any IMU sensor:

ωs×j=0 #12

dωs×jdj=ωs×j×ωsωs×j=0 #13

Given a data set of pure FE motion, j can be solved for by minimizing Equation (12) using the Gauss-Newton algorithm. Once we obtained j, we converted qFAAi  to the equivalent vFAAi, where vFAAi is the 3D rotation vector parameterization of the relative orientation[51]. With vFAAi and j, the rotation along the FE axis to get from frame Ai to frame FA is

dθ=vFAAi⋅j #14

We constructed a kinematic chain model using 4x4 homogenous transformation matrices and solved the forward kinematics problem[56] to estimate the wrist position with respect to the shoulder, pwriw (Figure 6). This sequence transforms the world frame with origin at the acromion to a frame partially aligned with the forearm sensor and with origin at the wrist (Figure 2). This final frame has only partial alignment with the forearm sensor because our model does not incorporate the degree of PS. This is because physically pronating or supinating the forearm generally does not change the actual wrist position in space, so the PS angle is not necessary to compute pwriw and therefore does not need to be incorporated into the kinematic model.

The first transformation matrix in this sequence, TAw, is

TAw=RAw03x101x31 #15

where RAw is the equivalent rotation matrix representation of qAw. The next transformation matrix, TAiA, is

TAiA=(RFAA)initialpAA01x31 #16

where (RFAA)initial is the rotation matrix representation of (qFAA)initial. Note that this rotation makes pwriw a function of dθ. Assuming that the x-axis of the arm sensor is aligned with the length of the arm,  pAA=lA,0, 0T is the vector containing the translation from the acromion to the antecubital fossa, and lA is the measured arm length. The following transformation matrix, TFAAi, is defined as

TFAAi=RFAAi03x101x31 #17

where RFAAi is the rotation matrix describing a rotation of dθ about the FE axis of rotation j. Assuming that the x-axis of the forearm sensor is aligned with the length of the forearm, the final transformation, TwriFA, is

TwriFA=I3x3pFAFA01x31 #18

where I3x3 is the 3x3 identity matrix, pFAFA=[lFA, 0,0] is the vector containing the translation from the antecubital fossa to the center of the wrist, and lFA is the measured forearm length. Using Equations (15) to (19), the net transformation matrix, Twriw, is

Twriw=TAwTAiATFAAiTwriFA=Rwriwpwriw 01x31 #19

where pwriw is the position vector from the shoulder to the wrist expressed in the world frame.

Regarding the data calibration procedure for a typical recording session, maintaining the static, neutral pose for 30-60 seconds provided raw IMU measurements that we used while processing the data. Specifically, we concatenated these raw IMU measurements to the beginning of the recorded data to extend the period available for filter convergence. This allowed us to select lower gains in the setting of more severe magnetometer and accelerometer distortion. Additionally, this static period provided qAw and qFAw estimations for baseline orientation removal. Furthermore, the subsequent passive FE motion provided gyroscope data for determining j.

Vive-Based Tracking

The Vive automatically estimates each tracker’s position, rFAw  and rAw, and orientation, qFAw and qAw, expressed with respect to a common world frame. To compute pwriw, we utilized only the orientation estimations to construct the Vive’s UE kinematic model (Figure 7), which is relatively simple compared to the IMU’s kinematic model. The first transformation matrix, TAw, is defined by Equation (15). The second transformation matrix, TFAA, is

TFAA=RFAApAA01x31 #20

where RFAA is the rotation matrix representation of qFAA. The third transformation, TwriFA, is defined by Equation (18). We multiplied these three transformation matrices to obtain Twriw.