Supplementary Materials

Redox coupling of metals drives rate enhancement in thermochemical oxidative dehydrogenation

Isaac T. Daniel1†, Bohyeon Kim2†, Mark Douthwaite1*, Samuel Pattisson1, Richard J. Lewis1, Joseph Cline3, David J. Morgan1, Donald Bethell1, Christopher J. Kiely2,3, Steven McIntosh2*, and Graham J. Hutchings1*

Corresponding author: Mark Douthwaite (douthwaitejm@cardiff.ac.uk), Steven McIntosh (mcintosh@lehigh.edu) and Graham J. Hutchings (hutch@cardiff.ac.uk).

This file includes:

Supplementary Fig. 1 - 5

Supplementary Scheme 1 and 2

Supplementary Tables 1 to 3

Supplementary Fig. 1. XPS spectra from fresh and used samples of catalysts. (A) Au 4f region from fresh monometallic Au/C, used Au/C + Ir/C physical mixture and used Au/C + Pt/C physical mixture, with the expected binding energy for the metallic Au highlighted by the dashed lines. (B) Pd 3d region for fresh monometallic Pd/C and used Pd/C + Pt/C physical mixture, with metallic Pd highlighted by the dashed lines. (C) Pt 4f region for fresh monometallic Pt/C and used Pt/C + Pd/C physical mixture, with Pt2+ peaks highlighted by the dashed lines. (D) Ir 4f region for fresh monometallic Ir/C and used Ir/C + Au/C physical mixture, with Ir2+ peaks highlighted by the dashed lines.

Supplementary Fig. 2. Linear sweep voltammetry curves of (A) Au/C, (B) Pd/C, (C) Pt/C, and (D) Ir/C for HMFOR (black line) and ORR (red line). Reaction conditions (HMFOR): 0.4 M NaHCO3, 0.1 M HMF, 50 mL min-1 of N2. Reaction conditions (ORR): 0.4 M NaHCO3, 50 mL min-1 of O2. All LSV curves were measured at room temperature with a scan rate of 10 mV s-1. (E) Comparison between the jM and ODH activity at room temperature.

Supplementary Fig. 3. Tafel plots of (A) the various monometallic catalysts, (B) corresponding comparison of measured EM and EM values obtained from the Tafel plot, and (C) comparison of current and ODH activity. Reaction conditions: NaHCO3 (0.4 M), HMF (0.1 M), O2 (50 mL min-1), 50 ℃.

Supplementary Fig. 4. Activity and selectivity of the five physically mixed binary catalytic systems. (A) Au/C + Pd/C, reproduced with permission (5) (B) Au/C + Ir/C, (C) Pd/C + Ir/C, (D) Au/C + Pt/C, and (E) Pt/C + Pd/C. Reaction conditions: H2O (16 mL), HMF (0.1 M), NaHCO3 (0.4 M), 3 bar O2, 80 ˚C, 30 minutes. The total metal content is constant in these bimetallic systems (HMF : metal = 200 : 1 (mol : mol)). The sum of the monometallic activities are taken from the monometallic activities of each respective metal, for the specific molar quantity. All catalysts have approximately 1 wt.% total metal loading.

Supplementary Fig. 5. Linear sweep voltammetry curves at 50 ℃ of (A) Au/C (HMFOR), Pd/C (ORR), and Ir/C (ORR), and (B) Au/C (HMFOR), Ir/C (HMFOR), and Pd/C (ORR). (A) explains why ORR of Ir/C is limiting compared to that of Pd/C and (B) explains why HMF OR of Ir/C is limiting compared to that of Au/C. HMFOR LSVs at room temperature (RT) for (C) Au/C, Pd/C, Au/C + Pd/C; (D) Pd/C, Ir/C, Ir/C + Pd/C; (E) Au/C, Ir/C and Au/C + Ir/C. The average value of the respective monometallic catalysts is also given (dashed lines). Reaction conditions (HMFOR): 0.4 M NaHCO3, 0.1 M HMF, 50 mL min-1 of N2. Reaction conditions (ORR): 0.4 M NaHCO3, 50 mL min-1 of O2.

Supplementary Scheme 1. The ODH of HMF can proceed via two pathways, with either the aldehyde (Route 1) or alcohol (Route 2) moiety transformed first to produce 2,5-diformylfuran (DFF) or 5-hydroxymethyl-2-furancarboxylic acid (HMFCA), respectively.

Supplementary Scheme 2: Experimental configurations utilized in electrochemical measurements.

| Catalyst | Au/C | Pd/C | Ir/C | Pt/C |
| --- | --- | --- | --- | --- |
| Weight percent of metal by ICP-MS | 1.01 | 1.04 | 0.94 | 1.01 |

Supplementary Table 1. ICP-MS analysis of each of the four monometallic catalysts used in this study. Target weight of 1 wt.%.

| Catalytic System | ORR Onset Potential (V) | HMFOR Onset Potential (V) | Mixed Potential, EM (V), measured* | Mixed Potential, EM (V), predicted** | Mixed Current Density, jM (A mmol-1), predicted** |
| --- | --- | --- | --- | --- | --- |
| Au/C | 0.72 | 0.47 | 0.658 ± 0.003 | 0.670 ± 0.013 | 3.1 ± 0.39 |
| Pd/C | 0.84 | 0.68 | 0.808 ± 0.007 | 0.800 ± 0.009 | 1.23 ± 0.28 |
| Pt/C | 0.86 | 0.56 | 0.769 ± 0.005 | 0.770 ± 0.014 | 3.1 ± 0.16 |
| Ir/C | 0.82 | 0.46 | 0.753 ± 0.002 | 0.730 ± 0.002 | 1.27 ± 0.09 |

*1 The measured EM was obtained with 0.1 M HMF, and 0.4 M NaHCO3 solution under O2.

**2 The predicted EM and jM values are taken from Fig. 2 for each monometallic catalytic system.

Supplementary Table 2. The ORR and HMFOR onset potential, and the predicted EM and jM values for each monometallic catalyst from Fig. 2. Reaction conditions can be found in caption for Fig. 2. Measured EM values are included for comparison.

| Catalytic PM System (x & y) | Molar Ratio (x : y) | Activity Enhancement (x 10-8, mol s-1) |
| --- | --- | --- |
| Au/C + Pd/C | 1 : 1 | 90.9 |
| Ir/C + Pd/C | 3 : 2 | 51.8 |
| Au/C + Ir/C | 2 : 3 | 36.5 |

Supplementary Table 3. Maximum activity enhancement at the associated molar ratio observed for the three bimetallic catalytic systems that show evidence of a CORE effect. Full reaction data be found in Supplementary Fig. 6. Reaction conditions: H2O (16 mL), HMF (0.1 M), NaHCO3 (0.4 M), 3 bar O2, 80 ˚C, 30 minutes.