Table 1. Rock layer name, annotation to Figure 2 Stratum name & Hermeneutics; Lithology; Thickness (m); Feature Description. Macroporous soil; 50.0; Sediments mainly composed of silt and containing a certain proportion of fine sand, extremely fine sand, and clay particles. Laterite; 35.0; Reddish brown with high water content, low density, and high strength. Siltstone #1; 15.0; Contains more quartz, followed by feldspar and rock debris, with a hard texture and low density. Fine-grained sandstone; 18.7; It is generally light grey, containing feldspar and a small amount of rock debris. Main roof; Medium grained sandstone #1; 21.3; Mainly fine sandstone, grey, dark grey with thin layer of siltstone. Immediate roof; Medium grained sandstone #2; 5.2; Fine-grained sandstone is mainly grey and light grey, with medium sorting, containing muscovite flakes and dark minerals intercalated with thin layers of siltstone, in an interlayered and wavy bedding.. No.5 Coal seam; Coal; 5.4; Proctor hardness reaches 2.38, thickness is stable. Immediate floor; Siltstone #2; 20.0; Light grey, intercalated with thin layers of fine-grained sandstone, interbedded, containing fragments of plant fossils.. Main floor; Argillaceous rocks; 30.0; Grey, light grey, well sorted, locally calcareous cementation, containing a small amount of plant fossils, wavy bedding.. Refilling bearing structure; Inorganic high water filling material + scattered coal/Gangue; 5.4; Refill the roadway space via two methods: accumulation compaction grouting or mixing water spray cementation. Crawler gangue filling machine; Suitable for all-around filling of coal mine roadways, with the function of ejection compaction, high filling density, and good roof connection effect.