Enabling lower carbon clinical trials: Development and prototype testing of a method to quantify the carbon footprint of clinical trials to inform future lower carbon clinical trial design

Guidance and method to calculate the carbon footprint of a clinical trial

Data collation quick guide and worksheet

This guidance provides information on how to carbon footprint a clinical trial for the purposes of the NIHR-funded project ‘enabling lower carbon clinical trials.’

Within the guidance, clinical trial processes have been sub-divided into the following modules:

- Trial set up

- CTU emissions

- Trial specific meetings and travel

- Treatment intervention

- Data collection and exchange

- Trial supplies and equipment

- Trial specific patient assessments

- Samples

- Laboratory

- Trial close out

This list is not exhaustive, and it is expected that further activities and modules may need to be added to account for specialist processes in all clinical trial types.

NB: analysis of data does not need to be calculated separately, it is covered by the emissions attributed to trial staff FTE in “CTU emissions” and calculations included within “Data Collection and exchange”.

In addition to this quick guide and worksheet, we have produced a detailed guidance and method document defining the project scope, limitations and assumptions. The detailed guidance contains a more in depth look and explanation of the calculations found in this document, including emission factor and benchmark data sources, and should be referred to when using this worksheet.

Introduction to calculating carbon footprint

A carbon footprint is a measure of greenhouse gases, usually quoted in kg or tonnes of carbon dioxide equivalent (CO2e). To calculate the carbon footprint of a particular clinical trial process, both ‘activity data’ and ‘emission factors’ are required.

An emission factor, also known as a conversion factor, “is a coefficient which allows you to convert activity data into greenhouse gas emissions. It is the average emission rate of a given source, relative to units of activity or process/processes.”

The activity data is provided by the user and multiplied by the emission factors provided in this guidance document.

Data collation quick guide and worksheet

This data collation quick guide should be used in conjunction with the “Enabling lower carbon clinical trials: Development and prototype testing of a method to quantify the carbon footprint of clinical trials to inform future lower carbon clinical trial design - Detailed Guidance and method to calculate the carbon footprint of a clinical trial”. The guidance document provides the detailed explanation of how calculations should be considered and calculated. This quick guide should be used to collate the trial-specific processes, necessary activity data and to record the subsequent calculations. It is important to avoid double-counting activities i.e., modules must not include activities already covered elsewhere in the clinical trial process map. Please complete this worksheet for each trial to be carbon footprinted.

NB: We are using the term ‘CTU’ to describe the organisation that manages all aspects of central trial management. For some institutions some of those tasks maybe done by groups outside the CTU team e.g., sponsor office/CRO etc.

PRIMETIME carbon footprint calculations

PRIMETIME is a post-operative avoidance of radiotherapy trial which recruited 1962 patients across 64 sites in the UK. The trial used electronic data capture for collection of data, data linkage with NHS routine data sources and involved shipment of tissue samples for immunohistochemistry testing and subsequent long-term sample storage.

| Module | Examples | Trial activity data | Calculation | Results |
| --- | --- | --- | --- | --- |
| 1. Trial set-up |  |  |  |  |
| - Production of trial documentation to be sent to sites or participants | E.g. Site Investigator File and contents, Site Pharmacy File and contents, CRF Folder and contents, PIS/Cs, GP letters | 67 sites initially set up (64 recruited patients) 3123 patients registered to pre-screening, 1962 patients consented and registered to main study SIF: 260 pages, 67 sent (17420 total pages) GP letter: 2 pages, 1962 (3924 total pages) PIS/C form 1: 6 pages, 3123 patients (18738 total pages) PIS/C form 2: 15 pages, 1962 (29430 total pages) Number of folders used to send trial documentation = 67 large ring binders (0.5kg) | Paper: [no. of page] x 0.005 = paper weight (kg) b/w printing: Kg of paper x 0.22438 = kgCO2e Colour printing: Kg of paper x 0.31786 = kgCO2e Materials (paper): Kg of paper x 0.919 = kgCO2e Folders: Kg (of cardboard) x 0.821 = kgCO2e Assumption: Weight of lever arch = 0.5kg Assumption: Weight of ring binder = 0.3kg SIF: 17420 pages x 0.005= 87.1 kg Printing: 87.1 kg x 0.22438 = 19.5 KgCO2e Materials (paper): 87.1 kg x 0.919 = 80 kgCO2e Materials (folder): 0.5 kg x 67= 33.5kg 33.5 kg x 0.821 = 27.5 kgCO2e PIS/C: 48168 x 0.005 kg = 240.8 kg Printing: 240.8 kg x 0.31786 = 76.5 kgCO2e Materials: 240.8 kg x 0.919 = 221.3 kgCO2e GP letter: 3924 x 0.005 kg = 19.6 kg Printing: 19.6 kg x 0.22438 = 4.4 kgCO2e Materials: 19.6 kg x 0.919 = 18 kgCO2e | 447.2 kg CO2e |
| - Provision/postage of trial documentation to sites - Provision/postage of documentation to participants by CTU or participating sites - Provision/postage of incentives to participant | E.g. Site Investigator File and contents, Site Pharmacy File and contents, CRF Folder and contents, PIS/Cs, GP letters | Estimated weight of SIF: (260 pages x 0.005kg) + 0.5 kg = 1.8kg / 0.00177 tonnes Estimated distance of deliveries: Average site distance from ICR-CTSU is 301.8 km Number of deliveries: SIF delivered to 67 sites | Delivery weight (tonnes) x distance (km) = t.km For road freight: t.km x 0.19443 = kgCO2e For air freight: t.km x required emission factor below = kgCO2e - Domestic (to/from UK) = 4.98549 - Short-haul (to/from UK) = 2.55439 - Long-haul (to/from UK) = 1.13047 - International (to/from non-UK) = 1.13047 For delivery of trial supplies to patients or GP, if unknown, use 17.4km as distance from hospital to patient, or hospital to GP. Calculations: SIF delivery: - 0.00177 x 301.8km = 0.53 t.km - 0.53 x 0.19443 = 0.103 - x 67 sites = 6.9 kgCO2e GP letter: Weight of letter = 2x5g + envelope (7g)= 17g x1962 patients = 33354g/0.033 tonnes 0.033 x 17.4km = 0.5742 t.km 0.5742 t.km x 0.19443 = 0.11 kgCO2e | 7.01 kgCO2e |
| 2. CTU emissions |  |  |  |  |
| - Energy consumption at CTU according to trial staff FTE | E.g. energy consumption per square metre of air-conditioned office space | Trial duration: 10 years Trial staff FTE: the total FTE over the 10-year trial period was 7.1 | Energy consumption for 1 FTE for 1 year = 193.28 kgCO2e Multiply by the number of years and FTE applicable 7.1 X 193.28 kgCO2e = 1372.3 kgCO2e | 1372.3 kgCO2e |
| - Heating | E.g. energy consumption at coordination centre attributed to heating (natural gas) | Trial duration: 10 years Trial staff FTE: the total FTE over the 10-year trial period was 7.1 | Heating for 1 FTE for 1 year = 431.56 kgCO2e Multiply by the number of years and FTE applicable 7.1 x 431.56 kgCO2e = 3064 kgCO2e | 3064 kgCO2e |
| - Trial team commuting | E.g. Car, rail, bus, walking etc | Trial duration: 10 years Trial staff FTE: the total FTE over the 10-year trial period was 7.1 | For 1 FTE for 1 year, total average commuting emissions = 1027.8 kgCO2e Multiply by the number of years and FTE applicable 7.1 x 1027.8 kgCO2e = 7297 kgCO2e | 7297 kgCO2e |
| 3. Trial specific meetings and travel |  |  |  |  |
| - Visits and travel to site - Travel to meetings | E.g. Feasibility, site initiation and monitoring visits, audits, inspections, Trial Management Group (TMG), Trial Steering Committee (TSC), Independent Data Monitoring Committee (IDMC), and investigator meetings, Patient and Public Involvement and Engagement (PPIE), conferences, scientific meetings etc | No feasibility visits, monitoring, audits, or conferences. Site initiation visits 66 virtual (+6 refresher sessions), 1 in person IDMC meetings 5 IDMC meetings, all virtual, total of 31 instances of attendance TMG meetings Total distances travelled by participants or number of virtual attendees: - Meeting 1- 1760.6km - Meeting 2- 1437.3km - Meeting 3 – virtual, 12 attendees - Meeting 4 – 1624.8km - Meeting 5 – virtual, 25 attendees - Meeting 6 – virtual, 18 attendees - Meeting 7 – virtual, 23 attendees TSC meetings 5 attendees to each TSC meeting, 1 hour duration, some teleconference and some in person. Distance staff travelled from CTU = 22.5km | Number of passengers x total distance (km) = p.km For national rail: p.km x 0.04441= kgCO2e For flights: p.km x relevant emission factor below: - Domestic (average): 0.27278 - Short-haul (average) to/from UK: 0.17034 - Long-haul (average) to/from UK: 0.21423 - International (average) to/from non-UK: 0.20373 NB: Distances may be calculated using google maps and calculated from CTU to destination Videoconferencing = 157 grams CO2e per hour. Calculations: Site Initiation Visits: Virtual SIVs: 72 X 157 g CO2e/hr = 11304g/11.3 kgCO2e 1 in person SIV: 1 attendee, 24.1KM. 1 x 2 x 24.1 km = 48.2 p.km 48.2 p.km x 0.04441= 2.1 kgCO2e IDMC IDMC: 31 x 157 g CO2e/hr = 4867 g/4.9 kgCO2e TMG meetings - assumption: rail used to travel TMG meeting 1: 1x2x1760.6km = 3521.2 p.km. 3521.2 p.km x 0.04441 = 156.4 kgCO2e TMG meeting 2: 1x2x1437.3km= 2874.6 p.km 2874.6 p.km x 0.04441 = 127.7 kgCO2e TMG meeting 3: teleconference, 2 hours, 12 external attendees. 157 g CO2e/hr x 2 x 12 = 3768g/3.77 kgCO2e TMG meeting 4: 1 x 2 x 1624.8km = 3249.6 p.km x 0.04441 = 144.3 kgCO2e TMG meeting 5: 13-14:30 via zoom, 25 attendees. 157 g CO2e/hr x 1.5 x 25 = 5887.5 g/5.89 kgCO2e TMG meeting 6: 157 g CO2e/hr x 1.5 x 18 = 4239 g/4.24 kgCO2e TMG meeting 7: 1hr, Teams meeting, 23 attendees - 157 g CO2e/hr x 23 = 3611g/3.6 kgCO2e TSC 2016, teleconference: 157 g CO2e/hr x 5 = 785g/0.8 kgCO2e 2017: 5x(2x22.5km) = 225 p.km x 0.04441= 10 kgCO2e 2018 = 10 kgCO2e 2019 = 10 kgCO2e 2021 = 10 kgCO2e Other: - 2 virtual conferences: 157g CO2e/hr x 2 = 0.314 kgCO2e - Public Health England (PHE) teleconference, 2 attendees: 2 x 157gCO2e/hr = 0.314 kgCO2e - Sample management and logistics planning meeting, 5 attendees: 5 x 157 g CO2e/hr = 0.785 kgCO2e - Data management meetings, 20 instances of teleconferencing: 20 x 157 g CO2e/hr = 3140g/3.14 kgCO2e - LCRN Meeting re CPMS Uploads, 9 attendees - 9 x 157 g CO2e/hr = 1.413 kgCO2e - Primetime Team meetings, 132 instances of people teleconferencing for 1 hour. 132 x 157 = 20724 g/20.7 kgCO2e - Cl_TC updates, 303 instances of people teleconferencing for 1 hour: 303 x 157 g CO2e/hr = 47571 g/47.6 kgCO2e | 579.3 kgCO2e |
| - Hotel stays | E.g. TMG meetings, monitoring visits, audits, inspections etc | Number of rooms: 8 rooms (for one person for one night) | For UK: number of hotel rooms x number of nights x 13.9 = kgCO2e For UK (London) = number of rooms x number of nights x 13.8 = kgCO2e. For other countries use conversion factors from orange ‘hotel stay’ tab: conversion-factors-2021-full-set-advanced-users.xlsm (live.com) Alternatively, you may use cost-based method: £ spent x 0.388 = kgCO2e Assumption: 8 stays (for TMG meetings) 8 x 13.9 = 250.2 kgCO2e | 250.2 kgCO2e |
| - Sustenance | E.g. meeting lunches, hotel dinners | Number of lunches or dinners: 3 in-person TMG meetings, 52 total attendees 4 in person TSC meetings, 20 total attendees Total = 72 lunches for TMG and TSC meeting participants | Meeting lunches or hotel dinners (vegetarian) = 2.6 kgCO2e per meal per person Meeting lunches or hotel dinners (meat) = 5.92 kgCO2e per meal per person Assumption: unable to determine if meat or vegetarian therefore assumed 50/50 (36 x 2.6) + (36 x 5.92) = 306.7 kgCO2e | 306.7 kgCO2e |
| 4. Intervention 4.1. Physical (IMP) 4.2. Clinical (non-IMP) 4.3. Other (not captured above) Please fill out the section most relevant to the intervention being investigated. |  |  |  |  |
| 4.1. Physical |  |  |  |  |
| 4.1.1, 4.1.2. Movement of intervention, or materials required to deliver the intervention | E.g. movement of intervention from manufacturing site to distribution site, shipment of IMP to participating sites or direct to participant | Total weight of deliveries: Total distance of deliveries: | Carry out freight calculation as described in section 1.2. For refrigerated freight, increase the total kgCO2e associated with freight by 15%. Frozen freight: Dry ice has a carbon footprint of 1.81kg CO2e for 1 kg dry ice produced/used. When calculating the overall emissions of frozen freight, as well as the 1.81 kgCO2e per 1kg attributed to manufacture, include the weight (kg) of dry ice used in the weight of the freight calculation in section 1.2. In the absence of activity data, assume 1kg of dry ice is used per individual sample shipping box. | N/A – Avoidance of radiotherapy trial |
| 4.1.3. Materials required for the packaging and shipment of IMP | E.g. cardboard, cold storage boxes, polystyrene | Number of cold storage boxes: Kg of cardboard/ polystyrene: | Single use sample cold storage box = 25.2 kgCO2e per box Reusable sample cold storage box = 2.2 kgCO2e per box Kg (cardboard) x 0.821 = kgCO2e Kg (polystyrene) x 3.778 = kgCO2e | N/A – avoidance of radiotherapy trial |
| 4.1.4. Destruction of overage | E.g. incineration of IMP | Estimated weight of overage incinerated: | Kg of waste x 2.4252 = kgCO2e | N/A – avoidance of radiotherapy trial |
| 4.2. Clinical e.g., radiotherapy, device, surgical. NB: not all calculations will be relevant to all interventions. This section of the method will be further developed as we carbon footprint more trials, so please inform us if your protocol specifies an activity that has not been included, and we will help to determine the associated carbon footprint. |  |  |  |  |

| 4.2.1 Movement of the intervention, or resources required to deliver the intervention | E.g. movement of intervention from manufacturing site to distribution site, shipment of intervention to participating sites. | Estimated total weight of delivery: Estimated total distance of deliveries: | Please refer to section 1.2 and 4.1.1, 4.1.2. | N/A – avoidance of radiotherapy trial |
| --- | --- | --- | --- | --- |
| 4.2.2 Materials required for the packaging and shipment of the intervention | E.g. cardboard, cold storage boxes, polystyrene | Number of cold storage boxes: Kg of cardboard/ polystyrene: | Please refer to section 4.1.3. | N/A – avoidance of radiotherapy trial |
| 4.2.3 Utilities required for delivery of the intervention | E.g. Hospital utilities if the intervention is delivered within a hospital | Hospital staff FTE required: | Please refer to section 7.3 to calculate the emissions attributed to hospital utilities if required to deliver the intervention. | N/A – avoidance of radiotherapy trial |
| 4.2.4 Activities or resources required/relating to delivery of the intervention | E.g. consumables, surgical time, specialist equipment, incineration of surgical waste |  | Please refer to section 7.2 for consumables, surgery and other activities that may be relevant to the delivery of the intervention, but please take care to avoid double counting. To calculate the emissions attributed to incineration, e.g. of surgical waste, please refer to section 4.1.4. | N/A – avoidance of radiotherapy trial |

4.3. Other NB: not all calculations will be relevant to all interventions. This section of the method will be further developed as we carbon footprint more trials, so please inform us if your protocol specifies an activity that has not been included, and we will help to determine the associated carbon footprint.

| 4.3.1 Movement of the intervention to the participant or participating site | E.g. shipment of intervention to participating sites or direct to participant | Estimated weight of delivery: Estimated distance of deliveries: | Please refer to section 1.2. | N/A – avoidance of radiotherapy trial |
| --- | --- | --- | --- | --- |
| 4.3.2 Materials required for packaging and shipment of the intervention | E.g. cardboard, cold storage boxes, polystyrene | Number of cold storage boxes: Kg of cardboard/ polystyrene: | Please refer to section 4.1.3. | N/A – avoidance of radiotherapy trial |
| 4.3.3 Materials or resources required for delivery of the intervention | E.g. software, booklets, specialist equipment |  | For printing and paper, please refer to section 1.1. | N/A – avoidance of radiotherapy trial |
| 4.3.4 Travel required to facilitate delivery of the intervention | E.g. to deliver training, conduct interviews etc | Estimated distance travelled, and number of passengers (p.km): | Please refer to section 3.1, 3.2. | N/A – avoidance of radiotherapy trial |

| 5. Data collection and exchange NB: analysis of data does not need to be calculated separately, it is covered by the emissions attributed to trial staff FTE in “CTU emissions” and calculations included within “Data Collection and exchange”. |  |  |  |  |
| --- | --- | --- | --- | --- |
| - Data collection and query exchange between CTU and sites | E.g. CRFs, EDC completion and query resolution, scans copied to CDs | Postage from sites to ICR-CTSU: Only the eligibility, pre-screening and registration checklists posted to CTU from sites. -Total pages from all sites = 14094. -Average site distance from ICR-CTSU: 301.8 km Printing by sites for their SIF: Estimate 419 pages printed by each site for their SIF, 63 sites = 26397 total pages Paper CRFs: 28 folders/box files containing paper CRFs in ICR-CTSU. Assumption: box file contains 500 pieces of paper 500 x 28 = 14000 total pages Site documents stored in ICR-CTSU: 18 folders with TMFs and site documents stored in CTU. Assumption: folder holds 350 pages. 18 x 350 = 6300 total pages | Postage of documents from sites: 14094 x 0.005 = 70.47 kg / 0.07 tonnes 0.07 x 301.8 = 21.126 t.km 21.126 t.km x 0.19443 = 4.1 kgCO2e Printing by sites for their SIF: 26397 x 0.005 = 131.985 kg Printing: 131.985 kg x 0.22438 = 29.6 kgCO2e Material: 131.985 kg x 0.919 = 121.3 kgCO2e Paper CRFs 14000 x 0.005 = 70kg Printing: 70 kg x 0.22438 = 15.7 kgCO2e Materials: 70kg x 0.919 = 64.3 kgCO2e Site documents stored in ICR-CTSU: 6300 x 0.005= 31.5 kg Printing: 31.5 kg x 0.22438 = 7.1 kgCO2e Materials: 31.5 kg x 0.919 = 29 kgCO2e | 271.1 kgCO2e |
| - Data sent direct from participants to CTU or participating sites | E.g. Questionnaires, patient diaries, wearables | Estimated weight and distance of delivery (t.km): Device used and time taken to complete electronic questionnaires: | For paper questionnaires, please refer to section 1.1. for the carbon footprint of producing the materials and section 1.2. for postage (freight). For use of smart watches and other devices see section 6.3. Electronic questionnaires Add the emissions attributed to data storage and transmission to the emissions attributed to using a device to complete the questionnaire. Web surfing (data storage and transmission) = 9.441 g CO2e/hr (10 mins = 1.57 g CO2e) Choose from the below: - Desktop computer = 0.18079 kg CO2e per hour - Laptop = 0.028719 kg CO2e per hour - Tablet = 0.027397 kg CO2e per hour - Smartphone = 0.015068 kg CO2e per hour | N/A |
| - Data from labs to CTU - Data from other collaborators to CTU | E.g. Laboratory patient results, data linkage | GB required for data storage/transmission: Duration of data storage: Total £ spent on computer services: £49000 | Data storage and transmission = estimate 1.365 kg CO2e per GB per year. For computer services such as data linkage: £ x 0.149 = kgCO2e £49000 x 0.149 = 7301 kgC02e | 7301 kgC02e |
| 6. Trial supplies and equipment |  |  |  |  |
| - Equipment used by CTU | E.g. computers, laptops, printers, software | Total £ spent on office machinery and computers for trial: £4500 | For any new office machinery and computers purchased specifically for trial: £ x 0.387 = kgCO2e £4500 x 0.387 = 1741.5 kgCO2e | 1741.5 kgCO2e |
| - Equipment and supplies used by participating sites supplied by CTU | E.g. centrifuge, fridge, freezer | Estimated weight and distance of deliveries (t.km): | For the shipment of equipment to participating sites, please refer to section 1.2. For the use of a centrifuge, please refer to section 9.2., for a fridge or freezer please refer to section 9.3. | N/A |
| - Equipment and supplies provided to participants specifically for the trial | E.g. wearables, smartphone, tablet | Number of devices and duration of their usage: Estimated weight and distance of deliveries (t.km): | Smartphone = 55 kgCO2e from manufacture and add 5.5 kgCO2e per year of usage. Tablet = 119 kgCO2e from manufacture and add 10 kgCO2e per year of usage. Wearables/smart watch = 30.1 kg CO2e for manufacture and add 1.633 kg CO2e per year of usage. To calculate the carbon footprint associated with shipment of the devices, please refer to section 1.2. | N/A |
| 7. Trial specific patient assessments |  |  |  |  |
| - Patient travel for study visits that are in addition to standard of care | E.g. Eligibility and screening assessments, trial-specific assessments and procedures | Number of times patient is required to travel (in addition to standard of care): 1623 patients had to travel for 5 additional above standard of care mammograms | Emissions associated with one patient visit to hospital (UK) = 5.8 kgCO2e (this includes both the out and back journeys) Emissions associated with one patient visit to GP surgery (UK) = 1.12 kgCO2e (this includes both the out and back journeys) 5.8 kgCO2e x 5 x 1623 = 47067 kgCO2e | 47067 kgCO2e |
| - Materials and activities required for study assessments that are in addition to standard of care | E.g. Laboratory tests, imaging assessments, clinical activities relating to intervention for example administering of study drug, biopsy. | 5 above standard of care mammograms for each patient in very low risk and no RT group (1623) Assumption: 2 visits where consumables required | Consumables = 0.30 kgCO2e per patient per trial appointment where consumables (such as gloves) required - 1 MRI = 24.7 kg CO2e - 1 CT scan = 9.2 kgCO2e - 1 CXR = 0.8 kgCO2e/scan - 1 hour in surgery = 53 kg CO2e - 1 low intensity (general ward) bed day = 37.9 kg CO2e - 1 high intensity (ICU) bed day = 103 kgCO2e - 15 sessions of breast radiotherapy = 5.7 kg CO2e - 20 sessions of prostate radiotherapy = 15.3 kg CO2e Blood tests: - 82 g CO2e for coagulation profile - 116 g CO2e for full blood examination - 49 g CO2e for arterial gas assessment - 99 g CO2e for urea and electrolyte assessment - 0.5 g CO2e for C-reactive protein Please note that the above figures for blood tests include the materials and consumables required for sample collection, phlebotomy and analysis, as well as power consumption by pathology analysers. Calculations: Consumables: - 0.30 kgCO2e x 2989 (number of patients who received ki67 testing) = 896.7 kgCO2e - 0.30 x 1962 (for physical appointment for those registered to study) = 588.6 kgCO2e - Total = 1485.3 kgCO2e Scans: 5 x 0.8 kgCO2e x 1623 = 6,492 kgCO2e | 7977.3 kgCO2e |
| - Utilities required for study assessments that are in addition to standard of care | E.g. energy consumption per square metre of hospital space according to trial staff FTE, taking into account time required for CRF completion and study assessments, consent etc | Hospital trial staff FTE: total estimated FTE = 5.5 from 2017 -2022 | Electricity: 1 FTE 1 year = 364.9 kgCO2e Multiply by the number of years and FTE applicable 364.9 kgCO2e x 5.5 = 2007 kgCO2e Heating: 1 FTE, 1 year = 685.7 kgCO2e Multiply by the number of years and FTE applicable 685.7 kgCO2e x 5.5 = 3771.4 kgCO2e | 5778.4 kgCO2e |
| 8. Samples |  |  |  |  |
| - Materials involved | E.g. sample collection kit and packaging for shipment | Kg of material: - 30.6 kg slide container (pp) - 243.8 kg jiffy bag - 39 kg paper | The emissions attributed to sample collection consumables for common blood tests are included in the blood tests listed in section 7.2. To calculate the carbon footprint of other common materials, multiply the weight in kg by the relevant emission factor below to produce kgCO2e. - Average plastics: 3.116 - Plastics (average film): 2.754 - Plastics (Average rigid): 3.277 - Plastics (PP): 3.105 - Plastics (PET): 4.032 - Glass: 3.413 - Paper: 0.919 - Board: 0.821 Slide containers = 30.6 kg x 3.105 (pp) = 95 kgCO2e Paper (materials) = 39kg x 0.919 = 35.8 kgCO2e Paper (printing) = 39kg x 0.22438 = 8.8 kgCO2e Jiffy bag = 243.8 kg x 0.919= 224.1 kgCO2e Tracking slip: 2 copies of each tracking slip on top of one included in the kit– 2989 x 2 = 5978 pages Printing: 5978 pages x 5g = 29890g/29.89kg 29.89 kg x 0.22438 = 6.7 kgCO2e 26.9kg x 0.919 = 27.5 kgCO2e | 397.9 kgCO2e |
| - Movement of sample kit materials from manufacturer to CTU - Movement of sample kits from CTU/distributor to participating Sites - Movement of samples from participating sites or patients to central laboratory. | E.g. shipment of blood tubes for sample kits to CTU | - Movement of sample kit materials from manufacturer to CTU Envelopes: - Weight = 0.004 tonnes, total - Total distance transported = 849.6km (141.6km x 6 deliveries) Slide mailing containers: -Weight = 0.00204 tonnes -Total distance transported = 6910.3 km (363 km x 19 deliveries) - Movement of sample kits from CTU to participating Sites: -Average distance from CTU to participating site = 301.8km - Shipment weight = 0.00053 tonnes - 630 sample kit shipments - Movement of samples from participating sites to laboratory. -Cambridge lab – average distance to participating site = 239.5km, 938 shipments -RMH lab - average distance to participating site = 258km, 1094 shipments -Glasgow lab - average distance to participating site = 467.8km, 957 shipments | Please refer to section 1.2. for freight and 4.1. for refrigerated or frozen freight. 8.2: Movement of sample kit materials from manufacturer to CTU Envelopes: 0.004 tonnes x 849.6km = 3.3984 t.km 3.3984 t.km x 0.19443 = 0.66 kgCO2e Slide mailing containers: 0.00204 tonnes x 6910.3 km = 14.097 t.km 14.097 t.km x 0.19443 = 2.7 kgCO2e - Movement of sample kits from CTU/distributor to participating Sites 0.00053 x (630 x 295.88) = 100.77 t.km x 0.19443 = 19.6 kgCO2e - Movement of samples from participating sites or patients to central laboratory. Cambridge laboratory: 0.0000935 x (239.5km x 938) x 0.19443 = 4.1 kgCO2e RMH laboratory: 0.0000935 X (258 km X 1094) X 0.19443 = 5.1 kgCO2e Glasgow laboratory: 0.0000935 x (467.8km x 957) x 0.19443 = 8.1 kgCO2e | 40.26 kgCO2e |
| 9. Laboratory |  |  |  |  |
| - Emissions attributed to lab utilities according to staff FTE | E.g. energy consumption per square metre of laboratory space according to trial staff FTE | Trial staff FTE and trial duration: 3 labs, 1 staff member in each. Trial is 5% of each of their FTE for 4 years and 2% for 1 year. | Electricity: 1747.2 kgCO2e per FTE per year Multiply 1747.2 kgCO2e by the number of years and FTE applicable 1747.2 kgCO2e x 0.05 = 87.36 kgCO2e 87.36 kgCO2e x 4 years = 349.44 kgCO2e 1747. 2 kgCO2e x 0.02 = 34.944 kgCO2e 349.44 + 34.944 = 384.4 kgCO2e 384.4 x 3 lab staff = 1153.2 kgCO2e Heating: 1376 kgCO2e per FTE per year Multiply 1376 kgCO2e by the number of years and FTE applicable 1376 kgCO2e x 0.05 = 68.8 kgCO2e 68.8 kgCO2e x 4 = 275.2 kgCO2e 1376 x 0.02 kgCO2e = 27.52 kgCO2e 275.2 kgCO2e + 27.52 kgCO2e = 302.72 kgCO2e Total = 302.72 kgCO2e x 3 = 908.16 kgCO2e | 2061.4 kgCO2e |
| - Materials/equipment/consumables used in processing and analysis of samples | E.g. centrifuges, refrigerators | kWh usage of equipment, hours used: Kg of material: | To avoid double counting, use of equipment will be included in lab staff FTE if calculated. If the trial does not involve a central lab, but there is still sample processing on site, please see below. For storage of samples, please see section 9.3. To calculate the emissions of a piece of equipment, multiply the power consumption by hours used to get a kWh value. Finally multiply kWh by the electricity emission factor (0.273). Example: use of a 310-Watt centrifuge for 15 minutes - 310 Watts x 0.25 (hours) = 77.5 kWh - 77.5 kWh x 0.273 = 21.2 kgco2e Consider the centrifuge capacity and multiply by the number of uses required. | N/A |
| - Storage of samples | E.g. utilities and ultra-low temperature freezer | Length of storage: Amount of refrigerator/freezer space required: | Storage in fridge/-20 freezer: - 298.9 kgCO2e per year - Multiply by number of years stored Storage in an ultra-low/-80 freezer: - 2192.2 kgCO2e per year - Multiply by number of years stored NB: this is for a whole freezer; you will need to make an assumption about the amount of space in the freezer that the trial samples take up. | N/A |
| 10. Trial close out |  |  |  |  |
| 10.1. Storage and archiving of essential trial documentation and data | E.g. Hospital files, lab files, trial guidance documents etc | Duration of storage: 10 years Amount of space required for storage (m2): 4m2 Storage location: CTU (office) | Carbon footprint associated with 1m2 for 1 year: - Office: 16.1 kgC02e - Laboratory: 43.7 kgc02e - Warehouse: 7.4 kgC02e - Health building: 22.1 kgCO2e Choose the most suitable building type and multiply by number of years and m2 necessary. Heating Carbon footprint associated with 1 m2 for 1 year: - Office: 36 kgCO2e - Laboratory: 34.4 kgCO2e - Warehouse: 13.3 kgCO2e - Health building: 41.6 kgCO2e Choose the most suitable building type and multiply by number of years and m2 necessary. For electronic storage, estimate 1.365 kg CO2e per GB per year. Calculations: Electricity: 16.1 kgCO2e x 4 x 10 = 644 kgCO2e Heating: 36 kgCO2e x 4 x 10 years = 1428 kgCO2e Electronic storage: PRIMETIME trial database = ~ 4GB Assumption: storage for 10 years 1.365 kg CO2e x 4 x 10 = 54.6 kgCO2e | 2126.6 kgCO2e |
| 10.2. Storage and destruction of biological samples | E.g. blood, tissue, urine etc | Duration of storage: 10 years Amount of space required for storage (m2): 2m2 Storage location: Health building | See section 9.3. for storage of refrigerated or frozen samples. See section 10.1 for storage of ambient samples. Electricity: 22.1 kgCO2e x2 x 10 = 442 kgCO2e Heating: 41.6 kgCO2e x 2 x 10 = 832 kgCO2e | 1274 kgCO2e |
| 10.3. Return of equipment and supplies from participating sites to CTU | E.g. wearables, unused or expired equipment and supplies | Estimated weight and distance of delivery (t.km): | See section 1.2. (freight). | N/A |

PRIMETIME carbon footprint summary

| Module | KgCO2e |
| --- | --- |
| Trial set up | 454.2 |
| CTU emissions | 11733.3 |
| Trial staff meetings and travel | 1136.2 |
| Treatment intervention | 0 |
| Data collection and exchange | 7572.1 |
| Trial supplies and equipment | 1741.5 |
| Trial specific patient assessments | 60822.7 |
| Samples | 438.2 |
| Laboratory | 2061.4 |
| Analysis and trial close out | 3400.6 |
| Total = | 89360.2 kgCO2e/ 89.4 tonnes |

References

What is an emission factor? [Internet]. Climfoot-project.eu. [cited 2023 May 11]. Available from: https://climfoot-project.eu/en/what-emission-factor

For all emission factor and benchmark data sources, please refer to the accompanying “Detailed Guidance and method to calculate the carbon footprint of a clinical trial.”

As per assumptions detailed in the guidance, manufacture of the intervention is considered out of scope. This section defines all processes relating to providing and delivering the trial intervention that are over and above routine care.

Hospital trial staff FTE was estimated over 6 years rather than the 10-year trial duration because the final 4 years are follow up via routine data sources.