Enabling lower carbon clinical trials: Development and prototype testing of a method to quantify the carbon footprint of clinical trials to inform future lower carbon clinical trial design

Guidance and method to calculate the carbon footprint of a clinical trial

Data collation quick guide and worksheet

This guidance provides information on how to carbon footprint a clinical trial for the purposes of the NIHR-funded project ‘enabling lower carbon clinical trials.’

Within the guidance, clinical trial processes have been sub-divided into the following modules:

- Trial set up

- CTU emissions

- Trial specific meetings and travel

- Treatment intervention

- Data collection and exchange

- Trial supplies and equipment

- Trial specific patient assessments

- Samples

- Laboratory

- Trial close out

This list is not exhaustive, and it is expected that further activities and modules may need to be added to account for specialist processes in all clinical trial types.

NB: analysis of data does not need to be calculated separately, it is covered by the emissions attributed to trial staff FTE in “CTU emissions” and calculations included within “Data Collection and exchange”.

In addition to this quick guide and worksheet, we have produced a detailed guidance and method document defining the project scope, limitations and assumptions. The detailed guidance contains a more in depth look and explanation of the calculations found in this document, including emission factor and benchmark data sources, and should be referred to when using this worksheet.

Introduction to calculating carbon footprint

A carbon footprint is a measure of greenhouse gases, usually quoted in kg or tonnes of carbon dioxide equivalent (CO2e). To calculate the carbon footprint of a particular clinical trial process, both ‘activity data’ and ‘emission factors’ are required.

An emission factor, also known as a conversion factor, “is a coefficient which allows you to convert activity data into greenhouse gas emissions. It is the average emission rate of a given source, relative to units of activity or process/processes.”

The activity data is provided by the user and multiplied by the emission factors provided in this guidance document.

Data collation quick guide and worksheet

This data collation quick guide should be used in conjunction with the “Enabling lower carbon clinical trials: Development and prototype testing of a method to quantify the carbon footprint of clinical trials to inform future lower carbon clinical trial design - Detailed Guidance and method to calculate the carbon footprint of a clinical trial”. The guidance document provides the detailed explanation of how calculations should be considered and calculated. This quick guide should be used to collate the trial-specific processes, necessary activity data and to record the subsequent calculations. It is important to avoid double-counting activities i.e., modules must not include activities already covered elsewhere in the clinical trial process map. Please complete this worksheet for each trial to be carbon footprinted.

NB: We are using the term ‘CTU’ to describe the organisation that manages all aspects of central trial management. For some institutions some of those tasks maybe done by groups outside the CTU team e.g., sponsor office/CRO etc.

CASPS carbon footprint calculations

CASPS is an international phase II trial of CEDIRANIB in the treatment of patients with alveolar soft part sarcoma. The trial enrolled 47 patients across twelve sites in the UK, Spain, and Australia and involved an internationally shipped IMP, completion and shipment of paper CRF, on site visits for site initiation, monitoring visits and audits, additional hospital visits for patients for trial-specific patient assessments and provision of samples.

| Module | Examples | Trial activity data | Calculation | Results |
| --- | --- | --- | --- | --- |
| 1. Trial set-up |  |  |  |  |
| - Production of trial documentation to be sent to sites or participants | E.g. Site Investigator File and contents, Site Pharmacy File and contents, CRF Folder and contents, PIS/Cs, GP letters | 12 sites, 47 patients SIF: 401 pages, 12 sent (4812 pages) Site pharmacy file: 301 pages, 11 sent (3311 pages) CRF folder: 76 pages in set up pack, 17 sent (1292 pages) PIS/C 1 = 17 pages, 27 patients (799 pages) PIS/C 2 = 16 pages, 47 patients (762) Letters inc. GP = 8 pages, 47 patients (376) Total = 11352 pages Number of folders used to send trial documentation: estimate 23 large ring binders (0.5kg) and 18 small (0.3kg) | Paper: [no. of page] x 0.005 = paper weight (kg) b/w printing: Kg of paper x 0.22438 = kgCO2e Colour printing: Kg of paper x 0.31786 = kgCO2e Materials (paper): Kg of paper x 0.919 = kgCO2e Folders: Kg (of cardboard) x 0.821 = kgCO2e Assumption: Weight of lever arch = 0.5kg Assumption: Weight of ring binder = 0.3kg Paper: 11352 pages x 0.005 = 56.76kg Printing: 56.76 kg x 0.22438 = 12.7 kgCO2e Materials: 56.76 kg x 0.919 = 52.2 kgCO2e Folders: (23 x 0.5) + (18 x 0.3) = 16.9kg 16.9kg x 0.821 = 13.9 kgCO2e | 78.8 kgCO2e |

| - Provision/postage of trial documentation to sites - Provision/postage of documentation to participants by CTU or participating sites - Provision/postage of incentives to participant | E.g. Site Investigator File and contents, Site Pharmacy File and contents, CRF Folder and contents, PIS/Cs, GP letters | Weight and distance of deliveries (t.km): UK sites - RMH - 0.00519 tonnes x 24.1km = 0.125079 t.km - Christie - 0.0092 x 355.7km = 3.275997 t.km - Bristol - 0.0063 tonnes x 196.3 km= 1.23669 t.km - UCLH - 0.00519 tonnes x 25.7km = 0.133383 t.km - Newcastle - 0.008725 tonnes x 506.9km = 4.4227 t.km - Nottingham - 0.00519 tonnes x 259.1km = 1.344729 t.km Australia 0.00587 tonnes x (17018.5km+16544km) = 197 t.km Spain 0.00519 x (1147km + 1244km + 1091km) = 18.07 t.km GP letter Weight of GP letter: 2x5g + envelope (7g) = 17g /0.000017tonnes 0.000017 tonnes x 47 patients = 0.000799 tonnes | Delivery weight (tonnes) x distance (km) = t.km For road freight: t.km x 0.19443 = kgCO2e For air freight: t.km x required emission factor below = kgCO2e - Domestic (to/from UK) = 4.98549 - Short haul (to/from UK) = 2.55439 - Long-haul (to/from UK) = 1.13047 - International (to/from non-UK) = 1.13047 UK Total t.km = 10.539289 x 0.19443 = 2.05 kgCO2e Australia 197 t.km x 1.13047 = 222.7 kgCO2e Spain 18.07 t.km x 2.55439 = 46.2 kgCO2e GP letter postage: For delivery of trial supplies to patients or GP, if unknown, use 17.4km as distance from hospital to patient, or hospital to GP. 0.000799 tonnes x 17.4km = 0.014 t.km 0.014 t.km x 0.19443 = 0.002722 kgCO2e 6 letters sent to each patient - 0.002722 x 6 = 0.016 kgCO2e | 271 kgCO2e |
| --- | --- | --- | --- | --- |
| 2. CTU emissions |  |  |  |  |
| - Energy consumption at CTU according to trial staff FTE | E.g. energy consumption per square metre of air-conditioned office space | Trial duration: 6 years Trial staff FTE: UK - 1, Spain - 0.3, Australia - 0.5 | Energy consumption for 1 FTE for 1 year = 193.28 kgCO2e Multiply by the number of years and FTE applicable UK CTU emissions = 193.28 kgCO2e x 6 = 1159.7 kgCO2e Australia CTU - location: Victoria - Office building energy consumption = 231.39 kwh/m2 source: baseline-energy-consumption-part_1-report-2012.pdf. energy split is 90% electricity, 10% gas. Therefore electricity = 208.251 kwh/m2 - Energy usage per staff (per 12 m2) = 12 m2 x 208.251 kWh =2499.012 kwh - Victoria electricity emission factor = 0.98 kg CO2-e/kWh (Source: EERS release 2020–21 (cleanenergyregulator.gov.au)) - 2499.012 kWh x 0.98 = 2449kgCO2e per FTE per year - 2449 kgCO2e x 0.5 (FTE) x 6 years = 7347kgCO2e Spain trial staff - UK benchmark data used as a proxy for average office building energy consumption because like the UK, Spain has a reasonably contribution from renewables. - 193.284 kgCO2e x 6 x 0.3 = 347.9 kgCO2e | 8854.6 kgCO2e |
| - Heating | E.g. energy consumption at coordination centre attributed to heating (natural gas) | Trial duration: 6 years Trial staff FTE: UK - 1, Spain - 0.3, Australia - 0.5 | Heating for 1 FTE for 1 year = 431.56 kgCO2e Multiply by the number of years and FTE applicable UK = 431.56 kgCO2e x 6 = 2589.4kgCO2e Australia - In office buildings energy split is 90% electricity, 10% gas. Office gas intensity = 23.139 kwh/m2 - Energy usage per staff (per 12 m2) = 12m2 x 23.139 kWh = 277.668 kWh - 277.668 kWh = 1GJ - Australian National Greenhouse Accounts Factors (dcceew.gov.au) Emission factor for natural gas: Scope 1 = 51.3 kgCO2e/GJ, Scope 3 = 4.0 kgCO2e/GJ - (1x 51.53 kgCO2e/GJ) + (1x4.0 kgCO2e/GJ) - = 55.53 kgCO2e - 55.53 kgCO2e x 0.5 (FTE) x 6 years = Total = 166.59 kgCO2e Spain - UK benchmark and emission factor data used as a proxy for average office building energy consumption. - 431.56 kgCO2e x 0.3 x 6 = 776.8 kgCO2e | 3532.8 kgCO2e |
| - Trial team commuting | E.g. Car, rail, bus, walking etc | Trial duration: 6 years Trial staff FTE: UK - 1, Spain - 0.3, Australia - 0.5 | For 1 FTE for 1 year, total average commuting emissions = 1027.8 kgCO2e Multiply by the number of years and FTE applicable UK = 1027.8 x 6 = 6166.8 kgCO2e Australia = 1027.8 kgCO2e x 0.5 x 6 years = 3083.4 kgCO2e Spain = 1027.8 kgCO2e x 0.3 x 6 years = 1850 kgCO2e | 11100.2 kgCO2e |
| 3. Trial specific meetings and travel |  |  |  |  |
| - Visits and travel to site - Travel to meetings | E.g. Feasibility, site initiation and monitoring visits, audits, inspections, Trial Management Group (TMG), Trial Steering Committee (TSC), Independent Data Monitoring Committee (IDMC), and investigator meetings, Patient and Public Involvement and Engagement (PPIE), conferences, scientific meetings etc | Australia feasibility visit: - Flight from Melbourne to Brisbane - Estimated distance travelled = 1375 km - Number of passengers = 2 Site Initiation visits: Assumption: Rail used for all travel UK site visits UK total p.km = 3868.6 p.km Australia total p.km = 9736 p.km Spain: 6 teleconferences, 34 total attendees Monitoring visits: UK total p.km = 12957.4 p.km Australia total p.km = 15310 p.km Spain total p.km = 3280 p.km Audits Long haul to/from UK flights = 29442 p.km Short haul = 13464 p.km International = 11444 p.km Other - In person visit to Spanish CTU, 2 ICR-CTSU travelled to Barcelona: 2x2x(1309km) = 5236 p.km - RMH lab visit, 115.8 p.km Meetings Joint IDMC/TSC: all teleconferences, 5 meetings, 3 external attendees TMG: 5 meetings, all teleconferences, 16 attendees Conferences: - 2 attendees to a conference in Bristol, p.km = 785.2 - 1 attendee to conference in Nottingham, p.km = 518.2 | Number of passengers x total distance (km) = p.km For national rail: p.km x 0.04441= kgCO2e For flights: p.km x relevant emission factor below: - Domestic (average): 0.27278 - Short-haul (average) to/from UK: 0.17034 - Long-haul (average) to/from UK: 0.21423 - International (average) to/from non-UK: 0.20373 NB: Distances may be calculated using google maps and calculated from CTU to destination Videoconferencing = 157 grams CO2e per hour. Feasibility visits Australia: - 2 x (2 x 1375) = 5500 p.km - 5500 p.km x 0.20373 = 1120.5 kgCO2e Site initiation and pharmacy visits UK: 3868.6 p.km x 0.0441 = 171.8 kgCO2e 1 teleconference , 4 attendees – 0.157 kgCO2e/hr x 4 = 0.628 kgCO2e Australia: 9736 p.km x 0.20373 = 1983.5 kgCO2e Spain (all teleconference) = 34 x 0.157 kgCO2e/hr = 5.3 kgCO2e Monitoring visits UK: 12957.4 p.km x 0.04441 = 575.4 kgCO2e Australia: 11074 p.km x 0.20373 = 3119.1 kgCO2e Spain: 3280 p.km x 0.17034 = 558.7 kgCO2e Audits Long haul to/from UK flights = 29422 p.km x 0.21423 = 6303.1 kgCO2e Short-haul = 13464 p.km x 0.17034 = 2293.5 kgCO2e International = 11444 p.km x 0.20373 = 2331.5 kgCO2e Other - ICR-CTSU trial staff visit to Barcelona: 5236 p.km x 0.17034 = 891.9 kgCO2e - Lab visit: 115.8p.km x 0.04441 = 5.14 kgCO2e Meetings IDMC/TSC: 5x3x 0.157 kgCO2e/hr = 2.4 kgCO2e TMG: 5 x 18 x 0.157 kgCO2e/hr = 12.6 kgCO2e Conferences - Bristol: 785.2 p.km x0.04441 = 34.9 kgCO2e - Nottingham: 518.2 p.km = 23 kgCO2e | 19433 kgCO2e |
| - Hotel stays | E.g. monitoring visits, audits, inspections etc | UK hotel stays: 30 stays Assumption: Hotel stays for all visits to Christie, Nottingham and Newcastle (same day visits to RMH, Bristol and UCLH) Visit to Spain CTU: 1 room, 7 nights Australia: 1 room, 3 nights | For UK: number of hotel rooms x number of nights x 13.9 = kgCO2e For UK (London) = number of rooms x number of nights x 13.8 = kgCO2e. For other countries use conversion factors from orange ‘hotel stay’ tab: conversion-factors-2021-full-set-advanced-users.xlsm (live.com) Alternatively, you may use cost-based method: £ spent x 0.358 = kgCO2e Trial calculations: UK hotel stays for monitoring visits: 13.9 x 30 = 417 kgCO2e Visits to Spain CTU: 7 x 18.7 = 130.9 kgCO2e Australia hotel stays: 42.6 x 3 = 127.8 kgCO2e | 675.7 kgCO2e |
| - Sustenance | E.g. meeting lunches, hotel dinners | Number of lunches or dinners: Or £ spent: £503.65 | Meeting lunches or hotel dinners (vegetarian) = 2.6 kgCO2e per meal per person Meeting lunches or hotel dinners (meat) = 5.92 kgCO2e per meal per person Or cost-based approach = £ x 0.708 = kgCO2e £503.65 x 0.708 = 356.6 kgCO2e | 356.6 kgCO2e |
| 4. Intervention 4.1. Physical (IMP) 4.2. Clinical (non-IMP) 4.3. Other (not captured above) Please fill out the section most relevant to the intervention being investigated. |  |  |  |  |
| 4.1. Physical |  |  |  |  |
| 4.1.1, 4.1.2. Movement of intervention, or materials required to deliver the intervention | E.g. movement of intervention from manufacturing site to distribution site, shipment of IMP to participating sites or direct to participant | NB: all shipping of IMP ambient Movement of IMP from manufacturing site to QP site for QP declaration: Assumption: 1 shipment Assumption: 1 pill bottle contains 35 tablets - weight ~30g Assumption: ~4000 IMP bottles shipped Flight distance from Puerto Rico to QP site = 6697 km Shipment weight = 4000x 30g = 120000g = 0.118 tonnes Movement of IMP from QP site to distribution/packaging site: Distance from QP site to distribution/packaging site = 352.4 km Shipment weight = 0.118 tonnes Movement of IMP from distribution/packaging site to participating sites: UK deliveries: 7.7 t.km Spain deliveries: 13 t.km Australia deliveries: 184.6 t.km | Carry out freight calculation as described in section 1.2. Movement of IMP from manufacturing site to Macclesfield for QP declaration: - Long-haul (to/from UK) = 1.13047 - 0.118 tonnes x 6697km = 790.246 t.km - 790.246 t.km x 1.13047 = 893.4 kgCO2e Movement of IMP from manufacturing site to distribution/packaging site (Fisher Clinical services): - 0.118 tonnes x 352.4km = 41.5832 t.km - 41.5832 t.km x 0.19443 = 8.1 kgCO2e Movement of IMP from distribution/packaging site to participating sites: UK deliveries: 7.7 t.km x 0.19443 = 1.5 kgCO2e Spain deliveries: - short - haul (to/from UK) = 2.55439 - 13 t.km x 2.55439 = 33.3 kgCO2e Australia deliveries: - Long-haul (to/from UK) = 1.13047 - 184.6 t.km x 1.13047 = 208.7 kgCO2e | 1145 kgCO2e |
| 4.1.3. Materials required for the packaging and shipment of IMP | E.g. cardboard, cold storage boxes, polystyrene | Kg of cardboard/ polystyrene: | E.g. Single use sample cold storage box = 25.2 kgCO2e per box E.g. Reusable sample cold storage box = 2.2 kgCO2e per box E.g. Kg (cardboard) x 0.821 = kgCO2e E.g. Kg (polystyrene) x 3.778 = kgCO2e | Data for IMP shipment packaging unobtainable from trial records. Item not included for CASPS. |
| 4.1.4. Destruction of overage | E.g. incineration of IMP | Estimated weight of overage incinerated: - Total bottles destroyed = 1464 - 1464 x 30g = 43.92 kg | Kg of waste x 2.4252 = kgCO2e 43.92 kg x 2.4252 =106.5 kgCO2e | 106.5 kgCO2e |
| 5. Data collection and exchange NB: analysis of data does not need to be calculated separately, it is covered by the emissions attributed to trial staff FTE in “CTU emissions” and calculations included within “Data Collection and exchange”. |  |  |  |  |
| - Data collection and query exchange between CTU and sites | E.g. CRFs, EDC completion and query resolution, scans copied to CDs | CASPS hospital files 35 x large folders (500 pages) - 500 x 35 = 17500 104 x small (150 pages) - 104 x 150 = 15600 Total pages = 17500 + 15600 = 33100 pages Scans 3 scans copied onto a CD per patient, 47 patients total 5 mins laptop time to burn CD per patient Postage of scans: UK: 0.278671 t.km Australia: 3.25 t.km Spain: 0.28 t.km CRFs Postage of CRF booklets to sites: UK sites, total t.km = 4.41 t.km Australia sites, total t.km = 102.2 t.km Spain sites, total t.km= 8.58963 t.km Postage of CRFs from sites to CTU: UK sites, total t.km = 6.37 Australia sites, total t.km = 132.9 Spain sites, total t.km = 13.8 | CASPS hospital files 33100 x 0.005kg = 165.5 kg Paper (printing) = 165.5kg x 0.22438= 37 kgCO2e Paper (materials) = 165.5kg x 0.919 = 152.1 kgCO2e Scans The carbon footprint of manufacturing a CD = 0.83 kgCO2e per CD 0.83 kgCO2e x 47 = 39.01 kgCO2e The carbon footprint of copying the scans on to a CD using a computer = 0.18079 kgCO2e per hour 5mins x 47 = 235 minutes = 3.91666667 hours 0.18079 kgCO2e x 3.91666667 hours = 0.71 kgCO2e Postage of scans: UK: 0.26115 t.km x 0.19443 = 0.05 kgCO2e Australia: 3.25 t.km x 1.13047 = 3.7 kgCO2e Spain: 0.28 t.km x 2.55439 = 0.72 kgCO2e CRFs Postage of CRF booklets to sites: UK: 4.41 t.km x 0.19443 = 0.86 kgCO2e Australia: 102.2 t.km x 1.13047 = 115.5 kgCO2e Spain: 8.58963 t.km x 2.55439 = 21.9 kgCO2e Postage of CRFs from sites to CTU: UK: 6.37 t.km x 0.19443 = 1.2 kgCO2e Australia: 132.9 t.km x 1.13047 = 150.2 kgCO2e Spain: 13.8 t.km x 2.55439 = 35.3 kgCO2e | 558.3 kgCO2e |
| - Data sent direct from participants to CTU or participating sites | E.g. Questionnaires, patient diaries, wearables | Estimated weight and distance of delivery (t.km): Device used and time taken to complete electronic questionnaires: | For paper questionnaires, please refer to section 1.1. for the carbon footprint of producing the materials and section 1.2. for postage (freight). For use of smart watches and other devices see section 6.3. Electronic questionnaires Add the emissions attributed to data storage and transmission to the emissions attributed to using a device to complete the questionnaire. Web surfing (data storage and transmission) = 9.441 g CO2e/hr (10 mins = 1.57 g CO2e) Choose from the below: - Desktop computer = 0.18079 kg CO2e per hour - Laptop = 0.028719 kg CO2e per hour - Tablet = 0.027397 kg CO2e per hour - Smartphone = 0.015068 kg CO2e per hour | N/A |
| - Data from labs to CTU - Data from other collaborators to CTU | E.g. Laboratory patient results, data linkage | GB required for data storage/transmission: Duration of data storage: Total £ spent on computer services: | Data storage and transmission = estimate 1.365 kg CO2e per GB per year. For computer services such as data linkage: £ x 0.149 = kgCO2e | N/A |
| 6. Trial supplies and equipment |  |  |  |  |
| - Equipment used by CTU | E.g. computers, laptops, printers, software | Total £ spent on office machinery and computers for trial: £1350 | For any new office machinery and computers purchased specifically for trial: £ x 0.387 = kgCO2e £1350 x 0.387 = 522.45 kgCO2e | 522.45 kgCO2e |
| - Equipment and supplies used by participating sites supplied by CTU | E.g. centrifuge, fridge, freezer | Estimated weight and distance of deliveries (t.km): | For the shipment of equipment to participating sites, please refer to section 1.2. For the use of a centrifuge, please refer to section 9.2., for a fridge or freezer please refer to section 9.3. | N/A |
| - Equipment and supplies provided to participants specifically for the trial | E.g. wearables, smartphone, tablet | Number of devices and duration of their usage: Estimated weight and distance of deliveries (t.km): | Smartphone = 55 kgCO2e from manufacture and add 5.5 kgCO2e per year of usage. Tablet = 119 kgCO2e from manufacture and add 10 kgCO2e per year of usage. Wearables/smart watch = 30.1 kg CO2e for manufacture and add 1.633 kg CO2e per year of usage. To calculate the carbon footprint associated with shipment of the devices, please refer to section 1.2. | N/A |
| 7. Trial specific patient assessments |  |  |  |  |
| - Patient travel for study visits that are in addition to standard of care | E.g. Eligibility and screening assessments, trial-specific assessments and procedures | Number of patients: 47 On average patients were on trial 72 weeks. Number of times patient were required to travel (in addition to standard of care): 880 total patient visits (496 total visits by patients receiving IMP, 384 total visits by patients receiving placebo) | Emissions associated with one patient visit to hospital (UK) = 5.8 kgCO2e (this includes both the out and back journeys) Emissions associated with one patient visit to GP surgery (UK) = 1.12 kgCO2e (this includes both the out and back journeys) 5.8 kgCO2e x 880 = 5104 kgCO2e | 5104 kgCO2e |
| - Materials and activities required for study assessments that are in addition to standard of care | E.g. Laboratory tests, imaging assessments, clinical activities relating to intervention for example administering of study drug, biopsy. | On average patients were on trial 72 weeks. Within that time: - CT scans per patient - 754 total visits requiring consumables | Consumables = 0.30 kgCO2e per patient per trial appointment where consumables (such as gloves) required - 1 MRI = 24.7 kg CO2e - 1 CT scan = 9.2 kgCO2e - 1 hour in surgery = 53 kg CO2e - 1 low intensity (general ward) bed day = 37.9 kg CO2e - 1 high intensity (ICU) bed day = 103 kgCO2e - 15 sessions of breast radiotherapy = 5.7 kg CO2e - 20 sessions of prostate radiotherapy = 15.3 kg CO2e Blood tests: - 82 g CO2e for coagulation profile - 116 g CO2e for full blood examination - 49 g CO2e for arterial gas assessment - 99 g CO2e for urea and electrolyte assessment - 0.5 g CO2e for C-reactive protein Please note that the above figures for blood tests include the materials and consumables required for sample collection, phlebotomy and analysis, as well as power consumption by pathology analysers. Trial calculations: CT scans: 47 x 9 x 9.2 kgCO2e = 3891.6 kgCO2e Consumables: 0.30 kgCO2e x 754 = 226.2 kgCO2e | 4117.8 kgCO2e |
| - Utilities required for study assessments that are in addition to standard of care | E.g. energy consumption per square metre of hospital space according to trial staff FTE, taking into account time required for CRF completion and study assessments, consent etc | Trial staff FTE (FTEs added up over whole trial duration): UK – 0.85 Spain – 0.25 Australia – 0.23 Australia benchmark and emission factor data: - Australian average hospital energy intensity = 1676 MJ = 465.55 kWh/m2. - Energy intensity per employee 16.5m2 x 465.55 = 7681.6 kWh -50:50 Electricity/gas split, therefore 3840.8 kWh electricity, 3840.8 kWh natural gas - Australia hospital sites located in NSW and QLD where electricity emission factor is 0.81 kgCO2e/kWh. - Australian National Greenhouse Accounts Factors (dcceew.gov.au) Emission factor for natural gas: Scope 1 = 51.3 kgCO2e/GJ, scope 3 = 13.1 kgCO2e/GJ | 1 FTE 1 year = 364.9 kgCO2e Multiply by the number of years and FTE applicable UK: 364.9 kgCO2e x 0.85 = 310.2 kgCO2e Spain: 364.9 kgCO2e x 0.25 = 91.2 kgCO2e Australia: 3840.8 kWh x 0.81 x 0.23 (FTE) = 715.5 kgCO2e Heating: 1 FTE, 1 year = 685.7 kgCO2e Multiply by the number of years and FTE applicable UK: 685.7 kgCO2e x 0.85 = 582.8 kgCO2e Spain: 685.7 kgCO2e x 0.25 = 171.4kgCO2e Australia: 3840.8 kWh = 13.82688 GJ - (13.82688 GJ x 51.53 kgCO2e/GJ) + (13.82688 GJ x 13.1 kgCO2e/GJ) - = 893.6 kgCO2e - x 0.23 (FTE) = 205.5 kgCO2e | 2076.6 kgCO2e |
| 8. Samples |  |  |  |  |
| - Materials involved | E.g. sample collection kit and packaging for shipment | Kg of material: - Safety kit (average plastic): 3kg - Cryotubes (PP): 6.88kg - Falcon tubes (PP): 6.88kg - PAXgene tube (PET): 6kg - Sea airbags (PET): 1.31kg - Labels (PET): 1.6kg - CTAD tubes (glass): 3.6335 kg - CPT (glass): 3.2kg - Jiffy bags (paper): 6kg - Paper: 1.53kg - Dry ice: 85kg - Sample cold storage box: 29 single use boxes | The emissions attributed to sample collection consumables for common blood tests are included in the blood tests listed in section 7.2. To calculate the carbon footprint of other common materials, multiply the weight in kg by the relevant emission factor below to produce kgCO2e. - Average plastics: 3.116 - Plastics (average film): 2.754 - Plastics (Average rigid): 3.277 - Plastics (PP): 3.105 - Plastics (PET): 4.032 - Glass: 1.403 - Paper: 0.919 - Board: 0.821 - Dry ice: 1.81kg CO2e for 1 kg dry ice produced/used - Sample cold storage box (SU): 25.2 kgCO2e per cold storage box Safety kit: 3kg x 3.116 = 9.35 kgCO2e Cryotubes + falcon tubes: 13.76kg x 3.105 = 42.7 kgCO2e PAXgene, sea airbags and labels: 8.9 1kg x 4.032 = 35.9 kgCO2e CTAD+CPT: 6.8225 kg x 3.413 = 9.57 kgCO2e Jiffy bags: 6kg x 0.881 (paper and board mixed) = 5.29 kgCO2e Paper: 1.53kg x 0.919 = 1.4 kgCO2e Paper (printing): 1.53kg x 0.22438 = 0.34 kgCO2e Dry ice: 85kg x 1.81 = 153.85 kgCO2e Sample boxes: 29 boxes x 25.2 kgCO2e = 730.8 kgCO2e | 989.2 kgCO2e |
| - Movement of sample kit materials from manufacturer to CTU - Movement of sample kits from CTU/distributor to participating Sites - Movement of samples from participating sites or patients to central laboratory. | E.g. shipment of blood tubes for sample kits to CTU | - Movement of sample kit materials from manufacturer to CTU - T.km = 11.9 - Movement of sample kits from CTU/distributor to participating Sites: - UK sites, total t.km = 11.9 t.km - Australia, total t.km = 19.8528 t.km - Spain, total t.km = 10.2 t.km - Movement of samples from participating sites to laboratory. - UK sites, total t.km = 32.7 t.km - Australia, total t.km = 663.65 t.km - Spain, total t.km = 51.486 t.km Movement of samples between laboratories: - 0.8676 t.km | Please refer to section 1.2. for freight and 4.1. for refrigerated or frozen freight. Movement of sample kit materials from manufacturer to CTU: - 11.9 t.km X 0.19443 = 2.31 kgCO2e Movement of sample kits from CTU/distributor to participating Sites: - UK sites:11.9 t.km x 0.19443 = 2.3 kgCO2e - Australia: 19.8528 t.km x 1.13047 = 22.4 kgCO2e - Spain: 10.2 t.km x 2.55439 = 26 kgCO2e - Movement of samples from participating sites to laboratory. - UK sites: 32.7 t.km x 0.19443 = 6.4 kgCO2e - Australia: 663.65 t.km x 1.13047 = 750.2 kgCO2e - Spain: 51.486 t.km x 2.55439 = 131.5 kgCO2e Movement of samples between laboratories: - 0.8676 t.km x 0.19443 = 0.17 kgCO2e | 941.3 kgCO2e |
| 9. Laboratory |  |  |  |  |
| - Emissions attributed to lab utilities according to staff FTE | E.g. energy consumption per square metre of laboratory space according to trial staff FTE | Trial duration: Trial staff FTE: | Electricity: 1747.2 kgCO2e per FTE per year Multiply 1747.2 kgCO2e by the number of years and FTE applicable Heating: 1376 kgCO2e per FTE per year Multiply 1376 kgCO2e by the number of years and FTE applicable | N/A - No central labs within the scope of this trial. This module refers to the site laboratories where initial processing of samples and sample storage occurs. |
| - Materials/equipment/consumables used in processing and analysis of samples | E.g. centrifuges, refrigerators | Centrifuges required for processing the following samples: CTAD samples - Samples centrifuged for 15 mins (0.25 hours) - 310 watts - 310 x 0.25 = 77.5 kWh - 313 samples CPT samples - Samples centrifuged for 45 mins (0.583 hours) - 310 Watts - 310 x 0.583 = 180.73 kWh - 261 samples | To avoid double counting, use of equipment will be included in lab staff FTE if calculated. If the trial does not involve a central lab, but there is still sample processing on site, please see below. For storage of samples, please see section 9.3. To calculate the emissions of a piece of equipment, multiply the power consumption by hours used to get a kWh value. Finally multiply kWh by the electricity emission factor (0.273). Consider the centrifuge capacity and multiply by the number of uses required. Centrifuge – CTAD Samples: - 77.5 kWh x 0.273 = 21.2 kgCO2e / 24 = 0.881kgCO2e per sample (Assumption: centrifuge capacity is 24 samples) - 0.881 x 313 = 275.9 kgCO2e Centrifuge – CPT samples: - 180.73 kWh x 0.273 = 49.33929 kgCO2e / 24 = 2.1 kgCO2e per sample (Assumption: centrifuge capacity is 24 samples) - 2.1 x 261 samples = 536.6 kgCO2e Total = 275.9 kgCO2e + 536.6 kgCO2e = 812.5 kgCO2e | 812.5 kgCO2e |
| - Storage of samples | E.g. utilities and ultra-low temperature freezer | Storage of blood samples in fridge/freezer: Length of storage = 10 years Amount of refrigerator/freezer space required = 1/5 of a fridge, 1/3 of a freezer Storage of ambient tissue: - 10 years, 2011-2021 - Assumption: tissue samples occupy 1m2 | Storage in fridge/-20 freezer: - 298.9 kgCO2e per year - Multiply by number of years stored Storage in an ultra-low/-80 freezer: - 2192.2 kgCO2e per year - Multiply by number of years stored NB: this is for a whole freezer; you will need to make an assumption about the amount of space in the freezer that the trial samples take up. Fridge/-20: 298.9 kgCO2e x 10 years = 2989 kgCO2e 2989 kgCO2e x 0.20 = 597.8 kgCO2e Storage in an ultra-low/-80 freezer: 2192.2 kgCO2e per year x 10 = 21922 kgCO2e 21922 kgCO2e x 0.333 = 7307.3 kgCO2e Ambient storage of tissue: Electricity: - Health building = 22.1 kgCO2e/ m2 - 22.1 kgCO2e/ m2 x 10 = 221 kgCO2e Heating: 41.5 kgCO2e x 10 = 415 kgCO2e | 8541.1 kgCO2e |
| 10. Trial close out |  |  |  |  |
| 10.1. Storage and archiving of essential trial documentation and data | E.g. Hospital files, lab files, trial guidance documents etc | Duration of storage: 10 years Amount of space required for storage (m2): Assumption: archiving documents requires 4m2 Storage location: CTU (office) | Carbon footprint associated with 1m2 for 1 year: - Office: 16.1 kgCO2e - Laboratory: 43.7 kgCO2e - Warehouse: 7.4 kgCO2e - Health building: 22.1 kgCO2e Choose the most suitable building type and multiply by number of years and m2 necessary. 16.1 kgC02e x 4 x 10 = 644 kgCO2e Heating Carbon footprint associated with 1 m2 for 1 year: - Office: 35.69 kgCO2e - Laboratory: 34.4 kgCO2e - Warehouse: 13.3 kgCO2e - Health building: 41.5 kgCO2e Choose the most suitable building type and multiply by number of years and m2 necessary. 35.69 kgCO2e x 4 x 10 = 1427.6 kgCO2e For electronic storage, estimate 1.35 kgCO2e per GB per year. Size of CASPS on server = ~1GB 1.365 kgCO2e x 10 = 13.65 kgCO2e | 2085.3 kgCO2e |
| 10.2. Storage and destruction of biological samples | E.g. blood, tissue, urine etc | Duration of storage: 10 years Amount of space required for storage (m2): 1 Storage location: hospital/health building | See section 9.3. for storage of refrigerated or frozen samples. See section 10.1 for storage of ambient samples. Health building Electricity: 22.1 kgCO2e x 10 = 221 kgCO2e Heating: 41.5 kgCO2e x 10 = 415 kgCO2e | 636 kgCO2e |
| 10.3. Return of equipment and supplies from participating sites to CTU | E.g. wearables, unused or expired equipment and supplies | Estimated weight and distance of delivery (t.km): | See section 1.2. (freight). | N/A |

CASPS carbon footprint summary

| Module | KgCO2e |
| --- | --- |
| Trial set up | 349.8 |
| CTU emissions | 23487.6 |
| Trial staff meetings and travel | 20465.3 |
| Treatment intervention | 1251.5 |
| Data collection and exchange | 558.3 |
| Trial supplies and equipment | 522.5 |
| Trial specific patient assessments | 11298.4 |
| Samples | 1930.5 |
| Laboratory | 9353.6 |
| Analysis and trial close out | 2721.3 |
| Total = | 71938.8 kgCO2e / 71.9 tonnesCO2e |

References

What is an emission factor? [Internet]. Climfoot-project.eu. [cited 2023 May 11]. Available from: https://climfoot-project.eu/en/what-emission-factor

For all emission factor and benchmark data sources, please refer to the accompanying “Detailed Guidance and method to calculate the carbon footprint of a clinical trial.”

Assumption: as the international sites are also located in cities, the same patient to GP distance was used for Australia and Spain

Assumption: the average commuting statistics for the UK were found to be similar to those for Spain and Australia and therefore also applied to the international CTUs.

As per assumptions detailed in the guidance, manufacture of the intervention is considered out of scope. This section defines all processes relating to providing and delivering the trial intervention that are over and above routine care.