| Mooring | Location | Water Depth [m] | Instrument | Height of Most Proximal Data Above Bed [m] | Height of Most Distal Data Above Bed [m] | Bin size [m] | Deployment Date | Recovery Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C02 | Open Slope | 1768 | Aanderaa Seaguard Current Meter | 3 & 5 |  |  | 2013-03-11 | 2014-10-25 |
| C03A | Tungue Canyon | 1014 | Aanderaa Seaguard Current Meter | 3 & 5 |  |  | 2013-03-11 | 2014-04-20 |
| C03B | Tungue Canyon | 1710 | Aanderaa Seaguard Current Meter | 3 & 5 |  |  | 2014-04-25 | 2014-10-25 |
| C04 | Open Slope | 993 | Aanderaa Seaguard Current Meter | 3 & 5 |  |  | 2013-03-11 | 2014-10-26 |
| C05A | Tungue Canyon | 603 | Upward-looking RDI 300 kHz ADCP | 15 | 111 | 2 | 2013-03-12 | 2013-12-04 |
|  |  |  | Aanderaa Seaguard Current Meter | 3 & 5 |  |  |  |  |
| C05B | Open Slope | 595 | Upward-looking RDI 300 kHz ADCP | 15 | 113 | 2 | 2013-12-05 | 2014-10-25 |
|  |  |  | Aanderaa Seaguard Current Meter | 3 & 5 |  |  |  |  |
| C06 | Open Slope | 199 | Aanderaa Seaguard Current Meter | 3 & 5 |  |  | 2013-03-09 | 2014-10-26 |
| C08 | Open Slope | 520 | Aanderaa Seaguard Current Meter | 3 & 5 |  |  | 2013-03-10 | 2014-10-26 |
| C09B | Afungi Canyon | 1822 | Aanderaa Seaguard Current Meter | 3 & 5 |  |  | 2014-04-27 | 2014-10-25 |
| C11B | Mebuisi Canyon | 1506 | Aanderaa Seaguard Current Meter | 3 & 5 |  |  | 2014-04-26 | 2014-10-25 |
| C12 | Open Slope | 197 | Aanderaa Seaguard Current Meter | 3 & 5 |  |  | 2013-03-09 | 2014-10-25 |
| CM06A | Tungue Canyon Head | 116 | Downward-looking 600 kHz ADCP | 10.5 | 21 | 0.5 | 2011-12-23 | 2012-04-26 |
| CM06B | Vamizi Canyon | 1126 | Downward-looking 600 kHz ADCP | 10.5 | 20 | 0.5 | 2012-10-05 | 2015-03-31 |
| CM08A | Open Slope | 686 | Downward-looking 600 kHz ADCP | 10.5 | 23 | 0.5 | 2011-12-24 | 2012-05-05 |
| CM08B | Open Slope | 1531 | Downward-looking 600 kHz ADCP | 10.5 | 15 | 0.5 | 2011-09-29 | 2015-01-15 |
| CM09A | Afungi Canyon | 431 | Downward-looking 600 kHz ADCP | 10.5 | 21.5 | 0.5 | 2011-12-24 | 2013-02-05 |
| CM09B | Afungi Canyon | 948 | Downward-looking 600 kHz ADCP | 10.5 | 20 | 0.5 | 2013-02-10 | 2013-05-10 |
| CM09C | Afungi Canyon | 449 | Downward-looking 600 kHz ADCP | 10.5 | 18 | 0.5 | 2013-09-07 | 2014-07-01 |
| CM10 | Gully | 546 | Downward-looking 600 kHz ADCP | 10.5 | 16.5 | 0.5 | 2012-09-27 | 2015-03-31 |
| CM11 | Tungue Canyon | 1583 | Downward-looking 600 kHz ADCP | 10.5 | 15 | 0.5 | 2011-12-25 | 2015-04-02 |
| CM12A | Afungi Canyon | 1842 | Downward-looking 600 kHz ADCP | 10.5 | 17 | 0.5 | 2011-12-25 | 2012-04-02 |
| CM12B | Afungi Canyon | 1766 | Downward-looking 600 kHz ADCP | 10.5 | 13.5 | 0.5 | 2012-10-04 | 2015-03-29 |
| CM13A | Open Slope | 1503 | Downward-looking 600 kHz ADCP | 10.5 | 17 | 0.5 | 2012-09-29 | 2013-06-26 |
| CM13B | Open Slope | 1521 | Downward-looking 600 kHz ADCP | 10.5 | 16 | 0.5 | 2013-11-16 | 2015-04-01 |
| CM15 | Gully | 1572 | Downward-looking 600 kHz ADCP | 10.5 | 19 | 0.5 | 2013-02-05 | 2015-01-17 |
| CM16 | Vamizi Canyon | 1811 | Downward-looking 600 kHz ADCP | 10.5 | 17.5 | 0.5 | 2013-02-04 | 2015-03-31 |
| D1 | Gully | 717 | Downward-looking 600 kHz ADCP | 11 | 35 | 1 | 2014-07-28 | 2015-04-18 |
| D2 | Gully | 1213 | Downward-looking 600 kHz ADCP | 11 | 34 | 1 | 2014-07-27 | 2015-04-16 |
| D3 | Mebuisi Canyon | 923 | Downward-looking 600 kHz ADCP | 11 | 36 | 1 | 2014-07-21 | 2015-04-18 |
| D4 | Open Slope | 249 | Downward-looking 600 kHz ADCP | 11 | 38 | 1 | 2014-07-20 | 2015-04-20 |
| D5 | Mebuisi Canyon | 1606 | Downward-looking 600 kHz ADCP | 11 | 30 | 1 | 2014-07-26 | 2015-04-15 |
| D6 | Gully | 1522 | Downward-looking 600 kHz ADCP | 11 | 28 | 1 | 2014-07-27 | 2015-04-14 |
| D7 | Gully | 996 | Downward-looking 600 kHz ADCP | 11 | 35 | 1 | 2014-07-20 | 2015-04-17 |
| D8 | Tungue Canyon | 746 | Downward-looking 600 kHz ADCP | 11 | 34 | 1 | 2014-07-21 | 2015-04-20 |

Table S1. Specifications and location information for the moorings and instruments deployed offshore North Mozambique.