Table S1. Concentration of heavy metals found in Certified Reference Materials NMIJ CRM 7303-a lake sediment (means ± standard errors, in mg/kg as dry wt.) by AAS (n = 3).

| Element | Certified value | Measured value | Recovery (%) |
| --- | --- | --- | --- |
| As | 8.6±1.0 | 8.2±0.04 | 95 |
| Cr | 39.1±2.8 | 38.7±0.21 | 99 |
| Cd | 0.342±0.017 | 0.344±0.02 | 101 |
| Pb | 31.3±1.1 | 31.5±0.89 | 101 |

Table S2. Mathematical expressions and physical significances of environmental and ecological indices.

| Indices | Mathematical expressions | Classification | Contamination degree | References |
| --- | --- | --- | --- | --- |
| Geo-accumulation index, Igeo | Igeo= Log2Cn1.5 ×Bn Where, Cn is the measured elemental concentration in sediment and Bn is the background heavy metal concentrations | Igeo < 0 | Uncontaminated | Müller, (1981) |
|  |  | 1 ≤ Igeo < 2 | Uncontaminated to moderately contaminated |  |
|  |  | 2 ≤ Igeo < 3 | Moderately to heavily contaminated |  |
|  |  | 3 ≤ Igeo < 4 | Heavily contaminated |  |
|  |  | 4 ≤ Igeo < 5 | Heavily to extremely contaminated |  |
|  |  | Igeo ≥ 5 | Extremely contaminated |  |
| Contamination factor, CF | CF= CnBn Where, Cn and Bn are the measured and background elemental concentrations in sediment & water, respectively. | CF < 1 | Low contamination | Håkanson, (1980) |
|  |  | 1 ≤ CF < 3 | Moderate contamination |  |
|  |  | 3 ≤ CF < 6 | Considerable contamination |  |
|  |  | CF ≥ 6 | Very high contamination |  |
| Modified degree of contamination, mCd | mCd=1ni=1nCFi where, CF is the contamination factor | mCd < 1.5 | Uncontaminated | Håkanson, (1980); Abrahim, and Parker, (2008) |
|  |  | 1.5 ≤ mCd < 2 | Slightly contaminated |  |
|  |  | 2 ≤ mCd < 4 | Moderately contaminated |  |
|  |  | 4 ≤ mCd < 8 | Moderately to heavily contaminated |  |
|  |  | 8 ≤ mCd < 16 | Heavily contaminated |  |
|  |  | 16 ≤ mCd < 32 | Severely contaminated |  |
|  |  | mCd > 32 | Extremely contaminated |  |
| Degree of contamination, Cd | Cd=i=1nCFi where, CF is the contamination factor | Cd < 8 | Low degree of contamination | Håkanson, (1980) |
|  |  | 8 ≤ Cd < 16 | Moderate degree of contamination |  |
|  |  | 16 ≤ Cd < 32 | Considerable degree of contamination |  |
|  |  | Cd > 32 | High degree of contamination |  |
| Enrichment factor, EF | EF= (Cn/Fe)Sample(Bn/Fe)Background Where, Cn is the measured elemental concentration in sediment and Bn is the background heavy metal concentrations | EF < 1 | Elemental mobilization | (Birch & Olmos, 2008) |
|  |  | EF = 1 | Baseline data |  |
|  |  | 1 ≤ EF < 1.5 | Possible anthropogenic origin |  |
|  |  | 1.5 ≤ EF < 3 | Minor enrichment |  |
|  |  | 3 ≤ EF < 5 | Moderate enrichment |  |
|  |  | 5 ≤ EF < 10 | Severe modification |  |
|  |  | EF > 10 | Very severe modification |  |
| Pollution load index, PLI | PLI=(CF1× CF2×… CFn)1n where, CF is the contamination factor | PLI = 0 | Perfection | Sadhu et al. (2012) |
|  |  | PLI < 1 | Baseline level |  |
|  |  | PLI > 1 | Contaminated |  |
| Potential ecological risk factor, Eri | Eri=Tri×CFi Where, Tri is the biological toxic metal response factor (Zn=1, Ni=5, Mn=1, Pb=5, Cu=5, Cr=2 and Cd=30) and CF is the contamination factor. | Eri < 40 | Low risk | Håkanson, (1980) |
|  |  | 40 ≤ Eri < 80 | Moderate risk |  |
|  |  | 80 ≤ Eri < 160 | Considerable risk |  |
|  |  | 160 ≤ Eri < 320 | High risk |  |
|  |  | Eri ≥ 320 | Very high risk |  |
| Potential ecological risk index (PERI) | RI=i=1nEri Where Eri is the potential ecological risk factor. | RI < 150 | Low risk | Håkanson, (1980) |
|  |  | 150 ≤ RI < 300 | Moderate risk |  |
|  |  | 300 ≤ RI < 600 | Considerable risk |  |
|  |  | RI ≥ 600 | High risk |  |

Figure S1. Pollution load index of heavy metals in sediments of an urban river (Kirtankhola) in Bangladesh during summer and winter season.

Figure S2. Toxic unit (TU) of heavy metals in sediments of an urban river (Kirtankhola) in Bangladesh during summer and winter season.

Figure S3. PCA for heavy metals in water and sediment of an urban river (Kirtankhola) in Bangladesh.

Table S3. Contamination factors, degree of contamination, contamination level, and pollution load of heavy metals in sediment collected from river of Kirtankhola, Bangladesh

| Contamination factors () |  |  |  |  |  |  |  |  | Degree of contamination (Cd) |  | Contamination level |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sites | As |  | Cr |  | Cd |  | Pb |  | Summer | Winter | Summer | Winter |
|  | Summer | Winter | Summer | Winter | Summer | Winter | Summer | Winter |  |  |  |  |
| S1 | 0.31 | 0.48 | 0.57 | 0.76 | 0.13 | 0.16 | 0.28 | 0.36 | 1.29 | 1.77 | Low | Low |
| S2 | 0.35 | 0.59 | 0.33 | 0.45 | 0.12 | 0.14 | 0.60 | 0.84 | 1.40 | 2.02 | Low | Low |
| S3 | 0.30 | 0.38 | 0.38 | 0.51 | 0.06 | 0.11 | 0.65 | 0.93 | 1.39 | 1.92 | Low | Low |
| S4 | 0.44 | 0.61 | 0.67 | 0.74 | 0.11 | 0.13 | 0.51 | 0.64 | 1.73 | 2.12 | Low | Low |
| S5 | 0.33 | 0.57 | 0.63 | 0.80 | 0.19 | 0.25 | 0.59 | 0.86 | 1.74 | 2.48 | Low | Low |
| S6 | 0.27 | 0.46 | 0.53 | 0.71 | 0.14 | 0.17 | 0.51 | 0.56 | 1.45 | 1.91 | Low | Low |
| S7 | 0.22 | 0.56 | 0.44 | 0.58 | 0.22 | 0.27 | 0.65 | 0.91 | 1.54 | 2.31 | Low | Low |
| S8 | 0.39 | 0.64 | 0.81 | 1.06 | 0.28 | 0.31 | 0.95 | 1.13 | 2.43 | 3.15 | Low | Low |
| S9 | 0.40 | 0.63 | 0.74 | 1.16 | 0.29 | 0.34 | 1.07 | 1.27 | 2.50 | 3.40 | Low | Low |
| S10 | 0.49 | 0.84 | 0.78 | 0.95 | 0.33 | 0.41 | 0.74 | 0.95 | 2.34 | 3.15 | Low | Low |
| S11 | 0.50 | 0.70 | 0.93 | 1.06 | 0.29 | 0.37 | 0.83 | 1.06 | 2.55 | 3.18 | Low | Low |
| S12 | 0.36 | 0.47 | 0.58 | 0.69 | 0.21 | 0.24 | 0.56 | 0.77 | 1.71 | 2.17 | Low | Low |
| S13 | 0.38 | 0.54 | 0.46 | 0.57 | 0.10 | 0.10 | 0.51 | 0.70 | 1.45 | 1.92 | Low | Low |
| S14 | 0.43 | 0.55 | 0.50 | 0.56 | 0.18 | 0.19 | 0.59 | 0.71 | 1.70 | 2.02 | Low | Low |

Table S4. Varimax rotated factor loadings of the elements for water and sediment of an urban river (Kirtankhola) in Bangladesh

| Samples | Initial Eigen value |  |  |  |  | Rotation Sums of Squared Loadings |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Component | Total | %variance | Cumulative% |  | Total | % variance | Cumulative% |
| Water | 1 | 3.13 | 78.26 | 78.26 |  | 1.69 | 42.33 | 42.33 |
|  | 2 | 0.465 | 11.61 | 89.87 |  | 1.13 | 28.46 | 70.80 |
|  | 3 | 0.241 | 6.03 | 95.91 |  | 1.00 | 25.11 | 95.91 |
| Sediment | 1 | 2.98 | 74.53 | 74.53 |  | 1.58 | 39.70 | 39.70 |
|  | 2 | 0.46 | 11.50 | 86.04 |  | 1.12 | 28.06 | 67.77 |
|  | 3 | 0.38 | 9.49 | 95.53 |  | 1.11 | 27.75 | 95.53 |

Table S5. Correlation coefficient of heavy metals in water and sediment from Kirtankhola River, Bangladesh

| Water | Cr | As | Cd | Pb |
| --- | --- | --- | --- | --- |
| Cr | 1 |  |  |  |
| As | 0.650 | 1 |  |  |
| Cd | 0.828 | 0.580 | 1 |  |
| Pb | 0.770 | 0.639 | 0.779 | 1 |
| Sediment |  |  |  |  |
| Cr | 1 |  |  |  |
| As | 0.684 | 1 |  |  |
| Cd | 0.792 | 0.591 | 1 |  |
| Pb | 0.618 | 0.572 | 0.696 | 1 |

*. Correlation is significant at the 0.05 level (2-tailed).

Figure S1. Pollution load index (PLI) of heavy metals (summer and winter season) in Kirtankhola River, Bangladesh

Figure S2. Geoaccumulation index of heavy metals in Kirtankhola River, Bangladesh

Figure S3. Toxic units of heavy metals in Kirtankhola River, Bangladesh