# Supplementary information for:

# Enhanced electric field and charge polarity modulate the microencapsulation and stability of electrosprayed probiotic cells (Streptococcus thermophilus, ST44)

Panagiota Dima, Peter R. Stubbe, Ana C. Mendes*, Ioannis S. Chonakis*

Technical University of Denmark, DTU-Food, Research Group for Food Production Engineering, Laboratory of Nano-BioScience, Kemitorvet 202, 2800, Kgs. Lyngby, Denmark

*Corresponding authors: anac@food.dtu.dk, ioach@food.dtu.dk

### Calculations for freeFEM++:

Assuming no or negligible current and a steady state, it was found from Maxwell’s equations that the electric field E obeys:

∇⋅E=ρε0

( 1)

∇×E=0

( 2)

E=-∇U

where ρ is the charge density and ε0 is the permittivity of vacuum. The field was found as the negative gradient of the electrostatic potential U. The condition ∇×∇U=0 is automatically fulfilled.

Before starting electrospraying ρ=0, so (1) and (2) can be expressed as:

-ΔU=0

In cylindrical coordinates that is equal to:

-1r∂∂rr∂U∂r-1r2∂2U∂φ2-∂2U∂z2=0

For an axisymmetric case the φ dependent term vanishes. Multiplying with r led to:

-∂∂rr∂U∂r-r∂2U∂z2=0

The equation was multiplied with a helper function υ∈H01Ω with zero trace, and then integrated over the solution domain Ω:

-Ωυ∂∂rr∂U∂r-Ωυr∂2U∂z2=0

Integrating by parts and invoking Green’s theorem gave the weak form:

Ωr∂υ∂r∂U∂r-∂Ωrυ∂U∂r+Ωr∂υ∂z∂U∂z-∂Ωrυ∂U∂z=0

Since υ has zero trace, the two integrals on the boundary vanish and the equation is finally given by:

Ωr∂υ∂r∂U∂r+Ωr∂υ∂z∂U∂z=0

( 3)

For solving the problem in freeFEM++, first the boundary of the solution domain needed to be defined by parametric curves with the solution domain on the left side of the curve.

border b0(t=0., 1.){x=0.; y=W/2.-W*t;label=Ch;}

border b1(t=0., 1.){x=L*t;y=-W/2.;label=Ch;}

border b2(t=0., 1.){x=L;y=-W/2.+W*t;label=Ch;}

border b3(t=0., 1.){x=L-L*t;y=W/2.;label=Ch;}

Here W, L and Ch should be defined in advance as:

real W=5.5; // width of chamber

real L=5.5; // length of chamber

int Ch=1;

The boundary segments could be used to create a mesh representing the solution domain as:

mesh Th=buildmesh(b0(30)+b1(30)+b2(30)+b3(30)+b4(-1)+b5(-6)+b6(-1)+b7(-6)+b10(-20)+b11(-1)+b12(-20)+b13(-1));

The mesh used to define a finite element space was:

fespace Vh(Th,P3);

P3 was the type of elements selected. In this case triangular elements with polynomial interpolation functions. Other elements could be P1 with linear or P2 with polynomial interpolation functions instead of linear.

Vh uh, vh;

uh and vh were the functions defined in all of the solution domain. They were respectively U and υ from (3). The integral over Ω was implemented by the function int2d(), and the partial derivatives were implemented by the functions dx() and dy(). r and z were substituted by y and x. This led to the definition:

problem Electro(uh,vh) = int2d(Th)(y*dx(uh)*dx(vh) + y*dy(uh)*dy(vh))

+ on(Ch,uh=0) // surrounding

+ on(Ne,uh=-15000.) // needle

+ on(Co,uh=5000.); // collector

The problem was the solved by:

Electro;

The variable uh was the solution to the problem representing the electric potential. When the field strength was wanted, it could be calculated by the magnitude of the gradient as:

Vh field;

field=sqrt(dx(uh)^2+dy(uh)^2);

Figure 1. Visualization of the solution domain that has been divided in an optimized mesh of smaller triangular elements. The closer to the electrodes the smaller the triangles to better model the field. The grid (A) without the auxiliary ring-electrode, (B) with 1 ring-electrode, and (C) with 2 ring-electrodes, between the nozzle and the collector is presented.

### Code for freeFEM++:

load "Element_P3";

real starterror=0.02;

real[int] viso=[0,1e4,2e4,3e4,4e4,5e4,6e4,7e4,8e4,9e4,10e4,11.5e4,13e4,14.5e4,16e4,17.5e4,19e4,21e4,24e4,27e4,30e4];

real[int] isopot=[-15000,-14000,-13000,-12000,-11000,-10000,-9000,-8000,-7000,-6000,-5000,-4000,-3000,-2000,-1000,0,1000,2000,3000,4000,5000];

real W=5.5;  // width of chamber

real L=5.5;  // length of chamber

real ln=0.011;  // length of needle

real wn=0.00041; // width of needle

real xn=L/2.-0.05-ln;  // position of needle

real Rring=0.04;  // radius of ring

real rring=0.0005;  // dimension of ring material

real xring=W/2.-0.03;  // position of ring

real Rring2=0.04;  // radius of ring2

real rring2=0.00125;  // dimension of ring2 material

real xring2=W/2.-0.01;  // position of ring2

real wcoll=0.1;  // width of collector

real lcoll=0.002;  // thickness of collector

real xcoll=W/2.+0.05;  // position of collector

int Ch=1;

int Ne=2;

int Co=3;

int R1=4;

int R2=5;

// shape of chamber

border b0(t=0., 1.){x=0.; y=W/2.-W*t;label=Ch;}

border b1(t=0., 1.){x=L*t;y=-W/2.;label=Ch;}

border b2(t=0., 1.){x=L;y=-W/2.+W*t;label=Ch;}

border b3(t=0., 1.){x=L-L*t;y=W/2.;label=Ch;}

// shape of needle

border b4(t=0., 1.){x=xn;y=wn/2.-wn*t;label=Ne;}

border b5(t=0., 1.){x=xn+ln*t;y=-wn/2.;label=Ne;}

border b6(t=0., 1.){x=xn+ln;y=-wn/2.+wn*t;label=Ne;}

border b7(t=0., 1.){x=xn+ln-ln*t;y=wn/2.;label=Ne;}

// shape of ring

border b8(t=0., 2.*pi){x=xring+rring*cos(t); y=Rring+rring*sin(t);label=R1;}

border b9(t=0., 2.*pi){x=xring+rring*cos(t); y=-Rring+rring*sin(t);label=R1;}

border b10(t=0., 2.*pi){x=xring2+rring2*cos(t); y=Rring+rring2*sin(t);label=R2;}

border b11(t=0., 2.*pi){x=xring2+rring2*cos(t); y=-Rring+rring2*sin(t);label=R2;}

// shape of collector

border b12(t=0., 1.){x=xcoll; y=wcoll/2.-wcoll*t;label=Co;}

border b13(t=0., 1.){x=xcoll+lcoll*t; y=-wcoll/2.;label=Co;}

border b14(t=0., 1.){x=xcoll+lcoll; y=-wcoll/2.+wcoll*t;label=Co;}

border b15(t=0., 1.){x=xcoll+lcoll-lcoll*t; y=wcoll/2.;label=Co;}

////////////////////////////////////////////////////////////////////////////

{// 0 rings

// build mesh of all the contours

mesh Th=buildmesh(b0(30)+b1(30)+b2(30)+b3(30)+b4(-1)+b5(-6)+b6(-1)+

b7(-6)+b12(-20)+b13(-1)+b14(-20)+b15(-1));

plot(Th, wait=true);  // show mesh

fespace Vh(Th,P3);

Vh uh,vh,xd,yd,field;

real error=starterror;

problem Electro(uh,vh) = int2d(Th)( y*dx(uh)*dx(vh) + y*dy(uh)*dy(vh) ) // bilinear

+ on(Ch,uh=0) // surrounding

+ on(Ne,uh=-15000.) // needle

+ on(Co,uh=5000.); // collector

for(int i=0;i<4;i++){

Electro;  // solve

Th=adaptmesh(Th, uh, err=error); //refine mesh

error=error/2;

}

plot(Th, ps="grid0n.ps", wait=true);

plot(Th, ps="grid0nz.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], wait=true);

plot(uh, ps="isopot0n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], value=false, viso=isopot, wait=true); // plot isopotential

plot(uh, ps="isopotfill0n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], value=false, viso=isopot, fill=true, wait=true); // plot isopotential

// gradient

xd=dx(uh);

yd=dy(uh);

field=sqrt(dx(uh)^2+dy(uh)^2);

plot(field, ps="field0n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], value=false, fill=true, viso=viso, wait=true);

field=field>3e5?3e5:field;

plot(field, ps="fieldcapped0n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], value=false, fill=true, viso=viso, wait=true);

//  plot([xd,yd], ps="gradient0n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], cmm="field direction", wait=true);  // plot gradient

{

ofstream file("potentials0n.txt");

for(y=-0.1;y<=0.1;y+=0.2/19.)

for(x=xn+ln-0.05;x<=xn+ln+0.15;x+=0.2/19.)

file<<x<<" "<<y<<" "<<dx(uh)<<" "<<dy(uh)<<" "<<uh<<endl;

}

{

ofstream file("center0n.txt");

y=0.;

for(x=xn+ln;x<=xcoll;x+=(xcoll-xn-ln)/40)

file<<x<<"\t"<<xd<<"\t"<<yd<<"\t"<<uh<<endl;

}

{

ofstream file("coll0n.txt");

x=xcoll-0.0001;

for(y=-wcoll/2.;y<=wcoll/2.;y+=wcoll/40)

file<<y<<"\t"<<dx(uh)<<"\t"<<dy(uh)<<"\t"<<uh<<endl;

}

}

/////////////////////////////////////////////////////////////////////////////

{// 1 ring

// build mesh of all the contours

mesh Th=buildmesh(b0(30)+b1(30)+b2(30)+b3(30)+b4(-1)+b5(-6)+b6(-1)+

b7(-6)+b8(-20)+b9(-20)+b12(-20)+b13(-1)+b14(-20)+b15(-1));

plot(Th, wait=true);  // show mesh

fespace Vh(Th,P3);

Vh uh,vh,xd,yd,field;

real error=starterror;

problem Electro(uh,vh) = int2d(Th)( y*dx(uh)*dx(vh) + y*dy(uh)*dy(vh) ) // bilinear

+ on(Ch,uh=0) // surrounding

+ on(Ne,uh=-15000.) // needle

+ on(R1,uh=-5000.) // ring

+ on(Co,uh=5000.); // collector

for(int i=0;i<4;i++){

Electro;  // solve

Th=adaptmesh(Th, uh, err=error);

error=error/2;

}

plot(Th, ps="grid1n.ps", wait=true);

plot(Th, ps="grid1nz.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], wait=true);

plot(uh, ps="isopot1n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], value=false, viso=isopot, wait=true); // plot isopotential

plot(uh, ps="isopotfill1n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], value=false, viso=isopot, fill=true, wait=true); // plot isopotential

// gradient

xd=dx(uh);

yd=dy(uh);

field=sqrt(xd^2+yd^2);

plot(field, ps="field1n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], value=false, fill=true, viso=viso, wait=true);

field=field>3e5?3e5:field;

plot(field, ps="fieldcapped1n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], value=false, fill=true, viso=viso, wait=true);

//  plot([xd,yd], ps="gradient1n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], cmm="field direction", wait=true);  // plot gradient

{

ofstream file("potentials1n.txt");

for(y=-0.1;y<=0.1;y+=0.2/19.)

for(x=xn+ln-0.05;x<=xn+ln+0.15;x+=0.2/19.)

file<<x<<" "<<y<<" "<<dx(uh)<<" "<<dy(uh)<<" "<<uh<<endl;

}

{

ofstream file("center1n.txt");

y=0.;

for(x=xn+ln;x<=xcoll;x+=(xcoll-xn-ln)/40)

file<<x<<"\t"<<xd<<"\t"<<yd<<"\t"<<uh<<endl;

}

{

ofstream file("coll1n.txt");

x=xcoll-0.0001;

for(y=-wcoll/2.;y<=wcoll/2.;y+=wcoll/40)

file<<y<<"\t"<<dx(uh)<<"\t"<<dy(uh)<<"\t"<<uh<<endl;

}

}

////////////////////////////////////////////////////////////////////////////

{// 2 rings

// build mesh of all the contours

mesh Th=buildmesh(b0(30)+b1(30)+b2(30)+b3(30)+b4(-1)+b5(-6)+b6(-1)+

b7(-6)+b8(-20)+b9(-20)+b10(-20)+b11(-20)+b12(-20)+

b13(-1)+b14(-20)+b15(-1));

plot(Th, wait=true);  // show mesh

fespace Vh(Th,P3);

Vh uh,vh,xd,yd,field;

real error=starterror;

problem Electro(uh,vh) = int2d(Th)( y*dx(uh)*dx(vh) + y*dy(uh)*dy(vh) ) // bilinear

+ on(Ch,uh=0) // surrounding

+ on(Ne,uh=-15000.) // needle

+ on(R1,uh=-5000.) // ring

+ on(R2,uh=-5000.)

+ on(Co,uh=5000.); // collector

for(int i=0;i<4;i++){

Electro;  // solve

Th=adaptmesh(Th, uh, err=error);

error=error/2;

}

plot(Th, ps="grid2n.ps", wait=true);

plot(Th, ps="grid2nz.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], wait=true);

plot(uh, ps="isopot2n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], value=false, viso=isopot, wait=true); // plot isopotential

plot(uh, ps="isopotfill2n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], value=false, viso=isopot, fill=true, wait=true); // plot isopotential

// gradient

xd=dx(uh);

yd=dy(uh);

field=sqrt(xd^2+yd^2);

plot(field, ps="field2n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], value=false, fill=true, viso=viso, wait=true);

field=field>3e5?3e5:field;

plot(field, ps="fieldcapped2n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], value=false, fill=true, viso=viso, wait=true);

//  plot([xd,yd], ps="gradient2n.ps", bb=[[L/2.-0.2,-0.2],[L/2.+0.2,0.2]], cmm="field direction", wait=true);  // plot gradient

{

ofstream file("potentials2n.txt");

for(y=-0.1;y<=0.1;y+=0.2/19.)

for(x=xn+ln-0.05;x<=xn+ln+0.15;x+=0.2/19.)

file<<x<<" "<<y<<" "<<dx(uh)<<" "<<dy(uh)<<" "<<uh<<endl;

}

{

ofstream file("center2n.txt");

y=0.;

for(x=xn+ln;x<=xcoll;x+=(xcoll-xn-ln)/40)

file<<x<<"\t"<<xd<<"\t"<<yd<<"\t"<<uh<<endl;

}

{

ofstream file("coll2n.txt");

x=xcoll-0.0001;

for(y=-wcoll/2.;y<=wcoll/2.;y+=wcoll/40)

file<<y<<"\t"<<dx(uh)<<"\t"<<dy(uh)<<"\t"<<uh<<endl;

}

}