### Supplementary Material

Both in MCO and LO, the optimization problem is performed following the hierarchical order of a treatment site specific list of requests, a so called Wish-List (WL). The WLs are generated by imitating the plan discussion process between radiation oncologists and planners, and are characterized by clinical and planning constraints (CC and PC, respectively), which cannot be violated, and a list of prioritized objective functions according to their importance degree [19, 21]. While the Erasmus MC Cancer Institute of Rotterdam firstly introduced and implemented LO in the iCycle software [13], now mCycle is implemented into the Monaco TPS research version (v5.59.13).

In the previous study, an initial WL tweaking has been performed aiming to reproduce the manual plans (WL01). In this study a second WL was generated to investigate the possibility to improve the plan quality in terms of organs-at-risk (OARs) sparing without affecting plan delivery accuracy.

The two WLs are presented in Tables 1 and 2. In both cases, the fulfilment of the bowel bag constraint is indicated as clinical constraint because its violation implies plan rejection most of the time. It is followed by dose gradient requests (PC). The main differences between the two WLs regards the PTV coverage and the OAR mean doses requests priority order. In the WL02, a less strict PTV coverage request is performed as a 1st-priority request, and a last-priority level request was added to achieve PTV coverage as high as possible, while the bladder and rectum mean doses claims are swapped with the right- and left-femoral heads ones. In both WLs, if there was a double PTV with the same dose prescription (PTV uterus and PTV pelvis) the requests were doubled and kept both as first-priority objectives function.

Table 1 mCycle Wish-List 01 for auto-planning of cervical cancer at 50 Gy in 25 fractions.

| Wish-List 01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Clinical constraints |  |  |  |  |
|  | Structure | Cost Function (parameter) | Margin (cm) | Limit |
|  | PTV | Quadratic Overdose (52 Gy) |  | < 0.02 Gy |
|  | Bowel bag | Overdose DVH (45 Gy) |  | < 195.0 cm3 |
|  | External | Maximum Dose |  | 53.4 Gy |
| Planning Constraints |  |  |  |  |
|  | Structure | Cost Function (parameter values) | Margin (cm) | Limit |
|  | External | Quadratic Overdose (50 Gy) | 0.0 | < 0.1 Gy |
|  | External | Quadratic Overdose (45 Gy) | 0.3 | < 0.2 Gy |
|  | External | Quadratic Overdose (40 Gy) | 0.6 | < 0.2 Gy |
|  | External | Quadratic Overdose (35 Gy) | 0.9 | < 0.2 Gy |
|  | External | Quadratic Overdose (25 Gy) | 2.5 | < 0.3 Gy |
| Objectives |  |  |  |  |
| Priority | Structure | Cost Function (parameter values) | Margin (cm) | Goal value (sufficient) |
| 1 | PTV | Target EUD (0.5) |  | 50.0 Gy |
| 1 | PTV | Target Penalty (99%) |  | 50.0 Gy |
| 2 | Rectum | Parallel (40 Gy, k = 3) | 0.3 | < 30.0% |
| 2 | Bladder | Parallel (40 Gy, k = 3) | 0.3 | < 33.5% |
| 3 | Rectum | Serial (k = 15) |  | < 46.0 Gy |
| 3 | Bowel bag | Parallel (40 Gy, k = 3) | 0.3 | < 20.0% |
| 3 | Bowel bag | Serial (k = 15) |  | < 43.0 Gy |
| 3 | Bladder | Serial (k = 15) |  | < 47.0 Gy |
| 4 | Bowel bag | Overdose DVH (45 Gy) |  | < 14.0% (7.0%) |
| 4 | Right femoral head | Serial (k = 15) |  | < 38.0 Gy |
| 4 | Left femoral head | Serial (k = 15) |  | < 38.0 Gy |
| 5 | External | Conformality |  | < 0.75 |
| 6 | Right femoral head | Serial (k = 1) |  | < 30.0 Gy |
| 6 | Left femoral head | Serial (k = 1) |  | < 30.0 Gy |
| 7 | Rectum | Serial (k = 1) |  | < 30.0 Gy |
| 8 | Bladder | Serial (k = 1) |  | < 35.0 Gy |

Priority: order list according to which the objectives (cost functions) are optimized. Margin: creates a buffer zone between the PTV and overlapping structures to avoid conflict between the applied cost functions of each structure. Abbreviations: PTV: planning target volume, DVH: dose volume histogram, EUD: equivalent uniform dose.

Table 2 mCycle Wish-List 02 for auto-planning of cervical cancer at 50 Gy in 25 fractions.

| Wish-List 02 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Clinical constraints |  |  |  |  |  |
|  | Structure | Cost Function (parameter) | Margin (cm) | Limit |  |
|  | PTV | Quadratic Overdose (52.5 Gy) |  | < 0.02 Gy |  |
|  | Bowel bag | Overdose DVH (45 Gy) |  | < 195.0 cm3 |  |
|  | External | Maximum Dose |  | 53.5 Gy |  |
| Planning Constraints |  |  |  |  |
|  | Structure | Cost Function (parameter values) | Margin (cm) | Limit |  |
|  | External | Quadratic Overdose (50 Gy) | 0.0 | < 0.1 Gy |  |
|  | External | Quadratic Overdose (45 Gy) | 0.3 | < 0.2 Gy |  |
|  | External | Quadratic Overdose (40 Gy) | 0.6 | < 0.2 Gy |  |
|  | External | Quadratic Overdose (35 Gy) | 0.9 | < 0.2 Gy |  |
|  | External | Quadratic Overdose (25 Gy) | 2.5 | < 0.3 Gy |  |
| Objectives |  |  |  |  |
| Priority | Structure | Cost Function (parameter values) | Margin (cm) | Goal value (sufficient) |  |
| 1 | PTV | Target Penalty (95%) |  | 47.5 Gy |  |
| 1 | PTV | Target Penalty (50%) |  | 50.0 Gy |  |
| 2 | Rectum | Parallel (40 Gy, k = 3) |  | < 30.0% |  |
| 2 | Bladder | Parallel (40 Gy, k = 3) |  | < 40.0% |  |
| 3 | Rectum | Serial (k = 15) |  | < 46.0 Gy |  |
| 3 | Bowel bag | Parallel (40 Gy, k = 2) |  | < 20.0% |  |
| 3 | Bladder | Serial (k = 20) |  | < 47.5 Gy |  |
| 4 | Bowel bag | Overdose DVH (45 Gy) |  | < 14.0% (7.0%) |  |
| 4 | Right femoral head | Serial (k = 15) |  | < 38.0 Gy |  |
| 4 | Left femoral head | Serial (k = 15) |  | < 38.0 Gy |  |
| 5 | External | Conformality |  | < 0.75 |  |
| 6 | Rectum | Serial (k = 1) |  | < 30.0 Gy (34.5%) |  |
| 7 | Bladder | Serial (k = 1) |  | < 35.0 Gy |  |
| 8 | Left femoral head | Serial (k = 1) |  | < 30.0 Gy |  |
| 8 | Right femoral head | Serial (k = 1) |  | < 30.0 Gy |  |
| 9 | PTV | Underdose DVH (47.5 Gy) |  | > 99.0% |  |

Priority: order list according to which the objectives (cost functions) are optimized. Margin: creates a buffer zone between the PTV and overlapping structures to avoid conflict between the applied cost functions of each structure. Abbreviations: PTV: planning target volume, DVH: dose volume histogram, EUD: equivalent uniform dose.