Supplementary Table

Supplementary Table 1:  Nonlinear adsorption isotherms.

| Isotherms | Equation |
| --- | --- |
| Freundlich | Qe= Kf Ce1/n |
| Hill | Qe= QhCenhKd+ Cenh |
| Khan | Qe= QkBkCe(1+BkCe)Ak |
| Redlich–Peterson | Qe= KRCe1+ aRCebR |
| Toth | Qe= KtCe(at+Ce)1t |
| Koble-Corrigan | Qe= ACeD1+BCeD |

Note: Kf -Adsorption capacity Freundlich constant ((mg/mg) (mg/L)-1/n); n - Freundlich constant for intensity    (dimensionless); Kd –Constant for Hill model; Qh - Hill isotherm uptake value (mg/L); nh- Hill coefficient for binding interaction; Qk- Theoretical value for isotherm saturation capacity (mg/mg); Ak- Khan model exponent; Bk- Constant for Khan model; aR- Redlich model constant (1/mg); KR- Constant for Redlich–Peterson (L/mg); bR- Redlich–Peterson exponent; Kt- Toth model constant (mg/mg); t- Toth constant; at- Constant for Toth isotherm (L/mg); A- Koble–Corrigan model constant (Lnmg1−n/mg); B- Constant for Kobel (L/mg); D- Intensity value for adsorption.

Supplementary Table 2: Kinetic models Equations

| Isotherm | Equation |
| --- | --- |
| Pseudo-first order | logqe-qt=logqe-K12.303t |
| Pseudo-second order | tqt =1K2qe2+tqe |
| Intraparticle diffusion | qt=Kidt1/2+C |
| Experimental value |  |

Note:  qe: Final  NH3-N concentration  at  equilibrium  (mg/g); qt: Final  NH3-N concentration in t time duration (min); K1: Kinetic  constant for Pseudo-first  order  (min-1); K2: kinetic  constant for Pseudo-second  order  model (g  mg-1 min-1); Kid- Rate constant related to Intra-particle diffusion (mg g-1 min-1/2)  C- Boundary layer thickness