## Details of MRI acquisition

MR images were acquired using a 1.5-T system unit (Intera Achieva; Philips Healthcare, Best, The Netherlands) with a 16-channel head-neck combined coil. The MRI protocol included an axial turbo spin echo (TSE) T1-weighted sequence (TR/TE 545/14 ms; field of view [FOV], 23 cm; slice thickness, 4 mm; gap, 4 mm; and matrix size 328 × 220), an axial TSE T2-weighted sequence (TR/TE 3193/80 ms; FOV, 23 cm; slice thickness 5 mm; gap, 5 mm; and matrix size, 228 × 185), a coronal short time inversion recovery (STIR) sequence (TR/TE 3224/165 ms; FOV, 26 cm; slice thickness 5 mm; gap, 5 mm; and matrix size, 312 × 163), and an axial and a sagittal contrast-enhanced TSE T1-weighted sequence with fat saturation. Parameters of contrast-enhanced T1-weighted imaging (T1WI) were the same as those of unenhanced T1WI except for the use of the fat-suppression technique. Contrast-enhanced T1WI was performed after intravenous administration of 0.1 mmol/kg gadopentetate dimeglumine (Magnevist, Bayer Schering).

Radiomics feature extraction

279 radiomics features derived from six statistical image descriptors (Histogram, Co-occurrence matrix, Run-length matrix, Absolute gradient, Autoregressive model and Wavelet) were extracted. The texture features details are described in Table S1.

Table S1: Radiomics features calculated by using MaZda4.6.

| Statistical descriptors | Features |
| --- | --- |
| Histogram | A total 9 of features :Mean, skewness, kurtosis, variance, percentiles 1%, 10%, 50%, 90%, and 99% |
| Co-occurrence matrix | A total of 220 features :Angular second moment, contrast, correlation, sum of squares, inverse difference moment, sum average, sum variance, sum entropy, entropy, difference variance, and difference entropy. Features are computed for 5 between-pixel distances (1, 2, 3, 4, 5) and for 4 different directions (vertical, horizontal, 0, 135). |
| Run-length matrix | A total of 20 features: Run-length nonuniformity, gray-level nonuniformity, long-run emphasis, short run emphasis, and fraction of image in runs. Features are computed for 4 different directions (vertical, horizontal, 0, 135). |
| Absolute gradient | A total of 5 features : Mean, variance, skewness, kurtosis, and percentage of pixels with nonzero gradient |
| Autoregressive model | A total of 5 features : Theta 1 (θ1) Theta 2 (θ2), Theta 3 (θ3), Theta 4 (θ4), and Sigma (σ) |
| Wavelet | A total of 20 features: WavEn (wavelet energy). Features are computed at 5 scales within 4 frequency bands LL, LH, HL, and HH. |