Table S1. Physicochemical properties of minocycline.

| Chemical structure |  |
| --- | --- |
| Molecular formula | C23H27N3O7 |
| Molecular weight | 457.48 (Not including H+) |
| Water solubility | Freely soluble in water |

Table S2. Gradient table of mobile phase concentration in I-Class-VION-IMS-Qtof (v:v)

| Time (min) | Flow Rate (mL/min) | Composition A (%) | Composition B (%) | Curve |
| --- | --- | --- | --- | --- |
| 0.00 | 0.400 | 90.0 | 10.0 | Initial |
| 1.00 | 0.400 | 80.0 | 20.0 | 6 |
| 2.00 | 0.400 | 15.0 | 85.0 | 6 |
| 3.00 | 0.400 | 0.0 | 100.0 | 6 |
| 4.00 | 0.400 | 0.0 | 100.0 | 6 |
| 5.00 | 0.400 | 95.0 | 5.0 | 6 |
| 6.00 | 0.400 | 95.0 | 5.0 | 6 |

Table S3. MS mode operating parameters are in I-Class-VION-IMS-Qtof.

| Parameters |  | Parameters |  |
| --- | --- | --- | --- |
| Source type | ESI | Polarity | Positive |
| Low mass | 50 m/z | High mass | 2000 m/z |
| Capillary voltage | 2.00kV | Scan time | 1.000 s |
| Cone gas | 50 L/h | Desolvation gas | 800 L/h |
| Source temperature | 120 ℃ | Desolvation temperature | 500℃ |

Table S4. Characteristics of water samples used in this study.

|  | Water samples |  |
| --- | --- | --- |
|  | Xi'an Moat | Tap Water |
| pH | 8.03 | 7.86 |
| UV254(cm-) | 0.021 | 0.013 |
| Fe (ppm) | Not detection | ＜0.03 |
| Cu (ppm) | 66.12 | ＜0.03 |
| CO3-(mM) | Not detection | Not detection |
| Ca(ppm) | 29.81 | 26.38 |
| Mg(ppm) | 3.94 | 3.49 |
| K(ppm) | 3.57 | 1.48 |
| Na(ppm) | 6.86 | 2.72 |

Table S5. The chemical elements analysis of the fresh NBo powder by XRF.

| Elements | Fresh NBo | Elements | Fresh NBo |
| --- | --- | --- | --- |
| Cu(wt%) | 38.53 | Si(wt%) | 2.75 |
| Fe(wt%) | 34.27 | Mg(wt%) | 0.64 |
| S(wt%) | 20.46 | Mn(wt%) | 0.33 |
| Zn(wt%) | 0.24 | Al(wt%) | 0.11 |
| Ca(wt%) | 1.46 | Pb(wt%) | 0.81 |

Total other elements ＜0.4%

Table S6. Final pH and MNC removal through NBo/PS or NBo/H2O2 system under various treatment conditions ([MNC]0 = 10 mg/L, NBo=0.15g/L t = 180 min).

| Oxidant dose （uM） | AOPs system |  |  |  |
| --- | --- | --- | --- | --- |
|  | NBo/PS |  | NBo/H2O2 |  |
|  | pHfinal | Removal（%） | pHfinal | Removal（%） |
| 0 | 4.52 | 1.44 | 4.52 | 1.44 |
| 25 | 4.31 | 38.33 | 5.25 | 41.65 |
| 50 | 4.07 | 64.78 | 4.32 | 58.40 |
| 250 | 3.62 | 79.55 | 4.27 | 81.66 |
| 500 | 3.53 | 86.44 | 4.13 | 87.47 |
| 1000 | 3.65 | 85.35 | 4.42 | 91.10 |

Table S7. Final pH and MNC removal through NBo/PS or NBo/H2O2 system under various treatment conditions ([MNC]0 = 10 mg/L, oxidant=500uM, t = 180 min).

| NBo dose （g/L） | AOPs system |  |  |  |
| --- | --- | --- | --- | --- |
|  | NBo/PS |  | NBo/H2O2 |  |
|  | pHfinal | Removal（%） | pHfinal | Removal（%） |
| 0 | 5.16 | 14.67 | 5.30 | 11.07 |
| 0.05 | 3.89 | 64.7 | 4.68 | 43.46 |
| 0.1 | 3.63 | 83.85 | 5.19 | 79.91 |
| 0.15 | 3.53 | 86.44 | 4.83 | 87.47 |
| 0.2 | 3.62 | 78.72 | 5.22 | 78.49 |

Table S8. Final pH and MNC removal through NBo/PS or NBo/H2O2 reaction under different pH ([MNC]0 = 10 mg/L, [oxidant]0 =500uM, NBo=0.15g/L，t =180 min).

| pHinitial | AOPs system |  |  |  |
| --- | --- | --- | --- | --- |
|  | NBo/PS |  | NBo/H2O2 |  |
|  | pHfinal | Removal（%） | pHfinal | Removal（%） |
| 2.5 | 2.43 | 80.1 | 2.46 | 83.23 |
| 3.5 | 3.16 | 83.6 | 3.02 | 87.30 |
| 4.5 | 3.53 | 86.44 | 4.13 | 87.47 |
| 5.5 | 3.63 | 81.75 | 4.22 | 78.98 |
| 6.5 | 3.58 | 67.37 | 4.32 | 54.52 |
| 7.5 | 3.74 | 80.03 | 4.28 | 60.77 |
| 8.5 | 3.81 | 72.87 | 4.26 | 60.76 |
| 9.5 | 3.95 | 59.65 | 3.48 | 48.91 |

Table S9. Final pH of NBo/PS or NBo/H2O2 reaction solutions with the presence of differnent anions ([MNC]0 = 10 mg/L, [oxidant]0 =500uM, NBo=0.15g/L，t =180 min).

| Anions | AOPs system |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NBo/PS |  |  | NBo/H2O2 |  |  |
|  | Concentration（mM） |  |  | Concentration（mM） |  |  |
|  | 10 | 20 | 50 | 10 | 20 | 50 |
| Cl- | 3.52 | 3.50 | 3.50 | 3.96 | 3.98 | 3.86 |
| HCO3- | 7.6 | 7.65 | 7.85 | 7.68 | 7.78 | 8.02 |
| NO3- | 3.51 | 3.48 | 3.48 | 3.93 | 3.93 | 3.92 |

Fig. S1 Effect of pH on MNC degradation kinetics by (a) NBo/PS and (b) NBo/H2O2 processes. Experimental conditions: [MNC]0=10mg/L, [H2O2]0=[PS]0=500uM, [NBo]0=0.15g/L. pHinitial was adjusted by 0.1M NaOH or H2SO4. Error represent the standard deviation (n=3).

Fig. S2 Luminescence inhibition rate of MNC solution in NBo/PS and NBo/H2O2 processes. Experimental conditions: [MNC]0=10mg/L, [H2O2]0=[PS]0=500uM, [NBo]0=0.15g/L. Error represent the standard deviation (n=3).

Fig.S3 Changes of hydroxyl radical concentration in NBo/PS and NBo/H2O2 processes (a) and persulfate concentration in NBo/PS process. Experimental conditions: [MNC]0=10mg/L, [H2O2]0=[PS]0=500uM, [NBo]0=0.15g/L. Error represent the standard deviation (n=3).

Fig.S4 XRD spectra of the fresh and used NBo.

Fig.S5 The total ion chromatograms of MNC and its degradation products identified by I-Class-VION-IMS-Qtof in positive ionization mode via (a) NBo/PS and (b) NBo/H2O2 processes.

Fig.S6 (a), (b), (c), (d), (e), and (f) represene the MS/MS spectra of the degradation products of MNC in NBo/H2O2 process.

Fig.S7 (a), (b), (c), (d), (e), and (f) represene the MS/MS spectra of the degradation products of MNC in NBo/PS process.